Observations analysis, statistical analysis and interpolation for the Gippsland Basin bioregion

Product 2.1 - 2.2 from the Gippsland Basin Bioregional Assessment

2018

The Bioregional Assessment Programme

The Bioregional Assessment Programme is a transparent and accessible programme of baseline assessments that increase the available science for decision making associated with coal seam gas and large coal mines. A bioregional assessment is a scientific analysis of the ecology, hydrology, geology and hydrogeology of a bioregion with explicit assessment of the potential direct, indirect and cumulative impacts of coal seam gas and large coal mining development on water resources. This Programme draws on the best available scientific information and knowledge from many sources, including government, industry and regional communities, to produce bioregional assessments that are independent, scientifically robust, and relevant and meaningful at a regional scale.

The Programme is funded by the Australian Government Department of the Environment. For more information, visit http://www.bioregionalassessments.gov.au.

Department of the Environment

The Office of Water Science, within the Australian Government Department of the Environment, is strengthening the regulation of coal seam gas and large coal mining development by ensuring that future decisions are informed by substantially improved science and independent expert advice about the potential water related impacts of those developments. For more information, visit http://www.environment.gov.au/coal-seam-gas-mining/.

Bureau of Meteorology

The Bureau of Meteorology is Australia’s national weather, climate and water agency. Under the Water Act 2007, the Bureau is responsible for compiling and disseminating Australia's water information. The Bureau is committed to increasing access to water information to support informed decision making about the management of water resources. For more information, visit http://www.bom.gov.au/water/.

Department of Environment, Land, Water and Planning

The Department of Environment, Land, Water and Planning creates liveable, inclusive and sustainable communities that support jobs and growth in Victoria. We recognise the link between the built and natural environment in the quality of our lives, and work to accommodate population growth while maintaining world class liveability and protecting our heritage for future generations. For more information, visit http://www.delwp.vic.gov.au.

Geological Survey of Victoria

The Geological Survey of Victoria (GSV) is Victoria’s geoscience agency and leading authority on Victorian geology. The GSV is the custodian of over 160 years of geological knowledge. The GSV provides objective and authoritative geoscientific data and advice, creates and delivers innovative geoscience information, on a wide range of earth resources which underpin sound Victorian government policy, programs and projects. For more information, visit http://www.energyandresources.vic.gov.au/earth-resources/geology-of-victoria/geological-survey-of-victoria.

Department of Economic Development, Jobs, Transport and Resources

Agriculture Victoria works with the agriculture and fisheries industries on research, development and extension to improve production, connect the sector with international markets, support development and maintain effective biosecurity controls.  For more information, visit http://agriculture.vic.gov.au

ISBN: 978-1-925315-88-2

Citation

DELWP (2017) Observations analysis, statistical analysis and interpolation report for the Gippsland bioregion.

Copyright

© Commonwealth of Australia 2015
With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence http://www.creativecommons.org/licenses/by/3.0/au/deed.en. 
The Bioregional Assessment Programme requests attribution as ‘© Commonwealth of Australia (Bioregional Assessment Programme http://www.bioregionalassessments.gov.au)’.

Disclaimer

The information contained in this report is based on the best available information at the time of publication. The reader is advised that such information may be incomplete or unable to be used in any specific situation. Therefore decisions should not be made based solely on this information or without seeking prior expert professional, scientific and technical advice

Cover photograph

Lake Victoria, Victoria, 2013

Credit: Hashim Carey

Contents

2.1	Observations analysis and 2.2 statistical analysis and interpolation	10

2.1.1	Geology	10

2.1.1.1	Hydrogeology and groundwater quality	11

2.1.2	Surface water hydrology and water quality	10

2.1.3	Surface water - groundwater interactions	12

2.1.4	Geography	12

2.1.4.1	Methods	13

2.1.4.2	Observed data	13

2.1.4.2.1	Digital Elevation Model data	13

2.1.4.2.2	Surface water courses and catchment boundaries	16

2.1.4.2.3	Spatial soil data layer and attribution	16

2.1.4.2.4	Land use data	19

2.1.4.2.5	Climate data	20

2.1.4.2.6	Flood data	27

2.1.4.3	Statistical analysis and interpolation	27

2.1.4.3.1	DEM	27

2.1.4.3.2	Surface water courses and catchment boundaries	27

2.1.4.3.3	Soil map	28

2.1.4.3.4	Land use	28

2.1.4.3.5	Climate data	28

2.1.4.3.6	Flood mapping	31

2.1.4.4	Gaps	32

2.1.5	Geology	33

2.1.5.1	Methods	33

2.1.5.2	Observed data	33

2.1.5.2.1	Deep wells dataset	34

2.1.5.2.2	Gravity interpretation	38

2.1.5.2.3	Seismic stratigraphy	39

2.1.5.2.4	Victorian Aquifer Framework	39

2.1.5.2.5	Latrobe Valley Coal Model	40

2.1.5.2.6	Digital Elevation Model	40

2.1.5.3	Statistical analysis and interpolation	40

2.1.5.3.1	Method	40

2.1.5.3.2	Data selection and processing	40

2.1.6	Hydrogeology and groundwater quality	45

2.1.6.1	Methods	45

2.1.6.1.1	Aquifer assignment	45

2.1.6.1.2	Water levels	45

2.1.6.1.3	Hydraulic parameters	46

2.1.6.1.4	Groundwater quality	46

2.1.6.2	Observed data	46

2.1.6.3	Statistical analysis and interpolation	49

2.1.6.3.1	Groundwater level	49

2.1.6.3.2	Hydraulic parameters	50

2.1.6.3.3	Groundwater quality	51

2.1.6.3.4	Licensed extraction volume	51

2.1.6.4	Gaps	53

2.1.7	Surface water hydrology and water quality	55

2.1.7.1	Methods	55

2.1.7.1.1	Latrobe River	55

2.1.7.1.2	Mitchell River	56

2.1.7.1.3	Thomson/Macalister rivers	56

2.1.7.1.4	Avon River	57

2.1.7.1.5	Snowy River	57

2.1.7.1.6	Flooding	58

2.1.7.2	Observed data	61

2.1.7.3	Statistical analysis and interpolation	71

2.1.7.4	Gaps	80

2.1.8	Surface water – groundwater interactions	81

2.1.8.1	Methods	81

2.1.8.2	Observed stream and bore water level data	82

2.1.8.3	Statistical analysis and interpolation	84

2.1.8.4	Gaps	92

References	93

Datasets	96

Figures

Figure 1	Elevation of ground surface (mAHD) based on the 1 second digital elevation model (Gallant et al., 2011)	14

Figure 2 	Aspect derived from the 1 second digital elevation model. Aspect is reported as the angle in degrees from north	15

Figure 3	Slope derived from the 1 second digital elevation model	16

Figure 4	Spatial pattern of the key soil classifications in the Gippsland region	18

Figure 5	Land use classified into broad grouping	20

Figure 6	Zones of influence attributed to climate stations within the study area	21

Figure 7	Interpolated mean annual rainfall (mm/yr) for the period 1957–2012	22

Figure 8  	Interpolated average daily minimum temperature (ºC) for the period 1957–2012	23

Figure 9	Interpolated average daily maximum temperature (ºC) for the period 1957–2012	24

Figure 10	Interpolated average daily solar radiation (MJ/m2) for the period 1957–2012	25

Figure 11	Potential mean annual evaporation (mm/yr)	26

Figure 12 	Comparative histograms derived from the SILO climate station data and the interpolated ANUClim for mean annual (left) and corresponding probability of exceedance curve of the percent variation (right) (Dataset 12)	29

Figure 13	Comparative histograms derived from the SILO climate station data and the interpolated ANUClim for average daily minimum temperature (left) and corresponding probability of exceedance curve of the percent variation (right) (Dataset 12)	30

Figure 14	Comparative histograms derived from the SILO climate station data and the interpolated ANUClim for average daily maximum temperature (left) and corresponding probability of exceedance curve of the %  variation (right) (Dataset 12)	30

Figure 15	Comparative histograms derived from the SILO climate station data and the interpolated ANUClim for average potential evaporation (left) and corresponding probability of exceedance curve of the percent variation (right) (Dataset 12)	30

Figure 16	Comparative histograms derived from the SILO climate station data and the interpolated ANUClim for average daily solar radiation (left) and corresponding probability of exceedance curve of the percent variation (right) (Dataset 12)	31

Figure 17  	Stratigraphic units of the Gippsland study area (source; Schaeffer 2008), Dataset 1	34

Figure 18  	Location of petroleum wells used (Dataset 3)	38

Figure 19 	Isostatically corrected Bouguer Gravity image for the Gippsland Basin (Fitzgerald et al., 2009). (Dataset 4)	39

Figure 20 	Location of observation bores in the Gippsland region by GMU	48

Figure 21 	Summary of monitoring records for bores in the Gippsland region	50

Figure 22	Annual extraction volume (ML) attributed to licensed, stock and domestic, mine and offshore bores for 1970–2012	52

Figure 23	Annual extraction volume (ML) attributed to aquifer system for 1970–2012	53

Figure 24	Major rivers of the Gippsland Basin	58

Figure 25 	Location and extent of 1-in-100 year flooding events in the Gippsland area (sourced from the Victorian Flood Database)	59

Figure 26	Location and extent of major recent flooding events (sourced from the Victorian Flood Database	60

Figure 27 	Location of selected stream gauges and surface-groundwater connectivity in the Gippsland region (surface water-groundwater interaction data sourced from DSE, 2012)	83

Figure 28 	Summary of monitoring records for the 50 selected gauges in the Gippsland	87

Figure 29 	Data quality of monitoring records for the 50 selected gauges for surface water-groundwater interaction analysis in the Gippsland	87

Figure 30 	Summary of monitoring records for the 34 selected observation bores for surface water-groundwater interaction analysis in the Gippsland Basin	89

Figure 31 	Groundwater hydrographs of a transect of bores located near Mitchell River west of Bairnsdale plotted against daily stream water level of Mitchell River at Rose (gauge station no. 224203)	90

Tables

Table 1	Percentiles based on the percentage deviation of interpolated mean annual estimated derived using ANUClim to observed climate station data (Dataset 12)	31

Table 2  	Selected petroleum wells used for stratigraphic interpretation	34

Table 3  	Petroleum well database ‘well picks’	41

Table 4 	Data tables from the various sources used for hydrogeology and water quality analysis in the Gippsland region	48

Table 5 	Number of bores by aquifer system showing variability of aquifer depth in the Gippsland region	49

Table 6 	Summary of water level records for bores in the Gippsland region	50

Table 7 	Summary of hydraulic parameter’s typical range reported for the main aquifers in the Gippsland region	51

Table 8 	Summary of groundwater salinity for the main aquifers in the Gippsland region	51

Table 9 	A summary of groundwater extraction data for the bores in the Gippsland region	52

Table 10	Extents of historical flood events (sourced from the Victorian Flood Database)	60

Table 11	East Gippsland Basin streamflow site details	62

Table 12	La Trobe Basin streamflow site details	63

Table 13	Mitchell Basin streamflow site details	65

Table 14	Snowy River Basin streamflow site details	66

Table 15	South Gippsland Basin streamflow site details	67

Table 16	Tambo Basin streamflow site details	68

Table 17	Thompson Basin streamflow site details	69

Table 18	Proposed national streamflow quality code scheme	71

Table 19	National water quality code assigned to Victorian water monitoring codes	72

Table 20	Proportion of basin stream flow data assigned to each national streamflow quality code	74

Table 21	Proportion of stream flow data assigned to each national streamflow quality code	74

Table 22 	Number of stream gauges selected for analysis of surface-groundwater interaction in each river basin in the Gippsland region	83

Table 23 	Number of bores selected for analysis of surface-groundwater interaction in each river basin in the Gippsland region	84

Table 24	Proportion of the streamflow data assigned to each national streamflow quality code	85

Table 25 	Summary of selected bores for surface-groundwater interaction analysis	88

Table 26 	Estimated Baseflow Index (BFI) in 51 unregulated catchments in the Gippsland region (sourced from SKM 2012)	90

## Observations analysis and 2.2 statistical analysis and interpolation

### Geology

Summary

A review of available surface and subsurface datasets in the onshore and offshore Gippsland Basin was conducted to build a regional geological model.

The Gippsland Basin geological model represents an interpretation of the underground geometries and lithologies. The interpretation is based on bores, wells, outcrop and geophysical data and accounts for the uncertainties and resolution inherent in these data. The well data helps to define the limits between the different geological units as well as the lithology. Geophysical data (mostly seismic reflection data) were used to extrapolate the data calibrated at the wells. Geophysical data (mostly potential fields) was also used to constrain basin architecture away from wells and seismic surveys. The uncertainty in these data increases as the distance between wells geophysical surveys increases.

As the density of both wells and geophysical data varies throughout the Gippsland Basin, isopach maps (i.e. thickness maps) were used to define the basin-scale architecture of the sequences.

The Gippsland Basin geological model can be updated and refined with additional or new datasets and can be populated with hydraulic properties derived from stratigraphic facies.

### Surface water hydrology and water quality

Summary

The Gippsland Basin covers seven catchments. The Gippsland Basin bioregion contains 197 streamflow gauges. The Victorian water monitoring scheme classifies streamflow data into 57 quality codes. In this analysis, these quality codes were grouped into six broad water quality codes based on the national water quality coding scheme, to enable consistency and comparison between states and territories. Based on an analysis of all daily streamflow gauge data: 72% of all records were good; 8% fair; 6% poor; 4% unknown; and 10% missing. The quality of individual streamflow data varied significantly whilst results suggest that streamflow monitoring in the South Gippsland Basin was of a higher quality than the monitoring in all other basins. Water chemistry and quality was not considered in this analysis.

#### Hydrogeology and groundwater quality

The hydrogeology and groundwater quality discussion forms the basis of the conceptualisation of the numerical groundwater model that was built for the region. Hydrogeological data was sourced from various Victorian Government agencies and was combined and integrated to form a dataset fit for modelling purposes.

Groundwater level and bore depths information was sourced for various aquifers in the Gippsland region including the alluvial aquifers, the fractured rock aquifers and the deep water bearing units. This information is important for understanding groundwater processes and groundwater model calibration. Hydraulic parameters provide critical input into the numerical groundwater model – the identification of uncertainties in the hydraulic parameter fields is essential as they can contribute significantly to the overall uncertainty associated with the modelling of impacts.

The gap analysis provides an overall assessment of data availability required to construct three-dimensional representation of aquifers of various depths.

### Surface water - groundwater interactions

Summary

Fifty stream gauges in unregulated catchments and 34 groundwater observation bores along main stems of some major rivers were selected for analysis of surface water – groundwater interaction in the Gippsland region. The 50 stream gauges cover the majority of main stems of major rivers in the region. The majority of these gauges have long-term record with a large number of high-quality daily streamflow. Forty-five of the 50 gauges record greater than 80% daily streamflow data classified as ‘Good’ or ‘Fair’ as defined by the national streamflow quality code. The 34 selected observation bores also have long-term record with high-quality groundwater level readings. Thirty have more than 100 readings with typical monitoring frequency ranging from monthly to quarterly. However, these selected bores only represent a small proportion of main stems of the major rivers in the region. There is no adequate water chemistry data from the gauges and bores for surface water – groundwater interaction analysis.

### Geography

Summary

This section covers data characteristics, including accuracy, for all datasets used in the Geography section of companion product 1.1 for the Gippsland Basin bioregion.

For physical geography, brief assessments are provided from the relevant literature for the: (i) digital elevation model (DEM) data, (ii) surface watercourses and basin/catchment boundary data, (iii) soil classes and (iv) land use.For human geography, concise descriptions are provided of the land use management.

For historical climate analysis, brief descriptions are provided for: (i) precipitation (P), (ii) maximum and minimum air temperature (Tmax and Tmin, respectively), (iii) vapour pressure (VP), (iv) net radiation (Rn), and (v) potential evapotranspiration(PET) and (vi) flooding.

Estimates of the errors attributed to the spatial assignment of key climate inputs are presented. The relative error was presented as the percentage deviation of the interpolated mean annual climate data to measured daily climate observations. For rainfall, the median relative error was 6% whereas for Tmin the median relative error was approximately 9% and for Tmax the median relative error was approximately 3%.  The error associated with interpolated rainfall reflects the significant variation in topography which ranges in elevation from sea level to 1,600mAHD. Similarly, the minimum air temperature is also influenced by topography, hence the high median relative error, whereas the maximum air temperature is modelled more accurately. For potential evaporation, the median relative error was 2.5%, for solar radiation it was 2.1%.

#### Methods

For physical geography, the main datasets used are: (i) digital elevation model (DEM) data, (ii) surface watercourses and basin or catchment boundary data, (iii) soil classes and (iv) land use. Each is briefly outlined in the following section.

#### Observed data

This section describes the nature, source, construct and the spatial and/or temporal resolution of all relevant geographic datasets that have been used in the Gippsland Basin bioregional assessment, and in particular the development of the groundwater model.

##### Digital Elevation Model data

The Digital Elevation Model (DEM) was obtained from the 1 second (~30 m resolution grid cell) Shuttle Radar Topography Mission (SRTM). This national elevation data set was developed by the National Elevation Data Framework (NEDF) partners comprising Geoscience Australia (GA), the Bureau of Meteorology (BoM), the Commonwealth Scientific and Industrial Research Organisation (CSIRO) and the Australian National University (ANU) (Gallant et al., 2011). The SRTM global elevation data was collected by the Space Shuttle Endeavour during 11 days in February 2000. The data was acquired by interferometric synthetic aperture radar based on interference patterns between radar signals collected by two antennas. The shuttle was oriented to point the antennas at 45° to the ground to optimise the effect of topography on the interference patterns. Obscured regions such as zones of large topographic variation were evaluated by collecting overlapping swathes from different orbits. The elevation of ground surface in metres above Australian Height Datum (mAHD) based on the 1 second DEM is shown in Figure 1. Derived map surfaces of aspect and slope based on the DEM are shown in Figure 2 and Figure 3 respectively.

Figure 1	Elevation of ground surface (mAHD) based on the 1 second digital elevation model (Gallant et al., 2011)

Figure 2 	Aspect derived from the 1 second digital elevation model. Aspect is reported as the angle in degrees from north

Figure 3	Slope derived from the 1 second digital elevation model

##### Surface water courses and catchment boundaries

The spatial map of surface water courses was sourced from the Victorian Department of Sustainability and Environment Spatial Information Infrastructure group, Vicmap Hydro data layer (www.land.vic.gov.au/vicmapdata). Vicmap Hydro is a topologically structured digital data set depicting Victoria’s natural and man-made water resources including selected water-related structures and coastal navigational features. This data layer contains line features delineating hydrological features including watercourses (ie channels, rivers and streams) and connectors. The drainage pattern covering Victoria is made up of point, line and polygon area features in a seamless and topologically structured data set. The line work and points were derived from the Vicmap Digital Topographic (VDT) map base which evolved from Victoria's printed 1:25,000 Topographic Map Series program. As such, data within the Vicmap Hydro spatial map has primarily been captured to 1:25 000 scale topographic specifications.

##### Spatial soil data layer and attribution

The spatial soil data layer was derived by merging broad scale Land Classification Survey data and 1:25,000 soil attribute coverage (Smith, 2002). Numerous Land Classification/Landform studies have been undertaken within the Gippsland Basin region (Sibley, 1966; Nicholson, 1974; Sargeant, 1975; Rowe, 1977; Nicholson, 1978; Schoknecht, 1982; Aldrick et al., 1988; Aldrick et al., 1992; Rees, 1996; Sargeant and Imhof, 2000) and have been mapped at varying scales ranging from 1:50 000 to 1:250 000 scale. Public land was typically mapped at 1:250 000 scale, freehold land was mapped at 1:100 000 scale whereas irrigated regions were mapped at 1:50 000 scale.

The spatial landform data attribution was based on information related to parent material, depositional processes, topography and climate. Soil pit studies were subsequently used to develop relationships between landform attributes, soil texture and soil profile information. Based on these relationships and the spatial landform data attribution a spatial soil map was generated for the study area. The merged spatial soil layer was attributed using the Factual Key of Northcote (1979) at the Principal Profile Form (PPF) level to classify different soil types. Within the study area, 169 different soil types were identified and spatially assigned. The spatially-assigned key primary level soil classifications are presented in Figure 4. The primary levels refer to Dr (texture contrast soils, red clay B horizons), Dy (texture contrast soils, yellow-grey clay B horizons), Gn (sodosol, gradational not calcareous throughout), NS (no soil representing water bodies), Oo (organsols), Uc (uniform coarse textured), Uf (uniform fine textured, not cracking), and Um (uniform medium textured).

The soil attribution through depth for each soil type was based on published data sources (McKenzie et al., 2000), field observations and pseudo-transfer functions (vanGenuchten et al., 1991) and includes soil water characteristics, bulk densities, hydraulic properties and impedance properties specified for each soil layer modelled. Additionally, each soil required specification of erodability, erosion and soil evaporation attributes.

Figure 4	Spatial pattern of the key soil classifications in the Gippsland region

##### Land use data

The land use spatial data layer for this study was sourced from the Catchment Scale Land Use Mapping for Australia update March 2010 (CLUM Update 03/10) dataset compiled by the Australian Bureau of Agricultural and Resource Economics (ABARE) and the Bureau of Rural Sciences (BRS). Land use data was classified based on the Australia Land Use and Management (ALUM) classification Version 6 (BRS, 2006) (http://adl.brs.gov.au).

The land use spatial data layer is a 50 m raster dataset and was collated as part of the Australian Collaborative Land Use and Management Program (ACLUMP). The March 2010 version used in this study represents the land use as mapped in 1997. Where land use data was not assigned, attribution was based on the land use specified in the corresponding Australian Bureau of Statistics mesh block with modifications based on: (1) the 1:250 000 scale topographic data for built up areas published by Geoscience Australia (GA) in 2006; (2) land tenure data compiled by the Bureau of Rural Sciences (BRS) in 2007; and (3) native and plantation forest data compiled by BRS in 2007.

The ALUM taxa describe land cover against which management strategies need to be specified. A total of 49 land use classifications were adopted in this study (Figure 5). In the case of cropping enterprises, crop rotations and cropping history was incorporated into the management scripts used in the catchment modelling.

Figure 5	Land use classified into broad grouping

##### Climate data

For this study, daily meteorological data was obtained from the Bureau of Meteorology/Queensland Department of Science, Information Technology, Innovation and the Arts (https://www.longpaddock.qld.gov.au/silo/) for each of the 199 climate stations located within the study area from 1957 to 2013.

For a given climate station, daily meteorological data was a combination of site-specific measurements and rectified data to infill any gaps in the record using interpolation methods discussed in Jeffery et al., (2001). To account for sparsely located climate stations within the study area, daily rainfall, temperature, evaporation and solar radiation data were scaled to each solution point within the study area. This was based on interpolated mean annual spatial layers created using the ANUClim software (Hutchinson, 2001). This approach combines a DEM and temporal climatic data to generate a smoothed surface for each key climate attribute.

Using this approach daily meteorological data assigned to each solution point within the study area is a function of the data from the 199 climate stations, landscape position and topography. Figure 6 shows the area influenced by each climate station. Figure 7, Figure 8, Figure 9, Figure 10 and Figure 11 show respectively, the smoothed surfaces for the mean annual rainfall, average daily minimum temperature, average daily maximum temperature, average daily radiation and mean average potential evaporation from 1957 to 2012.

Figure 6	Zones of influence attributed to climate stations within the study area

Figure 7	Interpolated mean annual rainfall (mm/yr) for the period 1957–2012

Figure 8  	Interpolated average daily minimum temperature (ºC) for the period 1957–2012

Figure 9	Interpolated average daily maximum temperature (ºC) for the period 1957–2012

Figure 10	Interpolated average daily solar radiation (MJ/m2) for the period 1957–2012

Figure 11	Potential mean annual evaporation (mm/yr)

##### Flood data

Flood data was sourced from the Victorian Flood Database and included information regarding location and extent of a 1-in-100 year event and historical flood events. The database contains flood extent, contour and spot height features derived from various flood studies, reports, plans and imagery. It includes both modelled and observed data and comprises 3 main data groups, containing a total of 26 separate layers, with data groups as follows:

Modelled Flood Extents: Extent_{n}y_ARI, where n = 5, 10, 20, 30, 50, 100, 200, 500, 1000 year intervals

Modelled Flood Contours: Contour_{n}y_ARI, where n = 5, 10, 20, 30, 50, 100 and 200 year intervals

Other flood data: Floodway, historic_extent, historic_height, historic_contour, flow direction, levee, levee spot height and running distance.

The mapped scale of input data ranges from 1:1 000 to 1:10 000. Figure 26 shows the location and extent of recent major floods and Table 10 presents the magnitude of each flooding event in the past 100 years (including area directly affected and duration). The extent of the 1-in-100 year flooding event only covers 4.8% of the total area of the Gippsland region. In the last 100 years, flooding events mainly occurred in small areas around the lower reaches of the Mitchell, Thomson, Latrobe and Snowy rivers. The area directly affected by flooding was generally less than 1% of the Gippsland region. The largest flood event occurred in June 2012 and covered 1.4% of the Gippsland region. Given the areas affected by historic flooding events were relatively small, it is believed that the impact of flooding on groundwater recharge is insignificant at the regional scale, but likely to be significant at the local scale.

#### Statistical analysis and interpolation

The observed datasets were interpolated to form derived datasets. A brief review of the accuracy of each data set is presented below.

##### DEM

As reported in Gallant et al., (2011), a comparison of SRTM with reference data (Rodriguez et al., 2006) showed that 90 of tested heights were within 6 m of the reference heights. In much of the clear flat areas of Australia the height errors are less than 3 m, although there are some areas where the errors are much larger.

##### Surface water courses and catchment boundaries

The accuracy of the Vicmap Hydro spatial data layer in terms of the degree to which features and attribute values of spatial objects agree with the information on the source material ranges between a 1% to 5% error (DSE, 2012).

The planimetric accuracy attainable is defined as the sum of errors from three sources: the positional accuracy of the source material, errors due to the conversion process and errors due to the manipulation process. For topographic base derived data, it is estimated this represents an error of 8.3 m on the ground for 1:25 000 data. A conservative estimate of 10 m for the standard deviation is therefore assigned to represent the data quality of the Vicmap Hydro spatial data layer. An alternate and equal way of expressing this error is that not more than 10% of well-defined points will be in error by more than 16 m. The worst-case error for the data is +/- 30 m. The vertical positional accuracy of points based on contour elevation is less than or equal to half the value of the contour interval.

##### Soil map

The derived spatial soil map is considered to be highly reliable. It is not appropriate to derive statistical analysis based on comparing the soil map attributes at those locations where soil pit data exists due to the following reasons:

- Sparse site specific soil pit data may not be representative of a soil unit and should therefore not be the sole data set used to define the spatial boundaries of a soil type. Soil maps based on soil pit data are typically derived for irrigated regions or small-scale investigations where soil cores are sampled at a high spatial intensity.

- Soil structure is a function of environmental conditions and depositional processes which are not accounted for were soil pit data to be extrapolated across a landscape with no consideration of landscape processes.

- The majority of the study area was attributed by an individual soil scientist such that the mapped units are considered consistent.

- The scale of the spatial soil map is consistent with the scale of the catchment modelling.

##### Land use

Catchment scale mapping is produced by combining land tenure and other types of land use information, fine-scale satellite data and information collected in the field. These data sets were combined and gridded at 50 m. It is acknowledged that the scale of the source data may vary significantly. The Victorian Government has recently invested in the Victorian Land Use Information System (VLUIS) to create regular and consistent land use data over time. This dataset describes land tenure, land use and land cover for each cadastral land parcel. The land use classification used is the Australian Valuation Property Classification Code (AVPCC) 2010. A comparison of the CLUM and 2008/09 VLUIS data sets for a limited number of similar land classifications in a region of the Gippsland Basin resulted in a greater than 85% correlation. A more rigorous analysis of reliability is not possible as VLUIS does not adopt the Australian Land Use Mapping (ALUM) classification categories which was used in the catchment modelling.

##### Climate data

The catchment-scale water balance modelling was based on discretising the study area into 865,611 landscape units of varying size based on land use, climate, topography and sub-catchment boundaries. Within each landscape unit a solution point was assigned a time varying climate dataset based on climate station records. To account for sparsely located climate stations within the study area, daily rainfall, temperature, evaporation and solar radiation data were scaled to each solution point within the study area according to interpolated mean annual spatial layers created using the ANUClim software (Hutchinson, 2001). This approach combines a DEM and temporal climatic data to generate a smoothed climate surface. As such, daily meteorological data assigned to each solution point within the study area was a function of the data from the 199 climate stations, landscape position and topography.

To assess the accuracy of the input climate data the error associated with the spatial interpolation of the key meteorological inputs of rainfall, Tmax, Tmin, potential evaporation and radiation was evaluated. The error was calculated by comparing the climate station daily recorded information to the interpolated mean annual estimate derived using the ANUClim software (Hutchinson, 2001) for the same climate station location. That is, a comparison of the mean annual 1976–2005 rainfall, Tmax, Tmin, potential evaporation and radiation for each climate station was assessed to estimate the error associated with the spatial assignment of climate data to each solution point.

The comparative histograms derived from the SILO climate station data and the interpolated ANUClim for mean annual rainfall, Tmin, Tmax, potential evaporation and radiation are shown in Figure 12 to Figure 16; also shown are the corresponding probability of exceedance curves (Dataset 12).

Figure 12 	Comparative histograms derived from the SILO climate station data and the interpolated ANUClim for mean annual (left) and corresponding probability of exceedance curve of the percent variation (right) (Dataset 12)

Figure 13	Comparative histograms derived from the SILO climate station data and the interpolated ANUClim for average daily minimum temperature (left) and corresponding probability of exceedance curve of the percent variation (right) (Dataset 12)

Figure 14	Comparative histograms derived from the SILO climate station data and the interpolated ANUClim for average daily maximum temperature (left) and corresponding probability of exceedance curve of the %  variation (right) (Dataset 12)

Figure 15	Comparative histograms derived from the SILO climate station data and the interpolated ANUClim for average potential evaporation (left) and corresponding probability of exceedance curve of the percent variation (right) (Dataset 12)

Figure 16	Comparative histograms derived from the SILO climate station data and the interpolated ANUClim for average daily solar radiation (left) and corresponding probability of exceedance curve of the percent variation (right) (Dataset 12)

Analysis of the climate data as reported in Table 1 suggests that the median variation between the observed climate station and interpolated ANUClim annual average rainfall, Tmin, Tmax, potential evaporation and radiation estimates are 6.0%, 8.8%, 2.9%, 2.5% and 2.1% respectively. The corresponding 90 percentile values are 1.3%, 2.2%, 0.9%, 0.5% and 0.7% respectively whereas the 10 percentile values are 13.8%, 25.6%, 5.6%, 14.6% and 4.7% respectively.

Table 1	Percentiles based on the percentage deviation of interpolated mean annual estimated derived using ANUClim to observed climate station data (Dataset 12)

##### Flood mapping

The positional accuracy of the Victorian Flood database is reported to vary from 10 cm to 100 m. Reliability and date fields as specified in the data files indicate the source and relative reliability of each map product. The attribute accuracy is considered mostly reliable, especially data sourced from recent flood studies.

#### Gaps

The characterisation of input data errors reported in Section 2.1.4.3 suggest knowledge gaps related to (1) enhancement of the digital elevation model for integration into the Vicmap Hydro spatial map; (2) up to date land use mapping; (3) soil map attribution (as opposed to soil map units); and (4) spatial interpolation of Bureau of Meteorology climate station daily observations.

It is acknowledged that the Vicmap Hydro layer currently is subject to cartographic generalisation of data (DSE, 2012). This is due to the conversion and manipulation processes which have contributed to the degradation of positional accuracy. Improvements in cartographic clarity and the derivation of topology and directional flow of stream mapping would enhance the accuracy of the water course spatial data layer. Such an enhancement would also assist in the creation of the next generation digital elevation model for Victoria.

Land use mapping is essential for water balance modelling. The catchment scale modelling associated with the current study assigns various farming system models to land management units based on land use mapping data. Each farming system model is then used to derive a daily time varying water balance at each land management unit across a landscape. Components of the water balance are then used as inputs into both surface water and groundwater modelling. Significant errors in estimates of recharge can be associated with incorrect land use mapping and management scripts assigned to irrigated regions.

Erroneous soil map attribution can result in up to 50% variation in water balance components depending on soil texture (Beverly, 2012). The soil map attribution relates to the parameterisation of a soil profile through depth and includes information for each texture classification within the soil profile such as water holding capacity, porosity, bulk density and vertical hydraulic conductivities. Whereas soil mapping defines the spatial extent of soil units, often soil unit attribution is not reported.

The network of Bureau of Meteorology climate stations is somewhat limited and would benefit from the install additional stations in regions of high climatic and topographic variability. Each station would benefit from being equipped to record solar radiation as currently solar radiation is only recorded at limited sites. Of equal importance is the deployment of tools enabling the spatial interpolation of point scale daily climate data. Whereas the gridded interpolated daily climate data (SILO Data Drill made by the Australian Bureau of Meteorology http://www.derm.qld.gov.au/services_resources) offers a solution to this problem, the 5 km x 5 km scale of gridded data is often too coarse for the catchment scale modelling approach used in the current study.

### Geology

Summary

A review of available surface and subsurface datasets in the onshore and offshore Gippsland Basin was conducted to build a regional geological model.

The Gippsland Basin geological model represents an interpretation of the underground geometries and lithologies. The interpretation is based on boreholes, wells, outcrop and geophysical data and accounts for the uncertainties and resolution inherent in these data. The well data helps to define the limits between the different geological units as well as the lithology. Geophysical data (mostly seismic reflection data) were used to extrapolate the data calibrated at the wells. Geophysical data (mostly potential fields) was also used to constrain basin architecture away from wells and seismic surveys. The uncertainty in these data increases as the distance between wells geophysical surveys increases.

As the density of both wells and geophysical data varies throughout the Gippsland Basin, isopach maps (i.e. thickness maps) were used to define the basin-scale architecture of the sequences.

The Gippsland Basin geological model can be updated and refined with additional or new datasets and can be populated with hydraulic properties derived from stratigraphic facies.

#### Methods

The main geological data sets used are: (1) the Victorian Aquifer Framework data, (2) the Latrobe Valley Coal model, (3) Gippsland Off-shore velocity model and (4) petroleum wells. Each is discussed in the following sections.

#### Observed data

This section described the nature, source, construct and the spatial and/or temporal resolution of all relevant geographic datasets.

There are four datasets that were utilised for the geological modelling task: petroleum wells dataset (GSV, 2014; Constantine 2001), Victorian Aquifer Framework (GHD, 2012), Latrobe Valley Coal Model (GHD, 2011) and geophysical datasets (McLean & Blackburn, 2013). The petroleum wells dataset consists of stratigraphic and lithological data compiled from well completion reports. The Victorian Aquifer Framework consists of hydrostratigraphic units extrapolated from the Victorian Groundwater database (GHD, 2012). The geophysical datasets comprise of a DEM model, Bathymetry, Bouguer Gravity and published interpretations of seismic reflection data. The 1:25 000 scale Structural Framework of the Onshore Gippsland geological map (Constantine et al., 1995) has also been used as a guide to determine the extent of the Strzelecki Group. The 1:250 000 Warragul and Sale geological maps (Vandenberg, 1997) have been used for other geological limits and the major structural trends (see Section 1.1.3 of companion product 1.1 for the Gippsland Basin bioregion). Figure 17 presents the major stratigraphic units of the sedimentary aquifers in the Gippsland Basin. The Strzelecki Group sits within the Cretaceous period not shown in Figure 17, and forms the bedrock aquifer system.

Figure 17  Stratigraphic units of the Gippsland study area (source; Schaeffer 2008), Dataset 1

##### Deep wells dataset

Well data provide information on the stratigraphy and the structure of the Gippsland Basin from geophysical logs, core and cutting analyses and test data. The Gippsland Basin is a mature petroleum province and an extensive deep well database with well completion reports from 1931 to 2013 by the Geological Survey of Victoria (GSV, 2014). Table 2 presents a list of 73 petroleum wells that have been drilled predominantly within the onshore and near shore sections of the Gippsland Basin. The list is presented in alphabetical order of the well name, not by formation. Further information on the wells can be found in the Victorian Petroleum Wells database or the Geoscience Australia Petroleum Wells database at://dbforms.ga.gov.au/www/npm.well.search.

Table 2  	Selected petroleum wells used for stratigraphic interpretation

| Well name | Date | Company | X coordinate (MGA54) | Y coordinate (MGA 54) | Total depth (m) |
| --- | --- | --- | --- | --- | --- |
| AVON-1 | 1990 | Mosaic Oil NL | 512128 | 5788952 | 934 |
| AVON-1 | 1990 | Mosaic Oil NL | 512128 | 5788952 | 934 |
| BALEEN-1 | 1981 | Hudbay Oil (Australia) Ltd | 626147 | 5792252 | 1030 |
| BAUDIN-1 | 1999 | Lakes Oil N.L. | 576812 | 5809375 | 426 |
| BELLBIRD-1 | 1963 | ARCO International Oil and Gas Company | 501088 | 5770580 | 763 |
| BURONG-1 | 1985 | Hartogen Energy Ltd | 517395 | 5759852 | 1259 |
| CARRS CREEK-1 | 1963 | ARCO International Oil and Gas Company | 523169 | 5761978 | 1679 |
| COLLIERS HILL-1 | 1970 | Woodside Petroleum Development Pty. Ltd. | 525651 | 5772246 | 1711 |
| CROSS ROADS-1 | 1971 | Halliday Enterprises Pty Ltd | 514243 | 5758004 | 1039 |
| CUTTLEFISH-1 | 1999 | Amity Oil NL | 592364 | 5794425 | 1226 |
| DARRIMAN-1 | 1955 | Frome Lakes | 500697 | 5744381 | 1497 |
| DEADMAN HILL-1 | 2002 | Lakes Oil N.L. | 515940 | 5772428 | 839 |
| DUCK BAY-1 | 1964 | ARCO Ltd. | 557961 | 5800239 | 1287 |
| DUTSON DOWNS-1 | 1966 | Woodside Petroleum Development Pty. Ltd. | 531711 | 5772182 | 1862 |
| EAST REEVE-1 | 1972 | Halliday Enterprises Pty Ltd | 548118 | 5783425 | 1622 |
| EAST SEACOMBE-1 | 1980 | Beach Petroleum NL | 546812 | 5784974 | 1362 |
| FAIRHOPE-1 | 1985 | Ampol Exploration Ltd | 551788 | 5803797 | 569 |
| GALLOWAY-1 | 2006 | Nexus Energy N.L. | 549307 | 5784519 | 2315 |
| GANGELL-1 | 2001 | Lakes Oil N.L. | 517317 | 5759405 | 2350 |
| GANNET-1 | 1970 | ENDEAVOUR | 599954 | 5804208 | 1460 |
| GOLDEN BEACH WEST-1 | 1965 | Woodside Petroleum Development Pty. Ltd. | 531155 | 5766790 | 2290 |
| GOLDEN BEACH-1A | 1967 | B.O.C. of Australia | 536914 | 5765607 | 2937 |
| GOON NURE-1 | 1931 | Amalgamated Oil Syndicate | 549052 | 5796136 | 893 |
| HEDLEY-1 | 1957 | Woodside Petroleum Development Pty. Ltd. | 460085 | 5722213 | 1223 |
| HOLLANDS LANDING-1 | 1940 | AGSO | 540425 | 5788170 | 1220 |
| INVESTIGATOR-1 | 1999 | Lakes Oil N.L. | 553976 | 5803731 | 697 |
| KEYSTONE-1 | 1972 | Halliday Enterprises Pty Ltd | 513733 | 5758005 | 1960 |
| LAKE REEVE-1 | 1965 | Woodside Petroleum Development Pty. Ltd. | 522307 | 5757974 | 2022 |
| LOY YANG-1A | 1994 | Capital Energy NL | 461493 | 5766123 | 1736 |
| MACALISTER-1 | 1988 | Crusader Ltd | 512130 | 5755414 | 1453 |
| MCCREESH-1 | 1998 | Roma Petroleum NL | 509260 | 5752833 | 1803 |
| MERRIMAN-1 | 1963 | ARCO International Oil and Gas Company | 515578 | 5755832 | 1830 |
| NORTH SEASPRAY-1 | 1962 | ARCO Ltd. | 517775 | 5761807 | 1524 |
| NORTH SEASPRAY-2 | 1965 | ARCO Ltd. | 517943 | 5760913 | 1633 |
| NORTH SEASPRAY-3 | 2000 | Lakes Oil N.L. | 517987 | 5761908 | 1170 |
| NORTHRIGHT-1 | 2001 | Eagle Bay Resources N.L. | 689037 | 5799642 | 391 |
| NUNTIN-2 | 1939 | Commonwealth Government | 518351 | 5791054 | 1085 |
| PATRICIA-1 | 1987 | Lasmo Energy Australia Ltd | 627058 | 5789884 | 900 |
| PATTIES PIES-1 | 2003 | Lakes Oil N.L. | 559436 | 5810660 | 441 |
| PAYNESVILLE-1 | 1985 | Ampol Exploration Ltd | 559230 | 5803603 | 708 |
| PELICAN POINT-1 | 1932 | Valve Oil Wells | 554178 | 5792837 | 704 |
| ROMAWI-1 | 1938 | Dept. of Nat. Res. & Environment - Govt. of Victoria | 552761 | 5797704 | 989 |
| ROSEDALE-1 | 1960 | APM Development Pty Ltd | 482123 | 5780238 | 1787 |
| SALT LAKE-1 | 1970 | Woodside Petroleum Development Pty. Ltd. | 507675 | 5744636 | 1644 |
| SEACOMBE SOUTH-1 | 1970 | Woodside Petroleum Development Pty. Ltd. | 542630 | 5778546 | 1186 |
| SEASPRAY-1 | 1964 | ARCO International Oil and Gas Company | 514125 | 5758085 | 1694 |
| SIGNAL HILL-1 | 1933 | Sale Oil Development Co | 527318 | 5767729 | 699 |
| SOUTH LONGFORD-1 | 1964 | ARCO International Oil and Gas Company | 507656 | 5772425 | 747 |
| SPERM WHALE HEAD-1 | 1939 | Valve Oil Wells | 561947 | 5797898 | 948 |
| SPOON BAY-1 | 1970 | Woodside Petroleum Development Pty. Ltd. | 540972 | 5785123 | 1400 |
| ST MARGARET ISLAND-1 | 1966 | Woodside Petroleum Development Pty. Ltd. | 485585 | 5723656 | 1422 |
| STRINGY BARK-1 | 1990 | Crusader Ltd | 491437 | 5736950 | 1047 |
| SUNDAY ISLAND-1 | 1966 | Woodside Petroleum Development Pty. Ltd. | 471251 | 5716127 | 1830 |
| SWEEP-1 | 1978 | Esso Australia | 643725 | 5786718 | 900 |
| TANJIL-PT.ADDIS-1 | 1939 | Tanjil Point Addis Co | 509431 | 5771869 | 485 |
| TANJIL-PT.ADDIS-2 | 1939 | Tanjil Point Addis Co | 514772 | 5766744 | 841 |
| TILDESLEY EAST-3 | 1986 | Arena Petroleum | 614646 | 5818306 | 300 |
| TRIFON-1 | 2000 | Lakes Oil N.L. | 516865 | 5760571 | 2570 |
| TRIFON-2 | 2004 | Lakes Oil N.L. | 517016 | 5760587 | 1267 |
| WAHOO-1 | 1969 | Esso Australia | 653413 | 5789770 | 746 |
| WELLINGTON PARK-1 | 1962 | Woodside (Lakes Entrance) Oil Company No Liability | 532832 | 5778804 | 3661 |
| WELLINGTON PARK-2 | 1970 | Woodside Petroleum Development Pty. Ltd. | 530663 | 5779256 | 1258 |
| WEST SEACOMBE-1 | 1972 | Halliday Enterprises Pty Ltd | 537066 | 5779217 | 1767 |
| WOMBAT-1 | 2004 | Lakes Oil N.L. | 514009 | 5755039 | 1990 |
| WOMBAT-2 | 2004 | Lakes Oil N.L. | 512512 | 5753661 | 1550 |
| WONGA BINDA-1 | 1988 | Crusader Ltd | 503644 | 5744336 | 1394 |
| WOODSIDE SOUTH-1 | 1965 | Woodside Petroleum Development Pty. Ltd. | 491984 | 5730785 | 1774 |
| WOODSIDE-1 | 1955 | Woodside Petroleum Development Pty. Ltd. | 494285 | 5728444 | 1831 |
| WOODSIDE-2 | 1957 | Woodside Petroleum Development Pty. Ltd. | 490830 | 5724681 | 2701 |
| WOODSIDE-4 | 1957 | Woodside Petroleum Development Pty. Ltd. | 486195 | 5720143 | 821 |
| WRIXONDALE-1 | 1985 | Beach Petroleum NL | 543724 | 5794768 | 987 |
| YORK-1 | 2002 | Lakes Oil Ltd | 487941 | 5729699 | 1200 |

Dataset 2

Figure 18 shows the varied distribution of deep wells within the Gippsland Basin. The highest concentration of wells are in the offshore near the Central Deep where both historical and current oil and gas production occurs. Onshore wells have been drilled proximal to Ninety Mile Beach from Yarram to Lakes Entrance. Although there has been considerable exploration drilling in the onshore, no oil or gas is produced in the onshore Gippsland Basin. In the west of the Gippsland Basin, the concentration of deep wells are considerably lower. The greatest uncertainty of the basin structure is to the east. The northern Gippsland Basin is lacking deep wells due to outcropping Paleozoic Basement indicating a lack of hydrocarbon resources. The depths of wells in the deep well dataset vary between 300 m measured depth (MD) (Tildesley East-3) and 3,661 m MD relative to the kelly bushing (KB) (Wellington Park-1).

Figure 18  Location of petroleum wells used (Dataset 3)

Major lithological variations were identified among various formations. These variations were typical of an environment of deposition ranging from fluvial to a deltaic environment to coal swamps, beach barrier sands, shallow marine and cold water carbonates.

Within the Seaspray Group, beach barrier sands to coal and beach barrier sands to cold water carbonates represent the highest level of lateral variation. The correlation between Traralgon coals onshore and offshore remains ambiguous given the well spacing and lack of seismic bridging over the onshore offshore transition. It is highly probable that although coals are continuous offshore, there is a degree of discontinuity and pinching out of individual seams onshore.

##### Gravity interpretation

Isostatic corrected Bouguer Gravity (Figure 19) was used to interpret the depth to Palaeozoic Basement within the west Gippsland region. Gravity was used due to the low density of petroleum wells, poor seismic data and lack of differentiation between Strzelecki Group and Palaeozoic Basement within the Victorian Aquifer Framework (GHD, 2009).

The Gravity images used for interpretation were processed by Intrepid Geophysics and the Geological Survey of Victoria. Further details of the processing are reported in Fitzgerald et al., (2009).

Figure 19 	Isostatically corrected Bouguer Gravity image for the Gippsland Basin (Fitzgerald et al., 2009). (Dataset 4)

##### Seismic stratigraphy

A regional velocity model built by Mclean and Blackburn (2013) was used to convert seismic interpreted stratigraphic surfaces to depth based on the approach reported by Keetley et al. (2010) and Blevin et al. (2013). Notably, the stratigraphic surfaces are confined to the offshore portions of the Gippsland Basin. The mapped accuracy of the seismic interpreted stratigraphic surfaces are highest closest to wells and decreases away from wells and seismic lines. This has produced greater accuracy in the central deep of the Gippsland Basin and lower accuracy within 5 km of the coastline.

##### Victorian Aquifer Framework

The Victorian Aquifer Framework (VAF, GHD 2012) provides the basis for grouping detailed geology into logical aquifer units allowing a consistent conceptualisation of the state’s aquifers and aquitards. The tiered system of geological units, hydrogeological units and aquifers as reported in the VAF has simplified complex geological sequences from a groundwater perspective while preserving the detail of the constituent units.

##### Latrobe Valley Coal Model

The Latrobe Valley Coal resource model has been developed utilising the past 40 years of stratigraphic drilling (GHD, 2011). The coal resource model specifies individual coal sequences as GIS surfaces, such M1A, M1B, T0, T1, T2, etc.

##### Digital Elevation Model

The surface topography represents a DEM model built from 1 second Shuttle Radar Topography Mission (SRTM) data (Geoscience Australia, 2008). The data was resampled to 100 m spatial resolution.

#### Statistical analysis and interpolation

Observed datasets (Section 2.1.6.1) were selected and processed to form derived datasets. These datasets were analysed and interpolated to propose a basin-scale three-dimensional geological model using Petrosys software.

##### Method

The method used to define the subsurface structural and stratigraphic architecture of the Gippsland Basin comprised:

- Selection and processing of spatial and point data to form derived datasets:

  - Reviewed all spatial data sets (VAF, seismic, coal model) and define stratigraphic sequence of differing units

  - Merge spatial data sets to form a single coverage for each geological unit across the region. Where data anomalies occurred, point data (petroleum wells) were used to assess which surface required adjustment

  - Incorporate Palaeozoic basement surface layer beneath Strzelecki geology.

- Error check layer surfaces:

  - Subtract geology tops from bottoms to determine residuals

  - Either correct or remove areas where residuals were less than zero

  - Map geology thickness for each unit across region for assessment.

##### Data selection and processing

The spatial distribution and density of the geological data in the Gippsland Basin is limited onshore in the west and good offshore in the east. Stratigraphic information provided by the petroleum well dataset was processed to constrain and inform the geological surfaces where no spatial information existed. Well completion reports from the 71 selected deep wells were analysed to determine each formation top depth, a listed in Table 3. The markers of the formation top depths in the wells are called ‘well picks’. The error in the formation top positions is a function of the well stratigraphic interpretation.

Table 3  	Petroleum well database ‘well picks’

| Well name | Formation top | Pick depth (m TVD ss) | Well name | Formation top | Pick depth (m TVD ss) |
| --- | --- | --- | --- | --- | --- |
| AVON-1 | La Trobe Group | 735 | ROMAWI-1 | La Trobe Group | 802 |
| AVON-1 | Lakes Entrance | 610 | ROMAWI-1 | Lakes Entrance | 597 |
| AVON-1 | Strzelecki Group | 890 | ROMAWI-1 | Strzelecki Group | 963 |
| BALEEN-1 | Lakes Entrance | 512 | ROSEDALE-1 | La Trobe Group | 52 |
| BALEEN-1 | Strzelecki Group | 707 | ROSEDALE-1 | Strzelecki Group | 714.8 |
| BAUDIN-1 | La Trobe Group | 368 | SALT LAKE-1 | La Trobe Group | 777.2 |
| BAUDIN-1 | Lakes Entrance | 304 | SALT LAKE-1 | Lakes Entrance | 675.1 |
| BELLBIRD-1 | La Trobe Group | 6.1 | SALT LAKE-1 | Strzelecki Group | 1588 |
| BELLBIRD-1 | Strzelecki Group | 295.7 | SEACOMBE SOUTH-1 | Lakes Entrance | 1009 |
| BURONG-1 | La Trobe Group | 655 | SEASPRAY-1 | La Trobe Group | 812.3 |
| BURONG-1 | Lakes Entrance | 552 | SEASPRAY-1 | Lakes Entrance | 655.3 |
| BURONG-1 | Strzelecki Group | 1251.2 | SEASPRAY-1 | Strzelecki Group | 1384.4 |
| CARRS CREEK-1 | Lakes Entrance | 584 | SIGNAL HILL-1 | La Trobe Group | 682.8 |
| CARRS CREEK-1 | Strzelecki Group | 1618 | SIGNAL HILL-1 | Lakes Entrance | 555.4 |
| COLLIERS HILL-1 | Lakes Entrance | 451 | SOUTH LONGFORD-1 | Strzelecki Group | 204 |
| CROSS ROADS-1 | La Trobe Group | 815.3 | SOUTH LONGFORD-1 |  | 0 |  |
| CROSS ROADS-1 | Lakes Entrance | 655.3 | SPERM WHALE HEAD-1 | La Trobe Group | 816.9 |
| CUTTLEFISH-1 | Lakes Entrance | 792 | SPERM WHALE HEAD-1 | Lakes Entrance | 664.5 |
| CUTTLEFISH-1 | Strzelecki Group | 998 | SPERM WHALE HEAD-1 | Strzelecki Group | 903.4 |
| DARRIMAN-1 | Lakes Entrance | 452 | SPOON BAY-1 | La Trobe Group | 1027.8 |
| DARRIMAN-1 | Strzelecki Group | 1290 | SPOON BAY-1 | Lakes Entrance | 976.884 |
| DEADMAN HILL-1 | La Trobe Group | 101 | SPOON BAY-1 | Strzelecki Group | 1305.153 |
| DEADMAN HILL-1 | Lakes Entrance | 82 | ST MARGARET ISLAND-1 | La Trobe Group | 604.1 |
| DUCK BAY-1 | La Trobe Group | 696 | ST MARGARET ISLAND-1 | Lakes Entrance | 548.6 |
| DUCK BAY-1 | Lakes Entrance | 579 | ST MARGARET ISLAND-1 | Strzelecki Group | 926.3 |
| DUCK BAY-1 | Strzelecki Group | 818.1 | STRINGY BARK-1 | La Trobe Group | 373.5 |
| DUTSON DOWNS-1 | Lakes Entrance | 578.5 | STRINGY BARK-1 | Lakes Entrance | 319.5 |
| DUTSON DOWNS-1 | Strzelecki Group | 1829 | STRINGY BARK-1 | Strzelecki Group | 1018 |
| EAST REEVE-1 | Lakes Entrance | 989 | SUNDAY ISLAND-1 | La Trobe Group | 399 |
| EAST REEVE-1 | Strzelecki Group | 1410 | SUNDAY ISLAND-1 | Lakes Entrance | 295.6 |
| EAST SEACOMBE-1 | La Trobe Group | 1094 | SUNDAY ISLAND-1 | Strzelecki Group | 506 |
| EAST SEACOMBE-1 | Lakes Entrance | 1076 | SWEEP-1 | La Trobe Group | 842 |
| FAIRHOPE-1 | La Trobe Group | 533 | SWEEP-1 | Lakes Entrance | 615 |
| FAIRHOPE-1 | Lakes Entrance | 496 | TANJIL-PT.ADDIS-1 | La Trobe Group | 367.9 |
| GALLOWAY-1 | Lakes Entrance | 965 | TANJIL-PT.ADDIS-1 | Strzelecki Group | 420 |
| GANGELL-1 | La Trobe Group | 683 | TANJIL-PT.ADDIS-2 | La Trobe Group | 397 |
| GANGELL-1 | Lakes Entrance | 642 | TANJIL-PT.ADDIS-2 | Lakes Entrance | 343.5 |
| GANGELL-1 | Strzelecki Group | 1251 | TANJIL-PT.ADDIS-2 | Strzelecki Group | 804.7 |
| GANNET-1 | La Trobe Group | 440 | TILDESLEY EAST-3 | Lakes Entrance | 159 |
| GANNET-1 | Strzelecki Group | 825.6 | TRIFON-1 | La Trobe Group | 689 |
| GOLDEN BEACH WEST-1 | Lakes Entrance | 585 | TRIFON-1 | Lakes Entrance | 620 |
| GOLDEN BEACH-1A | Lakes Entrance | 556 | TRIFON-1 | Strzelecki Group | 1236 |
| GOON NURE-1 | La Trobe Group | 786.5 | TRIFON-2 | La Trobe Group | 688 |
| GOON NURE-1 | Lakes Entrance | 756.9 | TRIFON-2 | Lakes Entrance | 563 |
| GOON NURE-1 | Strzelecki Group | 892 | TRIFON-2 | Strzelecki Group | 1236 |
| HEDLEY-1 | Strzelecki Group | 63 | WAHOO-1 | Lakes Entrance | 369 |
| HOLLANDS LANDING-1 | Lakes Entrance | 731 | WAHOO-1 | Strzelecki Group | 591 |
| INVESTIGATOR-1 | La Trobe Group | 584 | WELLINGTON PARK-1 | La Trobe Group | 727 |
| INVESTIGATOR-1 | Lakes Entrance | 529 | WELLINGTON PARK-1 | Lakes Entrance | 655 |
| KEYSTONE-1 | La Trobe Group | 826.3 | WELLINGTON PARK-1 | Strzelecki Group | 1158 |
| KEYSTONE-1 | Lakes Entrance | 645.6 | WELLINGTON PARK-2 | La Trobe Group | 718.1 |
| KEYSTONE-1 | Strzelecki Group | 1398.7 | WELLINGTON PARK-2 | Lakes Entrance | 613.9 |
| LAKE REEVE-1 | Lakes Entrance | 749 | WELLINGTON PARK-2 | Strzelecki Group | 1092.7 |
| LAKE REEVE-1 | Strzelecki Group | 1604 | WEST SEACOMBE-1 | La Trobe Group | 813 |
| LOY YANG-1A | Strzelecki Group | 148 | WEST SEACOMBE-1 | Lakes Entrance | 615.7 |
| MACALISTER-1 | La Trobe Group | 792.5 | WEST SEACOMBE-1 | Strzelecki Group | 1378.6 |
| MACALISTER-1 | Lakes Entrance | 675.5 | WOMBAT-1 | La Trobe Group | 694 |
| MACALISTER-1 | Strzelecki Group | 1384 | WOMBAT-1 | Lakes Entrance | 589 |
| MCCREESH-1 | La Trobe Group | 792.5 | WOMBAT-1 | Strzelecki Group | 1767 |
| MCCREESH-1 | Lakes Entrance | 694 | WOMBAT-2 | La Trobe Group | 776 |
| MCCREESH-1 | Strzelecki Group | 1380 | WOMBAT-2 | Lakes Entrance | 614 |
| MERRIMAN-1 | Lakes Entrance | 625 | WONGA BINDA-1 | La Trobe Group | 605.5 |
| NORTH SEASPRAY-1 | La Trobe Group | 585.83 | WONGA BINDA-1 | Lakes Entrance | 530.5 |
| NORTH SEASPRAY-1 | Lakes Entrance | 518.2 | WONGA BINDA-1 | Strzelecki Group | 1344 |
| NORTH SEASPRAY-1 | Strzelecki Group | 1104.29 | WOODSIDE SOUTH-1 | La Trobe Group | 592.3 |
| NORTH SEASPRAY-2 | La Trobe Group | 646 | WOODSIDE SOUTH-1 | Lakes Entrance | 511.5 |
| NORTH SEASPRAY-2 | Lakes Entrance | 515 | WOODSIDE SOUTH-1 | Strzelecki Group | 991.5 |
| NORTH SEASPRAY-2 | Strzelecki Group | 1219 | WOODSIDE-1 | La Trobe Group | 777.3 |
| NORTH SEASPRAY-3 | La Trobe Group | 575 | WOODSIDE-1 | Lakes Entrance | 701 |
| NORTH SEASPRAY-3 | Lakes Entrance | 511 | WOODSIDE-1 | Strzelecki Group | 1191.8 |
| NORTH SEASPRAY-3 | Strzelecki Group | 1087 | WOODSIDE-2 | La Trobe Group | 759 |
| NORTHRIGHT-1 | Lakes Entrance | 250 | WOODSIDE-2 | Lakes Entrance | 682.7 |
| NORTHRIGHT-1 | Strzelecki Group | 337 | WOODSIDE-2 | Strzelecki Group | 1158 |
| NUNTIN-2 | La Trobe Group | 920.5 | WOODSIDE-4 | La Trobe Group | 705.3 |
| NUNTIN-2 | Lakes Entrance | 712.3 | WOODSIDE-4 | Lakes Entrance | 610.8 |
| PATRICIA-1 | Lakes Entrance | 655 | WRIXONDALE-1 | La Trobe Group | 783.5 |
| PATRICIA-1 | Strzelecki Group | 800 | WRIXONDALE-1 | Lakes Entrance | 629 |
| PATTIES PIES-1 | La Trobe Group | 297 | WRIXONDALE-1 | Strzelecki Group | 948.5 |
| PATTIES PIES-1 | Lakes Entrance | 250 | YORK-1 | La Trobe Group | 532 |
| PAYNESVILLE-1 | La Trobe Group | 569 | YORK-1 | Lakes Entrance | 468.5 |
| PAYNESVILLE-1 | Lakes Entrance | 529.5 | YORK-1 | Strzelecki Group | 954.5 |
| PELICAN POINT-1 | Lakes Entrance | 703.8 |  |  |  |

Dataset 2

Three representative regional formation tops were selected from the Petroleum well database. These were used to extend the existing representative regional formations from the onshore to the offshore.

Key regional geological formation tops in the Gippsland Basin included:

•	Lakes Entrance Formation

•	La Trobe Group

•	Strzelecki Group.

The Boisdale Formation and Gippsland Limestone were not included as they were too shallow for data to be recorded in Petroleum Well Completion Reports.

### Hydrogeology and groundwater quality

#### Methods

##### Aquifer assignment

Assigning a bore to a specific aquifer is based on the screened interval data and the aquifer geometry. In many cases, it is impossible to assign the screened interval of a bore to an aquifer, as there is insufficient information on stratigraphy and screened intervals. Although the Victorian groundwater database contains stratigraphic data for some groundwater bores in the Gippsland region, stratigraphic information is not available for many groundwater bores.

All available groundwater observation bores data was compiled and included in this analysis. The compiled dataset comprised 686 onshore bores as recorded in the Victorian state groundwater database and 136 bores managed by other institutions (namely around the coal mines). Following a stratigraphic assessment for the Gippsland region, and after a quality check of the data, bores were assigned to aquifers by associating their screen intervals and depth with aquifer geometry.

The following steps were followed during the aquifer assignment:

Assess the boundaries of aquifers as outlined in Section 1.1.4.1 of companion product 1.1 for the Gippsland region.

Determine the screen intervals of bores onshore. The screen interval information was extracted from the groundwater database in the Water Measurement Information System (WMIS) (DEPI, 2014). For the bores without screen interval information in the database, the bore depth information was extracted from the site information table in the WMIS. For these bores, it is assumed that the screens start from the bottoms of the bores.

Determine the screen interval of bores for the bores offshore. The screen interval and bore depth information was extracted from the data provided the Victorian Water Management Information System (WMIS).

Filter bores for a specific area using a shape file or coordinates.

Cross-check the final datasets against expert knowledge and spatial context of aquifers.

##### Water levels

Water levels for the Gippsland region were obtained from the WMIS dataset (onshore bores) and Exxon Mobil (offshore production wells). The water level in the WMIS database is presented in three forms – depth from top of casing (m), the depth of water below the ground surface (m) and reduced water level (mAHD). For a small number of bores, reduced water levels are not available as their elevations are missing from the database. For these bores, the bore elevations were determined from a high resolution digital elevation model. Water level information for the offshore production wells are presented in the form of reduced water level (mAHD) instead of pressure head equivalents.

##### Hydraulic parameters

There are over 150,000 groundwater bores with in the Victorian groundwater database, some of which have undergone pumping tests. Notably, hydraulic parameters are not available in the state groundwater databases although the databases do contain limited pumping test data such as the date and duration of the test, initial water level, pumping rate, and maximum drawdown. To improve the accessibility and present the information in a more useful form, DSE (2011) created a Victorian Pump Test Database (PTD) which reports aquifer parameters derived from pumping tests referenced to the pump bore used, and linked to the reporting document. The PTD forms the main source of hydraulic parameters for this project.

Other hydrogeological data sources (e.g. university theses, consultancy reports and government reports) were explored and hydrogeological records were incorporated into the dataset where appropriate. The key previous studies explored include:

West Gippsland CMA Groundwater Model-Transient Model Development Report (GHD, 2012a)

East Gippsland CMA Groundwater Model-Transient Model Development Report (GHD, 2012b)

Scaling Point Based Aquifer Data for Developing Regional Groundwater Models: Application to the Gippsland Groundwater System (Schaeffer, 2008).

##### Groundwater quality

Groundwater quality was sampled in the Gippsland Basin bioregion to assist in the characterisation of the aquifers in the Gippsland Basin. Historical data and literature was used to inform the site selection and selection of analytes to be sampled for. Three rounds of sampling were undertaken between December 2014 and April 2015 from 29 bores. Samples were taken using low flow sampling techniques. Bores were sampled for major ions, methane, BTEX, TPH, hydrocarbons and nitrogen. Full results of the sampling are available from the Onshore natural gas water science studies website (http://onshoregas.vic.gov.au/science-studies/about-the-water-studies).

This report presents the groundwater salinity records (uS/cm) from the Victorian Measurement Information System (WMIS) to show the distribution and general quality of groundwaters in the Gippsland Basin and a summary of the groundwater quality from the groundwater sampling program.

#### Observed data

The groundwater observation data for the Gippsland region was primarily sourced from the Victorian groundwater databases in the WMIS (onshore bores). For the offshore bores, the observation data was obtained from Exxon Mobile. The datasets used in the analysis are listed in Table 4.

Groundwater licence data was primarily sourced from Southern Rural Water (the licensing authority for groundwater in the Gippsland Basin). The data includes license number, bore location and annual licensed volume. It is noted that a groundwater licence may be associated with more than one bore. As the annual allocation volume is only associated with licence, for those licences with more than one bore or service point, one representative service point was selected to represent the location of the licence. The service points under the same licence are generally close to each other. It is believed that this assumption would cause minimum error. The extraction volumes for stock and domestic bores was provided by DSE. Licensed volumes and extraction data associated with Latrobe Valley coal mines was sourced via annual coal mine water usage reports. Groundwater extractions associated with offshore oil and gas operations were compiled by the Victorian Department of State Development, Business and Innovation via Exxon Mobile. The data used in the EcoMarkets project (GHD, 2012a; 2012b) was also incorporated into this dataset.

As screen information is necessary to assign a bore to particular aquifers, bores are assigned to aquifers based on the screen interval and when a bore’s screen information is not available, its depth is used to assign the bore to a particular aquifer.

The alluvium and fractured rock aquifer subgroups represent the most accessed aquifer with 487 and 46 bores respectively. Most bores are either screened in the alluvium or the outcrops of underlying bedrock aquifers. Figure 20 shows the spatial distribution of these extraction bores.

Table 4 	Data tables from the various sources used for hydrogeology and water quality analysis in the Gippsland region

| Observed data | Title | Dataset name | URI |
| --- | --- | --- | --- |
| Bore construction | Victorian Water Measurement Information System | Bore | data.water.vic.gov.au/monitoring.htm |
|  | Victorian Water Measurement Information System | Construction details | data.water.vic.gov.au/monitoring.htm |
|  | Victorian allocation of aquifer for screened unit of monitoring bores | Aquifers assignment | data.water.vic.gov.au/monitoring.htm |
| Groundwater level | Victorian Water Measurement Information System | Groundwater level | data.water.vic.gov.au/monitoring.htm |
|  | Exxon Mobile | Digital data supply |  |
| Hydraulic parameters | Various previous studies | Refer to Schaeffer 2008 |  |
| Allocation | Southern Rural Water/DEPI | Victorian Water Accounts |  |

Figure 20 	Location of observation bores in the Gippsland region by GMU

Table 5 	Number of bores by aquifer system showing variability of aquifer depth in the Gippsland region

| Aquifer system | Number of bores | Depth range* (m,BGL) | Modelled layer numbers1 |
| --- | --- | --- | --- |
| Quaternary alluvium aquifer | 487 | 1 – 335 | 1 |
| Upper Tertiary aquifer | 36 | 8 – 670 | 2 – 8 |
| Mid-Tertiary aquifer | 60 | 3 – 223 | 9 – 16 |
| Lower Tertiary aquifer | 174 | 3 – 680 | 17 – 22 |
| Pre-Tertiary bedrock aquifer | 46 | 1 – 1779 | 23 – 30 |

*The depth column presents the depth range of bores associated with a specific aquifer

1The modelled layer numbers column refers to the groundwater model layers associated with a specific aquifer

#### Statistical analysis and interpolation

##### Groundwater level

A summary of the measured water level data used in this analysis is provided in Table 6. More than a third of the 822 observation bores identified for this project are constructed in the Tertiary aquifers (model layers 2 to 22) and the majority are located within groundwater management units (GMUs). Those bores located within GMUs generally have longer and more frequent monitoring records. Records of groundwater level for bores constructed in located Quaternary alluvium aquifer (model layer 1) are also significant, with 75% of the total records. Conversely, the number of records for the observation bores constructed in Pre-Tertiary bedrock aquifer (model layers 23 to 30) are limited due to the relatively small number of bores and the shorter monitoring history. The bores offshore generally only have one indicative reading estimated from wellhead pressure test data provided by Exxon Mobile.  Table 6 summarises the number of groundwater monitoring records in the Victorian state groundwater database, by aquifer system, for those bores located in the Gippsland region. An analysis of groundwater level trends for bores in the Gippsland region will be reported in the Groundwater numerical modelling report (companion product 2.6.2 for the Gippsland region).

Figure 21 	Summary of monitoring records for bores in the Gippsland region

Table 6 	Summary of water level records for bores in the Gippsland region

| Aquifer system | Number of bores | Average and range of groundwater depth (m,BGL) | Number of monitoring records |
| --- | --- | --- | --- |
| Quaternary alluvium aquifer | 487 | 5 (2 to 61) | 75,612 |
| Upper Tertiary aquifer | 36 | 10 (2 to 41) | 6,096 |
| Mid-Tertiary aquifer | 60 | 13 (3 to 126) | 6,970 |
| Lower Tertiary aquifer | 174 | 39 (3 to 188) | 9,453 |
| Pre-Tertiary bedrock aquifer | 46 | 48 (4 to 796) | 2,935 |

##### Hydraulic parameters

There are a limited number of pumping test records within the DSE pumping test database for the bores located in the Gippsland region. The majority of the pumping tests were conducted in the 1970s (e.g. Nahm 1977; Nahm & Reid, 1979a; 1979b; 1979c). Supplementary reports including commissioned studies (e.g. GHD, 2012a; 2012b) and university theses (e.g. Schaeffer, 2008) were also referenced to compile the hydraulic parameter’s values for the aquifers of interest. Table 7 summarises the parameter ranges in the dominant aquifers within the Gippsland region. Ss refers to specific storage, Sy refers to specific yield and Kxy refers to horizontal hydraulic conductivity.

Table 7 	Summary of hydraulic parameter’s typical range reported for the main aquifers in the Gippsland region

| Aquifer system | Ss (m-1) | Sy | Kxy (m/day) |
| --- | --- | --- | --- |
| Quaternary Alluvium Aquifer | 1.0E-05 to 1.0E-04 | 0.04 to 0.15 | 2 to 50 |
| Upper Tertiary Aquifer | 1.0E-05 to 1.0E-03 | 0.05 to 0.2 | 1 to 30 |
| Mid-Tertiary Aquifer | 3.0E-06 to 5.0E-04 | 0.015 to 0.1 | 1.5 to 10 |
| Lower Tertiary Aquifer | 1.0E-06 to 4.7E-04 | 0.03 to 0.1 | 1 to 40 |
| Pre-Tertiary Bedrock Aquifer | <3.0E-06 to 5.0E-03 | 0.005 to 0.02 | - o 1 |

##### Groundwater quality

The observed groundwater quality data for the Gippsland region are sourced from the Victorian groundwater database in the WMIS. Of the 686 onshore observation bores, 128 bores have at least one salinity reading. Notably, the majority of these bores are constructed in the Quaternary and Tertiary aquifers within the central region of the Gippsland Basin. A summary of the groundwater salinity based on analysis of the groundwater bore salinity data for the main aquifers is presented in Table 8.

Groundwater quality is mapped by salinity for each of the major aquifer systems, and classed according to the SEPP Groundwaters of Victoria beneficial use classes. Most of the licensed extraction points are located in areas of low salinity (< 1000 mg/L).

Table 8 	Summary of groundwater salinity for the main aquifers in the Gippsland region

| Aquifer system | Number of bores | Total number of records | Median groundwater salinity (µS/cm) |
| --- | --- | --- | --- |
| Quaternary Alluvium Aquifer | 65 | 781 | 1114 |
| Upper Tertiary Aquifer | 34 | 677 | 1355 |
| Mid-Tertiary Aquifer | 17 | 129 | 1000 |
| Lower Tertiary Aquifer | 8 | 11 | 708 |
| Pre-Tertiary Bedrock Aquifer | 4 | 69 | 1682 |

##### Licensed extraction volume

The data for licensed extraction bores is summarised in Table 9. The majority of the licensed bores are located in groundwater management areas and extractions are from the major tertiary aquifers. In addition to licensed extractions, there are a large number of stock and domestic bores. Each of these bores has a small estimated extraction volume (ranging from 1 to 2 ML/yr) such that the total volume of groundwater extraction from these bores is relatively small compared with the total extracted volume. The extracted volume attributed to licensed, stock and domestic, mine and offshore bores from 1970 to 2012 is shown in Figure 22. The corresponding extracted volume by aquifer system is shown in Figure 23.

Table 9 	A summary of groundwater extraction data for the bores in the Gippsland region

| Aquifer system | Number of licensed bores | Number of stock and domestic bores | Number of mine bores | Number of offshore bores | Mean annual volume* (ML) |
| --- | --- | --- | --- | --- | --- |
| Quaternary Alluvium Aquifer | 894 | 4081 | 27 | 0 | 40,859 |
| Upper Tertiary Aquifer | 85 | 438 | 0 | 0 | 9,635 |
| Mid-Tertiary Aquifer | 142 | 520 | 11 | 0 | 8,604 |
| Lower Tertiary Aquifer | 104 | 234 | 78 | 22 | 116,282 |
| Pre-Tertiary Bedrock Aquifer | 133 | 812 | 0 | 0 | 7,930 |

* Mean annual volume calculated from 1970–2012 inclusive

Figure 22	Annual extraction volume (ML) attributed to licensed, stock and domestic, mine and offshore bores for 1970–2012

Figure 23	Annual extraction volume (ML) attributed to aquifer system for 1970–2012

#### Gaps

It is noteworthy that the groundwater observation bore network in the Gippsland Basin region is sufficient to monitor the key water resource aquifers. Adjacent to existing mines there is good coverage with shallow groundwater observation bores and regular reporting of groundwater dynamics. While there are many groundwater bores and a large number of groundwater observation bores, an on-going issue relates to future monitoring and maintenance. This aside, there are some distinct data and knowledge gaps that could potentially influence achieving realistic simulation and modelling based analysis. These include:

There are limited deep groundwater observation bores, as 74% of all groundwater observation bores are less than 50 m deep and less than 17% are in the deep water-bearing units at depths greater than 150 m. As such, the existing groundwater monitoring network is likely to capture only a small component of the hydrodynamics of the Gippsland Basin subregion.

Critical hydraulic information, including hydraulic properties and water chemistry, of key aquifers is currently missing. More information about hydraulic properties of key aquifer units needs to be obtained. In particular, information about the deep water-bearing units is required for the numerical model development to assess the impacts associated with coal seam gas development.

There is a general lack of nested (multi-level) bore sites throughout the Gippsland Basin bioregion. While groundwater dynamics in shallow alluvial aquifers are relatively well understood, there is very limited knowledge on characteristics such as groundwater flow direction or inter-aquifer hydraulic head gradients throughout much of the bioregion.

There are significant gaps in the groundwater databases. Of the observation bores identified in the Gippsland Basin bioregion, 62% has incomplete screen depth information.

There is limited information associating a groundwater bore to a specific ‘aquifer’ layer where the screened interval of bores is assigned to a specific aquifer in the Victorian Aquifer Framework or equivalent.

The hydraulic significance of faults is poorly understood due to the lack of nested (multi-level) groundwater monitoring sites. Only limited understanding exists about the role of faults as potential pathways or barriers for aquifer interconnectivity or groundwater flow to the surface. More research is required to identify the spatial extent and properties of faults.

Uncertainty in recharge estimates impacts the robustness of model predictions. Numerous studies have identified significant variations in recharge. It is recommended that an evaluation of recharge estimates and their method of derivation be undertaken and documented.

There remains incomplete time-varying groundwater pumping volume data. This information is critical in any assessment of groundwater response to proposed extractions. It is recommended that a database of time-varying extractions volumes (both measured and infilled) for all groundwater pumps be developed. A quality code should also be assigned to each record to identify the source of the measurement (actual, interpolated or infilled). In addition, the on-going installation of metering and monitoring of groundwater pumps is required in the future. This information would avoid variations in the estimated historical groundwater extraction volumes and unnecessary duplication in effort and resources undertaken by various modelling studies.

There is limited groundwater quality data in the groundwater databases. Of the onshore groundwater observation bores, only 18% have at least one salinity record and less than 10% have other constituent information. For example, nutrient data is infrequently recorded and only available for 8% of all bores listed in the groundwater database.

### Surface water hydrology and water quality

Summary

The Gippsland Basin bioregion contains 197 streamflow gauges. The Victorian water monitoring scheme classifies streamflow data into 57 quality codes. In this analysis, these quality codes were grouped into six broad water quality codes based on the national water quality coding scheme to enable consistency and comparison between states and territories. Based on an analysis of all daily streamflow gauge data, 72% of all records were good, 8% fair, 6% poor, 4% unknown and 10% missing. The quality of individual streamflow data varied significantly whilst results suggest that the streamflow monitoring program in the South Gippsland Basin was of a higher quality than the monitoring in all other basins. Water chemistry and quality was not considered in this analysis.

#### Methods

The Gippsland region includes the South Gippsland, Latrobe, Thomson, Macalister, Avon, Mitchell, Tambo and Nicholson, Snowy and East Gippsland river basins, as shown in Figure 24. The Latrobe, Thomson-Macalister, Avon, Tambo, Nicholson and Mitchell rivers flow into the Gippsland Lakes, while the Tarwin, Snowy and Cann rivers flow into small estuaries or to the sea. The Tarwin, Agnes and Tarra Rivers flow steeply from the southern face of the Strzelecki Ranges to the coast. The Gippsland Lakes form the largest estuarine lake system in Australia. Along with Ninety Mile Beach they are major features of the region. Dunes and wetlands are common around the lakes and along the coast. There are also some large dams such as Blue Rock, Thomsons Dam and Glenmaggie Dam, and extensive channel and drain networks in the Macalister Irrigation District (MID).

##### Latrobe River

The Latrobe Basin includes the Latrobe, Tanjil, Tyers, Moe, Morwell and Traralgon river systems. The Latrobe River rises at an elevation of approximately 750 m due west of Noojee. Its headwaters, together with the Ada, Loch and Toorongo rivers, drain the eastern and southern slopes of the Yarra Ranges, which form a southern extension of the Great Dividing Range. Approximately 70 km from its source, the Latrobe River emerges from the foothills onto its floodplain. The river flows through the Latrobe Valley and discharges into Lake Wellington, at the western end of the Gippsland Lakes. The Latrobe River is the highest contributor of freshwater inflows to the Gippsland Lakes contributing 44% of mean annual inflow. The Latrobe River system has been most affected by regulation and extraction, with a 33% reduction of inflows to Lake Wellington (Tilleard et al., 2009).

The Latrobe Valley houses two major water storages, Blue Rock Lake (capacity 208 GL) and Moondarra Reservoir (capacity 30 GL). Another much smaller storage, Lake Narracan, is situated on the main stem of the Latrobe River at Yallourn near Moe. Blue Rock and Lake Narracan are primarily used to supply Yallourn, Loy Yang A and Loy Yang B power stations.

The environmental condition of the Latrobe River varies from excellent in the headwaters to moderate and poor condition in the mid – lower reaches below the storages. Freshwater flows from the Latrobe Basin are critical for sustaining the health of the Ramsar-listed Gippsland Lakes, which underpin the region’s tourism industry.

##### Mitchell River

The Mitchell River is identified as a heritage river and as one of two iconic rivers in Victoria. This is largely because of its size, being the largest unregulated river in Victoria, and because it supports a wide range of environmental and social values. Land use in the upper catchment includes sections of the Alpine National Park and the Mitchell River National Park.

The Mitchell River system originates on the southern slopes of the Great Dividing Range, with the Wonnangatta and Dargo rivers being the two major rivers in this area, and forming the Mitchell River downstream of their confluence. The Mitchell River discharges to Lake King within the Gippsland Lakes. Other surface water systems include the Humfray, Dargo, Wongungarra and Wonnangatta rivers, Wentworth Creek and the Gippsland Lakes.

The river system is highly valued by the community for urban and rural water supply, recreation and its contribution to the Gippsland Lakes. The Mitchell River provides about one third of the total flow to the Lakes system on average.

Geologically 70% of the catchment consists of fractured rock systems in Palaeozoic-aged metasediments and intrusive rocks. Small areas of alluvial valley sequences occur in the highland valleys together with layered systems within Tertiary-aged volcanics. The southern third of the catchment contains the regional aquifer systems of the Gippsland Basin sequence (SKM, 2012a).

##### Thomson/Macalister rivers

The Thomson River flows from the Dividing Range and joins the Latrobe River to the south of Sale. The Macalister River flows from the Alpine National Park in the Snowy Ranges and joins the Thomson River between Maffra and Sale. The Thomson and Macalister rivers then continue towards and contribute flows to the Ramsar-listed Gippsland Lakes. The Thomson River above Cowwarr Weir is listed as a Heritage River.

There are two major storages within the Thomson-Macalister system, namely Lake Glenmaggie situated on the Macalister River and the Thomson Reservoir on the upper reaches of the Thomson River. Lake Glenmaggie is the main source of supply for the Macalister Irrigation District, the largest irrigation area south of the Great Dividing Range. The Thomson Dam provides some of Melbourne’s water supply system and the Macalister Irrigation District.

However, both the Thomson and Macalister rivers are amongst the most stressed rivers in Victoria. This is due to the altered flow regimes, through regulation and over allocation of water for irrigation and consumptive use. As such, river management is considered to have the greater impact on mitigating river stress than groundwater/baseflow management. The Thomson and Macalister rivers directly below the Thomson Dam and Lake Glenmaggie respectively have reduced annual flows and reversed seasonality as a result of the reservoirs, with high flows in January and February and low flows during the winter months. This altered flow regime has significant impacts on the breeding and migration cues for fish. The dams and weirs are also a barrier to fish movement. Poor water quality in the rivers is affecting water supplies for consumptive use and recreation, as well as the Gippsland Lakes.

##### Avon River

The Avon River rises on the slopes of Mount Wellington in the eastern highlands and flows south to Lake Wellington. A significant portion of the upper catchment is protected in the Avon Wilderness Area (295 km2). The upper catchment is steep, heavily forested and largely inaccessible. The channel is stable and confined by bedrock. The upper reaches of the Avon River including Turton and Dolodrook rivers and Ben Cruachan Creek have been listed as Natural Catchment Areas.

The Lower Avon catchment below the Valencia Creek confluence flows through a topographically flat area which has been predominantly cleared for agriculture. The Avon River above Stratford and Freestone Creek have experienced dramatic widening since European settlement and now flow through wide mobile gravel beds. The Perry River joins the Avon approximately 1 km upstream from where the Avon discharges into Lake Wellington.

In the lower part of the Avon catchment, both surface water and groundwater are utilised for irrigation. The river has experienced low flows over the past decade and as a consequence, frequent irrigation restrictions have been imposed (Jones et al., 2009).

##### Snowy River

The Snowy River originates on the slopes of Mount Kosciuszko, draining the eastern slopes of the Snowy Mountains in New South Wales, before flowing through the Alpine National Park and the Snowy River National Park in Victoria and emptying into Bass Strait. The river flow was drastically reduced in the mid-20th century after the construction of four large dams (Guthega, Island Bend, Eucumbene, and Jindabyne) and many smaller diversion structures in its headwaters in New South Wales, as part of the Snowy Mountains Scheme.

Figure 24	Major rivers of the Gippsland Basin

##### Flooding

Flooding contributes to episodic recharge and was considered in the Gippsland groundwater modelling study. For example, in early June 2012, much of Gippsland experienced heavy rainfall causing flooding across a number of municipalities, including Latrobe, Wellington and East Gippsland. Hydrograph analysis revealed that groundwater levels responded to this event.

For this study flood data was sourced from the Victorian Flood Database and included information regarding location and extent of a 1-in-100 year event (Figure 25) and historical flood events. Figure 26 shows the location and extent of recent major floods and Table 10 presents the magnitude of each flooding event in the past 100 years (including area directly affected and duration).

Although damage caused by some recent flooding events (e.g. June 2012) to towns and communities was widespread in the Gippsland region, the areas directly affected by flooding were relatively small compared with the total area of the region. The extent of the 1-in-100 year flooding event only covers 4.8% of the total area of the Gippsland region. In the last 100 years, flooding events mainly occurred in small areas around the lower reaches of the Mitchell, Thomson, Latrobe and Snowy rivers. The area directly affected by flooding was generally less than 1% of the Gippsland region. The largest flood event occurred in June 2012 and covered 1.4% of the Gippsland region. Given the areas affected by historic flooding events were relatively small, it is believed that the impact of flooding on groundwater recharge is insignificant at the regional scale, but likely to be significant at the local scale.

By examining groundwater hydrographs across the Gippsland region, noticeable impact of flooding on groundwater recharge is only evidenced in the shallow aquifer systems well connected to the lower reaches of Thomson, Mitchell and Latrobe rivers. It was found that recharge spikes in groundwater hydrographs of some shallow bores near the rivers are aligned to high river flow events.

Figure 25 	Location and extent of 1-in-100 year flooding events in the Gippsland area (sourced from the Victorian Flood Database)

Figure 26	Location and extent of major recent flooding events (sourced from the Victorian Flood Database

Table 10	Extents of historical flood events (sourced from the Victorian Flood Database)

| Date | Duration (days) | Area affected (ha) | % of total catchment area |
| --- | --- | --- | --- |
| Jan-1919 | Not known | 2,717 | 0.1 |
| Jan-1920 | Not known | 1,129 | 0.0 |
| Oct-1923 | Not known | 208 | 0.0 |
| Jan-1934 | Not known | 7,527 | 0.2 |
| Dec-1934 | Not known | 13,361 | 0.4 |
| May-1968 | 1 | 544 | 0.0 |
| Jun-1969 | 1 | 644 | 0.0 |
| Feb-1971 | Not known | 1,666 | 0.0 |
| Sep-1974 | Not known | 42 | 0.0 |
| Jan-1977 | Not known | 11,323 | 0.3 |
| Jul-1977 | 3 | 20,248 | 0.5 |
| Jan-1978 | Not known | 2,884 | 0.1 |
| May-1978 | 5 | 14,947 | 0.4 |
| Jun-1978 | 5 | 12,086 | 0.3 |
| Jul-1978 | 1 | 3,709 | 0.1 |
| Jan-1985 | Not known | 740 | 0.0 |
| Dec-1985 | 1 | 6,427 | 0.2 |
| Nov-1988 | 1 | 7,061 | 0.2 |
| Apr-1990 | 27 | 32,563 | 0.9 |
| Oct-1990 | 1 | 3,278 | 0.1 |
| Oct-1991 | 1 | 169 | 0.0 |
| Sep-1993 | 16 | 2,155 | 0.1 |
| Jun-1998 | 3 | 6,364 | 0.2 |
| Nov-1998 | 1 | 949 | 0.0 |
| Jun-2007 | 4 | 23,308 | 0.6 |
| Jul-2011 | Not known | 36 | 0.0 |
| Mar-2012 | 8 | 9,023 | 0.2 |
| Jun-2012 | 3 | 54,912 | 1.4 |
| 1-in-100 year extent | N/A | 181,490 | 4.8 |

Source data: Victorian Flood Database

#### Observed data

There are 197 streamflow gauges in the Gippsland Basin. For this analysis, each streamflow gauge was assigned to the major river basin in which it was located. The nominated major river basins were the East Gippsland region, the Latrobe River, the Mitchell River, the Snowy River, the South Gippsland region, the Tambo River and the Thompson River. Site details for each streamflow gauge is summarised in Table 11 to Table 17 for each river basin. Daily computed daily streamflow data were obtained from the Victorian Water Measurement Information System (http://data.water.vic.gov.au/monitoring.htm). Associated with each daily flow estimate is a quality code which reflects the accuracy of the data. The quality of the daily interpolated streamflow data is dependent on the accuracy and robustness of the rating curve used to define the relationship between streamflow and gauge stage height.

Table 11	East Gippsland Basin streamflow site details

| Site ID | Site name | Easting | Northing | Start date | Cease date |
| --- | --- | --- | --- | --- | --- |
| 221001 | GENOA RIVER @ ROCKTON | 705825 | 5887218 | 26/05/1993 |  |
| 221201 | CANN RIVER (WEST BRANCH) @ WEERAGUA | 694682 | 5861473 | 21/02/1957 |  |
| 221202 | GENOA RIVER @ WANGARABELL | 721597 | 5858291 | 13/10/1927 | 30/06/1929 |
| 221203 | BETKA RIVER @ MALLACOOTA | 737895 | 5836091 | 26/01/1966 | 24/07/1972 |
| 221204 | THURRA RIVER @ POINT HICKS | 699200 | 5822200 | 17/05/1966 | 28/07/1978 |
| 221205 | BEMM RIVER @ BEMM RIVER | 671894 | 5821411 | 28/06/1966 | 5/06/1975 |
| 221206 | CANN RIVER @ NOORINBEE | 694397 | 5854712 | 31/05/1967 | 3/02/1971 |
| 221207 | ERRINUNDRA RIVER @ ERRINUNDRA | 669418 | 5853798 | 28/07/1971 |  |
| 221208 | WINGAN RIVER @ WINGAN INLET NATIONAL PARK | 719710 | 5825780 | 14/12/1982 |  |
| 221209 | CANN RIVER (EAST BRANCH) @ WEERAGUA | 695470 | 5863398 | 23/10/1972 |  |
| 221210 | GENOA RIVER @ THE GORGE | 723046 | 5856007 | 22/08/1972 |  |
| 221211 | COMBIENBAR RIVER @ COMBIENBAR | 675439 | 5854433 | 12/08/1974 |  |
| 221212 | BEMM RIVER @ PRINCES HIGHWAY | 667855 | 5836059 | 30/04/1975 |  |
| 221213 | GOOLENGOOK RIVER @ D/S OF ARTE RIVER JUNCTION | 663700 | 5846900 | 1/05/1978 | 18/12/1986 |
| 221214 | CANN RIVER @ D/S OF CANN RIVER | 688655 | 5836283 | 26/07/1979 |  |
| 221216 | GENOA RIVER @ D/S OF BIG FLAT CREEK JUNCTION | 722309 | 5858210 | 23/05/1967 | 10/12/1970 |
| 221217 | GENOA RIVER @ GIPSY POINT (WOOD'S JETTY) | 736900 | 5848600 | 8/07/1992 | 3/11/1998 |
| 221218 | BETKA RIVER @ MINERS TRACK | 732688 | 5835801 | 16/01/1996 |  |
| 221222 | HENSLEIGH CREEK U/S COMBIENBAR | 679158 | 5863650 | 18/05/2006 | 17/11/2010 |
| 221223 | COMBIENBAR RIVER @ TIGER SNAKE CK | 680158 | 5864336 | 18/05/2006 | 17/11/2010 |
| 221224 | CANN RIVER U/S CANN RIVER OFFTAKE | 690544 | 5843171 | 11/03/2009 |  |
| 221225 | BEMM RIVER U/S OF PUMPHOUSE | 671929 | 5822377 |  |  |
| 221800 | RAINGAUGE (GENOA RIVER) @ THE GORGE | 722715 | 5855916 | 22/08/1972 |  |

Source data: Victorian flood database

Table 12	La Trobe Basin streamflow site details

| Site ID | Site name | Easting | Northing | Start date | Cease date |
| --- | --- | --- | --- | --- | --- |
| 226005 | LATROBE RIVER @ THOMS BRIDGE | 448418 | 5775480 | 24/05/1960 |  |
| 226006 | TYERS RIVER @ BOOLA | 448568.9 | 5781803 | 16/04/1958 |  |
| 226007 | TYERS RIVER @ BROWNS | 443476 | 5788600 | 17/08/1961 |  |
| 226008 | TYERS RIVER WEST BRANCH @ MORGANS MILL | 439256 | 5800120 | 27/04/1960 |  |
| 226012 | TANJIL RIVER EAST BRANCH @ TANJIL BREN | 429307.7 | 5812543 | 18/01/1961 | 11/08/1971 |
| 226016 | WATERHOLE CREEK @ MORWELL | 449045.2 | 5767997 | 22/08/1961 | 15/02/1982 |
| 226017 | JACOBS CREEK @ O'TOOLES | 446350 | 5788246 | 16/04/1962 |  |
| 226021 | NARRACAN CREEK @ MOE | 435969 | 5775832 | 26/06/1996 |  |
| 226023 | TRARALGON CREEK @ TRARALGON | 460064 | 5772541 | 18/09/1998 |  |
| 226027 | LA TROBE RIVER AT SWING BRIDGE | 511200 | 5778084 | 7/05/2010 |  |
| 226028 | TYERS R @ PUMP HOUSE | 451503 | 5778560 | 5/04/2007 |  |
| 226033 | LA TROBE RIVER @ SCARNES BRIDGE | 460763 | 5777041 | 20/12/1996 |  |
| 226039 | BILLY CREEK @ U/S OF OFFTAKE WEIR | 446116.2 | 5755156 | 2/03/1967 | 8/05/1968 |
| 226041 | LAKE WELLINGTON @ BULL BAY | 532876 | 5780415 | 25/02/1991 |  |
| 226202 | TYERS RIVER @ GOULD | 444451.2 | 5785660 | 19/05/1926 | 28/02/1941 |
| 226204 | LATROBE RIVER @ WILLOW GROVE | 426308 | 5784017 | 17/10/1966 |  |
| 226205 | LATROBE RIVER @ NOOJEE | 414035 | 5804037 | 1/05/1996 |  |
| 226206 | LATROBE RIVER @ APM MARYVALE | 452119.2 | 5774427 | 2/01/1946 | 3/05/1977 |
| 226209 | MOE RIVER @ DARNUM | 412633 | 5770880 | 19/10/1960 |  |
| 226216 | TANJIL RIVER @ TANJIL SOUTH | 433560 | 5783395 | 5/04/1955 |  |
| 226217 | LATROBE RIVER @ HAWTHORN BRIDGE | 419490.7 | 5796548 | 2/06/1955 | 9/01/1989 |
| 226218 | NARRACAN CREEK @ THORPDALE | 428843 | 5763687 | 22/06/1955 |  |
| 226219 | TOORONGA RIVER @ NOOJEE | 415838.9 | 5810012 | 1/10/1924 | 1/07/1933 |
| 226220 | LOCH RIVER @ NOOJEE | 412644 | 5808497 | 25/04/1978 |  |
| 226222 | LATROBE RIVER @ NEAR NOOJEE (US ADA R JUNCT.) | 402650 | 5806658 | 10/05/1971 |  |
| 226223 | LATROBE RIVER @ NEERIM EAST | 414328.5 | 5800194 | 16/03/1972 | 8/01/1979 |
| 226224 | LATROBE RIVER @ ROSEDALE (ANABRANCH) | 481555 | 5777709 | 1/12/1936 |  |
| 226226 | TANJIL RIVER @ TANJIL JUNCTION | 429155 | 5796100 | 25/05/1960 |  |
| 226227 | LATROBE RIVER @ KILMANY SOUTH | 492900 | 5778721 | 16/12/1976 |  |
| 226228 | LATROBE RIVER @ ROSEDALE (MAIN STREAM) | 481748 | 5778351 | 1/12/1936 |  |
| 226229 | TANJIL RIVER @ D/S OF BLUE ROCK DAM | 432764.2 | 5784647 | 11/05/1979 | 28/02/1982 |
| 226232 | TANJIL RIVER @ MOE-WALHALLA ROAD BRIDGE | 0 | 0 | 17/05/1990 | 5/06/1990 |
| 226233 | TANJIL RIVER @ U/S OF SERPENTINE CREEK | 435276.4 | 5783133 | 16/07/1992 | 12/12/2006 |
| 226235 | TYERS RIVER @ TYERS JUNCTION | 441506 | 5798197 | 20/08/2003 |  |
| 226244 | TYERS R EAST BRANCH @ CORRINGAL SCOUT CAMP | 441496 | 5798505 | 27/10/2004 | 18/09/2008 |
| 226245 | TYERS R WEST BRANCH U/S SOUTH FACE RD | 436327 | 5807354 | 1/11/2004 | 24/02/2009 |
| 226246 | TYERS R WEST BRANCH D/S SOUTH FACE RD | 436123 | 5807237 | 28/10/2004 | 24/02/2009 |
| 226247 | HOPE CK U/S SOUTHFACE ROAD BRIDGE | 434283 | 5810028 | 26/03/2007 | 24/02/2009 |
| 226248 | HOPE CK D/S SOUTHFACE ROAD BRIDGE | 434267 | 5809990 | 26/03/2007 | 24/02/2009 |
| 226400 | LATROBE RIVER @ YALLOURN | 437171.9 | 5776866 | 1/11/1923 |  |
| 226402 | MOE DRAIN @ TRAFALGAR EAST | 431070 | 5774351 | 7/06/1957 |  |
| 226407 | MORWELL RIVER @ BOOLARRA | 439351 | 5748787 | 9/05/1972 |  |
| 226408 | MORWELL R @ YALLOURN | 444702 | 5770110 | 21/04/2004 |  |
| 226410 | TRARALGON CREEK @ KOORNALLA | 459050 | 5758231 | 9/07/1953 |  |
| 226415 | TRARALGON CREEK @ TRARALGON SOUTH (JONES RD) | 459805 | 5764127 | 27/05/1976 |  |
| 226602 | AREA 2 SITE1 @ DOWDS MORASS NTH | 515887 | 5777885 | 17/06/2003 | 24/09/2009 |
| 226603 | AREA 4 SITE 2 @ DOWDS MORASS STH | 516572 | 5776987 | 17/06/2003 |  |
| 226605 | LAKE VICTORIA AT MCLENNAN'S STRAIT | 539492.4 | 5787708 | 17/11/2010 |  |
| 226606 | LAKE VICTORIA AT LOCH SPORT | 551177 | 5789006 | 26/11/2010 |  |
| 226607 | MCMILLAN STRAIT AT PAYNESVILLE | 564019 | 5803744 | 21/10/2010 |  |
| 226608 | LAKE KING AT METUNG | 576509 | 5807128 | 17/11/2010 |  |
| 226609 | CUNNINGHAM ARM AT BULLOCK ISLAND | 585591 | 5806582 | 19/10/2010 |  |
| 226700 | LOY YANG OUTFALL TO TRARALGON CREEK | 0 | 0 | 17/12/1997 | 7/02/2000 |
| 226702 | MORWELL GROSS POLLUTION TRAP D/S CIRCULAR PIPE | 447900 | 5766700 | 1/06/1998 | 7/10/1998 |
| 226801 | RAINGAUGE (REPRESENTATIVE BASIN) @ LATROBE NO. 2 | 399410 | 5810382 | 19/01/1982 | 17/08/1988 |
| 226802 | RAINGAUGE (REPRESENTATIVE BASIN) @ LATROBE NO. 3 | 398477.4 | 5802788 | 19/01/1982 | 6/03/1986 |
| 226804 | RAINGAUGE (SITE 3) @ WALHALLA ST - NEWBOROUGH | 438000 | 5772500 | 6/12/1994 | 16/10/1995 |
| 226805 | RAINGAUGE (SITE 4) @ MOE INDOOR RECREATION CENTRE | 437018.3 | 5772948 | 7/12/1994 | 16/10/1995 |
| 226814 | RAIN GAUGE @ MT TASSIE | 461784 | 5750077 | 3/12/1998 |  |
| 226815 | RAIN GAUGE (TRARALGON CK) @ TRARALGON - EPA YARD | 458982 | 5772514 | 25/05/1999 |  |
| 226816 | RAIN GAUGE @ MT HOOGHLY | 453401 | 5750509 | 29/09/1999 | 15/05/2008 |
| 226817 | RAIN GAUGE @ LE ROY QUARRY | 457656 | 5750650 | 30/09/1999 | 31/10/2009 |
| 226818 | RAIN GAUGE @ BALOOK | 459952 | 5746341 | 10/06/1999 |  |
| 226819 | RAIN GAUGE @ CALIGNEE NORTH | 462063 | 5758780 | 25/05/1999 |  |
| 226825 | RAINGAUGE @ MOE SOUTH | 434231.7 | 5769649 | 6/10/2009 |  |
| 226826 | RAINGAUGE AT YARRAGON SOUTH | 420511 | 5763367 | 21/10/2009 |  |
| 226827 | RAINGAUGE @ THORPDALE PEAK | 431275 | 5760002 | 17/09/2009 |  |
| 226828 | RAINGAUGE @ JEERALANG SOUTH HALLAMS RD | 451008.4 | 5752916 | 6/10/2009 | 16/11/2011 |
| 226829 | RAINGAUGE AT JERRALANG, DOBBINS ROAD | 450883.6 | 5754700 | 29/01/2013 |  |

Source data: Victorian flood database

Table 13	Mitchell Basin streamflow site details

| Site ID | Site name | Easting | Northing | Start date | Cease date |
| --- | --- | --- | --- | --- | --- |
| 224200 | MITCHELL RIVER @ BAIRNSDALE | 552241 | 5813267 |  |  |
| 224201 | WONNANGATTA RIVER @ WATERFORD | 514714 | 5850745 | 15/07/1976 |  |
| 224203 | MITCHELL RIVER @ GLENALADALE | 533010 | 5820334 | 22/02/1991 |  |
| 224205 | DARGO RIVER @ DARGO (UPPER SITE) | 524129.9 | 5861628 | 1/06/1953 | 26/06/1974 |
| 224206 | WONNANGATTA RIVER @ CROOKED RIVER | 507891 | 5859903 | 13/04/1977 |  |
| 224207 | WONGUNGARRA RIVER @ GUYS | 508800 | 5862200 | 7/05/1953 | 10/01/1989 |
| 224208 | MITCHELL RIVER @ HOWITT DAM SITE | 532433.8 | 5827894 | 14/02/1969 | 22/12/1975 |
| 224209 | COBBANNAH CREEK @ NEAR BAIRNSDALE | 530900 | 5831600 | 15/05/1970 | 1/07/1987 |
| 224210 | WONNANGATTA RIVER @ KINGSWELL BRIDGE (HAWKHURST) | 510185.8 | 5859582 | 12/02/1970 | 25/03/1981 |
| 224213 | DARGO RIVER @ LOWER DARGO ROAD | 523752 | 5850152 | 24/05/1973 |  |
| 224214 | WENTWORTH RIVER @ TABBERABBERA | 534808 | 5850099 | 4/07/1974 |  |
| 224215 | MITCHELL RIVER @ ANGUSVALE (TABBERABBERA) | 529928 | 5838289 | 28/05/1975 | 30/09/2008 |
| 224216 | CLIFTON CREEK @ WY YUNG | 554294.5 | 5816640 | 28/01/1977 | 4/01/1979 |
| 224217 | MITCHELL RIVER @ ROSEHILL | 550366 | 5814542 | 2/04/2003 |  |
| 224220 | RAIN GAUGE (BOGGY CREEK) @ BULLUMWAAL | 548565.5 | 5830013 | 26/02/1991 |  |
| 224222 | MITCHELL RIVER U/S GLENALADALE PUMPHOUSE | 532443 | 5823351 | 18/12/2009 |  |

Source data: Victorian flood database

Table 14	Snowy River Basin streamflow site details

| Site ID | Site name | Easting | Northing | Start date | Cease date |
| --- | --- | --- | --- | --- | --- |
| 222200 | SNOWY RIVER @ JARRAHMOND | 620071 | 5830901 | 30/06/1971 |  |
| 222201 | SNOWY RIVER @ ORBOST | 627898 | 5825644 | 27/03/1997 |  |
| 222202 | BRODRIBB RIVER @ SARDINE CREEK | 636752 | 5847240 | 16/06/1998 |  |
| 222203 | SNOWY RIVER @ MARLO JETTY | 634504.8 | 5815385 | 25/06/1934 | 18/01/1998 |
| 222204 | SNOWY RIVER @ BETE BELONG | 623487.8 | 5824590 | 21/10/1938 | 26/07/1949 |
| 222205 | SNOWY RIVER @ NEWMERELLA-LOCHEND ROAD | 632611.9 | 5821910 | 1/03/1960 | 27/01/1983 |
| 222206 | BUCHAN RIVER @ BUCHAN | 603711 | 5849459 | 27/03/1926 |  |
| 222207 | BUCHAN RIVER @ MURRINDAL | 609403 | 5848401 | 15/02/1951 | 21/11/1972 |
| 222209 | SNOWY RIVER @ MCKILLOP BRIDGE | 625612 | 5894902 | 5/04/1967 |  |
| 222210 | DEDDICK RIVER @ DEDDICK (CASEYS) | 626577.7 | 5894694 | 5/04/1973 |  |
| 222212 | SNOWY RIVER @ BASIN CREEK NEAR BUCHAN | 613458.7 | 5850738 | 4/05/1932 | 10/01/1934 |
| 222213 | SUGGAN BUGGAN RIVER @ SUGGAN BUGGAN | 618048 | 5909278 | 25/06/1974 |  |
| 222214 | ROCKY RIVER @ NEAR ORBOST | 645109.5 | 5833373 | 8/12/1969 | 17/03/1978 |
| 222216 | MURRINDAL RIVER @ BASIN ROAD (BUCHAN) | 608500 | 5849900 | 4/02/1976 | 30/06/1987 |
| 222217 | RODGER RIVER @ JACKSONS CROSSING | 620346 | 5858744 | 3/06/1976 |  |
| 222218 | LITTLE RIVER @ WULGULMERANG | 616400 | 5897200 | 21/04/1977 | 18/07/1984 |
| 222219 | SNOWY RIVER @ D/S OF BASIN CREEK | 612580 | 5848984 | 12/12/1978 |  |
| 222221 | BUCHAN RIVER @ EGW OFFTAKE | 603874 | 5852271 | 14/05/2009 |  |
| 222222 | ROCKY RIVER U/S OT THE WEIR | 645938 | 5834053 | 11/03/2009 |  |
| 222400 | MOYANGUL RIVER @ LOOKOUT NEAR TIN MINE | 613203.5 | 5941898 | 14/03/1955 | 14/11/1963 |
| 222401 | INGEEGOODBEE RIVER @ D/S OF TIN MINE HUTS | 613098.4 | 5933947 | 19/05/1955 | 14/11/1963 |
| 222403 | BUCHAN RIVER @ GLENMORE | 600499.4 | 5879844 | 27/01/1955 | 22/07/1969 |
| 222404 | MELLICK MUNJIE CREEK @ GILLINGALL | 598595.5 | 5877355 | 5/07/1955 | 22/07/1969 |

Source data: Victorian flood database

Table 15	South Gippsland Basin streamflow site details

| Site ID | Site name | Easting | Northing | Start date | Cease date |
| --- | --- | --- | --- | --- | --- |
| 227001 | MERRIMAN CREEK @ SEASPRAY | 514294.7 | 5754491 | 29/09/1966 | 17/06/1971 |
| 227200 | TARRA RIVER @ YARRAM | 471649 | 5734286 | 18/02/1976 |  |
| 227201 | BRUTHEN CREEK @ WOODSIDE | 488800 | 5735750 | 1/03/1946 | 30/09/1960 |
| 227202 | TARWIN RIVER @ MEENIYAN | 412165 | 5729150 | 22/06/1955 |  |
| 227203 | FRANKLIN RIVER @ HENWOODS BRIDGE | 436475.8 | 5725294 | 3/12/1946 | 8/01/1985 |
| 227205 | MERRIMAN CREEK @ CALIGNEE SOUTH | 469833 | 5755048 | 13/12/1965 |  |
| 227210 | BRUTHEN CREEK @ CARRAJUNG LOWER | 477651 | 5750309 | 7/08/1952 |  |
| 227211 | AGNES RIVER @ TOORA | 445387 | 5722973 | 10/01/1957 |  |
| 227213 | JACK RIVER @ JACK RIVER | 459739 | 5735728 | 1/12/1962 |  |
| 227216 | ALBERT RIVER @ HIAWATHA (BELOW FALLS) | 453863.5 | 5735534 | 25/06/1964 | 23/02/1989 |
| 227217 | LILLYPILLY CREEK @ STAIRCASE | 442292 | 5680223 | 2/09/1965 | 15/01/1974 |
| 227219 | BASS RIVER @ LOCH | 388609 | 5753722 | 1/04/1966 |  |
| 227220 | GREIG CREEK @ MUMFORDS | 473100 | 5743600 | 6/05/1968 | 2/12/1998 |
| 227221 | BODMAN CREEK @ BRIDGES | 476725.7 | 5746056 | 7/05/1968 | 20/10/1978 |
| 227222 | SPRING CREEK @ BOWDENS | 475282.4 | 5742353 | 20/05/1968 | 13/06/1975 |
| 227223 | MACKS CREEK @ RICHARDS | 467870.1 | 5741587 | 24/04/1968 | 15/06/1987 |
| 227224 | WOMERAH CREEK @ TARR VALLEY ROAD | 462201 | 5741377 | 29/04/1968 | 20/12/1982 |
| 227225 | TARRA RIVER @ FISCHERS | 461190 | 5741983 | 24/04/1968 |  |
| 227226 | TARWIN RIVER EAST BRANCH @ DUMBALK NORTH | 426838 | 5738399 | 8/01/1969 |  |
| 227227 | WILKUR CREEK @ LEONGATHA | 408948 | 5750056 | 31/07/1970 |  |
| 227228 | TARWIN RIVER EAST BRANCH @ MIRBOO | 433155.3 | 5737489 | 30/04/1971 | 18/06/1987 |
| 227231 | BASS RIVER @ GLEN FORBES SOUTH | 370463 | 5741320 | 30/03/1973 |  |
| 227232 | LANCE CREEK U/S LANCE CREEK RESERVOIR | 383783 | 5738738 | 31/03/2008 |  |
| 227234 | SPRING CREEK @ BEAUMONT | 475276.8 | 5744202 | 3/04/1975 | 15/12/1982 |
| 227235 | MIDDLE CREEK @ TALL TIMBERS | 460742.6 | 5742295 | 27/04/1978 | 18/01/1989 |
| 227236 | POWLETT RIVER @ D/S FOSTER CREEK JUNCTION | 387508 | 5731353 | 18/05/1979 |  |
| 227237 | FRANKLIN RIVER @ TOORA | 439771 | 5724088 | 7/04/1983 |  |
| 227238 | FOSTER CREEK @ DAM SITE | 387050.1 | 5739747 | 8/06/1979 | 17/01/1989 |
| 227239 | MERRIMAN CREEK @ STRADBROKE WEST | 492658 | 5764652 | 18/11/1983 |  |
| 227240 | MERRIMAN CREEK @ PROSPECT ROAD SEASPRAY | 514320 | 5754179 | 26/08/1983 |  |
| 227242 | MERRIMAN CREEK @ SEASPRAY TOWNSHIP | 516203 | 5752529 | 6/03/1990 |  |
| 227243 | BRUTHEN CREEK @ D/S REEDY CREEK | 484778 | 5747810 | 13/05/1992 |  |
| 227244 | DEEP CREEK @ FOSTER | 432312 | 5725707 | 29/04/1993 |  |
| 227245 | LITTLE BASS RIVER @ POOWONG U/S LITTLE BASS RES. | 39050 | 5753576 | 18/05/1999 |  |
| 227246 | COALITION CREEK | 396827 | 5748087 | 8/06/2004 |  |
| 227248 | BELLVIEW CREEK U/S BELLVIEW RESERVOIR | 396367 | 5749105 | 18/05/1999 |  |
| 227249 | RUBY CREEK @ ARAWATA | 402379 | 5748848 | 23/07/2008 |  |
| 227251 | TARRA RIVER @ TARRA WEIR OFFTAKE | 463301 | 5741305 | 27/10/2004 |  |
| 227264 | COALITION CREEK @ LEONGATHA (SPENCERS ROAD BRIDGE) | 409001 | 5743754 | 21/10/2008 |  |
| 227265 | GOLDEN CREEK @ BLACK SWAMP ROAD | 428266 | 5711026 | 15/02/2008 | 18/11/2008 |
| 227266 | TARWIN RIVER @ KOONWARRA | 408996 | 5735402 | 22/09/2008 |  |
| 227270 | FOSTER CREEK AT KORUMBURRA | 394617.6 | 5745540 | 13/10/2011 |  |

Source data: Victorian flood database

Table 16	Tambo Basin streamflow site details

| Site ID | Site name | Easting | Northing | Start date | Cease date |
| --- | --- | --- | --- | --- | --- |
| 223202 | TAMBO RIVER @ SWIFTS CREEK | 564619 | 5875047 | 8/03/1977 |  |
| 223204 | NICHOLSON RIVER @ DEPTFORD | 561543 | 5839104 | 12/05/1961 |  |
| 223205 | TAMBO RIVER @ D/S OF RAMROD CREEK | 576710 | 5830147 | 9/06/1965 |  |
| 223206 | TAMBO RIVER @ BINDI | 571424.2 | 5896694 | 8/08/1957 | 19/12/1974 |
| 223207 | TIMBARRA RIVER @ TIMBARRA | 592900 | 5869900 | 9/09/1957 | 4/06/1973 |
| 223208 | TAMBO RIVER @ BINDI (NEAR JUNCTION CREEK) | 568710 | 5887104 | 21/03/1974 | 14/07/2003 |
| 223209 | TAMBO RIVER @ BATTENS LANDING | 574796.1 | 5820758 | 26/01/1977 |  |
| 223210 | NICHOLSON RIVER @ SARSFIELD | 562646 | 5823050 | 21/09/1977 |  |
| 223212 | TIMBARRA RIVER @ D/S OF WILKINSON CREEK | 594066 | 5855069 | 6/05/1982 |  |
| 223213 | TAMBO RIVER @ D/S OF DUGGAN CREEK | 578509 | 5904321 | 16/09/1987 |  |
| 223214 | TAMBO RIVER @ U/S OF SMITH CREEK | 582588 | 5909736 | 2/03/1989 |  |
| 223215 | HAUNTED STREAM @ HELLS GATE | 573015 | 5851389 | 8/02/1990 |  |
| 223216 | TAMBO RIVER U/S SWIFTS CK OFFTAKE | 563729 | 5877748 | 20/05/2009 |  |
| 223217 | NICHOLSON RIVER AT PUMP HOUSE | 564088 | 5821594 | 20/01/2011 |  |
| 223402 | TIMBARRA RIVER @ NUNNIONG PLAINS | 587028.3 | 5888209 | 22/06/1955 | 16/02/1960 |
| 223403 | TAMBO RIVER @ NUNNIONG PLAINS | 583943.9 | 5892463 | 21/06/1955 | 16/02/1960 |
| 223800 | RAINGAUGE (TAMBO RIVER) @ MOUNT ELIZABETH | 582300 | 5850700 | 15/01/1985 | 20/10/2004 |
| 223801 | RAIN GAUGE (TAMBO RIVER) @ MT ELIZABETH HELIPAD | 581909 | 5850809 | 20/10/2004 | 19/01/2011 |
| 223802 | RAINGAUGE AT MOUNT ELIZABETH SOMMERVILLE TRACK | 580733 | 5851999 | 19/01/2011 |  |

Source data: Victorian flood database

Table 17	Thompson Basin streamflow site details

| Site ID | Site name | Easting | Northing | Start date | Cease date |
| --- | --- | --- | --- | --- | --- |
| 225019 | NORTH CASCADE CREEK @ THOMSON VALLEY ROAD | 441836.2 | 5815578 | 11/01/1962 | 6/06/1974 |
| 225105 | THOMSON RIVER @ NEWLAND ROAD | 427654.1 | 5819830 | 30/03/1954 | 1/05/1984 |
| 225114 | THOMSON RIVER @ D/S WHITELAWS CREEK | 436711 | 5825795 | 27/03/1987 |  |
| 225200 | THOMSON RIVER @ HEYFIELD | 480693 | 5795763 | 17/01/1991 |  |
| 225201 | AVON RIVER @ STRATFORD | 506676 | 5797653 | 1/11/1976 |  |
| 225204 | MACALISTER RIVER @ LAKE GLENMAGGIE (TAIL GAUGE) | 482885 | 5805021 | 28/09/1966 |  |
| 225207 | THOMSON RIVER @ WALHALLA | 449064.3 | 5798591 | 9/03/1950 | 22/05/1952 |
| 225209 | MACALISTER RIVER @ LICOLA | 466762 | 5835198 | 1/08/1952 |  |
| 225210 | THOMSON RIVER @ THE NARROWS | 447551 | 5805990 | 9/04/1957 |  |
| 225212 | THOMSON RIVER @ WANDOCKA | 489554 | 5792978 | 1/03/1977 |  |
| 225213 | ABERFELDY RIVER @ BEARDMORE | 450135 | 5810238 | 27/06/1963 |  |
| 225216 | JORDAN RIVER @ ABERFELDY | 439747.8 | 5829099 | 4/10/1971 | 18/07/1972 |
| 225217 | BARKLY RIVER @ GLENCAIRN | 461700 | 5842800 | 12/05/1966 | 4/01/1989 |
| 225218 | FREESTONE CREEK @ BRIAGALONG | 508366.4 | 5815233 | 14/07/1975 |  |
| 225219 | MACALISTER RIVER @ GLENCAIRN | 461689 | 5847757 | 7/04/1967 |  |
| 225221 | MACALISTER RIVER @ STRINGYBARK CREEK | 470738 | 5819709 | 18/03/1968 |  |
| 225222 | GLENMAGGIE CREEK @ SEATON (AUBREYS) | 471117.4 | 5803310 | 10/03/1970 | 18/12/1975 |
| 225223 | VALENCIA CREEK @ GILLIO ROAD | 499321 | 5822633 | 26/03/1991 |  |
| 225224 | AVON RIVER @ THE CHANNEL | 489868 | 5816150 | 12/07/1972 |  |
| 225225 | MACALISTER RIVER @ LAKE GLENMAGGIE (HEAD GAUGE) | 482418.6 | 5804322 | 26/01/1925 |  |
| 225228 | THOMSON RIVER @ COWWARR TIMBER WEIR H.G. | 469855.4 | 5794300 | 1/01/1958 |  |
| 225230 | GLENMAGGIE CREEK @ THE GORGE | 469772 | 5803687 | 2/05/1975 |  |
| 225231 | THOMSON RIVER @ U/S OF COWWARR WEIR | 467065 | 5796878 | 1/04/1976 |  |
| 225232 | THOMSON RIVER @ BUNDALAGUAH | 499308 | 5789148 | 3/11/1976 |  |
| 225233 | PERRY RIVER @ PERRY BRIDGE | 523288.7 | 5793398 | 24/12/1976 | 19/11/1982 |
| 225234 | AVON RIVER @ CLYDEBANK (CHINN'S BRIDGE) | 515238 | 5791515 | 29/06/2004 |  |
| 225236 | RAINBOW CREEK @ HEYFIELD | 480686 | 5793911 | 30/04/1992 |  |
| 225247 | MACALISTER RIVER @ RIVERSLEA | 498044.5 | 5791336 | 11/01/2001 |  |
| 225248 | BOGGY CREEK @ CORNWALLS ROAD | 492831 | 5793650 | 29/08/2008 |  |
| 225255 | AVON RIVER U/S VALENCIA CK JUNCTION | 498324 | 5813965 | 30/06/2004 |  |
| 225256 | MACALISTER R D/S MAFFRA (SMITHS BR.) | 498377 | 5793730 | 26/10/2005 |  |
| 225600 | LAKE WELLINGTON @ SALE | 519212 | 5781296 | 1/12/1973 | 1/07/1977 |
| 225703 | THOMSON RIVER DIVERSION CHANNEL @ COWWARR WEIR | 470010 | 5794250 | 5/02/2001 | 19/07/2007 |
| 225711 | LAKE WELLINGTON DRAIN @ 5 MILES 32 CHAIN MEASURING WEIR | 512208.2 | 5787288 | 12/11/1976 | 3/05/2001 |
| 225715 | CENTRAL GIPPSLAND DRAIN 3 @ NAMBROK (RD 1066M) | 488590 | 5786295 | 24/02/1980 | 15/04/2005 |
| 225716 | CENTRAL GIPPSLAND 1/2 DRAIN 3 @ DROP STRUCTURE (RD500) | 491500 | 5784700 | 24/02/1980 | 4/02/2004 |
| 225717 | CENTRAL GIPPSLAND 2/3 DRAIN @ US OF DRAIN 3 (RD 500M) | 491102 | 5783737 | 24/02/1980 | 15/04/2005 |
| 225721 | CENTRAL GIPPSLAND DRAIN NO 3/3 U/S OUTFALL | 487050 | 5788350 | 29/05/2000 | 13/02/2004 |
| 225722 | CENTRAL GIPPSLAND DRAIN NO 3 D/S NO3 O/FALL | 487100 | 5788300 | 29/05/2000 | 13/02/2004 |
| 225723 | CENTRAL GIPPSLAND DRAIN NO 1/3 @ D/S 11/4/1 JUNCTION | 488200 | 5784600 | 29/05/2000 | 13/02/2004 |
| 225724 | CHANNEL O/FALL 11/4/1 U/S JUNCTION CG DR 1/3 | 488150 | 5784500 | 29/05/2000 | 15/04/2005 |
| 225725 | CENTRAL GIPPSLAND DRAIN NO 3 D/S O/FALL 11/1 | 490400 | 5783500 | 26/05/2000 | 15/04/2005 |
| 225726 | CENTRAL GIPPSLAND DRAIN NO 3 D/S RAILWAY LINE | 491000 | 5781800 | 29/05/2000 | 15/04/2005 |
| 225727 | CENTRAL GIPPSLAND DRAIN NO 2/3 @ SOLDIERS RD | 490100 | 5785600 | 26/05/2000 | 13/02/2004 |
| 225737 | CENTRAL GIPPSLAND DRAIN NO 8/2 @ DENISON ROAD | 483978 | 5789779 | 28/05/2002 | 15/04/2005 |
| 225738 | CENTRAL GIPPSLAND DRAIN NO 2 @ SALE-TOONGABBIE RD | 487500 | 5782800 | 28/05/2002 | 15/04/2005 |
| 225739 | CENTRAL GIPPSLAND DRAIN NO 2/2 @ DESSENTS | 487311 | 5782836 | 28/05/2002 | 15/04/2005 |
| 225740 | CENTRAL GIPPSLAND DRAIN NO 2 U/S TINAMBA-ROSEDALE RD | 487800 | 5781900 | 16/05/2002 | 15/04/2005 |
| 225741 | CENTRAL GIPPSLAND DRAIN NO 2 U/S NAMBROK ROAD | 485335 | 5786592 | 16/05/2002 | 15/04/2005 |
| 225742 | CENTRAL GIPPSLAND DRAIN NO 2 U/S SALE-COWWARR ROAD | 484488 | 5788744 | 28/05/2002 | 15/04/2005 |
| 225743 | CENTRAL GIPPSLAND DRAIN NO 6/2 U/S DENISON ROAD | 483670 | 5788400 | 28/05/2002 | 15/04/2005 |
| 225747 | NOBLES O/FALL @ VALENCIA CK | 498563 | 5813297 | 24/02/2004 | 1/07/2005 |
| 225748 | TINAMBA MAIN O/FALL @ MENBURN PARK | 498214 | 5813767 | 25/02/2004 | 27/06/2005 |
| 225801 | RAIN GAUGE (MACALISTER RIVER) @ MURDERERS HILL | 460964 | 5810725 | 8/06/1970 |  |
| 225802 | RAIN GAUGE (MACALISTER RIVER) @ MOUNT TAMBORITHA | 472550 | 5853466 | 1/04/1986 |  |
| 225809 | RAIN GAUGE (AVON RIVER) @ MOUNT WELLINGTON | 487412 | 5850069 | 21/05/1997 |  |
| 225810 | RAIN GAUGE (AVON RIVER) @ REEVE KNOB | 500533.2 | 5846182 | 21/05/1997 |  |
| 225819 | RAINGAUGE @ MT USEFUL | 456353 | 5827872 | 27/05/2002 |  |
| 225823 | RAINGAUGE AT BLANKET HILL | 472533.6 | 5811892 | 26/07/2010 |  |
| 225824 | RAINGAUGE (MACALISTER RV) AT SNOWY RANGE | 0 | 0 | 22/02/2011 |  |
| 225825 | RAINGAUGE (MACALISTER RV) AT HIGH RIDGE | 0 | 0 |  |  |
| 225826 | RAINGAUGE (MACALISTER RV) AT MOUNT SUNDAY | 448995 | 5867264 | 6/04/2011 |  |

Source data: Victorian flood database

#### Statistical analysis and interpolation

Daily streamflow data was classified into unified six-class quality codes for each gauge (Viney et.al., 2011). The six unified quality categories are good, fair, poor, unverified, non-conforming and missing (Table 18). The categories are defined as follows:

- Good: data are an accurate representation of streamflow

- Fair: data are a moderately accurate representation of streamflow

- Poor: data are a poor representation of streamflow and may be unsuitable for some quantitative applications

- Unverified: data quality is not known

- Non-conforming: data are unsuitable for most applications requiring quantitative analysis, but may contain useful qualitative information

- Missing: data are missing or unusable.

The national streamflow quality code assigned to each of the Victorian quality codes for streamflow data are summarised in Table 19.

Table 18	Proposed national streamflow quality code scheme

| Code | Quality | Description |
| --- | --- | --- |
| G | Good | Data is an accurate representation of streamflow. |
| F | Fair | Data is a moderately accurate representation of streamflow |
| P | Poor | Data is a poor representation of streamflow and may be unsuitable for some quantitative applications |
| N | Non-conforming | Data is unsuitable for most applications requiring quantitative analysis, but may contain useful qualitative information. |
| U | Unknown | Data quality is not known. |
| M | Missing | Data is missing or unusable. |

Source data: Viney et al. (2011)

Table 19	National water quality code assigned to Victorian water monitoring codes

| Victorian code | Description | Proposed national code |
| --- | --- | --- |
| 1 | Good continuous records | G |
| 2 | Good quality edited data | G |
| 3 | Linear infill to first value in block (no data lost) | G |
| 4 | Temporary coded for currumbeene data | G |
| 5 | Drawdown - chart rating applies | G |
| 6 | Phased Rating Applicable-gradual changing of control and channel | G |
| 8 | Pool reading only | G |
| 9 | Pool dry - no data collected | G |
| 10 | Data transposed from recorder chart | G |
| 11 | Raw data used for operational purposes (not validataed) | U |
| 13 | Accurate derived data from multiple sources (level and flow) | G |
| 14 | Telemetry data not stored in archive | U |
| 15 | Minor editing | F |
| 20 | Edited to measurements | F |
| 26 | Daily read records (MW - Good periodic data) | F |
| 30 | Good Meas. - mult. point; 40 sec timing (Good acc. data - MW) | G |
| 31 | Good Meas. - Adeq. verts & obs; 40 sec timing (Good acc. data - MW) | G |
| 32 | Fair Measurement - Weighted mean gauge height; turbulent flow; flow angle extreme | F |
| 35 | Composite Measurement - Segments taken from several gaugings to create a composite | F |
| 36 | Measurement for sampling purposes. Rough estimate | P |
| 42 | Low velocity acoustic record affected by wind or gate opening | F |
| 50 | Medium editing >Q=15 (1996 on & MW) or HYMAN data import (pre1996) | P |
| 60 | Latrobe Valley Water Authority supplied data. | G |
| 65 | Other Authorities supplied data. Validation not supplied | U |
| 75 | Height correction applied | P |
| 76 | Reliable interpolation | P |
| 77 | Correlation with other station; same variable | P |
| 82 | Linear interpolation across gap in records. (<0.5 day) | P |
| 100 | Irregular data use with caution | U |
| 101 | Drawdown - normal rating does not apply. | U |
| 104 | Records estimated | P |
| 146 | Drawdown - no rating applies | F |
| 148 | Theoretical rating table applied | P |
| 149 | Raw data as received from Serco | U |
| 150 | Rating extrapolated due to insufficient gaugings (Unreliable data) | F |
| 151 | Data lost due to natural causes - NRE approved loss | M |
| 152 | Refer station file | U |
| 153 | Data not recorded. Probe out of water/below instrument threshold. | N |
| 155 | Data unreliable. Recording above instrument range. | N |
| 156 | Drawdown - normal rating does not apply | U |
| 160 | Backed-up by d/s influence. (Unreliable periodic data) | N |
| 161 | Debris Effecting Sensor. | N |
| 165 | Suspect or bad data supplied by other authority | P |
| 170 | Raw unedited data stored in archive (Unreliable. accumulated. data) | U |
| 180 | Equipment malfunction | M |
| 190 | Data unavailable station discontinued | M |
| 200 | Data available but not digitised | U |
| 201 | Data not recorded - no correlation available (Station not operational) | M |
| 235 | Poor Measurement - Not enough verticals or observations; not enough information | P |
| 236 | Suspect or Incomplete Measurement - Equipment suspect or giving problems causing measurement to be aborted. | M |
| 237 | Surface Velocities - Velocity measurements taken on surface only. | P |
| 238 | Control Leaking - Control leaking; either as noted on measurement or chart. | N |
| 239 | Backed up Flow Measurement - Measurement is affected by backup. | N |
| 240 | Not Coded Measurement - Measurement not coded as per HYDSYS System. | U |
| 250 | Rating table suspended | P |
| 254 | Rating table exceeded | P |
| 255 | No Data Exists | M |

Source data: Viney et al. (2011) and DEPI (2014)

The proportion of basin streamflow data assigned to each national streamflow quality code is summarised in Table 20. This summary presents the aggregated results derived from all gauges within each major basin and does not reflect the variations observed for each individual gauges as reported in Table 21.

Table 20	Proportion of basin stream flow data assigned to each national streamflow quality code

| Basin | Number of gauges | Good | Fair | Poor | Non-conforming | Unknown | Missing |
| --- | --- | --- | --- | --- | --- | --- | --- |
| East Gippsland | 19 | 78.5 | 4.2 | 4.5 | 0.0 | 1.8 | 11.0 |
| Latrobe | 38 | 64.4 | 10.4 | 5.5 | 0.1 | 8.6 | 11.0 |
| Mitchell | 15 | 73.2 | 6.6 | 8.2 | 0.0 | 0.6 | 11.4 |
| Snowy | 19 | 68.7 | 12.4 | 6.1 | 0.1 | 2.8 | 9.9 |
| South Gippsland | 41 | 83.6 | 6.1 | 6.8 | 0.3 | 0.9 | 2.4 |
| Tambo | 15 | 67.1 | 6.1 | 3.2 | 0.1 | 0.5 | 23.0 |
| Thomson | 50 | 76.7 | 6.3 | 4.7 | 0.1 | 4.0 | 8.3 |
| All above | 197 | 72.9 | 7.8 | 5.6 | 0.1 | 3.7 | 9.9 |

Table 21	Proportion of stream flow data assigned to each national streamflow quality code

| Gauge ID | Number of records | Good | Fair | Poor | Non-conforming | Unknown | Missing |
| --- | --- | --- | --- | --- | --- | --- | --- |
| East Gippsland Basin |  |  |  |  |  |  |  |
| 221001 | 7646 | 74.5 | 11.8 | 11.9 | 0.2 | 0.0 | 1.6 |
| 221201 | 20902 | 94.5 | 2.9 | 1.6 | 0.0 | 0.0 | 0.9 |
| 221202 | 19015 | 16.2 | 8.8 | 0.8 | 0.0 | 0.1 | 74.1 |
| 221203 | 2371 | 65.2 | 0.7 | 34.1 | 0.0 | 0.0 | 0.0 |
| 221204 | 6138 | 89.7 | 0.0 | 9.3 | 0.0 | 0.0 | 1.0 |
| 221205 | 3264 | 96.5 | 0.0 | 3.5 | 0.0 | 0.0 | 0.0 |
| 221206 | 1344 | 83.6 | 0.0 | 16.3 | 0.0 | 0.0 | 0.1 |
| 221207 | 16884 | 87.2 | 2.0 | 4.0 | 0.0 | 5.6 | 1.2 |
| 221208 | 12744 | 92.5 | 3.1 | 3.7 | 0.0 | 0.7 | 0.0 |
| 221209 | 15178 | 88.0 | 4.9 | 7.1 | 0.0 | 0.0 | 0.0 |
| 221210 | 15240 | 92.0 | 4.7 | 3.3 | 0.0 | 0.0 | 0.0 |
| 221211 | 14519 | 91.6 | 3.4 | 4.7 | 0.0 | 0.4 | 0.0 |
| 221212 | 14260 | 92.9 | 2.6 | 4.0 | 0.0 | 0.1 | 0.3 |
| 221213 | 3153 | 97.9 | 0.0 | 2.0 | 0.0 | 0.0 | 0.0 |
| 221214 | 3455 | 96.5 | 0.0 | 3.5 | 0.0 | 0.0 | 0.0 |
| 221216 | 1713 | 79.5 | 5.3 | 0.0 | 0.0 | 0.0 | 15.2 |
| 221218 | 6636 | 26.9 | 4.1 | 1.8 | 0.0 | 14.0 | 53.2 |
| 221224 | 1848 | 61.9 | 15.6 | 5.4 | 0.0 | 17.1 | 0.1 |
| 221225 | 1639 | 50.2 | 3.9 | 7.2 | 0.0 | 37.3 | 1.4 |
| Latrobe Basin |  |  |  |  |  |  |  |
| 226005 | 19131 | 68.1 | 3.4 | 2.6 | 0.0 | 25.9 | 0.0 |
| 226006 | 8590 | 35.9 | 0.0 | 0.8 | 0.0 | 0.0 | 63.3 |
| 226007 | 19270 | 64.3 | 1.3 | 6.1 | 0.0 | 12.7 | 15.6 |
| 226008 | 19712 | 33.5 | 2.5 | 0.7 | 0.0 | 1.1 | 62.2 |
| 226012 | 3848 | 95.9 | 0.0 | 0.3 | 0.0 | 0.0 | 3.9 |
| 226016 | 1755 | 98.9 | 0.0 | 0.0 | 0.0 | 0.0 | 1.1 |
| 226017 | 13852 | 34.4 | 1.7 | 5.6 | 0.0 | 0.5 | 57.8 |
| 226021 | 6552 | 41.9 | 21.2 | 11.0 | 0.0 | 25.9 | 0.0 |
| 226023 | 19583 | 65.7 | 19.2 | 4.3 | 0.3 | 0.1 | 10.4 |
| 226028 | 2092 | 90.8 | 3.8 | 0.0 | 0.0 | 4.8 | 0.6 |
| 226033 | 5988 | 86.8 | 7.6 | 4.2 | 0.0 | 1.3 | 0.0 |
| 226039 | 433 | 98.4 | 0.0 | 1.4 | 0.0 | 0.0 | 0.2 |
| 226202 | 514 | 90.9 | 0.0 | 8.9 | 0.0 | 0.0 | 0.2 |
| 226204 | 32738 | 38.9 | 45.6 | 8.4 | 0.0 | 7.2 | 0.0 |
| 226205 | 20909 | 79.2 | 6.1 | 2.9 | 0.0 | 11.0 | 0.7 |
| 226206 | 427 | 81.0 | 0.0 | 2.3 | 0.0 | 0.0 | 16.6 |
| 226209 | 23649 | 67.4 | 2.9 | 20.6 | 0.0 | 9.1 | 0.1 |
| 226216 | 21621 | 76.1 | 10.0 | 4.0 | 0.0 | 10.0 | 0.0 |
| 226217 | 12275 | 90.6 | 0.0 | 5.1 | 0.0 | 0.0 | 4.3 |
| 226218 | 21540 | 83.6 | 2.4 | 3.2 | 0.0 | 10.8 | 0.0 |
| 226219 | 6488 | 98.3 | 0.0 | 1.7 | 0.0 | 0.0 | 0.0 |
| 226220 | 20908 | 75.4 | 8.0 | 2.9 | 0.1 | 9.7 | 3.9 |
| 226222 | 15730 | 83.0 | 0.3 | 5.3 | 0.5 | 0.1 | 10.8 |
| 226223 | 1813 | 98.1 | 0.0 | 1.9 | 0.0 | 0.0 | 0.1 |
| 226224 | 28307 | 53.8 | 28.9 | 4.3 | 0.3 | 7.8 | 4.9 |
| 226226 | 19732 | 79.8 | 3.6 | 3.9 | 0.0 | 12.7 | 0.0 |
| 226227 | 13682 | 38.2 | 3.7 | 1.1 | 0.0 | 10.4 | 46.7 |
| 226228 | 28307 | 62.8 | 25.4 | 2.8 | 0.0 | 8.8 | 0.2 |
| 226229 | 1024 | 96.4 | 0.0 | 3.5 | 0.0 | 0.0 | 0.1 |
| 226233 | 5262 | 89.7 | 3.6 | 6.7 | 0.0 | 0.0 | 0.0 |
| 226235 | 4611 | 63.8 | 13.3 | 7.2 | 0.0 | 1.7 | 14.1 |
| 226400 | 2827 | 33.9 | 1.8 | 60.2 | 0.0 | 0.0 | 4.1 |
| 226402 | 20846 | 59.7 | 3.4 | 5.6 | 0.0 | 31.3 | 0.1 |
| 226407 | 20329 | 59.0 | 1.9 | 12.4 | 0.0 | 2.9 | 23.9 |
| 226408 | 10122 | 24.6 | 13.9 | 3.9 | 2.8 | 1.5 | 53.3 |
| 226410 | 18988 | 80.0 | 4.5 | 1.4 | 0.5 | 13.5 | 0.0 |
| 226415 | 6173 | 92.4 | 5.2 | 1.2 | 0.0 | 1.2 | 0.0 |
| 226700 | 929 | 96.8 | 3.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Mitchell River Basin |  |  |  |  |  |  |  |
| 224200 | 3995 | 23.0 | 9.8 | 57.1 | 0.0 | 0.0 | 10.1 |
| 224201 | 17701 | 60.6 | 2.4 | 36.1 | 0.0 | 0.9 | 0.0 |
| 224203 | 28047 | 74.6 | 20.3 | 4.9 | 0.0 | 0.3 | 0.0 |
| 224205 | 197 | 99.5 | 0.0 | 0.0 | 0.0 | 0.0 | 0.5 |
| 224206 | 19803 | 94.6 | 2.7 | 1.9 | 0.0 | 0.7 | 0.0 |
| 224207 | 10534 | 97.2 | 0.2 | 2.2 | 0.0 | 0.0 | 0.3 |
| 224208 | 378 | 84.7 | 0.0 | 15.1 | 0.0 | 0.0 | 0.3 |
| 224209 | 6256 | 99.6 | 0.0 | 0.4 | 0.0 | 0.0 | 0.0 |
| 224210 | 4059 | 96.9 | 0.0 | 3.0 | 0.0 | 0.0 | 0.0 |
| 224213 | 14979 | 93.0 | 3.6 | 3.1 | 0.1 | 0.2 | 0.0 |
| 224214 | 14527 | 94.7 | 0.8 | 3.3 | 0.0 | 0.8 | 0.5 |
| 224215 | 12179 | 35.5 | 1.4 | 0.8 | 0.0 | 0.0 | 62.3 |
| 224216 | 706 | 77.6 | 0.0 | 22.2 | 0.0 | 0.0 | 0.1 |
| 224217 | 13723 | 28.8 | 5.1 | 1.2 | 0.0 | 0.4 | 64.5 |
| 224222 | 1532 | 3.6 | 74.5 | 0.3 | 0.0 | 20.9 | 0.7 |
| Snowy River Basin |  |  |  |  |  |  |  |
| 222200 | 33647 | 43.6 | 17.0 | 1.6 | 0.0 | 0.3 | 37.5 |
| 222202 | 33623 | 50.9 | 29.5 | 2.1 | 0.0 | 0.0 | 17.5 |
| 222206 | 24324 | 42.9 | 25.8 | 30.6 | 0.0 | 0.7 | 0.0 |
| 222207 | 5703 | 87.3 | 0.6 | 2.9 | 0.0 | 1.9 | 7.2 |
| 222209 | 18416 | 90.6 | 2.2 | 3.6 | 0.1 | 0.2 | 3.3 |
| 222210 | 18309 | 88.7 | 2.2 | 2.9 | 0.1 | 1.7 | 4.4 |
| 222212 | 5477 | 85.5 | 0.0 | 14.5 | 0.0 | 0.0 | 0.0 |
| 222213 | 20746 | 72.5 | 8.4 | 4.5 | 0.6 | 14.0 | 0.0 |
| 222214 | 3021 | 99.2 | 0.0 | 0.7 | 0.0 | 0.0 | 0.0 |
| 222216 | 4164 | 96.1 | 0.0 | 3.8 | 0.0 | 0.0 | 0.0 |
| 222217 | 13706 | 96.7 | 1.5 | 1.7 | 0.0 | 0.0 | 0.0 |
| 222218 | 2645 | 95.6 | 0.0 | 4.3 | 0.0 | 0.0 | 0.0 |
| 222219 | 12936 | 92.7 | 3.9 | 2.5 | 0.0 | 0.9 | 0.0 |
| 222221 | 1790 | 0.0 | 18.7 | 1.8 | 0.0 | 68.0 | 11.6 |
| 222222 | 1638 | 36.3 | 14.7 | 2.8 | 0.0 | 39.5 | 6.7 |
| 222400 | 1331 | 98.5 | 0.7 | 0.0 | 0.0 | 0.8 | 0.1 |
| 222401 | 1331 | 99.5 | 0.2 | 0.0 | 0.0 | 0.2 | 0.1 |
| 222403 | 2421 | 92.6 | 5.4 | 0.0 | 0.0 | 1.9 | 0.0 |
| 222404 | 4145 | 91.7 | 2.6 | 0.0 | 0.0 | 2.6 | 3.1 |
| South Gippsland Basin |  |  |  |  |  |  |  |
| 227001 | 1722 | 99.1 | 0.0 | 0.8 | 0.0 | 0.0 | 0.1 |
| 227200 | 17982 | 91.9 | 0.8 | 7.0 | 0.0 | 0.3 | 0.0 |
| 227201 | 2131 | 0.0 | 96.0 | 4.0 | 0.0 | 0.0 | 0.0 |
| 227202 | 21523 | 89.7 | 4.9 | 4.5 | 0.0 | 0.8 | 0.0 |
| 227203 | 5411 | 96.9 | 0.0 | 3.1 | 0.0 | 0.0 | 0.0 |
| 227205 | 14364 | 87.3 | 3.7 | 4.4 | 1.1 | 0.3 | 3.1 |
| 227210 | 22539 | 76.3 | 21.1 | 2.4 | 0.0 | 0.1 | 0.1 |
| 227211 | 22544 | 95.5 | 1.4 | 2.8 | 0.0 | 0.2 | 0.1 |
| 227213 | 19596 | 89.6 | 6.8 | 2.9 | 0.0 | 0.7 | 0.0 |
| 227216 | 4748 | 97.2 | 0.0 | 2.8 | 0.0 | 0.0 | 0.0 |
| 227217 | 375 | 99.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.3 |
| 227219 | 17587 | 95.8 | 1.1 | 2.5 | 0.0 | 0.5 | 0.1 |
| 227220 | 9520 | 92.4 | 0.0 | 4.9 | 0.0 | 0.0 | 2.7 |
| 227221 | 3261 | 85.9 | 0.0 | 9.2 | 0.0 | 0.0 | 4.9 |
| 227222 | 1612 | 86.5 | 0.0 | 13.5 | 0.0 | 0.0 | 0.1 |
| 227223 | 4569 | 95.6 | 0.0 | 4.4 | 0.0 | 0.0 | 0.0 |
| 227224 | 3016 | 95.8 | 0.0 | 4.2 | 0.0 | 0.0 | 0.0 |
| 227225 | 16834 | 95.0 | 2.3 | 1.6 | 0.0 | 1.0 | 0.2 |
| 227226 | 16076 | 97.3 | 1.2 | 1.5 | 0.0 | 0.0 | 0.0 |
| 227227 | 14823 | 86.2 | 11.5 | 1.6 | 0.0 | 0.7 | 0.0 |
| 227228 | 5893 | 97.6 | 0.0 | 2.3 | 0.0 | 0.0 | 0.0 |
| 227231 | 15012 | 96.7 | 1.2 | 2.0 | 0.0 | 0.1 | 0.0 |
| 227232 | 1071 | 95.1 | 0.0 | 4.9 | 0.0 | 0.0 | 0.1 |
| 227234 | 2813 | 98.4 | 0.0 | 1.6 | 0.0 | 0.0 | 0.0 |
| 227235 | 3919 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 227236 | 12772 | 93.7 | 1.6 | 4.6 | 0.0 | 0.1 | 0.0 |
| 227237 | 12772 | 95.4 | 1.3 | 3.1 | 0.0 | 0.2 | 0.0 |
| 227238 | 3511 | 98.1 | 0.0 | 1.8 | 0.0 | 0.0 | 0.0 |
| 227239 | 11148 | 1.5 | 0.1 | 72.4 | 0.0 | 11.4 | 14.6 |
| 227240 | 11231 | 62.2 | 9.6 | 21.7 | 5.5 | 1.0 | 0.0 |
| 227243 | 7463 | 52.1 | 25.5 | 3.9 | 0.0 | 0.0 | 18.5 |
| 227244 | 7655 | 39.1 | 4.9 | 1.6 | 0.8 | 5.1 | 48.5 |
| 227245 | 2032 | 31.7 | 47.6 | 17.6 | 0.0 | 1.6 | 1.4 |
| 227246 | 1448 | 83.8 | 3.6 | 6.1 | 2.1 | 3.4 | 1.0 |
| 227248 | 2093 | 42.5 | 36.1 | 11.4 | 8.9 | 0.6 | 0.5 |
| 227249 | 2093 | 60.3 | 35.6 | 1.7 | 0.9 | 0.3 | 1.2 |
| 227251 | 3477 | 85.9 | 11.4 | 2.6 | 0.0 | 0.1 | 0.0 |
| 227264 | 2040 | 53.2 | 8.3 | 37.1 | 0.0 | 1.3 | 0.1 |
| 227265 | 245 | 24.9 | 75.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| 227266 | 2063 | 41.1 | 14.5 | 35.7 | 0.0 | 8.2 | 0.5 |
| 227270 | 952 | 67.5 | 2.9 | 15.9 | 0.0 | 3.6 | 10.1 |
| Tambo River Basin |  |  |  |  |  |  |  |
| 223202 | 24459 | 68.5 | 26.9 | 3.9 | 0.0 | 0.7 | 0.0 |
| 223204 | 19365 | 92.4 | 3.4 | 3.5 | 0.0 | 0.0 | 0.6 |
| 223205 | 17876 | 1.2 | 0.1 | 0.3 | 0.0 | 0.1 | 98.3 |
| 223206 | 6342 | 96.2 | 0.0 | 3.7 | 0.0 | 0.0 | 0.0 |
| 223207 | 9662 | 97.6 | 1.2 | 0.7 | 0.0 | 0.2 | 0.3 |
| 223208 | 10479 | 74.0 | 0.4 | 7.2 | 0.0 | 0.0 | 18.3 |
| 223209 | 13634 | 27.3 | 4.7 | 0.4 | 1.0 | 0.4 | 66.2 |
| 223210 | 13382 | 37.6 | 2.8 | 0.1 | 0.0 | 0.6 | 58.9 |
| 223212 | 11695 | 92.5 | 2.9 | 4.4 | 0.3 | 0.0 | 0.0 |
| 223213 | 9198 | 90.7 | 1.5 | 7.7 | 0.0 | 0.0 | 0.0 |
| 223214 | 9211 | 94.9 | 2.2 | 2.9 | 0.0 | 0.0 | 0.0 |
| 223215 | 8869 | 93.7 | 1.7 | 3.4 | 0.0 | 0.8 | 0.4 |
| 223216 | 1798 | 40.8 | 22.5 | 26.9 | 0.0 | 9.7 | 0.1 |
| 223402 | 1700 | 93.5 | 3.1 | 0.0 | 0.0 | 2.6 | 0.8 |
| 223403 | 1701 | 90.4 | 2.6 | 0.0 | 0.0 | 5.7 | 1.3 |
| Thompson River Basin |  |  |  |  |  |  |  |
| 225019 | 2956 | 98.4 | 0.0 | 1.5 | 0.0 | 0.0 | 0.0 |
| 225105 | 11012 | 54.3 | 2.7 | 2.4 | 0.0 | 36.2 | 4.3 |
| 225114 | 6146 | 55.7 | 2.2 | 29.9 | 0.0 | 1.4 | 10.8 |
| 225200 | 8071 | 55.0 | 23.0 | 2.7 | 0.0 | 19.3 | 0.0 |
| 225201 | 13692 | 94.0 | 3.2 | 2.2 | 0.0 | 0.6 | 0.0 |
| 225204 | 19790 | 84.6 | 5.9 | 1.8 | 0.0 | 7.4 | 0.2 |
| 225207 | 454 | 59.5 | 0.0 | 0.0 | 0.0 | 2.9 | 37.7 |
| 225209 | 22482 | 93.9 | 3.9 | 0.5 | 0.0 | 0.4 | 1.3 |
| 225210 | 17024 | 92.3 | 0.1 | 7.6 | 0.0 | 0.0 | 0.0 |
| 225212 | 18690 | 89.7 | 2.0 | 2.3 | 0.0 | 0.7 | 5.2 |
| 225213 | 18590 | 92.9 | 3.6 | 2.3 | 0.0 | 1.2 | 0.0 |
| 225216 | 2630 | 97.1 | 1.4 | 1.4 | 0.0 | 0.0 | 0.0 |
| 225217 | 5137 | 97.4 | 0.0 | 2.6 | 0.0 | 0.0 | 0.0 |
| 225218 | 17193 | 93.5 | 3.7 | 2.3 | 0.0 | 0.5 | 0.0 |
| 225219 | 17209 | 90.5 | 6.3 | 2.7 | 0.0 | 0.4 | 0.2 |
| 225221 | 16877 | 76.5 | 5.0 | 9.8 | 0.3 | 8.4 | 0.0 |
| 225222 | 2109 | 52.7 | 0.0 | 4.6 | 0.0 | 0.0 | 42.7 |
| 225223 | 15585 | 89.5 | 3.1 | 4.8 | 0.0 | 0.3 | 2.3 |
| 225224 | 15275 | 91.1 | 5.9 | 2.9 | 0.0 | 0.1 | 0.1 |
| 225228 | 4867 | 69.5 | 15.6 | 12.8 | 0.1 | 0.0 | 1.9 |
| 225230 | 13639 | 59.1 | 7.2 | 23.6 | 0.0 | 9.3 | 0.7 |
| 225231 | 13935 | 92.9 | 2.9 | 3.6 | 0.0 | 0.4 | 0.2 |
| 225232 | 13728 | 74.2 | 1.9 | 3.2 | 0.0 | 2.3 | 18.4 |
| 225233 | 2156 | 98.4 | 0.0 | 1.5 | 0.0 | 0.0 | 0.0 |
| 225234 | 13527 | 13.4 | 9.5 | 1.0 | 1.0 | 1.7 | 73.4 |
| 225236 | 8092 | 64.5 | 13.7 | 4.7 | 0.1 | 16.9 | 0.1 |
| 225247 | 4893 | 56.6 | 19.7 | 18.0 | 0.9 | 3.5 | 1.2 |
| 225248 | 4045 | 60.2 | 30.5 | 4.5 | 0.6 | 0.4 | 3.7 |
| 225255 | 3016 | 64.1 | 29.8 | 5.1 | 0.1 | 0.0 | 0.9 |
| 225703 | 2355 | 32.1 | 65.4 | 2.3 | 0.0 | 0.3 | 0.0 |
| 225711 | 8820 | 49.3 | 0.2 | 0.3 | 0.0 | 15.9 | 34.3 |
| 225715 | 5215 | 35.8 | 1.1 | 1.4 | 0.0 | 0.0 | 61.7 |
| 225716 | 4779 | 31.5 | 0.0 | 0.1 | 0.0 | 0.3 | 68.1 |
| 225717 | 5215 | 31.7 | 1.6 | 3.6 | 0.0 | 0.6 | 62.4 |
| 225721 | 1355 | 94.7 | 2.6 | 0.2 | 0.4 | 0.1 | 2.0 |
| 225722 | 1355 | 63.0 | 35.0 | 0.0 | 0.0 | 0.1 | 2.0 |
| 225723 | 1355 | 84.8 | 12.9 | 0.0 | 0.0 | 2.2 | 0.1 |
| 225724 | 1782 | 96.7 | 3.3 | 0.0 | 0.0 | 0.0 | 0.1 |
| 225725 | 1785 | 98.2 | 1.8 | 0.0 | 0.0 | 0.0 | 0.1 |
| 225726 | 1782 | 66.6 | 20.1 | 13.2 | 0.0 | 0.0 | 0.1 |
| 225727 | 1358 | 78.5 | 6.0 | 14.7 | 0.0 | 0.1 | 0.6 |
| 225737 | 1053 | 84.1 | 6.5 | 8.9 | 0.4 | 0.0 | 0.1 |
| 225738 | 1053 | 83.2 | 15.3 | 1.4 | 0.0 | 0.0 | 0.1 |
| 225739 | 1053 | 66.2 | 28.2 | 5.5 | 0.0 | 0.0 | 0.1 |
| 225740 | 1065 | 67.0 | 31.5 | 1.3 | 0.0 | 0.0 | 0.1 |
| 225741 | 1054 | 87.8 | 9.7 | 2.5 | 0.0 | 0.0 | 0.1 |
| 225742 | 1053 | 97.3 | 1.8 | 0.8 | 0.0 | 0.0 | 0.1 |
| 225743 | 1053 | 73.6 | 17.5 | 8.8 | 0.0 | 0.0 | 0.1 |
| 225747 | 493 | 4.3 | 91.5 | 0.0 | 0.0 | 0.0 | 4.3 |
| 225748 | 488 | 50.2 | 48.2 | 1.4 | 0.0 | 0.0 | 0.2 |

#### Gaps

The Gippsland Basin has an extensive network of streamflow gauges. Information recorded in the Victorian Water Measurement Information System is easily accessible and well managed. Data and knowledge gaps that could potentially influence achieving realistic simulation and modelling-based analysis include:

As summarised above, the quality of streamflow data is highly variable. Investment is further required to improve the reliability of monitored data.

The flood inundation data also requires additional resources to improve data reliability. Currently there is limited spatial information and no inundation duration data attributed to each inundation event. Such information would greatly enhance the utility of the data and relevance/ability to integrate the data into a predictive modelling framework.

Strategic investment in the installation of additional stream gauges to increase the monitoring of water quality information. To date there exists gaps in the spatial extent of the water quality network.

Improved frequency in water quality sampling.

Commitment for on-going maintenance and monitoring of priority stream gauges.

### Surface water – groundwater interactions

Summary

Fifty stream gauges in unregulated catchments and 34 groundwater observation bores along main stems of some major rivers were selected for analysis of surface water – groundwater interaction in the Gippsland region. The 50 stream gauges cover majority of main stems of the major rivers in the region. Majority of these gauges have long-term record with a large number of high-quality daily streamflow. Forty-five of them have greater than 80% daily streamflow data classified as ‘Good’ or ‘Fair’ according to the national streamflow quality code. The 34 selected observation bores also have long-term record with high-quality groundwater level readings. Thirty of them have more than 100 readings with typical monitoring frequency ranging from monthly to quarterly. However, these selected bores only represent small proportion of main stems of the major rivers in the region. There is no adequate water chemistry data from the gauges and bores for surface water – groundwater interaction analysis.

#### Methods

There are no direct observations or measurements of the degree of interaction between surface water and groundwater in the Gippsland region. However, there is empirical evidence regarding surface water – groundwater interactions as provided by analysis of both the hydrographs of groundwater and stream, and the chemistry of groundwater and surface water. To investigate surface water – groundwater interactions in the region, relevant surface water and groundwater information along the main stems of the rivers in the following river basins was collected:

Latrobe River

Mitchell River

Thomson/Macalister Rivers/Avon River

Tambo River

Snowy River

East Gippsland

South Gippsland.

The degree and direction of exchange between stream water and local groundwater in the Gippsland region is implied by comparing local groundwater levels to adjacent stream stage height. Previous studies (e.g. DSE, 2012, SKM, 2012a, 2012b; Hofmann, 2011) and the analysis of groundwater and surface water information have provided some level of quantification of surface water– groundwater interaction. Based on the outcomes from previous studies, DSE (2012) have classified surface water and groundwater interaction into four broad groups: neutral/losing, gaining, variable and unclassified.

Key inputs into this study included results from: (1) baseflow separation analysis for stream gauges on unregulated rivers undertaken by SKM (2012a; 2012b); and (2) the relative exchange between stream water and local groundwater using hydrogeochemistry and radon as a tracer as reported by Hofmann (2011).

#### Observed stream and bore water level data

There are two key observed datasets used for the analysis of the surface water – groundwater interaction in the Gippsland region. One is stream flow monitoring data at stream gauges in the region. A total of 197 river gauges were identified in the Gippsland region representing all of the major rivers. Daily gauge level data was sourced from the Water Measurement Information System (WMIS, http://data.water.vic.gov.au/monitoring.htm). The length of monitoring record and data quality vary significantly from one gauge to another. Details of this stream data are described in Section 2.1.7. Of these 197 stream gauges, 50 gauges in unregulated catchments were selected to investigate the surface water – groundwater along only main stems of the major rivers (including 33 currently active gauges). Locations of these gauges are shown in Figure 27 and the availability of flow data is summarised in Table 23.

Another key dataset is groundwater level data measured at observation bores. A total of 686 observation bores were identified in the Gippsland region (onshore). All water levels for these bores were sourced from the WMIS (http://data.water.vic.gov.au/monitoring.htm). The water levels in the WMIS database are presented in three form: depth from top of casing (m); the depth of water below the ground surface (m); and reduced water level (m, AHD). The nature, variation, spatial and temporal resolution of groundwater bore data is described in Section 2.1.6. Of the 686 bores, 34 observation bores were analysed (including 23 currently active observation bores) to evaluate the temporal surface water – groundwater interactions. The locations of these bores are shown in Figure 20 and associated water level data is summarised in Table 23. It is important to note that there is no suitable bore available for the analysis of surface water-groundwater interaction in the East Gippsland and Tambo River basins.

Figure 27 	Location of selected stream gauges and surface-groundwater connectivity in the Gippsland region (surface water-groundwater interaction data sourced from DSE, 2012)

Table 22 	Number of stream gauges selected for analysis of surface-groundwater interaction in each river basin in the Gippsland region

| River basin | No. of gauge |  | No. of daily records |
| --- | --- | --- | --- |
|  | Active | Ceased |  |
| East Gippsland | 7 | 1 | 6138 to 20902 |
| Latrobe River | 7 | 4 | 1755 to 32738 |
| Mitchell River | 2 | 2 | 197 to 14979 |
| Snowy River | 5 | 1 | 4164 to 33623 |
| South Gippsland | 7 | 4 | 4569 to 22539 |
| Tambo River | 3 | 2 | 6342 to 19365 |
| Thomson River | 2 | 3 | 2956 to 17209 |

Table 23 	Number of bores selected for analysis of surface-groundwater interaction in each river basin in the Gippsland region

| River basin | No. of bore |  | No. of record | RWL (m, AHD) |
| --- | --- | --- | --- | --- |
|  | Active | Ceased |  |  |
| Latrobe River | 8 | 5 | 34 to 400 | -19.66 to 91.81 |
| Mitchell River | 8 | 3 | 99 to 330 | 6.03 to 33.35 |
| Snowy River | 1 | 1 | 120 to 130 | 2.82 to 4.98 |
| South Gippsland (Tawin River) | 1 | 1 | 65 to 122 | 2.43 to 22.65 |
| Thomson River | 2 | 0 | 198 to 221 | 13.48 to 25.9 |
| Thomson River (Avon River) | 3 | 1 | 174 to 232 | 0.32 to 17.99 |

#### Statistical analysis and interpolation

As Section 2.1.7.3 describes the statistical analysis of the daily streamflow information from all 197 stream gauges in the Gippsland region, this section only presents analysis based on the 50 gauges selected to assess surface water-groundwater interactions. The majority of these 50 gauges have long-term record with a large number of daily streamflow readings (Table 24 and Figure 28). Of the 50 gauges selected, 47 gauges have more than 3,000 daily measurements (Figure 28). These gauges generally have high quality streamflow data. As shown in Figure 29, 45 of these 50 gauges have greater than 80% daily streamflow data classified as ‘Good’ or ‘Fair’ according to the national streamflow quality code. However, there are only 41 gauges with accurate elevation and 16 gauges with accurate coordinates for their locations.

Water quality data from the 50 selected gauges is limited. There is no comprehensive major ions information available for any of these gauges. Most abundant water quality information is stream salinity measurements. There are 30 gauges with various numbers of EC and/or TSS measurements (Table 24).

The statistical analysis of the groundwater level and quality for all bores in the Gippsland region was presented Section 2.1.6.3. This section only presents the analysis of groundwater level and quality for the 34 selected observation bores. A summary of the measured water level and water quality data for the 34 bores used in this analysis is provided in Table 25. The majority of these 34 bores have more than 10 years of record with a large number of readings (Table 25 and Figure 30). Of the 34 bores selected, 30 bores have more than 100 readings (Figure 30). Typical monitoring frequency ranges from monthly to quarterly. As these bores are part of the state observation bore network, they generally have high quality water level data and other bore information (e.g. bore depth and screen interval).

Groundwater salinity is the main water quality information from these bores. Nineteen (19) of the 34 bores have at least one salinity reading (Table 25).

As only a few selected bores and stream gauges can be used for establishing a groundwater level–river relationship (an example shown in Figure 31), baseflow separation analysis (SKM, 2012b) were used to assess the degree of groundwater–stream connectivity in the Gippsland region (Table 26). The analysis was based on the automated base flow separation and recession technique reported by Arnold et al., (1995) which uses a digital filter to match the slope of the base flow recession curve from stream flow records. Baseflow separation analysis suggests that groundwater contributes between 64% and 80% of streamflow in unregulated systems. As such, it is assumed that the groundwater is in good connection with rivers in these regions.

Table 24	Proportion of the streamflow data assigned to each national streamflow quality code

| Site ID | River basin | Monitoring period |  | Accuracy of elevation | No. of flow records | % of each quality category |  |  |  |  |  | No. of EC records | No. of TSS records |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Start | Stop |  |  | Good | Fair | Poor | Non-conforming | Unknown | Missing |  |  |
| 221201 | East Gippsland | 21/02/1957 | 14/05/2014 | Accurate | 20,902 | 94.54 | 2.93 | 1.60 | 0.00 | 0.05 | 0.89 | 292 | 297 |
| 221204 | East Gippsland | 17/05/1966 | 6/03/1983 | Accurate | 6,138 | 89.72 | 0.00 | 9.25 | 0.00 | 0.00 | 1.03 |  |  |
| 221207 | East Gippsland | 20/02/1968 | 12/05/2014 | Accurate | 16,884 | 87.22 | 2.01 | 4.00 | 0.00 | 5.60 | 1.17 | 292 | 294 |
| 221208 | East Gippsland | 22/06/1979 | 12/05/2014 | Accurate | 12,744 | 92.50 | 3.08 | 3.66 | 0.00 | 0.75 | 0.01 | 277 | 144 |
| 221209 | East Gippsland | 23/10/1972 | 13/05/2014 | Accurate | 15,178 | 87.97 | 4.93 | 7.09 | 0.00 | 0.00 | 0.01 | 7 |  |
| 221210 | East Gippsland | 22/08/1972 | 13/05/2014 | Accurate | 15,240 | 91.98 | 4.69 | 3.33 | 0.00 | 0.00 | 0.01 | 293 | 299 |
| 221211 | East Gippsland | 12/08/1974 | 12/05/2014 | Accurate | 14,519 | 91.62 | 3.35 | 4.66 | 0.00 | 0.37 | 0.01 | 200 | 201 |
| 221212 | East Gippsland | 30/04/1975 | 14/05/2014 | Accurate | 14,260 | 92.95 | 2.62 | 4.04 | 0.00 | 0.13 | 0.27 | 292 | 296 |
| 222202 | Snowy River | 24/04/1922 | 13/05/2014 | Accurate | 33,623 | 50.86 | 29.46 | 2.11 | 0.00 | 0.02 | 17.54 | 292 | 295 |
| 222206 | Snowy River | 8/10/1947 | 12/05/2014 | Estimated | 24,324 | 42.93 | 25.78 | 30.60 | 0.00 | 0.66 | 0.03 | 243 | 249 |
| 222210 | Snowy River | 26/03/1964 | 11/05/2014 | Accurate | 18,309 | 88.73 | 2.18 | 2.91 | 0.10 | 1.71 | 4.37 | 106 | 99 |
| 222213 | Snowy River | 24/07/1957 | 11/05/2014 | Accurate | 20,746 | 72.46 | 8.37 | 4.51 | 0.63 | 14.03 | 0.00 | 106 | 98 |
| 222216 | Snowy River | 4/02/1976 | 29/06/1987 | Accurate | 4,164 | 96.13 | 0.00 | 3.84 | 0.00 | 0.00 | 0.02 |  |  |
| 222217 | Snowy River | 3/06/1976 | 11/12/2013 | Accurate | 13,706 | 96.75 | 1.52 | 1.70 | 0.00 | 0.03 | 0.01 | 279 | 253 |
| 223204 | Tambo River | 12/05/1961 | 18/05/2014 | Accurate | 19,365 | 92.41 | 3.44 | 3.52 | 0.00 | 0.00 | 0.62 | 279 | 260 |
| 223206 | Tambo River | 8/08/1957 | 18/12/1974 | Accurate | 6,342 | 96.25 | 0.00 | 3.74 | 0.00 | 0.00 | 0.02 |  |  |
| 223207 | Tambo River | 23/07/1957 | 4/01/1984 | Accurate | 9,662 | 97.57 | 1.18 | 0.71 | 0.00 | 0.23 | 0.31 |  |  |
| 223212 | Tambo River | 6/05/1982 | 12/05/2014 | Accurate | 11,695 | 92.48 | 2.86 | 4.38 | 0.27 | 0.00 | 0.01 | 106 | 97 |
| 223215 | Tambo River | 8/02/1990 | 21/05/2014 | Accurate | 8,869 | 93.70 | 1.67 | 3.44 | 0.00 | 0.81 | 0.38 | 96 | 4 |
| 224205 | Mitchell River | 11/12/1973 | 25/06/1974 | Accurate | 197 | 99.49 | 0.00 | 0.00 | 0.00 | 0.00 | 0.51 |  |  |
| 224209 | Mitchell River | 15/05/1970 | 30/06/1987 | Accurate | 6,256 | 99.57 | 0.00 | 0.42 | 0.00 | 0.00 | 0.02 |  |  |
| 224213 | Mitchell River | 24/05/1973 | 27/05/2014 | Accurate | 14,979 | 93.02 | 3.61 | 3.10 | 0.08 | 0.19 | 0.01 | 291 | 297 |
| 224214 | Mitchell River | 13/08/1974 | 21/05/2014 | Accurate | 14,527 | 94.66 | 0.77 | 3.32 | 0.00 | 0.80 | 0.45 |  |  |
| 225019 | Thomson River | 28/04/1966 | 31/05/1974 | Accurate | 2,956 | 98.44 | 0.00 | 1.52 | 0.00 | 0.00 | 0.03 |  |  |
| 225217 | Thomson River | 12/12/1974 | 3/01/1989 | Accurate | 5,137 | 97.41 | 0.00 | 2.57 | 0.00 | 0.00 | 0.02 | 8 | 4 |
| 225218 | Thomson River | 12/04/1967 | 7/05/2014 | Accurate | 17,193 | 93.46 | 3.73 | 2.28 | 0.00 | 0.52 | 0.01 | 178 | 180 |
| 225219 | Thomson River | 7/04/1967 | 18/05/2014 | Accurate | 17,209 | 90.45 | 6.30 | 2.73 | 0.00 | 0.35 | 0.17 | 14 | 3 |
| 225230 | Thomson River | 2/05/1975 | 2/09/2012 | Estimated | 13,639 | 59.10 | 7.24 | 23.62 | 0.00 | 9.30 | 0.75 | 71 | 78 |
| 226008 | Latrobe River | 27/04/1960 | 15/04/2014 | Accurate | 19,712 | 33.47 | 2.49 | 0.75 | 0.00 | 1.07 | 62.22 | 43 | 55 |
| 226012 | Latrobe River | 27/01/1961 | 10/08/1971 | Accurate | 3,848 | 95.87 | 0.00 | 0.26 | 0.00 | 0.00 | 3.87 |  |  |
| 226016 | Latrobe River | 24/03/1977 | 11/01/1982 | Accurate | 1,755 | 98.92 | 0.00 | 0.00 | 0.00 | 0.00 | 1.08 |  |  |
| 226017 | Latrobe River | 12/05/1976 | 14/04/2014 | Accurate | 13,852 | 34.38 | 1.72 | 5.61 | 0.00 | 0.51 | 57.78 | 130 | 140 |
| 226204 | Latrobe River | 23/10/1924 | 10/06/2014 | Estimated | 32,738 | 38.87 | 45.56 | 8.40 | 0.00 | 7.17 | 0.00 | 243 | 250 |
| 226217 | Latrobe River | 2/06/1955 | 8/01/1989 | Accurate | 12,275 | 90.61 | 0.00 | 5.14 | 0.00 | 0.00 | 4.25 |  |  |
| 226219 | Latrobe River | 15/10/1969 | 20/07/1987 | Accurate | 6,488 | 98.26 | 0.00 | 1.73 | 0.00 | 0.00 | 0.02 |  |  |
| 226220 | Latrobe River | 6/03/1957 | 2/06/2014 | Estimated | 20,908 | 75.43 | 8.03 | 2.86 | 0.09 | 9.70 | 3.88 | 9 | 1 |
| 226226 | Latrobe River | 2/06/1960 | 10/06/2014 | Estimated | 19,732 | 79.81 | 3.60 | 3.89 | 0.04 | 12.66 | 0.01 | 274 | 148 |
| 226407 | Latrobe River | 6/10/1958 | 2/06/2014 | Accurate | 20,329 | 59.01 | 1.88 | 12.37 | 0.00 | 2.85 | 23.88 |  |  |
| 226410 | Latrobe River | 31/05/1962 | 25/05/2014 | Accurate | 18,988 | 80.02 | 4.53 | 1.43 | 0.55 | 13.46 | 0.01 |  |  |
| 227203 | South Gippsland | 17/03/1970 | 7/01/1985 | Accurate | 5,411 | 96.91 | 0.00 | 3.07 | 0.00 | 0.00 | 0.02 |  |  |
| 227210 | South Gippsland | 7/08/1952 | 22/04/2014 | Accurate | 22,539 | 76.29 | 21.12 | 2.41 | 0.00 | 0.06 | 0.12 |  |  |
| 227213 | South Gippsland | 1/10/1960 | 26/05/2014 | Accurate | 19,596 | 89.61 | 6.83 | 2.89 | 0.00 | 0.67 | 0.01 |  |  |
| 227220 | South Gippsland | 8/11/1972 | 1/12/1998 | Accurate | 9,520 | 92.41 | 0.00 | 4.87 | 0.00 | 0.00 | 2.72 |  |  |
| 227223 | South Gippsland | 11/12/1974 | 14/06/1987 | Accurate | 4,569 | 95.56 | 0.00 | 4.42 | 0.00 | 0.00 | 0.02 |  |  |
| 227225 | South Gippsland | 24/04/1968 | 26/05/2014 | Accurate | 16,834 | 94.96 | 2.32 | 1.56 | 0.00 | 0.99 | 0.17 | 169 | 135 |
| 227226 | South Gippsland | 23/04/1970 | 27/04/2014 | Estimated | 16,076 | 97.28 | 1.19 | 1.52 | 0.00 | 0.01 | 0.01 | 7 |  |
| 227227 | South Gippsland | 25/10/1973 | 25/05/2014 | Estimated | 14,823 | 86.24 | 11.53 | 1.57 | 0.00 | 0.65 | 0.01 | 106 | 98 |
| 227228 | South Gippsland | 30/04/1971 | 17/06/1987 | Accurate | 5,893 | 97.64 | 0.00 | 2.34 | 0.00 | 0.00 | 0.02 |  |  |
| 227236 | South Gippsland | 18/05/1979 | 5/05/2014 | Estimated | 12,772 | 93.69 | 1.55 | 4.63 | 0.00 | 0.13 | 0.01 | 80 | 87 |
| 227237 | South Gippsland | 8/06/1979 | 26/05/2014 | Estimated | 12,772 | 95.37 | 1.32 | 3.12 | 0.00 | 0.19 | 0.01 | 280 | 256 |

Figure 28 Summary of monitoring records for the 50 selected gauges in the Gippsland

Figure 29 Data quality of monitoring records for the 50 selected gauges for surface water-groundwater interaction analysis in the Gippsland

Table 25 	Summary of selected bores for surface-groundwater interaction analysis

| Bore ID | Monitoring period |  | No. of water level records | Mean RWL (m, AHD) | Mean BGL (m) | No. of EC records | Mean EC (µS/cm) | River basin | Aquifer monitored |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Start | Stop |  |  |  |  |  |  |  |
| 45761 | 4/10/1991 | 23/01/2011 | 170 | 18.00 | 1.81 | 9 | 4088 | Latrobe R | Alluvium |
| 45762 | 4/09/1991 | 20/04/2011 | 174 | 17.96 | 1.83 | 27 | 8589 | Latrobe R | Alluvium |
| 45763 | 4/09/1991 | 23/01/2011 | 171 | 17.89 | 1.93 | 19 | 7355 | Latrobe R | Alluvium |
| 45764 | 4/09/1991 | 23/01/2011 | 167 | 18.44 | 2.17 | 9 | 7553 | Latrobe R | Alluvium |
| 45765 | 4/09/1991 | 18/09/2003 | 144 | 18.58 | 2.05 | 25 | 6812 | Latrobe R | Alluvium |
| 56536 | 3/09/1970 | 7/02/2011 | 297 | 21.12 | 4.24 |  |  | Mitchell R | Alluvium |
| 56537 | 3/09/1970 | 18/11/2010 | 304 | 20.81 | 4.68 | 1 | 1000 | Mitchell R | Alluvium |
| 56538 | 16/09/1970 | 7/02/2011 | 306 | 20.51 | 3.33 | 1 | 268 | Mitchell R | Alluvium |
| 56550 | 14/02/1985 | 13/05/2014 | 193 | 21.30 | 4.62 |  |  | Mitchell R | Alluvium |
| 67441 | 28/12/1978 | 6/05/2014 | 152 | 23.78 | 9.92 |  |  | Latrobe R | Lower Tertiary (Latrobe Group) |
| 80760 | 4/06/1970 | 13/05/2014 | 326 | 9.00 | 2.30 |  |  | Mitchell R | Alluvium |
| 80761 | 26/02/1970 | 13/05/2014 | 317 | 8.75 | 3.25 | 1 | 224 | Mitchell R | Alluvium |
| 80762 | 26/02/1970 | 13/05/2014 | 330 | 9.09 | 3.34 |  |  | Mitchell R | Alluvium |
| 86464 | 21/08/1985 | 12/05/2014 | 215 | 5.98 | 0.39 | 1 | 446 | Avon R | Mid-Tertiary (Boisdale Formation) |
| 86669 | 15/11/1989 | 12/05/2014 | 189 | 12.14 | -5.57 | 2 | 429 | Avon R | Mid-Tertiary (Boisdale Formation) |
| 86670 | 15/11/1989 | 12/05/2014 | 174 | 4.74 | 1.68 | 2 | 507 | Avon R | Mid-Tertiary (Boisdale Formation) |
| 86726 | 2/10/1991 | 23/01/2011 | 232 | 4.48 | 2.42 | 43 | 3251 | Avon R | Alluvium |
| 89809 | 28/12/1978 | 6/05/2014 | 86 | 2.52 | 11.08 |  |  | Latrobe R | Lower Tertiary (Latrobe Group) |
| 90149 | 31/10/1986 | 7/05/2014 | 195 | 5.87 | -4.81 | 2 | 385 | Latrobe R | Mid-Tertiary (Boisdale Formation) |
| 95482 | 10/10/1990 | 6/05/2014 | 221 | 24.32 | 0.97 | 3 |  | Thomson R | Mid-Tertiary (Boisdale Formation) |
| 105196 | 27/11/1991 | 7/05/2014 | 198 | 16.20 | -1.70 | 2 | 375 | Thomson R | Mid-Tertiary (Boisdale Formation) |
| 105220 | 20/08/1993 | 26/08/2008 | 65 | 15.88 | 20.27 |  |  | South Gippsland (Tarra R) | Lower Tertiary (Latrobe Group) |
| 105222 | 25/08/1986 | 6/05/2014 | 122 | 11.60 | 9.10 | 1 | 316 | South Gippsland (Tarra R) | Lower Tertiary (Latrobe Group) |
| 105476 | 25/11/1991 | 13/05/2014 | 169 | 24.06 | 5.31 | 2 | 723 | Mitchell R | Alluvium |
| 105477 | 25/11/1991 | 13/05/2014 | 164 | 24.52 | 5.71 | 1 | 94 | Mitchell R | Alluvium |
| 105478 | 25/11/1991 | 13/05/2014 | 167 | 27.53 | 9.05 |  |  | Mitchell R | Upper Tertiary (Latrobe Valley Coal Measures) |
| 105479 | 25/11/1991 | 13/05/2014 | 99 | 28.46 | 8.15 |  |  | Mitchell R | Alluvium |
| 107970 | 15/05/1973 | 8/05/2014 | 400 | 65.98 | 14.61 |  |  | Latrobe R | Lower Tertiary (Older Volcanics) |
| 144468 | 19/12/2001 | 14/05/2014 | 130 | 3.29 | 1.86 | 23 | 1480 | Snowy R | Upper Tertiary (Curlip Gravels) |
| 144469 | 19/12/2001 | 15/11/2011 | 120 | 3.85 | 8.49 | 10 | 167 | Snowy R | Upper Tertiary (Curlip Gravels) |
| WRK990054 (S9037246/1) | 18/09/2009 | 8/05/2014 | 34 | 91.28 | -5.39 |  |  | Latrobe R | Lower Tertiary (Childers Formation) |
| WRK990055 (S9037246/4) | 18/09/2009 | 8/05/2014 | 34 | 76.04 | 10.15 |  |  | Latrobe R | Upper Tertiary (Hanuted Hills Formation) |
| WRK990056 (S9037246/3) | 18/09/2009 | 8/05/2014 | 34 | 71.75 | 14.43 |  |  | Latrobe R | Lower Tertiary (Yarragon Formation) |
| WRK990057 (S9037246/2) | 18/09/2009 | 8/05/2014 | 34 | 66.31 | 19.85 |  |  | Latrobe R | Lower Tertiary (Thorpdale Volcanics) |

Figure 30 Summary of monitoring records for the 34 selected observation bores for surface water-groundwater interaction analysis in the Gippsland Basin

Figure 31 Groundwater hydrographs of a transect of bores located near Mitchell River west of Bairnsdale plotted against daily stream water level of Mitchell River at Rose (gauge station no. 224203)

Table 26 	Estimated Baseflow Index (BFI) in 51 unregulated catchments in the Gippsland region (sourced from SKM 2012)

| Gauge ID | Gauge name | River basin | Av_BFI (Sum) | Av_BFI (Aut) | Av_BFI (Win) | Av_BFI (Spr) | Av_BFI (all) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 221201 | Cann River (West Branch) @ Weeragua | East Gippsland | 0.81 | 0.83 | 0.71 | 0.74 | 0.77 |
| 221204 | Thurra River @ Point Hicks | East Gippsland | 0.79 | 0.78 | 0.61 | 0.7 | 0.72 |
| 221207 | Errinundra River @ Errinundra | East Gippsland | 0.83 | 0.86 | 0.72 | 0.75 | 0.79 |
| 221208 | Wingan River @ Wingan Inlet National Park | East Gippsland | 0.82 | 0.79 | 0.6 | 0.71 | 0.73 |
| 221209 | Cann River (East Branch) @ Weeragua | East Gippsland | 0.81 | 0.8 | 0.67 | 0.72 | 0.75 |
| 221210 | Genoa River @ The Gorge | East Gippsland | 0.8 | 0.78 | 0.67 | 0.72 | 0.74 |
| 221211 | Combienbar River @ Combienbar | East Gippsland | 0.82 | 0.84 | 0.72 | 0.74 | 0.78 |
| 221212 | Bemm River @ Princes Highway | East Gippsland | 0.83 | 0.86 | 0.71 | 0.75 | 0.79 |
| 226008 | Tyers River West Branch @ Morgans Mill | Latrobe River | 0.83 | 0.83 | 0.7 | 0.71 | 0.77 |
| 226012 | Tanjil River East Branch @ Tanjil Bren | Latrobe River | 0.79 | 0.77 | 0.69 | 0.61 | 0.72 |
| 226016 | Waterhole Creek @ Morwell | Latrobe River | 0.9 | 0.82 | 0.68 | 0.73 | 0.78 |
| 226017 | Jacobs Creek @ Otooles | Latrobe River | 0.79 | 0.81 | 0.61 | 0.64 | 0.71 |
| 226204 | Latrobe River @ Willow Grove | Latrobe River | 0.86 | 0.87 | 0.72 | 0.73 | 0.8 |
| 226217 | Latrobe River @ Hawthorn Bridge | Latrobe River | 0.85 | 0.87 | 0.69 | 0.72 | 0.78 |
| 226219 | Tooronga River @ Noojee | Latrobe River | 0.85 | 0.88 | 0.7 | 0.72 | 0.79 |
| 226220 | Loch River @ Noojee | Latrobe River | 0.87 | 0.89 | 0.68 | 0.73 | 0.79 |
| 226226 | Tanjil River @ Tanjil Junction | Latrobe River | 0.83 | 0.87 | 0.67 | 0.65 | 0.75 |
| 226407 | Morwell River @ Boolarra | Latrobe River | 0.85 | 0.82 | 0.58 | 0.66 | 0.72 |
| 226410 | Traralgon Creek @ Koornalla | Latrobe River | 0.84 | 0.81 | 0.55 | 0.64 | 0.71 |
| 224205 | Dargo River @ Dargo (Upper Site) | Mitchell River | 0.79 | 0.8 | 0.57 | 0.53 | 0.67 |
| 224209 | Cobbannah Creek @ Near Bairnsdale | Mitchell River | 0.67 | 0.69 | 0.66 | 0.68 | 0.67 |
| 224213 | Dargo River @ Lower Dargo Road | Mitchell River | 0.79 | 0.83 | 0.48 | 0.56 | 0.66 |
| 224214 | Wentworth River @ Tabberabbera | Mitchell River | 0.78 | 0.86 | 0.7 | 0.7 | 0.76 |
| 222202 | Brodribb River @ Sardine Creek | Snowy River | 0.83 | 0.86 | 0.71 | 0.75 | 0.79 |
| 222206 | Buchan River @ Buchan | Snowy River | 0.77 | 0.83 | 0.6 | 0.61 | 0.7 |
| 222210 | Deddick River @ Deddick (Caseys) | Snowy River | 0.79 | 0.83 | 0.76 | 0.77 | 0.74 |
| 222213 | Suggan Buggan River @ Suggan Buggan | Snowy River | 0.77 | 0.84 | 0.57 | 0.58 | 0.69 |
| 222216 | Murrindal River @ Basin Road (Buchan) | Snowy River | 0.76 | 0.75 | 0.62 | 0.65 | 0.7 |
| 222217 | Rodger River @ Jacksons Crossing | Snowy River | 0.79 | 0.85 | 0.67 | 0.71 | 0.75 |
| 227203 | Franklin River @ Henwoods Bridge | South Gippsland | 0.84 | 0.79 | 0.61 | 0.67 | 0.73 |
| 227210 | Bruthen Creek @ Carrajung Lower | South Gippsland | 0.82 | 0.83 | 0.63 | 0.68 | 0.74 |
| 227213 | Jack River @ Jack River | South Gippsland | 0.83 | 0.84 | 0.61 | 0.68 | 0.74 |
| 227220 | Greig Creek @ Mumfords | South Gippsland | 0.84 | 0.85 | 0.67 | 0.68 | 0.76 |
| 227223 | Macks Creek @ Richards | South Gippsland | 0.84 | 0.81 | 0.68 | 0.7 | 0.76 |
| 227225 | Tarra River @ Fischers | South Gippsland | 0.84 | 0.86 | 0.68 | 0.73 | 0.78 |
| 227226 | Tarwin River East Branch @ Dumbalk North | South Gippsland | 0.85 | 0.79 | 0.52 | 0.62 | 0.69 |
| 227227 | Wilkur Creek @ Leongatha | South Gippsland | 0.84 | 0.74 | 0.45 | 0.6 | 0.64 |
| 227228 | Tarwin River East Branch @ Mirboo | South Gippsland | 0.85 | 0.76 | 0.54 | 0.62 | 0.69 |
| 227236 | Powlett River @ D/S Foster Creek Junction | South Gippsland | 0.9 | 0.8 | 0.42 | 0.6 | 0.67 |
| 227237 | Franklin River @ Toora | South Gippsland | 0.85 | 0.8 | 0.55 | 0.65 | 0.71 |
| 223204 | Nicholson River @ Deptford | Tambo River | 0.78 | 0.8 | 0.71 | 0.73 | 0.75 |
| 223206 | Tambo River @ Bindi | Tambo River | 0.78 | 0.84 | 0.57 | 0.58 | 0.69 |
| 223207 | Timbarra River @ Timbarra | Tambo River | 0.81 | 0.85 | 0.71 | 0.69 | 0.76 |
| 223212 | Timbarra River @ D/S Of Wilkinson Creek | Tambo River | 0.79 | 0.86 | 0.69 | 0.69 | 0.76 |
| 223215 | Haunted Stream @ Hells Gate | Tambo River | 0.77 | 0.86 | 0.71 | 0.73 | 0.77 |
| 225019 | North Cascade Creek @ Thomson Valley Road | Thomson River | 0.8 | 0.77 | 0.7 | 0.62 | 0.64 |
| 225217 | Barkly River @ Glencairn | Thomson River | 0.81 | 0.83 | 0.45 | 0.57 | 0.66 |
| 225218 | Freestone Creek @ Briagalong | Thomson River | 0.73 | 0.78 | 0.64 | 0.68 | 0.71 |
| 225219 | Macalister River @ Glencairn | Thomson River | 0.8 | 0.81 | 0.42 | 0.53 | 0.64 |
| 225230 | Glenmaggie Creek @ The Gorge | Thomson River | 0.75 | 0.8 | 0.67 | 0.71 | 0.73 |

#### Gaps

Data and knowledge gaps related to surface water - groundwater interactions that could potentially influence achieving realistic simulation and modelling based analysis include:

A significant number of stream gauges having a large amount of missing or low quality streamflow data.

Only a small proportion of the main stems of the major rivers have a groundwater observation bore suitably located adjacent to the river to enable the assessment of a groundwater level–river relationship.

There is limited water chemistry information available for selected stream gauges and groundwater observation bores to quantify surface water – groundwater interactions.

It is recommended that a standardised method of estimating baseflow be developed. To date numerous methods have been applied resulting in variations in predicted contribution of groundwater to streamflow. Given that baseflow is often used to calibrate both groundwater and river routing models, it may be necessary to further invest in the development of an agreed baseflow estimation approach.

References

Aldrick JM, Hook RA, van de Graaf RM, Nicholson BM, O'Beirne DA and Schoknecht NR (1988) A Study of the Land in the Catchment to the Gippsland Lakes, Volume 1. Technical Communication No. 17. former Land & Catchment Protection Branch, Dept. Conservation and Natural Resources.

Aldrick JM, Hook RA, van de Graaf RHM, Nicholson BM, O'Beirne DA and Schoknect NR (1992) A Study of the Land in the Catchment to the Gippsland Lakes, Volume 2. Technical Communication No. 17., former Land & Catchment Protection Branch, Dept. Conservation and Natural Resources.

Arnold JG, Allen PM, Muttiah R and Bernhardt G (1995) Automated base flow separation and recession analysis techniques, Groundwater, 33,6 1010-1018.

Beverly (2012) Sensitivity of farming system models to soil hydraulic properties, Department of Primary Industries, Victoria, pp56.

Blevin J, Cathro D, Nelson G, Vizy J and  Lee J D, (2013) Survey GDPI10 Interpretation Project, Southern Flank, Gippsland Basin. VicGCS Report 8, Department of Primary Industries, 246 p, Appendix 1-8.

BRS (2006) Guidelines for land use mapping in Australia: principles, procedures and definitions (3rd edn.), Australia, Bureau of Rural Science, Canberra, 83pp.

Constantine A (2001) Sedimentology, Stratigraphy and Palaeoenvironment of the Upper Jurassic Lower Cretaceous Non-Marine Strzelecki Group, Gippsland Basin, Southern Australia, PhD Thesis, Monash University

DSE (2011) Pump Test Database Project - Development Protocols and User Notes February 2011. Unpublished report, Department of Sustainability and Environment.

DSE (2012a) Product Description VICMAP Hydro, Spatial Information Infrastructure, Department of Sustainability and Environment, Document version 5.4, Feb 2012, pp72.

DSE (2012b) Victorian groundwater-surface water interaction spatial data. Compiled by Department of Sustainability and Environment for the Secure Allocation, Future Entitlements (SAFE) Project.

DEPI (2014a) Victorian Water Management Information System. Victorian Government. < http://data.water.vic.gov.au/monitoring.htm, June 2014)

DEPI (2014b) Water Monitoring Data Explanatory Notes, Victorian Department of Environment and Primary Industries, 25pp.

FitzGerald, D, Paterson, R, and Skladzien, PB, (2009) Isostatic and decompensative correction of Victorian gravity data. GeoScience Victoria 3D Victoria Report 3. Department of Primary Industries.

Gallant JC, Dowling TI, Read AM, Wilson N, Tickle P and Inskeep C (2011) 1 second SRTM Derived Digital Elevation Models User Guide. Geoscience Australia www.ga.gov.au/topographic-mapping/digital-elevation-data.html

GHD (2011) Latrobe Valley Coal Model completion report. A report prepared by GHD for Clean Coal Victoria, the Department of Primary Industries.

GHD (2012a) West Gippsland CMA Groundwater Model-Transient Model Development Report. An unpublished report for the Dept. of Sustainability and Environment, May 2010, pp.118.

GHD (2012b) East Gippsland CMA Groundwater Model-Transient Model Development Report. An unpublished report prepared by GHD for the Dept. of Sustainability and Environment, May 2010, pp.136.

GHD (2012c) Development of State Wide 3D Aquifer Surfaces. Report prepared by GHD for Department of Sustainability and Environment for the Secure Allocation, Future Entitlements (SAFE) Project, May 2012, pp. 116.

GSV (2014) Victorian dbMap database. Geological Survey of Victoria. Viewed 1 June 2014.

Hofmann H (2011) Understanding connectivity within groundwater systems and between groundwater and rivers. PhD Thesis, Monash University, Clayton, Victoria.

Hutchinson MF (2001) ANUClim Version 5.1, [Online] Available at http://cres.anu.edu.au/outputs/anuclim.php (verified 29 July 2004).

Jeffrey S, Carter J, Moodie K and Beswick A (2001) Using spatial interpolation to construct a comprehensive archive of Australian climate data. Environmental Modelling and Software 16, 309 330.

Keetley J, Nourollah M, Tait A, Hoffman N, Hall M and Asquith K, (2010) High resolution structural seismic stratigraphic analysis of the onshore and offshore Gippsland Basin, Victoria, Australia. Unpublished report.

McLean M and Blackburn G (2013) A new regional velocity model for the Gippsland Basin. VicGCS Report 9, Department of Primary Industries.

Nicholson BM (1974) Land Systems, Report on the East Gippsland Area. Land Conservation Council.

Nicholson BM (1978) A Study of the Land in the Gippsland Lakes Area. Technical Communication No. 12 former Soil Conservation Authority, Victoria.

Northcote K (1979) A factual key for the recognition of Australian Soils, Rellim Technical Publications Pty. Ltd.

Rees DB (1996) Land Inventory in East Gippsland - A Reconnaissance Study. Technical Report No. 23, Centre for Land Protection Research, DNRE, Victoria.

Rodríguez E, Morris CS and Belz JE (2006) A global assessment of the SRTM performance. Photogrammetric Engineering and Remote Sensing 72 (3), 249-260.

Rowe RK (1977) Land Systems. Report on the Alpine Study Area. Land Conservation Council.

Sargeant IJ (1975) Soil Survey in Westernport Bay Environmental Study 1973-74. Department of Agriculture, Victoria.

Sargeant IJ and Imhof MP (2000) Major Agricultural Soils of the West Gippsland Region. Department of Primary Industries, Victoria, Australia

Schaeffer J (2008) Scaling Point Based Aquifer Data for Developing Regional Groundwater Models: Application to the Gippsland Groundwater System. PhD thesis. Melbourne: University of Melbourne, pp.815.

Schoknecht NR (1982) A Study for the Land in the Gippsland Lakes Hinterland Area. former Land Conservation Council.

Sibley GT (1966) A Classification of the Land at Wilsons Promontory. Folio Report No. 914.
former Soil Conservation Authority.

SKM (2012a) Impacts of groundwater extraction on streamflow in selected catchments throughout Australia. Waterlines Report Series No 84. A report prepared by Sinclair Knight Merz for  National Water Commission, Canberra.

SKM (2012b) Baseflow separation and analysis. A report prepared by Sinclair Knight Merz for  Department of Sustainability and Environment for the Secure Allocation, Future Entitlements (SAFE) Project.

Smith C (2002) 1:250 000 Statewide Soil Attribute Coverage Documentation. Centre for Land Protection Research: Department of Primary Industries, Victoria.

Van de Graaf RHM and Hook RA (1980) Land Systems. Report on the South Gippsland Study Area - District 2. Land Conservation Council.

Viney, N R, T A McMahon and F H S Chiew (2011) Guidelines for presentation of the Bureau of Meteorology national streamflow database. CSIRO: Water for a Healthy Country National Research Flagship, 46pp.

Zhang YQ, Viney N, Frost A, Oke A, Brooks M, Chen Y and Campbell N (2013) Collation of Australian modeller’s streamflow dataset for 780 unregulated Australian catchments. CSIRO: Water for a Healthy Country National Research Flagship.

Datasets

Dataset 1: Gippsland 1 second digital elevation model https://data.bioregionalassessments.gov.au/datastore/dataset/1e74543b-d1ca-452a-ad09-53b2bc90b055

Dataset 2: Gippsland aspect derived from 1 second DEM https://data.bioregionalassessments.gov.au/datastore/dataset/1aed26bb-c6b1-43c5-abb3-1f960b7cca39

Dataset 3: Gippsland slope derived from 1 second DEM

https://data.bioregionalassessments.gov.au/datastore/dataset/9051bb0d-1054-42f4-b6c8-5742e40cc389

Dataset 4: Gippsland soil classification

https://data.bioregionalassessments.gov.au/datastore/dataset/d2e3a020-58df-4138-bf28-22c20be2e3a2

Dataset 5: Gippsland land use

Dataset 6: Gippsland climate station zones

https://data.bioregionalassessments.gov.au/datastore/dataset/cf8bfe31-bd59-4d82-b5d5-4d1b5d39df42

Dataset 7: Gippsland mean annual rainfall (mm/yr)

https://data.bioregionalassessments.gov.au/datastore/dataset/033319be-8058-48b5-ba2d-23f3931a25d4

Dataset 8: Gippsland average daily minimum (C)

https://data.bioregionalassessments.gov.au/datastore/dataset/26bd3c85-2262-4e6d-ae91-299672ff4526

Dataset 9: Gippsland average daily maximum temperature (C)

https://data.bioregionalassessments.gov.au/datastore/dataset/80ffe48e-8124-46cd-95d4-d09873db6556

Dataset 10: Gippsland average daily solar radiation (MJ/m2)

https://data.bioregionalassessments.gov.au/datastore/dataset/3dfaf28b-1e7a-4e44-a8c4-d86c3b2e2f1d

Dataset 11: Gippsland potential mean annual evaporation (mm/yr)

https://data.bioregionalassessments.gov.au/datastore/dataset/33dc8c50-9eb8-41af-9b4b-a7ea7b3c908e

Dataset 12: Gippsland climate geodatabase https://data.bioregionalassessments.gov.au/datastore/dataset/aa6108b8-a1d8-464c-9562-7d6a69304685

Dataset 13: Petroleum wells used for stratigraphic interpretation.

http://data.bioregionalassessments.gov.au/dataset/bb3f4d30-3572-4ac4-ac77-ee281846f071