Groundwater numerical modelling for the Gippsland Basin bioregion

Product 2.6.2 from the Gippsland Basin Bioregional Assessment

2018

The Bioregional Assessment Programme

The Bioregional Assessment Programme is a transparent and accessible programme of baseline assessments that increase the available science for decision making associated with coal seam gas and large coal mines. A bioregional assessment is a scientific analysis of the ecology, hydrology, geology and hydrogeology of a bioregion with explicit assessment of the potential direct, indirect and cumulative impacts of coal seam gas and large coal mining development on water resources. This Programme draws on the best available scientific information and knowledge from many sources, including government, industry and regional communities, to produce bioregional assessments that are independent, scientifically robust, and relevant and meaningful at a regional scale.

The Programme is funded by the Australian Government Department of the Environment. For more information, visit http://www.bioregionalassessments.gov.au.

Department of the Environment

The Office of Water Science, within the Australian Government Department of the Environment, is strengthening the regulation of coal seam gas and large coal mining development by ensuring that future decisions are informed by substantially improved science and independent expert advice about the potential water related impacts of those developments. For more information, visit http://www.environment.gov.au/coal-seam-gas-mining/.

Bureau of Meteorology

The Bureau of Meteorology is Australia’s national weather, climate and water agency. Under the Water Act 2007, the Bureau is responsible for compiling and disseminating Australia's water information. The Bureau is committed to increasing access to water information to support informed decision making about the management of water resources. For more information, visit http://www.bom.gov.au/water/.

Department of Environment, Land, Water and Planning

The Department of Environment, Land, Water and Planning creates liveable, inclusive and sustainable communities that support jobs and growth in Victoria. We recognise the link between the built and natural environment in the quality of our lives, and work to accommodate population growth while maintaining world class liveability and protecting our heritage for future generations. For more information, visit http://www.delwp.vic.gov.au.

Geological Survey of Victoria

The Geological Survey of Victoria (GSV) is Victoria’s geoscience agency and leading authority on Victorian geology. The GSV is the custodian of over 160 years of geological knowledge. The GSV provides objective and authoritative geoscientific data and advice, creates and delivers innovative geoscience information, on a wide range of earth resources which underpin sound Victorian government policy, programs and projects. For more information, visit http://www.energyandresources.vic.gov.au/earth-resources/geology-of-victoria/geological-survey-of-victoria.

Department of Economic Development, Jobs, Transport and Resources

Agriculture Victoria works with the agriculture and fisheries industries on research, development and extension to improve production, connect the sector with international markets, support development and maintain effective biosecurity controls.  For more information, visit http://agriculture.vic.gov.au

ISBN: 978-1-925315-87-5

Citation

DELWP (2017) Groundwater numerical modelling for the Gippsland Basin bioregion.

Copyright

© Commonwealth of Australia 2015
With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence http://www.creativecommons.org/licenses/by/3.0/au/deed.en. 
The Bioregional Assessment Programme requests attribution as ‘© Commonwealth of Australia (Bioregional Assessment Programme http://www.bioregionalassessments.gov.au)’.

Disclaimer

The information contained in this report is based on the best available information at the time of publication. The reader is advised that such information may be incomplete or unable to be used in any specific situation. Therefore decisions should not be made based solely on this information or without seeking prior expert professional, scientific and technical advice

Cover photograph

Lake Victoria, Victoria, 2013

Credit: Hashim Carey

Contents

2.6.2	Groundwater numerical modelling for the Gippsland Basin bioregion	16

2.6.2.1	Methods	17

2.6.2.2	Review of existing models	19

2.6.2.2.1	Introduction	19

2.6.2.2.2	Review of the Integrated Resource Model	20

2.6.2.2.3	Review of the ecoMarkets groundwater models	22

2.6.2.2.4	Review of the CSIRO groundwater model	26

2.6.2.2.5	Review of DPI Gippsland Basin groundwater model	28

2.6.2.2.6	Review of the Victorian Onshore Natural Gas Water Science Studies groundwater model	30

2.6.2.2.7	Summary of existing groundwater model attributes	31

2.6.2.3	Model development	34

2.6.2.3.1	Objectives	34

2.6.2.3.2	Hydrogeological conceptual model	35

2.6.2.3.3	Design and implementation	37

2.6.2.3.4	Modelled hydrogeological units	42

2.6.2.3.5	Modelling approach	63

2.6.2.3.6	Unsaturated modelling software	63

2.6.2.3.7	Groundwater modelling software and solver	63

2.6.2.3.8	Model complexity	64

2.6.2.3.9	Model simulation period	65

2.6.2.3.10	Model stress periods	66

2.6.2.3.11	Model groundwater extractions	66

2.6.2.3.12	Steady-state model conditions	67

2.6.2.3.13	Transient model initial conditions	67

2.6.2.3.14	Geometry and hydrostratigraphy	67

2.6.2.4	Boundary and initial conditions	116

2.6.2.4.1	Sources of boundary and calibration data	116

2.6.2.4.2	Groundwater model layer boundary conditions	117

2.6.2.4.3	Allowance for temperature and density	148

2.6.2.4.4	Allowance for faults	149

2.6.2.4.5	Allowance for flooding	149

2.6.2.4.6	Model recharge	154

2.6.2.4.7	Model evapotranspiration	159

2.6.2.4.8	Model rivers and drains	159

2.6.2.4.9	Surface water – groundwater interactions	179

2.6.2.5	Implementation and coal resource development pathway	188

2.6.2.5.1	Baseline and CRDP climate change signal	188

2.6.2.5.2	Baseline and CRDP recharge	189

2.6.2.5.3	Baseline and CRDP evapotranspiration	189

2.6.2.5.4	CRDP extractions	190

2.6.2.5.5	CRDP mine development	191

2.6.2.6	Parameterisation	192

2.6.2.7	Observations and predictions	226

2.6.2.7.1	Observation bore data	226

2.6.2.7.2	Baseflow estimates	255

2.6.2.7.3	Model calibration procedure	256

2.6.2.7.4	Calibration criteria	286

2.6.2.7.5	Steady-state predictions	288

2.6.2.7.6	Transient predictions	292

2.6.2.7.7	Sensitivity analysis	301

2.6.2.7.8	CRDP predictions	304

2.6.2.8	Limitations	315

References	318

Figures

Figure 1 	Grouping of hydrostratigraphic units incorporated into the IRM model layer structure (sourced from Schaeffer 2008)	21

Figure 2	Location of active cells and IRM model domain (sourced from Schaeffer 2008)	22

Figure 3	IRM boundary conditions for Layer 1 (source: Schaeffer 2008)	22

Figure 4	Domain and boundary conditions for the ecoMarkets East Gippsland groundwater model (source: GHD 2010c)	24

Figure 5	Domain and boundary conditions for the ecoMarkets West Gippsland groundwater models (source: GHD, 2010b)	Error! Bookmark not defined.

Figure 6	Locality map of the CSIRO large-scale model domain (source: CSIRO, unpublished data)	27

Figure 7	Conceptual model layers adopted in the CSIRO large-scale model (source: CSIRO, unpublished data)	27

Figure 8	Latrobe Valley finite difference model domain and boundary conditions (source: CSIRO, unpublished data)	28

Figure 9 	Extent of the DPI groundwater model. Also shown are previous DPI groundwater model extents (consistent with the IRM model domain) and the CSIRO sub-model region	29

Figure 10 	Spatial extent of the Victorian Onshore Natural Gas Water Science Studies groundwater model	31

Figure 11	Stylised conceptualisation of the dominant water balance components within the Gippsland region	36

Figure 12	Schematic flow conceptualisation of the Seaspray Depression showing the Baragwanath anticline and typical water quality variation offshore (source: DNRE unpublished data)	37

Figure 13 	Hydrogeological Unit (HGU) 1 representing the marine waters throughout the offshore portion of the modelled area	48

Figure 14 	Hydrogeological Unit (HGU) 2 representing the Quaternary Alluvium	48

Figure 15 	Hydrogeological Unit (HGU) 3 representing the Upper Tertiary Quaternary Alluvium	49

Figure 16 	Hydrogeological Unit (HGU) 4 representing the Upper Tertiary Quaternary Aquitard	49

Figure 17 	Hydrogeological Unit (HGU) 5 representing the Upper Tertiary Aquifer	50

Figure 18 	Hydrogeological Unit (HGU) 6 representing the Upper Tertiary Aquitard	50

Figure 19 	Hydrogeological Unit (HGU) 7 representing the Yallourn Coal seams	51

Figure 20 	Hydrogeological Unit (HGU) 8 representing the Yallourn interseam	51

Figure 21 	Hydrogeological Unit (HGU) 9 representing the Upper Mid-Tertiary Aquifer and Aquitard	52

Figure 22 	Hydrogeological Unit (HGU) 10 representing the Morwell 1A Coal Seam	52

Figure 23 	Hydrogeological Unit (HGU) 11 representing the interseam between the Morwell 1A Coal floor and the Morwell 1B Coal roof	53

Figure 24 	Hydrogeological Unit (HGU) 12 representing the Morwell 1B coal seams	53

Figure 25 	Hydrogeological Unit (HGU) 13 representing the Morwell 1B interseam between the Morwell 1B coal floor and Morwell 2 coal roof	54

Figure 26 	Hydrogeological Unit (HGU) 14 representing the Morwell 2 Coal Seam	54

Figure 27 	Hydrogeological Unit (HGU) 15 representing the Lakes Entrance Formation	55

Figure 28 	Hydrogeological Unit (HGU) 16 representing the Lower Mid Tertiary Aquifer	55

Figure 29 	Hydrogeological Unit (HGU) 17 representing the Lower Tertiary Basalts	56

Figure 30 	Hydrogeological Unit (HGU) 18 representing the Lower Tertiary Aquifer	56

Figure 31 	Hydrogeological Unit (HGU) 19 representing the Traralgon T0 and T1 coal seams and T0 inter-burden	57

Figure 32 	Hydrogeological Unit (HGU) 20 representing the Traralgon T1 aquifer and includes the T1 interseam	57

Figure 33 	Hydrogeological Unit (HGU) 21 representing the Traralgon T2 coal seams and minor interseams	58

Figure 34 	Hydrogeological Unit (HGU) 22 representing the Traralgon T2 inter-burden (aquifer) below the T2 coal seams	58

Figure 35 	Hydrogeological Unit (HGU) 23 representing the upper 500 m of the Strzelecki Group	59

Figure 36 	Hydrogeological Unit (HGU) 24 representing from 500 m to 1,000 m depth within the Strzelecki Group	59

Figure 37 	Hydrogeological Unit (HGU) 25 representing from 1,000 m to 2,000 m depth within the Strzelecki Group	60

Figure 38 	Hydrogeological Unit (HGU) 26 representing from 2,000 m to 3,000 m depth within the Strzelecki Group	60

Figure 39 	Hydrogeological Unit (HGU) 27 representing from 3,000 m to 4,000 m depth within the Strzelecki Group	61

Figure 40 	Hydrogeological Unit (HGU) 28 representing from 4,000 m to 7,000 m depth within the Strzelecki Group	61

Figure 41 	Hydrogeological Unit (HGU) 29 representing from 7,000 m to Palaeozoic Basement depth within the Strzelecki Group	62

Figure 42 	Hydrogeological Unit (HGU) 30 representing the Palaeozoic Basement	62

Figure 43 	Modelled 1970 to 2012 annual total extraction (ML/yr) by contribution. S&D refers to stock and domestic usage, Mine refers to mine dewatering, EG refers to east Gippsland usage and WG refers to West Gippsland usage	70

Figure 44 	State water account reporting regions (excluding Rosedale, Stratford and Yarram)	71

Figure 45 	Rosedale water account reporting regions (and associated depths)	72

Figure 46 	Stratford and Yarram water account reporting regions (and associated depths)	73

Figure 47 	Location of all modelled groundwater extraction bores (including stock and domestic bores)	74

Figure 48 	Location of modelled offshore production bores, mine dewatering and groundwater extraction bores (excluding stock and domestic bores)	75

Figure 49 	Location of extraction bores in modelled layer 2	76

Figure 50 	Location of extraction bores in modelled layer 3	77

Figure 51	 Location of extraction bores in modelled layer 4	78

Figure 52 	Location of extraction bores in modelled layer 5	79

Figure 53 	Location of extraction bores in modelled layer 6	80

Figure 54 	Location of extraction bores in modelled layer 9	81

Figure 55 	Location of extraction bores in modelled layer 17	82

Figure 56 	Location of extraction bores in modelled layer 18	83

Figure 57 	Location of extraction bores in modelled layer 19	84

Figure 58 	Location of extraction bores in modelled layer 23	85

Figure 59 	Location of extraction bores in modelled layer 30	86

Figure 60 	Thickness of modelled layer 1	87

Figure 61 	Thickness of modelled layer 2	88

Figure 62 	Thickness of modelled layer 3	89

Figure 63 	Thickness of modelled layer 4	90

Figure 64 	Thickness of modelled layer 5	91

Figure 65	Thickness of modelled layer 6	92

Figure 66 	Thickness of modelled layer 7	93

Figure 67 	Thickness of modelled layer 8	94

Figure 68 	Thickness of modelled layer 9	95

Figure 69 	Thickness of modelled layer 10	96

Figure 70 	Thickness of modelled layer 11	97

Figure 71 	Thickness of modelled layer 12	98

Figure 72 	Thickness of modelled layer 13	99

Figure 73 	Thickness of modelled layer 14	100

Figure 74 	Thickness of modelled layer 15	101

Figure 75 	Thickness of modelled layer 16	102

Figure 76 	Thickness of modelled layer 17	103

Figure 77 	Thickness of modelled layer 18	104

Figure 78 	Thickness of modelled layer 19	105

Figure 79 	Thickness of modelled layer 20	106

Figure 80 	Thickness of modelled layer 21	107

Figure 81 	Thickness of modelled layer 22	108

Figure 82 	Thickness of modelled layer 23	109

Figure 83 	Thickness of modelled layer 24	110

Figure 84 	Thickness of modelled layer 25	111

Figure 85 	Thickness of modelled layer 26	112

Figure 86 	Thickness of modelled layer 27	113

Figure 87 	Thickness of modelled layer 28	114

Figure 88 	Thickness of modelled layer 29	115

Figure 89 	Boundary conditions of modelled layer 1	119

Figure 90 	Boundary conditions of modelled layer 2	120

Figure 91 	Boundary conditions of modelled layer 3	121

Figure 92 	Boundary conditions of modelled layer 4	122

Figure 93 	Boundary conditions of modelled layer 5	123

Figure 94 	Boundary conditions of modelled layer 6	124

Figure 95 	Boundary conditions of modelled layer 7	125

Figure 96 	Boundary conditions of modelled layer 8	126

Figure 97 	Boundary conditions of modelled layer 9	127

Figure 98 	Boundary conditions of modelled layer 10.	128

Figure 99 	Boundary conditions of modelled layer 11	129

Figure 100 	Boundary conditions of modelled layer 12	130

Figure 101 	Boundary conditions of modelled layer 13	131

Figure 102 	Boundary conditions of modelled layer 14	132

Figure 103 	Boundary conditions of modelled layer 15	133

Figure 104 	Boundary conditions of modelled layer 16	134

Figure 105 	Boundary conditions of modelled layer 17	135

Figure 106 	Boundary conditions of modelled layer 18	136

Figure 107 	Boundary conditions of modelled layer 19	137

Figure 108 	Boundary conditions of modelled layer 20	138

Figure 109 	Boundary conditions of modelled layer 21	139

Figure 110 	Boundary conditions of modelled layer 22	140

Figure 111 	Boundary conditions of modelled layer 23	141

Figure 112 	Boundary conditions of modelled layer 24	142

Figure 113 	Boundary conditions of modelled layer 25	143

Figure 114 	Boundary conditions of modelled layer 26	144

Figure 115 	Boundary conditions of modelled layer 27	145

Figure 116 	Boundary conditions of modelled layer 28	146

Figure 117 	Boundary conditions of modelled layer 29	147

Figure 118 	Boundary conditions of modelled layer 30	148

Figure 119 	Location and extent of 1-in-100 year flooding events in the Gippsland region (source: Victorian Flood Database)	151

Figure 120 	Location and extent of major recent flooding events (source: the Victorian Flood Database)	152

Figure 121 	Drainage channels and monitoring sites in the Macalister Irrigation District	177

Figure 122 	Mean monthly flow (ML/month) at key gauges monitored by SRW in the Macalister Irrigation District	178

Figure 123 	Annual Flow (ML/yr) at key gauges monitored by SRW in the Macalister Irrigation District	179

Figure 124 	Surface-groundwater connectivity in the Gippsland region (source:  data compiled by DSE in 2012)	182

Figure 125 	Groundwater hydrographs of three nested bores located in MID west of Sale plotted against daily stream water level of Latrobe River at Rosedale (gauge station no. 226228)	186

Figure 126 	Groundwater hydrographs of a transect of bores located near Mitchell River west of Bairnsdale plotted against daily stream water level of Mitchell River at Rose (gauge station no. 224203)	186

Figure 127 	Groundwater hydrographs of two nested bores located near Snowy River west of Orbost plotted  against daily stream water level of Snowy River at Jarrahmond (gauge station no. 222200)	187

Figure 128 	Groundwater hydrographs of three nested bores located in the Macalister Irrigation District north of Sale plotted against daily stream water level of Avon River at Stratford (gauge station no. 225201)	187

Figure 129	Groundwater extractions applied to the model for 2013 to 2102	191

Figure 130	Lateral hydraulic conductivity (m/day) for modelled layer 2	212

Figure 131	Lateral hydraulic conductivity (m/day) for modelled layer 9	213

Figure 132 	Lateral hydraulic conductivity (m/day) for modelled layer 18	214

Figure 133 	Lateral hydraulic conductivity for modelled layer 19	215

Figure 134	Lateral hydraulic conductivity (m/day) for modelled layer 24	216

Figure 135	Vertical hydraulic conductivity (m/day) for modelled layer 2	217

Figure 136	Vertical hydraulic conductivity (m/day) for modelled layer 9	218

Figure 137	Vertical hydraulic conductivity (m/day) for modelled layer 18	219

Figure 138	Vertical hydraulic conductivity (m/day) for modelled layer 19	220

Figure 139	Vertical hydraulic conductivity (m/day) for modelled layer 24	221

Figure 140	Specific yield for modelled layer 2	222

Figure 141	Specific yield for modelled layer 18	223

Figure 142	Specific yield for modelled layer 19	224

Figure 143	Specific yield for modelled layer 24	225

Figure 144 	Location of observation bores used for model calibration	255

Figure 145	Spatial extent of zones used to calibrate modelled layer 1	257

Figure 146	Spatial extent of zones used to calibrate modelled layer 2	258

Figure 147 	Spatial extent of zones used to calibrate modelled layer 3	259

Figure 148 	Spatial extent of zones used to calibrate modelled layer 4	260

Figure 149 	Spatial extent of zones used to calibrate modelled layer 5	261

Figure 150 	Spatial extent of zones used to calibrate modelled layer 6	262

Figure 151 	Spatial extent of zones used to calibrate modelled layer 7	263

Figure 152 	Spatial extent of zones used to calibrate modelled layer 8	264

Figure 153 	Spatial extent of zones used to calibrate modelled layer 9	265

Figure 154 	Spatial extent of zones used to calibrate modelled layer 10	266

Figure 155 	Spatial extent of zones used to calibrate modelled layer 11	267

Figure 156 	Spatial extent of zones used to calibrate modelled layer 12	268

Figure 157 	Spatial extent of zones used to calibrate modelled layer 13	269

Figure 158 	Spatial extent of zones used to calibrate modelled layer 14	270

Figure 159 	Spatial extent of zones used to calibrate modelled layer 15	271

Figure 160 	Spatial extent of zones used to calibrate modelled layer 16	272

Figure 161 	Spatial extent of zones used to calibrate modelled layer 16	273

Figure 162 	Spatial extent of zones used to calibrate modelled layer 18	274

Figure 163 	Spatial extent of zones used to calibrate modelled layer 19	275

Figure 164 	Spatial extent of zones used to calibrate modelled layer 20	276

Figure 165 	Spatial extent of zones used to calibrate modelled layer 21	277

Figure 166 	Spatial extent of zones used to calibrate modelled layer 22	278

Figure 167 	Spatial extent of zones used to calibrate modelled layer 23	279

Figure 168 	Spatial extent of zones used to calibrate modelled layer 24	280

Figure 169 	Spatial extent of zones used to calibrate modelled layer 25	281

Figure 170 	Spatial extent of zones used to calibrate modelled layer 26	282

Figure 171 	Spatial extent of zones used to calibrate modelled layer 27	283

Figure 172 	Spatial extent of zones used to calibrate modelled layer 28	284

Figure 173 	Spatial extent of zones used to calibrate modelled layer 29	285

Figure 174 	Spatial extent of zones used to calibrate modelled layer 30	286

Figure 175 	Observed versus computed steady-state heads (mAHD)	289

Figure 176 	Residual error (observed head – modelled head) (m) for each steady-state calibration bore	289

Figure 177 	Simulated 1970 unconfined aquifer potentiometric surface (mAHD)	290

Figure 178 	Simulated 1970 steady-state depth to water table (m)	291

Figure 179 	1990 water table depth (m) within the West Gippsland CMA region	292

Figure 180 	Transient model mass balance components (m3/day per time step)	293

Figure 181 	Number of observation points throughout the simulation period	295

Figure 182 	Temporal trace of RMS error (%) throughout simulation period	295

Figure 183 	Temporal trace of mean residual error (m) throughout simulation period	295

Figure 184 	Temporal trace of mean absolute error (m) throughout the simulation period	296

Figure 185 	Spatial mean residual water level error (m) for 1980 based on observed and predicted heads	297

Figure 186 	Spatial mean residual water level error (m) for 1990 based on observed and predicted heads	298

Figure 187 	Spatial mean residual water level error (m) for 2000 based on observed and predicted heads	299

Figure 188 	Spatial mean residual water level error (m) for 2010 based on observed and predicted heads	300

Figure 189 	Sensitivity of the steady-state model to variations in recharge	302

Figure 190 	Sensitivity of the steady-state model to variations in potential groundwater evapotranspiration rate	303

Figure 191 	Sensitivity of the steady-state model to variations in river conductance	303

Figure 192 	Sensitivity of the steady-state model to variations in drain conductance	304

Figure 193	Predicted time varying mine dewatering extraction volume for the Hazelwood mine required to meet the CRDP targets. The predicted volume is the sum of the extraction from the base of the M2A seam (Haz_mine) and the aquifer below (Haz_below)	305

Figure 194	Predicted time varying mine dewatering extraction volume for the Loy Yang mine required to meet the CRDP pathway targets. The predicted volume is the sum of the extraction from the base of the M2A seam (Loy_mine) and the aquifer below (Loy_below)	306

Figure 195	Predicted time varying mine dewatering extraction volume for the Yallourn mine required to meet the CRDP targets. The predicted volume is the sum of the extraction from the base of the Yallourn coal seam (Yal_mine) and the aquifer below (Yal_below)	306

Figure 196	Average water table elevation within the open cut mine pits in response to CRDP mine work plans	307

Figure 197	2012 baseline depth to water table (m) from ground surface	308

Figure 198	Change in depth to water table (m) at 2042 under baseline conditions relative to 2012 condition	309

Figure 199	Change in depth to water table (m) at 2072 under baseline conditions relative to 2012 condition	310

Figure 200	Change in depth to water table (m) at 2102 under baseline conditions relative to 2012 condition	311

Figure 201	Change in depth to water table (m) at 2042 under CRDP conditions relative to 2042 baseline condition	312

Figure 202	Change in depth to water table (m) at 2072 under CRDP conditions relative to 2072 baseline condition	313

Figure 203	Change in depth to water table (m) at 2102 under CRDP conditions relative to 2102 baseline condition	314

Figure 204	Change in predicted annual baseflow (ML/yr) to the Latrobe River relative to the baseline condition	315

Tables

Table 1	Summary of key features of the top five ranked models by SKM (2011a) and the CSIRO model currently under development	33

Table 2	Groundwater model layers and associated Victorian Aquifer Framework (VAF) code, coal name and general description of stratigraphic groupings	38

Table 3 	Modelled hydrogeological units and associated Victorian Aquifer Framework (VAF) code, derivation method and thickness	42

Table 4	Summary of Confidence Level Classification criteria adopted in the Australian Groundwater Modelling Guidelines (Barnett et al. 2012)	65

Table 5	Gippsland Model appraisal based on the abbreviated guideline (Table 4)	65

Table 6 	Victorian Water Accounts (usage ML)	68

Table 7 	Modelled extractions within each GMA and WSPA reporting region	69

Table 8 	Source of time-series data incorporated into the groundwater model	117

Table 9 	Extents of historical flood events (sourced from the Victorian Flood Database)	153

Table 10 	Modelled flood extents and dates	154

Table 11	Spatially averaged groundwater potential evapotranspiration and recharge for each modelled time period	155

Table 12 	River gauges for East Gippsland	160

Table 13 	River gauges for Latrobe Basin	162

Table 14 	River gauges for Mitchell Basin	166

Table 15 	River gauges for Snowy Basin	167

Table 16 	River gauges for South Gippsland region	169

Table 17 	River gauges for Tambo Basin	172

Table 18 	River gauges for Thomson Basin	173

Table 19 	Gauges monitoring drain from the Macalister Irrigation District (sourced from SWR)	178

Table 20 	Estimated average baseflow index (BFI) (by season and annual) in 51 unregulated catchments in the Gippsland region (sourced from SKM 2012b)	183

Table 21 	Average annual rainfall estimates for the Gippsland Basin derived using various global climate models assuming a global warming of 1oC	189

Table 22	Range and value of specific yield (Sy), specific storage (Ss) and lateral hydraulic conductivity (Kxy) initially assigned to each modelled layer in the Gippsland model	193

Table 23	Calibrated model aquifer parameterisation	210

Table 24	Calibration bore attribution	227

Table 25	Statistical measure of model performance	287

Table 26	Steady-state mass balance predictions	288

Table 27	Steady-state model calibration statistics	288

Table 28	Transient model calibration statistics	294

Table 29	Key model assumptions	316

### 2.6.2	Groundwater numerical modelling for the Gippsland Basin bioregion

This product presents the modelling of groundwater quantity and availability within the Gippsland Basin subregion or bioregion. It describes:

the modelling undertaken

results for hydrological response variables identified in the conceptual model of causal pathways (product 2.3)

connections with surface water

limitations of the analysis.

Coal and coal seam gas development can potentially affect assets and receptors (either negatively or positively) through direct, indirect or cumulative impacts on groundwater hydrology. An assessment of these impacts on receptors in the Gippsland Basin bioregion has not been undertaken as part of the Gippsland Basin Bioregional Assessment, as per the revised Schedule with the Office of Water Science in 2015.

#### Methods

Summary

The bioregional assessment of the Gippsland Basin was based on results derived using a combination of a detailed multi-layered regional groundwater model and a catchment runoff model. This modelling approach was similar to the methodology adopted for the onshore natural gas water science study commissioned by the Victorian Department of Environment, Land, Water and Planning.

The catchment runoff model simulated the quickflow component of streamflow, calculated as the sum of runoff and sub-surface later flows, whereas the regional groundwater model simulated the groundwater baseflow contribution to streamflow. The total streamflow impact was the sum of these quickflow and baseflow contributions. The regional groundwater model also simulated the groundwater level within regions of interest.

The difference in simulated streamflow and groundwater level between a baseline condition and future expanded Coal resource development pathway of open-cut coal mine operations was evaluated and used to estimate the impact of the coal resource development pathway on groundwater dynamics and interactions with surface features.

A review of coal resource development pathways (CRDPs) in the Gippsland Basin, as outlined in technical product 2.3, identified three proposed developments from which model scenarios could be evaluated based on the continued expansion of the Yallourn, Hazelwood and Loy Yang open-cut coal mines. This review formed the context and construct of the modelling methodology and identified that large coal mining development has the potential to directly affect the regional groundwater system and that coal mine development can potentially affect assets and receptors (either negatively or positively) through a direct, indirect or cumulative impact on groundwater hydrology. Any impact on the groundwater systems in connection with the shallow alluvium aquifers has the potential to affect streamflow (and therefore surface water resources) within the Latrobe River basin in which the open-cut coal mines are located. Given the proximity of the Yallourn coal mine to the Latrobe River, and the Hazelwood coal mine to the Morwell River, it is anticipated that expanded open-cut coal mine operations will reduce runoff and groundwater baseflow which has the potential to affect streamflow directly.

The river networks within the Gippsland Basin are highly regulated. Numerous existing complex river routing models are used to inform surface water management. Explicit models exist for the Latrobe, Thomson, Mitchell and Avon rivers which account for diversion, releases and bypass flows. It was considered that the modelling of river management, or the routing of streamflow through the river networks, using the suite of existing river models was not necessary as the salient features of streamflow can be simulated solely using the combination of a catchment runoff model (to predict quickflow impacts) and a distributed groundwater model (to predict baseflow impacts).

A model, or model sequence, is needed that can simulate the impact of the coal resource development pathway upon the identified regional groundwater system and the stream network. Developing a single, coupled and integrated surface water and groundwater model is beyond the scope of this assessment. Therefore, for the bioregional assessment of the Gippsland Basin, a combination of two predictive numerical models were utilised consisting of a regional multi-layered groundwater model and a catchment runoff model. The modelling approach was based on the methodology adopted for the onshore natural gas water science study commissioned by the Department of Environment, Land, Water and Planning (Beverly et al., 2015).

The catchment runoff model adopted in the study was the Catchment Analysis Tool (CAT) (Beverly et al., 2005; Beverly, 2009) which comprises a suite of farming system models to estimate daily water balances. The CAT model simulates farm management units linked within a catchment framework with allowance for landscape connectivity and connection to a distributed, multi-layered groundwater model. The farm-scale models account for position in the landscape (topography, soil type, aspect and slope), climate, land use and land management and simulates water balance, nutrient transport and vegetative production on a daily time step. Daily recharge and evapotranspiration estimates from this model were used as inputs into the underlying distributed groundwater model.

The regional distributed groundwater model was based on MODFLOW-2005. This model was developed to simulate (1) the change in drawdown on economic receptors (groundwater bores) and ecological receptors (groundwater-dependent ecosystems); (2) the change in groundwater level within regions of interest; and (3) the change in surface water – groundwater flux. The predicted surface discharge flux from groundwater was then coupled with runoff and sub-surface lateral flow estimates derived from the CAT to predict total streamflow.

In the baseline model the impact of the existing coal mines (as defined by current extent and extractions) and their future closure pathways are simulated under modified climate conditions. The CRDP simulation includes the continued expansion of the Yallourn, Hazelwood and Loy Yang open-cut coal mines in accordance with mine planning proposals until mine closure. Both the baseline and CRDP mine closure dates are the same. The difference between the baseline and CRDP simulated streamflow and groundwater drawdowns is known as the additional coal resource development (ACRD).   The changes in streamflow and groundwater drawdown simulated under the ACRD are used to estimate the potential impact of the coal resource development pathway on the economic and ecological receptors within the reporting regions of interest. The details of the implementation of the CRDP are documented in Section 2.6.2.2.5 and includes an outline of the future climate change signal used to generate the daily modified climate conditions. The total streamflow impact between the baseline and coal resource development pathway conditions is calculated as the sum of the difference in the CAT simulated contribution to stream (defined as the sum of runoff and sub-surface lateral flows) and the MODFLOW-2005 simulated baseflow for each scenario. The time series of streamflow difference are summarised in a number of hydrologically relevant zones and the primary surface water receptor identified as the Latrobe River.

A comprehensive outline of the technical detail of the conceptualisation, parameterisation and CRDP implementation are documented in Section 2.6.2. Sensitivity analysis was undertaken to assess the impact of varying selected model input parameters on key modelled outputs. However, due to the complexity of the modelling framework, uncertainty analyses were not undertaken.

#### Review of existing models

Summary

A review of the existing groundwater management tools developed for the Gippsland Basin (SKM 2011a) commissioned by the then Victorian Department of Sustainability and Environment (DSE) identified 18 pre-existing groundwater models, ranging in complexity from impact assessment frameworks to distributed physics-based numerical models. Subsequent to this review, the DPI Gippsland Basin Model (Beverly et al., 2012) was developed based on the IRM Gippsland Basin model (Schaeffer, 2008). More recently, the CSIRO has developed a Latrobe Valley Coal Model (LVCM) to assess the likely water management issues associated with potential coal seam gas extractions in the Latrobe Valley (Freij-Ayoub et al., 2011a). The most recent regional groundwater model was developed for the Victorian Onshore Natural Gas Water Science Studies (VWSS) to assess the potential impacts of future onshore gas extractions and to understand the possible impacts of a potential onshore natural gas industry on groundwater and surface waters within the Gippsland region. This document summarises the key attributes of each model and has identified that the most recent regional groundwater model has the required spatial extent, vertical resolution and model grid to evaluate the bioregional assessment.

##### Introduction

A review of the existing groundwater management tools developed for the Gippsland Basin (SKM, 2011a) commissioned by the then Victorian Department of Sustainability and Environment (DSE) identified 18 models ranging in complexity from impact assessment frameworks to distributed physics-based numerical models. The models reviewed were:

- Stage 1 (Fraser, 1980) and review of Stage 1 report (Golder Associates, 1990)

- Stage 2 (Evans, 1983)

- Stage 3 (Bolger, 1987)

- Reservoir simulation of the Gippsland Basin (Henzell et al., 1985)

- Loy Yang (Bolger, 1990; Golder Associates, 1990)

- Latrobe Valley Resource Model — Update (Golder Associates, 1991)

- Latrobe Valley Resource Model — Update (Golder Associates, 1992a)

- Latrobe Valley Resource Model — Update (Golder Associates, 1992b)

- Latrobe Valley Resource Model — Update (Golder Associates, 1992c)

- Stage 4 (GeoEng, 1994)

- Stage 5 (GeoEng, 1996a)

- Gippsland Basin (SKM, 1995)

- Gippsland Basin (SKM, 1996)

- Yarram sub-regional model (SKM, 1999)

- Gippsland Basin (Nahm, 2002)

- Sale WSPA Groundwater Model (SKM, 2011a)

- Integrated Resource Model (IRM) of the Gippsland Basin (Schaeffer, 2008)

- ecoMarkets (GHD, 2008; 2010a; 2010b; 2010c).

Subsequent to the SKM review, the CSIRO developed a Latrobe Valley Coal Model (LVCM) to assess the likely water management issues associated with potential coal seam gas extractions in the Latrobe Valley (Freij-Ayoub et al., 2011a) and the DPI Gippsland Basin Model (Beverly et al., 2012) was developed to assess the long-term, equilibrium impacts of likely coal seam gas extractions and expanded open cut mining activity on aquifer potentiometric surfaces. The DPI Gippsland Basin Model was based in part on the IRM Gippsland Basin model (Schaeffer, 2008). At the time of this study, the most recent regional groundwater model was that utilised for the Victorian Onshore Natural Gas Water Science Studies (VWSS) to assess the potential impacts of future onshore gas developments and to understand the possible impacts of a potential onshore natural gas industry on groundwater and surface waters within the Gippsland region (Beverly et al., 2015). Summarised below are the key features of those models considered to have sufficient extent, grid resolution, design focus and attribution relevant to this study.

##### Review of the Integrated Resource Model

The Integrated Resource Model (IRM) was developed by Schaeffer (2008) to simulate the impacts of different groundwater abstractions and multiple pumping and artificial recharge scenarios on groundwater levels within the Gippsland Basin. The model represented 18 layers comprising the unconfined surface layer, the basement layer, eight regional aquifers and eight regional aquitards, as shown in Figure 1. Time dependent general head boundaries were assigned to the eastern and southern regions of the model domain, with no flow boundaries assigned to the western and northern regions of the model, as shown in Figure 2 and Figure 3. Spatially variable hydraulic conductivities were assigned based on lithological analysis and structural regions. Recharge was assigned to sub-cropping aquifers and groundwater extractions were incorporated to account for offshore pumping, coal mine dewatering and irrigation in the Yarram region.

Calibration of this model was based on matching simulated heads and groundwater trends with observation bore data from 1960 to 1999. The error criterion was 10 m with model validation based on data from 2000 to 2004.

The recommendations for future improvement of this model as reported by SKM (2011a) included improved calibration of the unconfined aquifer, refinement of the recharge input and extension of the eastern and southern boundaries to reduce boundary-induced effects when assessing the impacts of artificial recharge and pumping adjacent to these boundaries.

Figure 1 	Grouping of hydrostratigraphic units incorporated into the IRM model layer structure (sourced from Schaeffer 2008)

Figure 2	Location of active cells and IRM model domain (sourced from Schaeffer 2008)

Figure 3	IRM boundary conditions for Layer 1 (source: Schaeffer 2008)

##### Review of the ecoMarkets groundwater models

The ecoMarket groundwater models (GHD, 2008; 2010a) were developed to assess potential impacts on depth to the water table and baseflows for a range of land use change scenarios. Two models were developed with identical specifications: one for the West Gippsland Catchment Management Authority (CMA) region and one for the East Gippsland CMA region. Both models extend offshore to the freshwater–seawater interface. The offshore extent of the models was based on structural and stratigraphic mapping. Each model was constructed in MODFLOW (Harbaugh A W 2005) using a 200 m by 200 m grid and consisted seven layers.

In the case of the East Gippsland model, transient head boundaries were used offshore so the effects of offshore oil and gas pumping could explicitly be simulated at the southern model boundary, as shown in Figure 4. Stream cells were used over a large proportion of the high ground area to simulate the streams and rivers, whereas river cells were used to simulate lakes. General head boundary cells were set at an average sea level (0.1 mAHD) across the upper most active layer in all offshore areas. The peer reviewer concluded these boundary conditions were appropriate (PB, 2010a). However, it is noteworthy that the steady-state model for this area did not converge and consequently had a large water balance error. That is, an adequate steady-state calibration was not achieved for the East Gippsland ecoMarkets groundwater model, which has implications for the robustness of the associated transient model.

A review of the East Gippsland model identified that the ability of the model to simulate the actual groundwater flow systems was constrained by a lack of data to define those flow systems over the majority of the modelled area (PB, 2010a). Consequently, the usefulness of this model is effectively limited to a proportion of the lowland areas. The part of the model covering the elevated areas representing the Mesozoic basement outcrop is not reliable and is considered to be of limited use. Even in the lowland areas, the model has significant limitations regarding its primary purpose. The limitations relate to the effects of land use changes, which are likely to be poorly predicted because in many areas the computed heads, hydraulic gradients, trends in heads, and responses to stresses do not match the observed data. As reported by PB (2010a), more effort would be required in the calibration of localised areas for the East Gippsland ecoMarkets groundwater model to be fit for purpose in all areas for which groundwater level data are available.

Figure 4	Domain and boundary conditions for the ecoMarkets East Gippsland groundwater model (source: GHD 2010c)

In the case of the ecoMarkets West Gippsland groundwater model, the model extends to the onshore topographic divides across which no-flow boundaries are assigned, as shown in Error! Reference source not found.. The eastern boundary extends into East Gippsland along the Mitchell River, and the western boundary extends to the offshore freshwater - saltwater interface, along which a no-flow boundary is assigned that forces groundwater to flow up along the interface and discharge far out to sea. The offshore extent of the model is based on structural and stratigraphic mapping.

The shoreline is simulated in Layer 1 only, with a density corrected head of 0.1 mAHD. Stream cells representing third order and higher order streams are used over a large proportion of the high ground area, whereas first and second order streams are represented as drain cells. River boundaries are also used to represent surface water bodies on the lowlands. Time-varying drain cells with high conductance values (10,000 m2/d) were used to represent the dewatering of coal mines. A reviewer (PB, 2010b) noted that, while this is normal practice, it was not clear why this approach was not successful in reducing adjacent heads to observed levels, given that some heads were identified as being 100 m too high.

Steady-state calibration was based on 1970s pre-development water levels and 1957–2005 average recharge. While a steady-state solution was achieved, no comparative groundwater level contour maps were shown to demonstrate the degree of agreement with the observed groundwater level contour maps. The transient model calibration period was from 1970 to 1985, with monthly stress periods. A warm-up period was run from 1960 to 1969 to provide more sensible initial conditions (than steady-state heads) in a system disturbed by mining and characterised by declining heads. Verification was undertaken from 1986 to 1990. From the statistics and other calibration evidence, PB (2010b) concluded that the model was fit for the purpose as specified for the ecoMarkets study, which focused on the water table aquifer, but could not be used reliably in deeper layers where coal-mining effects are evident in observed datasets.

The peer review of the West Gippsland model by PB (2010b) highlighted that the West Gippsland model was unique among the ecoMarkets models in having to simulate extensive dewatering associated with large coal mining. The review identified that depressurisation caused by coal mining was poorly simulated and consequently the groundwater model could not be recommended for broader water resource management purposes. The review stated that the model might be suitable for this purpose in areas very distant from the mines. In conclusion, the review stated that there is uncertainty over the usefulness of the West Gippsland ecoMarkets groundwater model for the water resource management of deeper aquifers, given the poor simulation of mine depressurisation. This conclusion reinforces the limitation and suitability of this model to evaluate the bioregional assessment and mine depressurisation scenarios.

Figure 5	Domain and boundary conditions for the ecoMarkets West Gippsland groundwater models (source: GHD, 2010b)

##### Review of the CSIRO groundwater model

The CSIRO Gippsland Basin model was developed in collaboration with DPI Victoria (Geological Survey of Victoria) to assess coal seam gas water management in the Gippsland Basin. The objective of the study was to build and show the capability of the CSIRO in assessing the impact of coal seam gas production on aquifer pressure heads. The study also provided information related to the risk of land subsidence adjacent to coal seam gas production wells. The area selected for the study was a region in the Gippsland Basin with abundant coal mining activity.

The developed model qualitatively predicted the amount of recovered methane and water from single or multiple coal seams which could be separated by either an aquifer or a fine grained clayey-silty material typically considered to be an aquitard. The model coupled geomechanics and fluid flow and as such was capable of predicting possible land subsidence associated with reservoir depletion due to coal seam methane production. The modelling used the FLAC3D axisymmetric finite difference software, which assumes single-phase flow under which water and desorbed gas is modelled as one fluid. That is, the desorbed methane is treated as a chemical species carried with the fluid whereby the transport of methane is controlled by advection and diffusion in water. This model simulates the simultaneous production of two coal seams 50 m and 44 m in thickness separated by another 12 m thick layer of sediments (Freij-Ayoub et al., 2011b). The modelling explores the influence of coal permeability on the quantities of produced methane and water in a well bore.

In addition to the bore modelling using FLAC3D, the CSIRO study developed a numerical groundwater model based on Schaeffer (2008) to assess the impact of economically viable methane gas extractions on the hydraulic heads in aquifers underlying coal seams in the Latrobe Valley (Figure 6). This large-scale study used MODFLOW, which is limited to single phase water production. As such, the impact of methane extraction on the hydraulic heads in the produced or adjacent layers was predicted to be negligible at the basin-wide scale modelled (Freij-Ayoub et al., 2011b). The MODFLOW model considered a domain of 291 km by 186 km with a uniform grid resolution of 1 km by 1 km, and adopted a 10-layered simplified stratigraphic representation of the Gippsland Basin as shown in Figure 7. Of note was that the model assumed that the hydraulic head is hydrostatic. Constant head boundary conditions were assigned based on the assumption that the edge-effect boundary impacts associated with the scenario modelling would be negligible. The domain and boundary conditions are shown in Figure 8. In addition to the large-scale basin-wide model, a smaller scale model was developed specifically for the Latrobe Valley with a domain of 40 km by 20 km with a uniform grid resolution of 100 m by 100 m (Freij-Ayoub et al., 2011b). This model estimated the impact of coal seam gas extractions on hydraulic heads in the Latrobe Valley. In this case, the extraction estimates were predicted using the COMET3 modelling package.

Figure 6	Locality map of the CSIRO large-scale model domain (source: CSIRO, unpublished data)

Figure 7	Conceptual model layers adopted in the CSIRO large-scale model (source: CSIRO, unpublished data)

Figure 8	Latrobe Valley finite difference model domain and boundary conditions (source: CSIRO, unpublished data)

##### Review of DPI Gippsland Basin groundwater model

The DPI Gippsland Basin model (Beverly et al., 2012) was developed under a collaborative joint project involving the Natural Resources Management and Economics Division (NRME, Vic DPI) and Future Farming Systems Research (FFSR, Vic DPI) with input from GeoScience Victoria (Vic DPI) and the CSIRO Earth Sciences and Resources Engineering Division. The aim of the project was to develop and calibrate an integrated catchment model to assess the likely future impact on the water resource and agricultural sector within the Gippsland Basin due to altered extraction regimes from existing groundwater users, anticipated coal seam gas extractions and proposed energy and earth resource development.

The DPI groundwater model was based on the IRM model developed by Schaeffer (2008) and some offshore elements of the CSIRO basin-wide scale model (Freij-Ayoub et al., 2011b). The model was constructed in MODFLOW and comprised 18 layers of which layers 1–17 were similar to the IRM model extent with the addition of layer 18 representing the basement layer extending to the upland regions of the Gippsland Basin. Key refinements to the original IRM model included (1) finer uniform grid resolution of 200 m, (2) the incorporation of some offshore aquifer stratigraphic information as provided by the CSIRO and Earth Resources and (3) incorporation of a basement layer representing the upland outcropped regions of the Gippsland Basin. The extent of the model is shown in Figure 9.

The objective of this study was to assess the long-term, equilibrium impacts of likely coal seam gas extractions and expanded open cut mining activity on aquifer potentiometric surfaces. As such, the modelling assumed pseudo steady-state conditions based on observed conditions in year 2000.

The DPI model incorporated constant head and no-flow boundary conditions. A no-flow boundary condition was assigned along the onshore northern and western fringes of all layers as these boundaries represented the topographic divides of bedrock outcrops; fixed head boundary conditions were assigned along the coast. A constant head boundary of 0.7 m was assigned to all offshore layer 1 cells representing a density corrected hydrostatic head of the ocean in the offshore area.

Groundwater extractions were based on groundwater licence data provided by Southern Rural Water whereas the offshore extractions were as compiled by Schaeffer (2008). Measured time-varying extractions representing the Latrobe Valley mines were derived from the LVCM model, which also includes the Sale public water supply and the Australian Pulp Mill at Maryvale. In total the DPI groundwater model accounted for 179 groundwater users.

Recharge, potential groundwater evapotranspiration and groundwater extinction depths were derived using the Catchment Analysis Tool (CAT) (Beverly, 2009) which uses a combination of a suite of farming system models linked within a catchment framework with allowance for landscape connectivity and connection to a distributed, multi-layered groundwater model. The farm-scale models account for position in the landscape (topography, soil type, aspect and slope), climate, land use and land management to simulate water balance components (including recharge) and soil/water/plant interactions on a daily time step. The groundwater extinction depth controls the depth below which no groundwater evaporation demand is permitted. The groundwater extinction depths (as with evapotranspiration rates) were spatially varied based on the differing rooting depths of species used in the farming-systems models associated with the overlying land-use as derived using the CAT model.

Figure 9 	Extent of the DPI groundwater model. Also shown are previous DPI groundwater model extents (consistent with the IRM model domain) and the CSIRO sub-model region

##### Review of the Victorian Onshore Natural Gas Water Science Studies groundwater model

The Victorian Onshore Natural Gas Water Science Studies (VWSS) was commissioned in 2015 to assist the Victorian Government to determine if the existing moratorium on onshore gas exploration and fracking should be changed based on a review of associated water science impacts. This study required the development of a groundwater model for the Gippsland Basin (both on and offshore) to investigate and understand the potential impacts of future onshore gas developments and to understand the possible impacts of a potential onshore natural gas industry on groundwater and surface waters within the Gippsland region. The groundwater model (Beverly et al., 2015) was constructed in MODFLOW-2005 and adopted a uniform spatial resolution of 400 m with a spatial extent of 6,698,000 ha (66,980 km2), of which 3,629,000 ha (36,290 km2) exists onshore and 3,069,000 ha (30,690 km2) offshore, as shown in Figure 10. The model layer structure and attribution were based on a combination of (1) new stratigraphic mapping and interpretation developed by Geological Survey of Victoria, (2) the Victorian Aquifer Framework (VAF) (SKM, 2011b; GHD, 2012), (3) previous groundwater model data, and (4) existing maps and cross-sections.

The model comprised 30 layers and accounted for coal seams specified as discrete layers and Cretaceous sediments of the Strzelecki formation in discrete sub-layers. Key improvements to the previous DPI Gippsland Basin groundwater model include (1) new interpreted data sets that extend to the catchment boundaries, (2) finer grid resolution, (3) the incorporation of offshore stratigraphic information as developed by the CSIRO and Geological Survey of Victoria, and (4) incorporation of a basement layer representing the rocks underlying the sedimentary basin and extending to the upland regions of the Gippsland region where the basement outcrops.

The transient calibration period adopted annual stress periods from 1970 to 1990, and thereafter monthly stress periods throughout the 1991–2012 calibration/verification periods. These model stress periods were based on adequately capturing historical onshore and offshore extractions, specifically the Latrobe Valley coal mine and offshore oil and gas production wells. To this end, pre-1990 stress periods were assigned as annual within which 12 time-steps were adopted. For the more recent post-1990 calibration/verification period the stress period was based on quarterly time-steps so as to utilise seasonal water level oscillation and extractions data.

The assigned boundary conditions were based on the DPI groundwater model (Beverly et al., 2012) and included no-flow, well, river, drain and constant head boundary conditions. Recharge, potential groundwater evapotranspiration and groundwater extinction depths were derived using the Catchment Analysis Tool (CAT) (Beverly, 2009). Flood induced episodic recharge was simulated in the model by maintaining saturation in the surface soil layer in all regions within mapped flood extents. The duration of inundation was arbitrarily set as 5 days, unless specific information to the contrary was available.

A total of 8175 groundwater extraction bores were incorporated into the model representing groundwater usage bores, mine dewatering bores, stock and domestic bores and offshore production bores. A total of 22,573 river cells were also included in the model representing the main stems of the major rivers. The classification associated with the river spatial data layer was used to vary river incision depth (depth below the ground surface as defined by the digital elevation model) and width attributes. In the absence of recorded stage height information, river classification was used to estimate river stage heights. Drainage channels and man-made drainage features in the Macalister Irrigation District (MID) were included in the model based on available drainage network and groundwater discharge mapping and monitoring.

Seven hypothetical tight and shale gas and coal seam gas (CSG) development scenarios were considered under a dry future climate. Each scenario was designed to estimate the likely water usage of onshore gas development under varying well field designs and configurations.

An external review of the model concluded that the model was conceptualised and calibrated at a design level that meets the requirements of a moderate complexity regional scale groundwater model as defined in the Murray-Darling Basin Commission Groundwater Flow Modelling Guidelines (Middlemis, 2000) and meets the requirements of a Class 2 model confidence level classification as defined in the National Water Commission Australian Groundwater Modelling Guidelines (Barnett et al., 2012).

Figure 10 	Spatial extent of the Victorian Onshore Natural Gas Water Science Studies groundwater model

##### Summary of existing groundwater model attributes

A summary of the relevant model attributes and considerations required for this study is presented in Table 1.

For each of the top five ranked models as reported by SKM (2011a) and including the CSIRO (Freij-Ayoub et al., 2011a), DPI (Beverly et al., 2012) and VWSS (Beverly et al., 2015) models, a score is assigned based on the original ranking procedure adopted in SKM (2011a) using a Multi Criteria Assessment (MCA) approach involving the following steps:

definition of a set of criteria

assignment of weightings to the various criteria

identification of performance objectives for the agreed criteria

scoring and ranking of the alternatives.

An aggregated performance rating of 1 to 3 was used in which 1 refers to does not meet the set of criteria, 2 satisfies more than 50% of the criteria and 3 satisfies all criteria. It is noteworthy that the SKM (2011a) study compiled results from two independent reviewers.

In general, the MCA analysis identified that, with the exception of the VWSS groundwater model, no prior groundwater management tool adequately satisfies all of the model specifications required for this project. Specifically, the MCA analysis identified that the most recent regional groundwater model (VWSS) had the required spatial extent, vertical resolution and model grid to evaluate the bioregional assessment.

Table 1	Summary of key features of the top five ranked models by SKM (2011a) and the CSIRO model currently under development

| Attribute | Yarram sub-regional (SKM 1999) | Gippsland Basin (Nahm 2002) | Sale WSPA (SKM 2008) | Integrated Resource IRM (Schaeffer, 2008) | ecoMarkets (GHD, 2008, 2010a) | CSIRO (Freij-Ayoub et al., 2011a) | DPI (Beverly et al., 2012) | VWSS (Beverly et al., 2015) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extent focus | Onshore | offshore | onshore | onshore, offshore | Onshore | offshore | onshore | onshore, offshore |
| Inclusion of offshore extractions | 1 | 2 | 1 | 3 | 1 | 3 |  | 3 |
| Representation of within-Basin structural features | 2 | 2 | 2 | 2 | 2 | 2 |  | 2 |
| Number of layers | 4 | 7 | 4 | 18 | 7 | 10 | 18 | 30 |
| Grid resolution | 500–2,000 m | ? | 1 km | 1 km | 200 m | 1 km | 200 m | 400 m |
| Representation of major aquifers and aquitards | 1 | 2 | 1 | 3 | 2 | 3 | 3 | 3 |
| Representation of both onshore and offshore hydrogeology | 2 | 2 | 0 | 3 | 2 | 3 | 3 | 3 |
| Representation of appropriate boundary conditions | 1 | 2 | 1 | 3 | 3 | 3 | 3 | 3 |
| Accurate representation of groundwater/surface water interface | 1 | 2 | 2 | 2 | 1 | 2 | 3 | 3 |
| Ability to represent abstraction and injection wells | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
| Ability to determine extraction impacts on environment/users | 2 | 2 | 2 | 3 | 2 | 3 | 3 | 3 |
| Ability to identify shallow water tables | 2 | 2 | 2 | 3 | 2 | 3 | 3 | 3 |
| Ability to simulate steady-state conditions | 3 | ? | 3 | 3 | 2 | 3 | 3 | 3 |

#### Model development

Summary

To assess the impact of coal resource development on groundwater receptors, water table elevations and surface water groundwater exchange fluxes within the Gippsland Basin, a distributed, multi-layered groundwater model was developed in MODFLOW-2005. The groundwater model comprises 30 layers representing the key aquifers, aquitards and coal seams. Recharge and potential groundwater evapotranspiration are derived using a catchment modelling framework that accounts for land management practices at variable land management scales. The regional groundwater model incorporates comprehensive usage data to account for groundwater extractions, stock and domestic, mine dewatering and offshore production demands.

##### Objectives

The regional distributed groundwater model was developed to help quantify the potential groundwater and surface water impacts of coal resource development pathways in the Gippsland region. The model will be used to quantify impacts arising from potential developments (both individually and cumulatively) including:

the drawdown in hydraulic heads

the reduction in baseflow to rivers that drain the basin

the reduction in water availability to groundwater-dependent ecosystems.

The minimum specifications required to meet the project objectives were:

uniform finite difference grid of no more than 400 m cell size

model domain represents the entire Victorian Gippsland region and extends offshore to adequately represent oil and gas extractions

30 modelled layers representing the major geological units and consistent with existing documented hydrostratigraphic units

822 groundwater monitoring observation bores used for calibration

8,175 groundwater pumping bores representing licensed pumping, domestic and stock usage and offshore oil and gas platforms

recharge estimates (steady-state and transient) generated using the Catchment Analysis Tool (CAT) (Beverly, 2009) farming system models and provided by DEDJTR

allowance for episodic flood recharge events

groundwater evapotranspiration extinction depth to be based on land use

groundwater evapotranspiration rate to account for unsaturated vegetation evapotranspiration rates such that the summed saturated and unsaturated evapotranspiration does not exceed potential evapotranspiration as calculated from meteorological data

a normalised (scaled) root mean squared error (RMS) of less than 5% for the steady-state model based on matching hydraulic head observation data

a normalised (scaled) RMS of less than 10% for the transient model based on matching hydraulic head observation data at selected and agreed sites

existing mapped depth to water table and groundwater baseflow estimates to be considered

a transient split calibration/validation period of 10 and 13 years respectively (1990–99 and 2000–12)

a sensitivity analysis to assess the variability of modelled outputs to variations in key model input parameters

catchment-scale model water balance error of less than 2%

all significant catchment water balance features to be considered and reported

the source and a statement of quality of all input data sets to be reported

accuracy in drawdown outputs within 5 to 10 m

an annual time step for the simulation period 1970–1989 and thereafter monthly time steps for the simulation period 1990–2012 and beyond, which was considered to be sufficiently small to determine adverse impacts during periods of low surface water flow but also mindful of the impact on model run time.

##### Hydrogeological conceptual model

A schematic conceptual understanding of the water balance and hydrogeology within the Gippsland region is presented in Figure 11. The important water balance features considered within the model include:

- groundwater recharge

groundwater inflow and outflow

- groundwater abstraction

- groundwater evapotranspiration

- groundwater – surface water interaction.

Figure 11	Stylised conceptualisation of the dominant water balance components within the Gippsland region

The regional groundwater flow conceptualisation considers the Tertiary aquifers in the Gippsland region as behaving as three distinct flow systems underlain by the regional basement aquifer of the Strzelecki Group. The most significant of the Tertiary aquifers are the Moe Sub-basin, Latrobe Valley, Seaspray Depression (in which the catchments of the Thomson, Latrobe and Mitchell rivers occur) and Southern Terrace (which includes the catchment of the Tarra River).

Flow in the Northern Terrace and Seaspray Depression aquifers is dominantly west to east, with limited lateral interconnectivity due to both faulting and the Baragwanath Anticline (see Figure 12). Flow in the southern terrace is topographically driven off the southern slopes of the Strzelecki ranges.

Hatton et al (2004) noted the widespread declines in aquifer pressures in the Seaspray Depression ‘are clearly associated to some large but geographically variable degree with offshore oil and gas production’. These declines and the cone of depression associated with dewatering at the Loy Yang and Hazelwood mines are distinctly disconnected responses. The other basins are the Tarwin Basin (included in the model domain) which is effectively a disconnected basin flowing south, and the Westernport Basin flowing westerly towards Westernport Bay (and is not included in the model domain).

The conceptual model considers flows in the exposed meta-sediments in the high-relief areas of the Strzelecki and Eastern Highlands are primarily local flow systems discharging to nearby streams with some recharge to the basement rocks. Regional scale flows occur in deeper tertiary units where recharge primarily occurs on the margins of basins where the units sub-crop or outcrop, dominated by rainfall (GHD, 2010d).

Coal, silt and clay layers are effective aquitards over the region. However, the complex depositional history results in regional scale connections between aquifers within coal seams. The Balook Formation in particular provides a significant connection between the Morwell and Traralgon Formation aquifers and the Boisdale Aquifer.

The aquifers of the Latrobe Formation contain good quality water for many kilometres offshore indicating this groundwater is formation water associated with deposition and a result of long-term flow from onshore. However, near the seaward margin much higher salinities are encountered which are considered to pre-date the onset of oil and gas production and represent formation water.

Figure 12	Schematic flow conceptualisation of the Seaspray Depression showing the Baragwanath anticline and typical water quality variation offshore (source: DNRE unpublished data)

##### Design and implementation

The model consists of 30 layers and includes coal seams similar to those represented in the IRM model in addition to a basement layer that extends to the upland regions of the Gippsland region. Key refinements to the previous DEPI Gippsland Basin groundwater model include: (1) new interpreted data layers that extend to the catchment boundaries; (2) finer grid resolution; (3) the incorporation of offshore aquifer stratigraphic information as developed by the CSIRO and Geological Survey of Victoria; and (4) the incorporation of a basement layer representing the upland outcropped regions of the Gippsland region.

Table 2 summarises the grouping of coal and interseams into modelled layers, and also provides the associated Victorian Aquifer Framework (VAF) (SKM, 2011; GHD, 2012a) code and relevant references defining the extent and thicknesses of each modelled layer. Importantly, all dominant coal seams were specified as individual layers. Model layers 23 to 29 represent the Strzelecki Group. Rather than allocating this as a single layer, it was necessary to split this geological unit into six model layers to accommodate the likely scenarios which require depressurisation within parts of this unit.

Table 2	Groundwater model layers and associated Victorian Aquifer Framework (VAF) code, coal name and general description of stratigraphic groupings

| Model layer | Inputs and VAF code | Name | Coal name | Geology | Description / Comment | References |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Bathymetry | Marine Water |  | Marine water thickness | Marine water thickness | Whiteway (2009) |
| 2 | VAF 101 | QA |  | Quaternary | Quaternary | GHD (2012) SKM (2009) |
| 3 | VAF 102; -10m elevation contour | UTQA |  | Haunted Hill Formation | Haunted Hill Formation | GHD (2012) SKM (2009) |
| 4 | VAF 103 | UTQD |  | Nuntin clay | Nuntin clay | GHD (2012) SKM (2009) |
| 5 | VAF 105 | UTAF |  | Boisdale Formation | Boisdale Formation | GHD (2012) SKM (2009) |
| 6 | VAF 106 | UTD |  | Jemmy’s Point Formation and upper Hazelwood Formation | Jemmy’s Point Formation and upper Hazelwood Formation | GHD (2012) SKM (2009) |
| 7 | VAF 106 Yallourn Coal (isopach) | Y COAL | Yallourn Coal Seam | Yallourn coal seams; 1,1a, 1b and 2 | Y, Y1a, Y1b, Y2, Y1; y_all | GHD (2012) SKM (2009) GHD (2011) |
| 8 | VAF 106 Yallourn Coal Interseam | Y Interseam | Yallourn Aquifer & interseam | y_all floor & M1a_all top | Hazelwood Formation; y_all floor & M1a_all top | GHD (2012) SKM (2009) GHD (2011) |
| 9 | UMTA 107 UMTD 108 | UMT A&D | Lower M2 interseam, | Balook Formation Tambo River, Wuk Marl, Gippsland Limestone, Morwell Coals | Balook Formation Tambo River, Wuk Marl, Gippsland Limestone | GHD (2012) SKM (2009) |
| 10 | UMTA 107 Morwell 1A coal (isopach) | M1A COAL | M1A coal | Yarragon Formation, M10, M1a, M1b2, ML, M12; M1a_all | Yarragon Formation, M10, M1a, M1b2, ML, M12; M1a_all | GHD (2012) SKM (2009) GHD (2011) |
| 11 | UMTA 107 Morwell 1A Interseam | M1A Interseam | Morwell 1A interseam/aquifer | M1a_all_floor & M1b_top | M1a_all_floor and M1b_top | GHD (2012) SKM (2009) GHD (2011) |
| 12 | UMTA 107 Morwell 1B coal (isopach) | M1B COAL | Morwell 1B coal | M1b, M1b1, M1b2, ML, M12 | M1b, M1b1, M1b2, ML, M12 | GHD (2012) SKM (2009) GHD (2011) |
| 13 | UMTA 107 Morwell 1B Interseam | M1B Interseam | Morwell 1B interseam | Floor M1b_all & M2_all top | Floor M1b_all & M2_all top | GHD (2012) SKM (2009) GHD (2011) |
| 14 | UMTA 107 Morwell 2 coal (isopach) | M2 COAL | Morwell 2 | M2, M2A, M2B coal; M2_all | M2, M2A, M2B coal; M2_all | GHD (2012) SKM (2009) GHD (2011) |
| 15 | LEF Top; LEF Terminal Seal; LEF well picks | LEF |  | Lake Entrance Formation | Lakes Entrance Formation | McLean & Blackburn. (2013); (Blevin et al, 2013); Goldie Divko pers. comm. (2014) |
| 16 | LMTA 109 Morwell 2 interseam, | LMTA |  | M2c aquifer/Seaspray sands | M2c aquifer/Seaspray Sands | GHD (2012) SKM (2009) |
| 17 | LTB 112 | LTB |  | Thorpdale Volcanics | Thorpdale Volcanics | GHD (2012) SKM (2009) |
| 18 | LTA 111 Top Latrobe Petroleum wells | LTA |  | Upper Latrobe Group | Upper Latrobe Group | GHD (2012) SKM (2009) McLean & Blackburn (2013) |
| 19 | LTA 111 T0 & T1 Coal Petroleum wells | T1 COAL | T1 coal | TP, T1, TRU, TRM, TRL | TP, T1, TRU, TRM, TRL | GHD (2012) SKM (2009) GHD (2011) |
| 20 | LTA 111 T1-T2 interseam Petroleum wells | T1 Interseam | T1 interseam | Floor T1_all and Top T2_all | Floor T1_all and Top T2_all | GHD (2012) SKM (2009) GHD (2011) |
| 21 | LTA 111 T2 coal Petroleum wells | T2 COAL | T2 coal | T2 coal seams |  | GHD (2012) SKM (2009) GHD (2011) Osbourne et al. (2014) |
| 22 | LTA 111 T2 interseam Petroleum wells | T2 Interseam | T2 interseam | Lower Latrobe Group; T2_all floor | Lower Latrobe Group; T2_all floor | GHD (2012) SKM (2009) GHD (2011) Osbourne et al. (2014) |
| 23 | BSE 114 | STRZ |  | Strzelecki 500 m; >0–500 m | Strzelecki top 500 m | GHD (2012) SKM (2009) McLean and Blackburn (2013) |
| 24 | BSE 114 | STRZ1 |  | Strzelecki 500 m; >500–1000 m | Strzelecki 500–1000 m | GHD (2012) SKM (2009) McLean and Blackburn (2013) |
| 25 | BSE 114 | STRZ2 |  | Strzelecki 1 km; >1000–2000 m | Strzelecki 1–2 km | GHD (2012) SKM (2009) McLean and Blackburn (2013) |
| 26 | BSE 114 | STRZ3 |  | Strzelecki 2 km; >2000–3000 m | Strzelecki 2–3 km | GHD (2012) SKM (2009) McLean and Blackburn (2013) |
| 27 | BSE 114 | STRZ4 |  | Strzelecki 2 km; >3000–4000 m | Strzelecki 3–4 km | GHD (2012) SKM (2009) McLean and Blackburn (2013) |
| 28 | BSE 114 | STRZ5 |  | Strzelecki 3 km; >3000–7000 m | Strzelecki 4–6 km | GHD (2012) SKM (2009) McLean and Blackburn (2013) |
| 29 | BSE 114 | STRZ6 |  | Strzelecki; >7000 m | Strzelecki >6 km | GHD (2012) SKM (2009) McLean and Blackburn (2013) |
| 30 | BSE 114 | PALEO |  | Palaeozoic basement 200 m thick | Palaeozoic basement | GHD (2012) SKM (2009) McLean and Blackburn (2013) Stuart-Smith et al. (2010) Constantine (2009) |

##### Modelled hydrogeological units

The modelled hydrogeological units are summarised in Table 3. Included in this table are the associated Victorian Aquifer Framework (VAF) code, the hydrogeological unit, the method used to define the extent and the thickness of each modelled layer. The spatial extents of each modelled hydrogeological unit are shown in Figure 13 to Figure 42.

Table 3 	Modelled hydrogeological units and associated Victorian Aquifer Framework (VAF) code, derivation method and thickness

| Model layer | Inputs and VAF code | Hydrogeological Unit | Geology | Description of generation procedure | Thickness |
| --- | --- | --- | --- | --- | --- |
| 1 | Bathymetry | HGU 1 (Marine Water) | Represents marine waters throughout the offshore portion of the modelled area | The unit top is mean sea level and the unit bottom is the sea floor. Marine waters extend beyond the modelled area. Marine Water is clipped to the model extent in the south and east and the coastline in the northwest. Bathymetry is from Whiteway (2009). | The thickness of Marine Water ranges from 2 m at the coastline to 455 m in the southeast corner. |
| 2 | VAF 101 | HGU 2 (QA) | Represents the Quaternary Alluvium and includes Fluvial, lacustrine, alluvial and colluvial sediments. Quaternary Alluvium overlies the Haunted Hill Formation in the valleys and the Strzelecki Group and Palaeozoic Basement in the highlands | The QA HGU is based on the aquifer QA from (GHD, 2012; SKM 2009). QA extends to the Gippsland coast. | The thickness of Quaternary Alluvium ranges from 2 m to 105 m in small isolated pockets. |
| 3 | VAF 102; -10m elevation contour | HGU 3 (UTQA) | Represents the Upper Tertiary Quaternary Alluvium and includes the Haunted Hill Formation. | The UTQA HGU is based on the aquifer UTQA from (GHD, 2012; SKM, 2009). UTQA was extended offshore by re-gridding using bathymetry and a 1 km buffer zone. UTQA was modified to account for the offshore pinching out of the unit Haunted Hill Formation. The unit extends to the 10m bathymetric contour. | The thickness of UTQA ranges from 2 m to 175 m near Lake Wellington. |
| 4 | VAF 103 | HGU 4 (UTQD) | Represents the Upper Tertiary Quaternary Aquitard which includes the Haunted Hills Gravel, Eagle Point Sand and Boisdale Formation (Nuntin Clay Member). | The UTQD HGU is based on the aquitard UTQD from (GHD,2012; SKM, 2009). UTQD was extended offshore by re-gridding with bathymetry and using a 15 km buffer zone. UTQD was modified to account for the offshore pinching out of the unit. The surface extends to the 40m bathymetric contour. | The thickness of UTQD ranges from 2 m to 157 m west of Sale. |
| 5 | VAF 105 | HGU 5 (UTAF) | Represents the Upper Tertiary Aquifer which includes the Boisdale (Wurruk Sand Member). | The UTAF HGU is based on the aquifer UTAF from (GHD,2012; SKM,2009). UTAF was extended offshore by re-gridding with Bathymetry and using a 20 km buffer zone. UTAF was modified to account for the offshore pinching out of the Wurruk Sand Member. The surface extends to the 45m bathymetric contour. | The thickness of UTAF ranges from 2 m to 141 m beneath Lake Victoria. |
| 6 | VAF 106 | HGU 6 (UTD) | Represents the Upper Tertiary Aquitard and includes the Hazelwood Formation, Yallourn Formation and Jemmys Point Formation. | The UTD HGU is based on the aquitard UTD from (GHD,2012; SKM, 2009). UTD was extended offshore by re-gridding with Bathymetry and using a 40 km buffer zone. UTD was modified to account for the offshore pinching out of the Jemmys Point Formation. The surface extends to the 55 m bathymetric contour. | The thickness of UTD ranges from 2 m to 200 m southwest of Rosedale. |
| 7 | VAF 106 Yallourn Coal (isopach) | HGU 7 (Y COAL) | Represents the Yallourn Coal seams (Y Coal) and includes the Yallourn Y1A, Y1B and Y2 coal seams. | Coal seam isopachs and extents were determined from the Latrobe Valley coal model Yallourn Coal seams (Jansen and Maher, 2003; GHD, 2011b; Osbourne et al., 2014). These isopachs have been incorporated into UTD (6). | The thickness of Y Coal ranges from 2m to 228m north of Rosedale. |
| 8 | VAF 106 Yallourn Coal Interseam | HGU 8 (Y Interseam) | Represents the Yallourn interseam. Y Interseam represents the sediments below the Yallourn Coal floor and above the Morwell Coal roof. | This interseam was incorporated into UTD (6). | The thickness of Y Interseam ranges from 2m to 160m beneath Hazelwood. |
| 9 | UMTA 107 UMTD 108 | HGU 9 (UMT A&D) | Represents the Upper Mid-Tertiary Aquifer and Aquitard. This includes the Morwell, Balook and Tambo River Formations and the Gippsland Limestone. | The UMT HGU is based on the aquifer UMTA and aquitard UMTD from (GHD, 2012; SKM, 2009). To merge UMTA and UMTD the boundary was re-gridded to smooth conflicts. This occurred through the area of overlap between UMTA and UMTD. UMT was extended offshore by re-gridding with bathymetry and using a 40km buffer zone. The buffer was designed to account for the offshore pinching out of the Jemmys Point Formation. UMT was modified to account for the significant volume of the Gippsland Limestone. Proximal to corner inlet onshore, the UMT was re-gridded to align with petroleum wells and remove conflicts with Strzelecki. The re-gridding occurred over an area approximately 20kmx20km. The UMT extends beyond the study area where it has been clipped. | The thickness of UMT ranges from 2m to 428m offshore central deep. |
| 10 | UMTA 107 Morwell 1A coal (isopach) | HGU 10 (M1A COAL) | Represents the Morwell 1A Coal Seam (M1A). | Coal seam isopachs and extents were determined from the Latrobe Valley coal model M1A seams (Jansen and Maher, 2003; GHD 2011b; Osbourne et al., 2014). M1A was incorporated into UMT (9). | The M1A Coal thickness ranges from 2m to 228m east of Traralgon. |
| 11 | UMTA 107 Morwell 1A Interseam | HGU 11 (M1A Interseam) | Represents the interseam between the Morwell 1A Coal floor and the Morwell 1B Coal roof. | The M1A interseam was incorporated into UMT (9). | The thickness of the M1A Interseam ranges from 2m to 252m south of Rosedale. |
| 12 | UMTA 107 Morwell 1B coal (isopach) | HGU 12 (M1B COAL) | Represents the Morwell 1B (M1B) coal seams. | Coal seam isopachs and extents were determined from the Latrobe Valley coal model M1B seams (Jansen and Maher 2003; GHD, 2011b; Osbourne et al., 2014). M1B was incorporated into UMT 9. | The M1B Coal thickness ranges from 2 m to 252 m proximal to Morwell. |
| 13 | UMTA 107 Morwell 1B Interseam | HGU 13 (M1B Interseam) | Represents the Morwell 1B interseam between the Morwell 1B coal floor and Morwell 2 coal roof. | M1a Interseam was incorporated within UMT 9. | The thickness of the M1B Interseam ranges from 2m to 252m proximal to Rosedale. |
| 14 | UMTA 107 Morwell 2 coal (isopach) | HGU 14 (M2 COAL) | Represents the Morwell 2 Coal Seam (M2). | Coal seam isopachs and extents were determined from M2 coals, (Jansen and Maher, 2003; GHD 2011b; Osbourne et al., 2014). M2 was incorporated into UMT (9). | The M2 Coal thickness ranges from 2m to 239m east of Rosedale. |
| 15 | LEF Top; LEF Terminal Seal; LEF well picks | HGU 15 (LEF) | Represents the Lakes Entrance Formation. The Lakes Entrance Formation thickness ranges from 2m to 605m offshore in the central deep. | The LEF HGU is based on the top Lakes Entrance Formation from McLean & Blackburn (2013). LEF was extended onshore by re-gridding with petroleum well control. The surface extends to the terminal effective seal proposed by Blevin et al. (2013) although it is acknowledged that sealing characteristics will extend onshore beyond this line. | The Lakes Entrance Formation thickness ranges from 2m to 605m offshore in the central deep. |
| 16 | LMTA 109 Morwell 2 interseam, | HGU 16 (LMTA) | Represents the Lower Mid Tertiary Aquifer and contains the Morwell 2C aquifer and Seaspray Sands. | The LMTA HGU is based on the aquifer LMTA from (GHD, 2012; SKM, 2009). LMTA isopachs and extent were determined and were then incorporated above HGU 18. No offshore extension was modelled for this aquifer. | The LMTA thickness ranges from 2m to 123m proximal to Longford. |
| 17 | LTB 112 | HGU 17 (LTB) | Represents the Lower Tertiary Basalts. | The LTB HGU is based on the aquifer LTB from (GHD, 2012; SKM, 2009). LTB was not extended offshore. | The Lower Tertiary Basalts range in thickness ranges from 2m to 163m proximal to Yarragon. |
| 18 | LTA 111 Top Latrobe Petroleum wells | HGU 18 (LTA) | Represents the Lower Tertiary Aquifer and contains the Traralgon coals, Cobia Subgroup, Halibut Subgroup, Golden Beach Subgroup, and Emperor Subgroup. | The LTA HGU is based on the aquifer LTA from (GHD, 2012; SKM, 2009). LTA was extended offshore by re-gridding with top Latrobe (McLean & Blackman, 2013). Well intersections used to better constrain top LTA were applicable. This has particularly the case onshore proximal to 90 mile beach. The unit extends to the model extent in the east. In the southeast the LTA extent is based upon the interpretations from Blevin et al. (2013). | The Latrobe Aquifer ranges in thickness from 2m to 1192m offshore in the central deep. |
| 19 | LTA 111 T0 & T1 Coal Petroleum wells | HGU 19 (T1 COAL) | Represents the Traralgon T0 and T1 coal seams and T0 inter-burden (T1 Coal). | Coal isopachs and extents were determined for TP, TRU, TRM and TRL from the Latrobe Valley Coal Model (Jansen and Maher, 2003; GHD, 2011b; Osbourne et al., 2014). Additional Coal isopachs were determined for T0 and T1 from Holdgate et al. (2000). Coal model isopachs were aggregated and extended offshore by merging with T0/T1 isopachs. Depth control was derived the coal model and petroleum wells where available. T1 Coal was incorporated into LTA 18. | The T1 Coal ranges in thickness from 2 m to 133 m near Yarram and Longford. |
| 20 | LTA 111 T1-T2 interseam Petroleum wells | HGU 20 (T1 Interseam) | Represents the Traralgon T1 aquifer and includes the T1 interseam. | T1 Interseam isopachs and extents were determined by subtracting T2 Coal roof from T1 Coal floor. Petroleum wells were used for depth control where available. T1 Interseam was incorporated into LTA 18. | The T1 Interseam ranges in thickness from 2 m to 562 m offshore from Lake Wellington. |
| 21 | LTA 111 T2 coal Petroleum wells | HGU 21 (T2 COAL) | Represents the Traralgon T2 coal seams and minor interseams (T2 Coal). | Coal isopachs and extents were determined for T2A, T2B from the Latrobe Valley Coal Model (Jansen and Maher 2003; GHD 2011b; Osbourne et al., 2014). Additional Coal isopachs for the T2 coals were taken from Holdgate et al, (2000). Coal model isopachs were aggregated and extended offshore by merging with T2 isopachs. Depth control was derived from the coal model and petroleum wells where available. T2 Coal was incorporated into LTA 18. | The T2 coals range in thickness from 2 m to 448 m with the thicker coals tracing the Baragwaneth Anticline and thinner coals north of the Rosedale Fault. |
| 22 | LTA 111 T2 interseam Petroleum wells | HGU 22 (T2 Interseam) | Represents the Traralgon T2 inter-burden (aquifer) below the T2 coal seams. | T2 Interseam isopachs and extents were determined by subtracting LTA top from T2 Coal floor. Petroleum wells were used for depth control where available. T2 Interseam was incorporated into LTA 18. | The T2 Interseam ranges in thickness from 2 m to 2645 m offshore where top Strzelecki Group is deeper. |
| 23 | BSE 114 | HGU 23 (STRZ) | Represents the upper 500 m of the Strzelecki Group and includes the Wombat gas fields near Seaspray and the historical Black Coal mines at Wonthaggi. | Areas where the Palaeozoic Basement outcrops were clipped from BSE, GHD (2012), to produce the Strzelecki HGU onshore. This surface was then blended with the offshore top Strzelecki (McLean and Blackburn, 2013) using a 10km feathering distance. The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped. | Strzelecki ranges in thickness from 2 m to 624 m. |
| 24 | BSE 114 | HGU 24 (STRZ1) | Represents from 500 m to 1000 m depth within the Strzelecki Group. | The Strzelecki Group was arbitrarily subdivided by subtracting a 500 m thick slab from Strzelecki (23). The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped. | Strzelecki1 ranges in thickness from 2 m to 545 m. |
| 25 | BSE 114 | HGU 25 (STRZ2) | Represents from 1000 m to 2000 m depth within the Strzelecki Group. | The Strzelecki Group was arbitrarily subdivided by subtracting a 1000m thick slab from Strzelecki1 (23). The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped. | Strzelecki2 ranges in thickness from 2 m to 1000 m. |
| 26 | BSE 114 | HGU 26 (STRZ3) | Represents from 2000 m to 3000 m depth within the Strzelecki Group. | The Strzelecki Group was arbitrarily subdivided by subtracting a 1000m thick slab from Strzelecki2 (25). The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped. | Strzelecki3 ranges in thickness from 2 m to 1000 m. |
| 27 | BSE 114 | HGU 27 (STRZ4) | Represents from 3000 m to 4000 m depth within the Strzelecki Group. | The Strzelecki Group was arbitrarily subdivided by subtracting a 500-m thick slab from Strzelecki3 (26). The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped. | Strez4 ranges in thickness from 2 m to 1000 m. |
| 28 | BSE 114 | HGU 28 (STRZ5) | Represents from 4000 m to 7000 m depth within the Strzelecki Group. | The Strzelecki Group was arbitrarily subdivided by subtracting a 3000 m thick slab from Strzelecki4 (27). The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped. | Strzelecki5 ranges in thickness from 2 m to 3134 m. |
| 29 | BSE 114 | HGU 29 (STRZ6) | Represents from 7000 m to Palaeozoic Basement depth within the Strzelecki Group. | The Strzelecki Group was arbitrarily subdivided and Strzelecki6 represents the remaining Strzelecki Group not already incorporated into overlying layers. The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped. | Strzelecki6 ranges in thickness from 266 m to 4000 m. |
| 30 | BSE 114 | HGU 30 (PALEO) | Represents the Palaeozoic Basement. | Internal GSV grids were used as the foundation for the Palaeozoic (30) HGU. The Palaeozoic surface was then modified in the following ways: BSE (GH, 2012; SKM, 2009) was clipped to the extent of Palaeozoic outcrop and merged with Palaeozoic basement; Basement intersections from Constantine (2009) were used to constrain Palaeozoic basement depth; Strzelecki intersections from (Constantine 2009) were used to limit the upward extent; Interpretations based on isostatic gravity anomalies were used to mould the geometry of troughs; offshore basement surfaces from McLean & Blackman (2013) and Stuart-Smith et al. (2010) were merged together | The Palaeozoic HGU ranges in thickness from 2 m to 262 m. |

Figure 13 	Hydrogeological Unit (HGU) 1 representing the marine waters throughout the offshore portion of the modelled area

Figure 14 	Hydrogeological Unit (HGU) 2 representing the Quaternary Alluvium

Figure 15 	Hydrogeological Unit (HGU) 3 representing the Upper Tertiary Quaternary Alluvium

Figure 16 	Hydrogeological Unit (HGU) 4 representing the Upper Tertiary Quaternary Aquitard

Figure 17 	Hydrogeological Unit (HGU) 5 representing the Upper Tertiary Aquifer

Figure 18 	Hydrogeological Unit (HGU) 6 representing the Upper Tertiary Aquitard

Figure 19 	Hydrogeological Unit (HGU) 7 representing the Yallourn Coal seams

Figure 20 	Hydrogeological Unit (HGU) 8 representing the Yallourn interseam

Figure 21 	Hydrogeological Unit (HGU) 9 representing the Upper Mid-Tertiary Aquifer and Aquitard

Figure 22 	Hydrogeological Unit (HGU) 10 representing the Morwell 1A Coal Seam

Figure 23 	Hydrogeological Unit (HGU) 11 representing the interseam between the Morwell 1A Coal floor and the Morwell 1B Coal roof

Figure 24 	Hydrogeological Unit (HGU) 12 representing the Morwell 1B coal seams

Figure 25 	Hydrogeological Unit (HGU) 13 representing the Morwell 1B interseam between the Morwell 1B coal floor and Morwell 2 coal roof

Figure 26 	Hydrogeological Unit (HGU) 14 representing the Morwell 2 Coal Seam

Figure 27 	Hydrogeological Unit (HGU) 15 representing the Lakes Entrance Formation

Figure 28 	Hydrogeological Unit (HGU) 16 representing the Lower Mid Tertiary Aquifer

Figure 29 	Hydrogeological Unit (HGU) 17 representing the Lower Tertiary Basalts

Figure 30 	Hydrogeological Unit (HGU) 18 representing the Lower Tertiary Aquifer

Figure 31 	Hydrogeological Unit (HGU) 19 representing the Traralgon T0 and T1 coal seams and T0 inter-burden

Figure 32 	Hydrogeological Unit (HGU) 20 representing the Traralgon T1 aquifer and includes the T1 interseam

Figure 33 	Hydrogeological Unit (HGU) 21 representing the Traralgon T2 coal seams and minor interseams

Figure 34 	Hydrogeological Unit (HGU) 22 representing the Traralgon T2 inter-burden (aquifer) below the T2 coal seams

Figure 35 	Hydrogeological Unit (HGU) 23 representing the upper 500 m of the Strzelecki Group

Figure 36 	Hydrogeological Unit (HGU) 24 representing from 500 m to 1,000 m depth within the Strzelecki Group

Figure 37 	Hydrogeological Unit (HGU) 25 representing from 1,000 m to 2,000 m depth within the Strzelecki Group

Figure 38 	Hydrogeological Unit (HGU) 26 representing from 2,000 m to 3,000 m depth within the Strzelecki Group

Figure 39 	Hydrogeological Unit (HGU) 27 representing from 3,000 m to 4,000 m depth within the Strzelecki Group

Figure 40 	Hydrogeological Unit (HGU) 28 representing from 4,000 m to 7,000 m depth within the Strzelecki Group

Figure 41 	Hydrogeological Unit (HGU) 29 representing from 7,000 m to Palaeozoic Basement depth within the Strzelecki Group

Figure 42 	Hydrogeological Unit (HGU) 30 representing the Palaeozoic Basement

##### Modelling approach

The modelling tasks involved using an unsaturated biophysical catchment model to generate daily evapotranspiration demands and recharge estimates. This data was then used as input into the distributed groundwater model. As such the simulation procedure was uncoupled. This approach was considered to better represent farming systems and farm practices due to the construct of the unsaturated biophysical catchment model, functions of which are not commonly available in commercial groundwater modelling codes. Importantly this approach enables (1) finer resolution representation of farm management units to be captured and integrated into coarser resolution groundwater models, and (2) a holistic water balance that describes both soil-water-plant interactions and groundwater dynamics.

In addition to the unsaturated and groundwater modelling frameworks, customised software was also developed to assist in the pre- and post-processing of input data and simulation outputs. Each modelling approach is described below.

##### Unsaturated modelling software

Modelling of the unsaturated zone was undertaken using the Catchment Analysis Tool (CAT) (Beverly et al., 2005, Beverly, 2009). The CAT model utilises a suite of farming system models linked within a catchment framework with allowance for landscape connectivity and connection to a distributed, multi-layered groundwater model. The farm-scale models account for position in the landscape (topography, soil type, aspect and slope), climate, land use and land management and simulates water balance, nutrient transport and production on a daily time step.

##### Groundwater modelling software and solver

Two groundwater modelling frameworks have been deployed, namely:

USGS MODFLOW-2005:  A uniform rectilinear grid of the model domain was constructed and incorporated into the USGS source code version of MODFLOW-2005 finite difference modelling software. This model has provided predictions at a scale consistent with previous regional scale assessments.

Groundwater Vistas:  The commercial software version of MODFLOW-2005 embedded in Groundwater Vistas was used to independently check results derived using the USGS source code version of the MODFLOW-2005 finite difference modelling software. This was deemed prudent as Groundwater Vistas has implemented modifications to the solver algorithms enabling convergence to be met based on outer iterations only, the impacts of which required cross-checking with predictions derived using the original USGS version. The user interface of Groundwater Vistas aided visualisation and interrogation of model input data and simulation results.

Both models were developed using the same spatial and temporal data inputs. The groundwater solver used was the preconditioned conjugate gradient (PCG2) routine.

It is recognised that the adoption of a distributed, multi-layered groundwater model in this application is a complex groundwater model that has been developed for the Bioregional Assessment programme and differs philosophically from the use of a simplified Analytical Element (AE) model approach as applied in some other assessments. The AE models have been deliberately quite simple to enable fast convergence as required for uncertainty analysis that evaluates the plausible model input parameter space to produce a series of probabilistic predictions. Traditionally the AE models do not account for sloping aquifers and are not fully distributed. In this application, the functionality of the distributed modelling approach was considered essential to simulate the expansion of mine activities and associated spatial and temporal variations. The traditional calibration procedure produces a single prediction in a deterministic fashion from an optimised parameter set. However, uncertainty analysis can be undertaken using this form of deterministic model through the adoption of calibration constraints and post-posterior parameter sampling given sufficient computer resources.

##### Model complexity

The complexity of the Gippsland region groundwater model is consistent with the ‘Impact Assessment’ class described by Middlemis (2000). Based on a model appraisal the developed model conceptualisation is on average assumed to have moderate complexity and is suitable for predicting the impacts of proposed developments or management policies. This is further supported using the more recent Australian Groundwater Modelling Guidelines (Barnett et al., 2012) which adopts a Confidence Level Classification.

The Confidence Level Classification is a cornerstone of the national guidelines and is used to indicate the reliability of model predictions based on a number of criteria related to the available data with which the model is conceptualised and calibrated, the manner and accuracy of calibration and the manner in which the predictions are formulated. The assessment criteria are summarised in Table 4.

Three confidence level classes are defined with Class 1 being the lowest confidence and Class 3 the highest. On assessing the various relevant criteria as reported in Table 5, it is considered that the model should be targeted as Class 2 which is appropriate for assessing regional groundwater impacts. Factors limiting higher confidence include the limited availability of data in the offshore parts of the model domain and the absence of any historic coal seam gas or tight and shale gas developments and hence predictive scenarios include stresses and associated impacts that have yet to be demonstrated.

Table 4	Summary of Confidence Level Classification criteria adopted in the Australian Groundwater Modelling Guidelines (Barnett et al., 2012)

| Class | Data | Calibration | Prediction | Indicators |
| --- | --- | --- | --- | --- |
| 1 | Not much Sparse No metered usage Remote climate data | Not possible Large error statistic Inadequate data spread Targets incompatible with model purpose. | Timeframe >> calibration Long stress periods Transient prediction but steady-state calibration Bad verification. | Timeframe > 10x Stresses > 5x Mass balance > 1% (for single 5%). Properties <> field. Bad discretisation. No review. |
| 2 | Some Poor coverage Some usage information Baseflow estimates | Partial performance. Long-term trends wrong. Short time record. Weak seasonal replication. No use of targets compatible with model purpose. | Timeframe > calibration. Long stress periods. New stresses not in calibration. Poor verification. | Timeftame = 3-10x Stresses = 2-5x Mass balance < 1% Some properties <> field measurements. Some key coarse discretisation. Review by hydrogeol. |
| 3 | Lots Good aquifer geometry Good usage information Local climate information K measurements Hi-res DEM | Good performance stats. Long-term trends replicated. Seasonal fluctuations OK. Present day data targets. Head and flux targets. | Timeframe approx. calibration. Similar stress periods. Similar stresses to those in calibration. Steady-state prediction consistent with steady-state calibration. Good verification. | Timeframe < 3x Stresses < 2x Mass balance < 0.5% Properties approx. field measurements. Some key coarse discretisation. Review by modeller. |

Table 5	Gippsland Model appraisal based on the abbreviated guideline (Table 4)

| Class | Data | Calibration | Prediction | Indicators |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | X Timeframe > 10x |
| 2 | X - some | X – partial performance | X timeframe > calibration |  |
| 3 |  |  |  |  |

##### Model simulation period

Groundwater recharge in southern Australia is strongly correlated with annual rainfall. This is particularly true when the seasonal rainfall pattern follows that established by the long-term trend. In this condition most rainfall occurs during cold wet winters and in hot dry summers.

When evaluating the steady-state condition it is useful to adopt a period that has stable and consistent climate and land use over time. Such a period allows for an assessment of a quasi-equilibrium condition that is ultimately realised between surface water and groundwater. This approach was adopted as the basis of a steady-state groundwater model.

Choosing a representative annual daily climate series that best represents the long-term, zero trending climatic conditions from historical records, and which is supported by adequate and available calibration data within the same time period, is an essential part of constructing a steady-state simulation. Using this approach the representative steady-state condition was based on 1970 climatic conditions,  during which no groundwater extractions were assumed. This initial state was selected to represent predevelopment conditions, as post-1970 historic groundwater pumping in the region (on and offshore) had not achieved a quasi-equilibrium response as evidenced by available groundwater hydrograph trends.

The transient simulation period was from 1970 to 2012. This period captures both pre-mine development and a range of varying climatic conditions, including above average wet and dry sequences. The 1970 starting date enables the incorporation of historic groundwater extraction data into the model and provides sufficient lead time for the groundwater model to minimise the impact of initial conditions on model predictions associated with the period of interest, namely the calibration/validation period of 2000 to 2012.

##### Model stress periods

The transient calibration period has been formulated with annual stress periods for the period 1970 to1990, and monthly stress periods throughout the 1991 to 2012 calibration/verification periods. The model stress periods were based on adequately capturing historical onshore and offshore extractions, specifically the Latrobe Valley coal mines and offshore oil and gas production wells. To this end, pre-1990 stress periods were assigned as annual within which 12 time-steps were adopted. For the more recent post-1990 calibration/verification period, the stress period was based on representing seasonal water level oscillation and extractions data. As such, monthly time-steps were adopted post-1990. This temporal resolution was also considered adequate to enable an assessment of the potential impacts of water changes on water dependent ecosystems. It must be noted that throughout the simulation daily data was used in the generation of recharge and potential groundwater evapotranspiration stress periods.

##### Model groundwater extractions

Time series groundwater extraction data was sourced from various independent data sets, namely:

- Southern Rural Water – This data includes groundwater bore locations, bore construction date, entitlement and metered usage.

- GHD mine models – Pre-existing simulation model well files sourced to provide site specific groundwater extractions associated with the Latrobe Valley coal mines (GHD, 2011a, GHD, 2011b).

- EcoMarkets data – This data set captured the groundwater usage information incorporated in the East Gippsland and West Gippsland groundwater models developed by GHD Pty Ltd (GHD, 2010a, 2010b) as part of the ecoMarkets project commissioned by DSE in 2008.

- Stock and domestic – Victorian state-wide stock and domestic data was provided by DSE and included location, completion date, top of screen and annual extraction volume.

- Offshore extractions – Volume equivalent groundwater extractions associated with offshore oil and gas operations were sourced from Hatton, et al. (2004) and Varma, et al (2012).

All bores were assigned to aquifers according to the following criteria (in order of preference):

- as defined by the source of any historical abstraction data

- based upon bore construction (screen depth) information in conjunction with assigned GMU

- based upon bore construction (screen or bore depth) information

- bores without any GMU or construction data were assigned to the upper most (water table) aquifer.

A dataset for estimating annual groundwater use from all bores, in mega litres per year (ML/year), was compiled using the procedure outlined in Section 1.5.1.2.1 of product 1.5 for the Gippsland Basin bioregion. Limited monitored groundwater usage data was available for East Gippsland CMA bores from 1970 to 1990, but a more complete dataset was available for West Gippsland CMA bores. To address the data deficiency, the annualised West Gippsland CMA groundwater usage trend derived from 1970 to 1990 was assigned to East Gippsland CMA bores with missing data. Bore construction dates were used to ensure that extractions were assigned post-construction. Where data was available, entitlement volumes were used to constrain usage. Importantly, all abstractions within the model domain were forced to exactly match the Water Account usage volumes for the 2003 to 2012 reporting periods as summarised in Table 6.

The modelled average annual groundwater extractions , in giga litres per year (GL/yr), allocated to Groundwater Management Areas (GMA) and Water Supply Protection Areas (WSPA) within the study region are summarised in Table 7. The corresponding time-varying annual extractions are shown in Figure 43; also shown are the contributions from offshore, stock and domestic, mine dewatering, East Gippsland usage (referred to as EG_ML) and West Gippsland usage (referred to as WG_ML).

##### Steady-state model conditions

The steady-state calibration conditions represented the 1970 Latrobe Valley pre-development period, during which no groundwater extractions are assigned. Results derived based on the steady-state predictions reflect the long-term equilibrium conditions during which groundwater inputs and stresses are assumed constant throughout time.

##### Transient model initial conditions

The initial conditions assigned to the transient model adopted the 1970 pre-development condition represented by the steady-state simulation results.

##### Geometry and hydrostratigraphy

The layer thicknesses of each modelled layer are shown in Figure 60 to Figure 88.

Table 6 	State Water Accounts (usage ML)

| GMA/WSPA | 2003–04 | 2004–05 | 2005–06 | 2006–07 | 2007–08 | 2008-09 | 2009–10 | 2010–11 | 2011–12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Denison | 15,224 | 6,500 | 6,680 | 10,152 | 6,147 | 8,385 | 7,987 | 3,695 | 2,992 |
| Giffard | 2,719 | 2,520 | 3,260 | 3,719 | 3,205 | 3,662 | 1,717 | 865 | 845 |
| Leongatha | 648 | 515 | 441 | 625 | 600 | 344 | 158 | 31 | 72 |
| Moe | 1,098 | 1,084 | 990 | 1,447 | 1,414 | 1,081 | 1,095 | 191 | 330 |
| Orbost | 464 | 270 | 350 | 540 | 490 | 578 | 333 | 95 | 0 |
| Rosedale | 15,457 | 9,920 | 10,860 | 7,539 | 10,678 | 11,540 | 11,009 | 7,543 | 7,739 |
| Sale | 14,680 | 7,680 | 10,450 | 13,358 | 9,504 | 11,185 | 11,094 | 7,164 | 6,324 |
| Stratford | 27,355 | 17,230 | 17,690 | 19,182 | 24,099 | 26,897 | 27,896 | 24,904 | 26,042 |
| Tarwin | 18 | 14 | 12 | 12 | 2 | 6 | 6 | 9 | 15 |
| Wa De Lock | 12,095 | 9,403 | 8,059 | 10,509 | 7,194 | 9,517 | 10,386 | 4,832 | 3,767 |
| Wy Yung | 2,438 | 790 | 1,110 | 1,895 | 631 | 1,024 | 798 | 309 | 347 |
| Yarram | 12,205 | 8,100 | 11,070 | 16,009 | 12,048 | 13,911 | 11,778 | 6,882 | 6,740 |

Table 7 	Modelled extractions within each GMA and WSPA reporting region

|  | Depth upper (m) | Depth lower (m) | Licensed pumps | Mine pumps | S&D | Total pumps |
| --- | --- | --- | --- | --- | --- | --- |
| Denison WSPA | 0 | 25 | 183 | 0 | 118 | 301 |
| Giffard GMA | 50 | 200 | 16 | 0 | 152 | 168 |
| Leongatha GMA | 0 | basement | 27 | 0 | 130 | 157 |
| Moe GMA | 25 | basement | 67 | 0 | 249 | 316 |
| Orbost GMA | 20 | 45 | 11 | 0 | 13 | 24 |
| Rosedale GMA Zone 1 | 50 | 150 | 20 | 48 | 19 | 87 |
| Rosedale GMA Zone 2 | 25 | 350 | 229 | 0 | 506 | 735 |
| Rosedale GMA Zone 3 | 200 | 350 | 15 | 0 | 11 | 26 |
| Sale WSPA | 25 | 200 | 413 | 0 | 1011 | 1424 |
| Stratford GMA Zone 1 | 150 | basement | 1 | 8 | 7 | 16 |
| Stratford GMA Zone 2 | 350 | basement | 5 | 0 | 15 | 24 |
| Tarwin GMA | 0 | 25 | 1 | 0 | 23 | 24 |
| WaDeLock GMA | 0 | 25 | 301 | 0 | 323 | 627 |
| Wy Yung WSPA | 0 | 25 | 110 | 0 | 71 | 181 |
| Yarram WSPA Zone 1 | 0 | basement | 102 | 0 | 510 | 612 |
| Yarram WSPA Zone 2 | 200 | basement | 6 | 0 | 17 | 23 |

Figure 43 	Modelled 1970 to 2012 annual total extraction (ML/yr) by contribution. S&D refers to stock and domestic usage, Mine refers to mine dewatering, EG refers to east Gippsland usage and WG refers to West Gippsland usage

Figure 44 	State water account reporting regions (excluding Rosedale, Stratford and Yarram)

Figure 45 	Rosedale water account reporting regions (and associated depths)

Figure 46 	Stratford and Yarram water account reporting regions (and associated depths)

Figure 47 	Location of all modelled groundwater extraction bores (including stock and domestic bores)

Figure 48 	Location of modelled offshore production bores, mine dewatering and groundwater extraction bores (excluding stock and domestic bores)

Figure 49 	Location of extraction bores in modelled layer 2

Figure 50 	Location of extraction bores in modelled layer 3

Figure 51	 Location of extraction bores in modelled layer 4

Figure 52 	Location of extraction bores in modelled layer 5

Figure 53 	Location of extraction bores in modelled layer 6

Figure 54 	Location of extraction bores in modelled layer 9

Figure 55 	Location of extraction bores in modelled layer 17

Figure 56 	Location of extraction bores in modelled layer 18

Figure 57 	Location of extraction bores in modelled layer 19

Figure 58 	Location of extraction bores in modelled layer 23

Figure 59 	Location of extraction bores in modelled layer 30

Figure 60 	Thickness of modelled layer 1

Figure 61 	Thickness of modelled layer 2

Figure 62 	Thickness of modelled layer 3

Figure 63 	Thickness of modelled layer 4

Figure 64 	Thickness of modelled layer 5

Figure 65	Thickness of modelled layer 6

Figure 66 	Thickness of modelled layer 7

Figure 67 	Thickness of modelled layer 8

Figure 68 	Thickness of modelled layer 9

Figure 69 	Thickness of modelled layer 10

Figure 70 	Thickness of modelled layer 11

Figure 71 	Thickness of modelled layer 12

Figure 72 	Thickness of modelled layer 13

Figure 73 	Thickness of modelled layer 14

Figure 74 	Thickness of modelled layer 15

Figure 75 	Thickness of modelled layer 16

Figure 76 	Thickness of modelled layer 17

Figure 77 	Thickness of modelled layer 18

Figure 78 	Thickness of modelled layer 19

Figure 79 	Thickness of modelled layer 20

Figure 80 	Thickness of modelled layer 21

Figure 81 	Thickness of modelled layer 22

Figure 82 	Thickness of modelled layer 23

Figure 83 	Thickness of modelled layer 24

Figure 84 	Thickness of modelled layer 25

Figure 85 	Thickness of modelled layer 26

Figure 86 	Thickness of modelled layer 27

Figure 87 	Thickness of modelled layer 28

Figure 88 	Thickness of modelled layer 29

#### Boundary and initial conditions

Summary

The boundaries of the regional model coincide with the geological basin and are implemented as no flow boundaries. A total of 22,573 river boundary cells were incorporated into the model to represent major river reaches, whereas 410,504 drain cells were assigned to capture surface expressions of groundwater. Recharge and potential groundwater evapotranspiration rates were derived using a catchment modelling framework that accounted for episodic flooding and assigned as fixed inputs into the groundwater model. Initial conditions were based on 1970 observation data.

##### Sources of boundary and calibration data

Time series calibration information was sourced from on-line databases as summarised in Table 8. An assessment of the confidence of each key data set is also included based on criteria that considered:

expertise of those groups responsible for data collection and collation

processes for consistency of field measurement

processes used to quality assure data

data management and custodianship

degree of acceptance of data robustness by other groups and end-users.

Table 8 	Source of time-series data incorporated into the groundwater model

| Time-series data | Source | Confidence in data |
| --- | --- | --- |
| Groundwater level | Victorian Water Measurement Information System (http://data.water.vic.gov.au/monitoring.htm) | High |
| Surface water level | Victorian Water Measurement Information System (http://data.water.vic.gov.au/monitoring.htm) | High |
| Baseflow | SKM reports and derived estimates from Victorian Water Measurement Information System (http://data.water.vic.gov.au/monitoring.htm) | Moderate |
| Flood mapping | Department of Environmental and Primary Industries |  |
| Groundwater extraction (onshore) | Southern-Rural Water, Water, Department of Environmental and Primary Industries | Moderate to High; some data has been inferred and some actual |
| Groundwater extraction (offshore) | Volume equivalent groundwater extractions associated with offshore oil and gas operations were sourced from Hatton, et al (2004) and Varma, et al (2012) | Moderate |
| Mine floor elevations | Department of Environmental and Primary Industries | Moderate |
| Climatic data | Bureau of Meteorology (http://www.bom.gov.au/climate/) | High |

##### Groundwater model layer boundary conditions

Groundwater boundary conditions are features which influence groundwater flow. In a groundwater modelling context, boundary conditions are constraints imposed onto the model to reflect an area which is influenced by external features (such as wells, rivers, no-flow barriers, etc.). Groundwater flow boundary conditions considered in this study are listed below.

No flow boundary – represents locations where groundwater does not flow and/or the aquifer is absent; such features include groundwater divides (specified flow boundary type).

Well boundary – represents locations where fluxes are applied to the model (on a layer-by-layer basis). They are used to represent groundwater extraction from stock, domestic and industrial bores and from groundwater pumping in offshore oil and gas fields.

River boundary – represents a head dependent boundary condition where groundwater can either recharge or discharge into/from the model based upon a specified head elevation, the model-predicted head in neighbouring cells and a specified boundary conductance term. Rivers in this model have been grouped as either major or minor rivers. Major rivers represent primary tributaries and were assigned a width and depth of 20 m with a stage of 4 m from the base. Minor rivers were considered as significant tributaries and assigned a width and depth of 10 m and a stage of 2 m from the base. In the absence of river bed elevation data, the base elevation was assumed to be surface elevation less 5 m for major rivers and surface elevation less 2 m for minor rivers.

Drain boundary – represents a head dependent boundary condition where water is removed from the model depending on the specified head elevation, the predicted head in neighbouring cells and the specified boundary conductance term.

- Constant head boundary (time constant specified head) – represents flows into or out of the model domain where groundwater connects or interacts with features (and the ocean) outside the model domain.

Figure 89 to Figure 118 show the spatial assignment of boundary conditions to each of the modelled layers.

Figure 89 	Boundary conditions of modelled layer 1

Figure 90 	Boundary conditions of modelled layer 2

Figure 91 	Boundary conditions of modelled layer 3

Figure 92 	Boundary conditions of modelled layer 4

Figure 93 	Boundary conditions of modelled layer 5

Figure 94 	Boundary conditions of modelled layer 6

Figure 95 	Boundary conditions of modelled layer 7

Figure 96 	Boundary conditions of modelled layer 8

Figure 97 	Boundary conditions of modelled layer 9

Figure 98 	Boundary conditions of modelled layer 10.

Figure 99 	Boundary conditions of modelled layer 11

Figure 100 	Boundary conditions of modelled layer 12

Figure 101 	Boundary conditions of modelled layer 13

Figure 102 	Boundary conditions of modelled layer 14

Figure 103 	Boundary conditions of modelled layer 15

Figure 104 	Boundary conditions of modelled layer 16

Figure 105 	Boundary conditions of modelled layer 17

Figure 106 	Boundary conditions of modelled layer 18

Figure 107 	Boundary conditions of modelled layer 19

Figure 108 	Boundary conditions of modelled layer 20

Figure 109 	Boundary conditions of modelled layer 21

Figure 110 	Boundary conditions of modelled layer 22

Figure 111 	Boundary conditions of modelled layer 23

Figure 112 	Boundary conditions of modelled layer 24

Figure 113 	Boundary conditions of modelled layer 25

Figure 114 	Boundary conditions of modelled layer 26

Figure 115 	Boundary conditions of modelled layer 27

Figure 116 	Boundary conditions of modelled layer 28

Figure 117 	Boundary conditions of modelled layer 29

Figure 118 	Boundary conditions of modelled layer 30

##### Allowance for temperature and density

The model does not explicitly simulate non-isothermal processes, density driven flows, contaminant flow or subsidence. As such solute transport and water quality has not been explicitly modelled. It is acknowledged that temperature variation over the depth range considered, both onshore and offshore, is large as are the offshore salinity variations. Density gradients offshore are known to significantly influence pressure distributions resulting in flows towards the shoreline that are difficult to approximate using a constant density model. There are limitations to  ignoring density variations at the coast and off-shore but there was insufficient time to address the significance of these. However, the assumption of constant density is not considered to significantly compromise the onshore predictions of the shallow system response for the purpose of this study.

It is noteworthy that data used in CSIRO modelling that accounted for density (to assess potential for seawater intrusion) and temperature (due to geothermal resources in the basin) has been used in the calibration process. Specifically, the predevelopment and 2005 simulated potentiometric surfaces derived by CSIRO modelling for the Latrobe Group have been adjusted to remove the density and temperature impacts. These adjusted surfaces were then used in the calibration procedures. Given the limited available data this approach was deemed appropriate and pragmatic. Density head correction adjustments were also made to all groundwater observation bore data.

##### Allowance for faults

Geological faults and other geological discontinuities have not been explicitly modelled. Analysis of the stratigraphic data identified connection between the offshore and onshore units. Additionally, the properties were estimated to be continuous and not truncated. Faults were therefore considered to the extent that they were incorporated in the model geometry.

##### Allowance for flooding

Flooding contributes to episodic recharge and was considered in the Gippsland groundwater modelling study. For example, in early June 2012 much of Gippsland experienced heavy rainfall, causing flooding across a number of municipalities, including Latrobe, Wellington and East Gippsland. Hydrograph analysis revealed that groundwater levels responded to this event.

For this study flood data was sourced from the Victorian Flood Database and included information regarding the location and extent of a 1-in-100 year event (Figure 119) and historical flood events. Figure 120 shows the location and extent of recent major floods and

Table 9 presents the magnitude of each flooding event in the past 100 years (including area directly affected and duration).

Although damage caused by some recent flooding events (e.g. June 2012) to towns and communities was widespread in the Gippsland region, the areas directly affected by flooding were relatively small compared to the total area of the region. The extent of the 1-in-100 year flooding event only covers 4.8% of the total area of the Gippsland region. In the last 100 years, flooding events mainly occurred in small areas around the lower reaches of the Mitchell, Thomson, Latrobe  and Snowy rivers. The area directly affected by flooding was generally less than 1% of the Gippsland region. The largest flood event occurred in June 2012 and covered 1.4% of the Gippsland region. Given the areas affected by historic flooding events were relatively small, it is believed that the impact of flooding on groundwater recharge is insignificant at the regional scale, but likely to be significant at the local scale.

By examining groundwater hydrographs across the Gippsland region, a noticeable impact of flooding on groundwater recharge is only evidenced in the shallow aquifer systems well connected to the lower reaches of the Thomson, Mitchell and Latrobe rivers. It was found that recharge spikes in groundwater hydrographs of some shallow bores near the rivers are aligned to high river flow events.

Figure 119 	Location and extent of 1-in-100 year flooding events in the Gippsland region (source: Victorian Flood Database)

Figure 120 	Location and extent of major recent flooding events (source: the Victorian Flood Database)

Table 9 	Extents of historical flood events (source: Victorian Flood Database)

| Date | Duration (days) | Area affected (ha) | % of total catchment area |
| --- | --- | --- | --- |
| Jan-1919 | Not known | 2717 | 0.1 |
| Jan-1920 | Not known | 1129 | 0.0 |
| Oct-1923 | Not known | 208 | 0.0 |
| Jan-1934 | Not known | 7527 | 0.2 |
| Dec-1934 | Not known | 13361 | 0.4 |
| May-1968 | 1 | 544 | 0.0 |
| Jun-1969 | 1 | 644 | 0.0 |
| Feb-1971 | Not known | 1666 | 0.0 |
| Sep-1974 | Not known | 42 | 0.0 |
| Jan-1977 | Not known | 11323 | 0.3 |
| Jul-1977 | 3 | 20248 | 0.5 |
| Jan-1978 | Not known | 2884 | 0.1 |
| May-1978 | 5 | 14947 | 0.4 |
| Jun-1978 | 5 | 12086 | 0.3 |
| Jul-1978 | 1 | 3709 | 0.1 |
| Jan-1985 | Not known | 740 | 0.0 |
| Dec-1985 | 1 | 6427 | 0.2 |
| Nov-1988 | 1 | 7061 | 0.2 |
| Apr-1990 | 27 | 32563 | 0.9 |
| Oct-1990 | 1 | 3278 | 0.1 |
| Oct-1991 | 1 | 169 | 0.0 |
| Sep-1993 | 16 | 2155 | 0.1 |
| Jun-1998 | 3 | 6364 | 0.2 |
| Nov-1998 | 1 | 949 | 0.0 |
| Jun-2007 | 4 | 23308 | 0.6 |
| Jul-2011 | Not known | 36 | 0.0 |
| Mar-2012 | 8 | 9023 | 0.2 |
| Jun-2012 | 3 | 54912 | 1.4 |
| 1-in-100 year extent | N/A | 181490 | 4.8 |

Table 10 	Modelled flood extents and dates

| Flood event date | Recorded dates |
| --- | --- |
| 2012June_ext | 6/6, 7/6 |
| 2012March_ext | 3/3, 9/3,10/3 |
| 2011July_ext | 1/7 |
| 2007June | 26/6, 29/9 |
| 1998Nov_ext | 14/11 |
| 1998June_ext | 23/6, 24/6, 25/6 |
| 1993Sep_ext | 1/9, 14/9, 15/9, 16/9 |
| 1991Oct_ext | 3/10 |
| 1990Oct_ext | 11/10 |
| 1990April_ext | 1/4, 22/4, 23/4, 25/4, 27/4 |
| 1988Nov_ext | 18/11 |
| 1985Dec_ext | 12/12 |
| 1985Jan_ext | 1/1 |
| 1978July_ext | 9/7 |
| 1978June_ext | 1/6, 4/6, 5/6 |
| 1978May_ext | 21/5, 22/5, 25/5 |
| 1978Jan_ext | 1/1 |
| 1977July_ext | 1/7, 27/7 |
| 1977Jan_ext | 1/1 |
| 1974Sep_ext | 1/9 |
| 1971Feb_ext | ½ |
| 1969June_ext | 4/6 |

##### Model recharge

Dryland rainfall recharge, irrigation recharge and flood induced episodic recharge have been incorporated into the groundwater model based on predictions derived using the Catchment Analysis Tool (CAT) unsaturated catchment model (Beverly et al., 2005; Beverly, 2009a). Recharge derived from irrigation has been estimated using management scripts representing irrigation practices adopted by each irrigated agricultural industry. These management scripts are assigned to irrigated land units within the land use layer and describe irrigation triggers, irrigation application volumes and frequency of irrigation events. Flood induced episodic recharge has been simulated by maintaining saturation in the surface soil layer in all regions within mapped flood extents. The duration of inundation was arbitrarily set as 5 days, unless specific information to the contrary was available. The spatially averaged annual recharge is summarised in Table 11.

Table 11	Spatially averaged groundwater potential evapotranspiration and recharge for each modelled time period

| Stress period | Period | Time step (ts) | Days per timestep | Rain (mm/ts) | G’water Evap’n (mm/ts) | Recharge (mm/ts) | Rech/Rain (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 01/01/1971 – 31/12/1971 | 365 | 365 | 1022 | 783 | 65 | 6.31 |
| 2 | 01/01/1972 – 31/12/1972 | 366 | 731 | 626 | 619 | 20 | 3.14 |
| 3 | 01/01/1973 – 31/12/1973 | 365 | 1096 | 1035 | 644 | 76 | 7.37 |
| 4 | 01/01/1974 – 31/12/1974 | 365 | 1461 | 1353 | 678 | 173 | 12.78 |
| 5 | 01/01/1975 – 31/12/1975 | 365 | 1826 | 1064 | 682 | 108 | 10.18 |
| 6 | 01/01/1976 – 31/12/1976 | 366 | 2192 | 914 | 641 | 78 | 8.49 |
| 7 | 01/01/1977 – 31/12/1977 | 365 | 2557 | 840 | 639 | 78 | 9.28 |
| 8 | 01/01/1978 – 31/12/1978 | 365 | 2922 | 1378 | 639 | 166 | 12.02 |
| 9 | 01/01/1979 – 31/12/1979 | 365 | 3287 | 677 | 615 | 49 | 7.29 |
| 10 | 01/01/1980 – 31/12/1980 | 366 | 3653 | 879 | 662 | 50 | 5.72 |
| 11 | 01/01/1981 – 31/12/1981 | 365 | 4018 | 945 | 671 | 71 | 7.51 |
| 12 | 01/01/1982 – 31/12/1982 | 365 | 4383 | 671 | 616 | 29 | 4.38 |
| 13 | 01/01/1983 – 31/12/1983 | 365 | 4748 | 1021 | 569 | 102 | 10.04 |
| 14 | 01/01/1984 – 31/12/1984 | 366 | 5114 | 965 | 649 | 91 | 9.39 |
| 15 | 01/01/1985 – 31/12/1985 | 365 | 5479 | 1143 | 613 | 113 | 9.85 |
| 16 | 01/01/1986 – 31/12/1986 | 365 | 5844 | 782 | 626 | 75 | 9.55 |
| 17 | 01/01/1987 – 31/12/1987 | 365 | 6209 | 828 | 631 | 59 | 7.13 |
| 18 | 01/01/1988 – 31/12/1988 | 366 | 6575 | 995 | 626 | 72 | 7.25 |
| 19 | 01/01/1989 – 31/12/1989 | 365 | 6940 | 1031 | 655 | 122 | 11.81 |
| 20 | 01/01/1990 – 31/03/1990 | 90 | 7030 | 145 | 179 | 3 | 1.77 |
| 21 | 01/04/1990 – 30/06/1990 | 91 | 7121 | 283 | 96 | 19 | 6.86 |
| 22 | 01/07/1990 – 30/09/1990 | 92 | 7213 | 308 | 114 | 50 | 16.25 |
| 23 | 01/10/1990 – 31/12/1990 | 92 | 7305 | 238 | 262 | 22 | 9.15 |
| 24 | 01/01/1991 – 31/03/1991 | 90 | 7395 | 199 | 194 | 4 | 1.90 |
| 25 | 01/04/1991 – 30/06/1991 | 91 | 7486 | 243 | 92 | 12 | 4.92 |
| 26 | 01/07/1991 – 30/09/1991 | 92 | 7578 | 359 | 118 | 64 | 17.82 |
| 27 | 01/10/1991 – 31/12/1991 | 92 | 7670 | 165 | 241 | 7 | 4.53 |
| 28 | 01/01/1992 – 31/03/1992 | 91 | 7761 | 193 | 191 | 6 | 3.01 |
| 29 | 01/04/1992 – 30/06/1992 | 91 | 7852 | 216 | 76 | 12 | 5.77 |
| 30 | 01/07/1992 – 30/09/1992 | 92 | 7944 | 303 | 111 | 40 | 13.34 |
| 31 | 01/10/1992 – 31/12/1992 | 92 | 8036 | 377 | 231 | 47 | 12.50 |
| 32 | 01/01/1993 – 31/03/1993 | 90 | 8126 | 248 | 217 | 17 | 6.89 |
| 33 | 01/04/1993 – 30/06/1993 | 91 | 8217 | 125 | 94 | 11 | 9.11 |
| 34 | 01/07/1993 – 30/09/1993 | 92 | 8309 | 339 | 112 | 49 | 14.55 |
| 35 | 0110/1993 – 31/12/1993 | 92 | 8401 | 304 | 247 | 31 | 10.14 |
| 36 | 01/01/1994 – 31/03/1994 | 90 | 8491 | 266 | 213 | 15 | 5.77 |
| 37 | 01/04/1994 – 30/06/1994 | 91 | 8582 | 192 | 107 | 14 | 7.45 |
| 38 | 01/07/1994 – 30/09/1994 | 92 | 8674 | 143 | 111 | 13 | 9.14 |
| 39 | 01/10/1994 – 31/12/1994 | 92 | 8766 | 243 | 249 | 16 | 6.59 |
| 40 | 01/01/1995 – 31/03/1995 | 90 | 8856 | 207 | 202 | 5 | 2.32 |
| 41 | 01/04/1995 – 30/06/1995 | 91 | 8947 | 252 | 87 | 25 | 9.94 |
| 42 | 01/07/1995 – 30/09/1995 | 92 | 9039 | 203 | 121 | 26 | 12.65 |
| 43 | 0110/1995 – 31/12/1995 | 92 | 9131 | 378 | 234 | 38 | 10.11 |
| 44 | 01/01/1996 – 31/03/1996 | 91 | 9222 | 230 | 220 | 10 | 4.50 |
| 45 | 01/04/1996 – 30/06/1996 | 91 | 9313 | 191 | 92 | 18 | 9.45 |
| 46 | 01/07/1996 – 30/09/1996 | 92 | 9405 | 304 | 122 | 45 | 14.71 |
| 47 | 01/10/1996 – 31/12/1996 | 92 | 9497 | 211 | 246 | 13 | 6.08 |
| 48 | 01/01/1997 – 31/03/1997 | 90 | 9587 | 141 | 178 | 3 | 1.97 |
| 49 | 01/04/1997 – 30/06/1997 | 91 | 9678 | 174 | 73 | 5 | 2.75 |
| 50 | 01/07/1997 – 30/09/1997 | 92 | 9770 | 148 | 113 | 14 | 9.68 |
| 51 | 01/10/1997 – 31/12/1997 | 92 | 9862 | 158 | 223 | 3 | 1.96 |
| 52 | 01/01/1998 – 31/03/1998 | 90 | 9952 | 131 | 143 | 1 | 0.73 |
| 53 | 01/04/1998 – 30/06/1998 | 91 | 10043 | 295 | 69 | 10 | 3.32 |
| 54 | 01/07/1998 – 30/09/1998 | 92 | 10135 | 196 | 111 | 26 | 13.50 |
| 55 | 01/10/1998 – 31/12/1998 | 92 | 10227 | 317 | 264 | 17 | 5.44 |
| 56 | 01/01/1999 – 31/03/1999 | 90 | 10317 | 210 | 219 | 5 | 2.36 |
| 57 | 01/04/1999 – 30/06/1999 | 91 | 10408 | 156 | 91 | 9 | 5.99 |
| 58 | 01/07/1999 – 30/09/1999 | 92 | 10500 | 164 | 113 | 17 | 10.26 |
| 59 | 0110/1999 – 31/12/1999 | 92 | 10592 | 203 | 207 | 6 | 2.78 |
| 60 | 01/01/2000 – 31/03/2000 | 91 | 10683 | 168 | 188 | 3 | 1.84 |
| 61 | 01/04/2000 – 30/06/2000 | 91 | 10774 | 248 | 89 | 17 | 6.67 |
| 62 | 01/07/2000 – 30/09/2000 | 92 | 10866 | 235 | 112 | 29 | 12.26 |
| 63 | 01/10/2000 – 31/12/2000 | 92 | 10958 | 211 | 237 | 21 | 9.83 |
| 64 | 01/01/2001 – 31/03/2001 | 90 | 11048 | 199 | 179 | 3 | 1.58 |
| 65 | 01/04/2001 – 30/06/2001 | 91 | 11139 | 217 | 90 | 16 | 7.53 |
| 66 | 01/07/2001 – 30/09/2001 | 92 | 11231 | 285 | 113 | 42 | 14.73 |
| 67 | 01/10/2001 – 31/12/2001 | 92 | 11323 | 309 | 237 | 29 | 9.26 |
| 68 | 01/01/2002 – 31/03/2002 | 90 | 11413 | 216 | 205 | 7 | 3.17 |
| 69 | 01/04/2002 – 30/06/2002 | 91 | 11504 | 267 | 105 | 24 | 8.81 |
| 70 | 01/07/2002 – 30/09/2002 | 92 | 11596 | 145 | 128 | 17 | 11.73 |
| 71 | 01/10/2002 – 31/12/2002 | 92 | 11688 | 156 | 231 | 3 | 2.14 |
| 72 | 01/01/2003 – 31/03/2003 | 90 | 11778 | 104 | 121 | 0 | 0.45 |
| 73 | 01/04/2003 – 30/06/2003 | 91 | 11869 | 169 | 93 | 5 | 2.68 |
| 74 | 01/07/2003 – 30/09/2003 | 92 | 11961 | 228 | 130 | 22 | 9.45 |
| 75 | 01/10/2003 – 31/12/2003 | 92 | 12053 | 244 | 236 | 14 | 5.78 |
| 76 | 01/01/2004 – 31/03/2004 | 91 | 12144 | 111 | 143 | 2 | 1.40 |
| 77 | 01/04/2004 – 30/06/2004 | 91 | 12235 | 229 | 86 | 11 | 4.69 |
| 78 | 01/07/2004 – 30/09/2004 | 92 | 12327 | 220 | 119 | 22 | 10.13 |
| 79 | 01/10/2004 – 31/12/2004 | 92 | 12419 | 262 | 252 | 16 | 6.05 |
| 80 | 01/01/2005 – 3103/2005 | 90 | 12509 | 223 | 203 | 7 | 2.98 |
| 81 | 01/04/2005 – 30/06/2005 | 91 | 12600 | 116 | 86 | 3 | 2.74 |
| 82 | 01/07/2005 – 30/09/2005 | 92 | 12692 | 298 | 115 | 29 | 9.63 |
| 83 | 01/10/2005 – 31/12/2005 | 92 | 12784 | 223 | 266 | 10 | 4.48 |
| 84 | 01/01/2006 – 31/03/2006 | 90 | 12874 | 114 | 161 | 1 | 0.98 |
| 85 | 01/04/2006 – 30/06/2006 | 91 | 12965 | 239 | 86 | 12 | 5.08 |
| 86 | 01/07/2006 – 30/09/2006 | 92 | 13057 | 174 | 123 | 13 | 7.38 |
| 87 | 01/10/2006 – 31/12/2006 | 92 | 13149 | 86 | 186 | 1 | 1.23 |
| 88 | 01/01/2007 – 31/03/2007 | 90 | 13239 | 190 | 157 | 1 | 0.37 |
| 89 | 01/04/2007 – 30/06/2007 | 91 | 13330 | 326 | 99 | 14 | 4.18 |
| 90 | 01/07/2007 – 30/09/2007 | 92 | 13422 | 188 | 126 | 33 | 17.44 |
| 91 | 01/10/2007 – 31/12/2007 | 92 | 13514 | 251 | 254 | 11 | 4.56 |
| 92 | 01/01/2008 – 31/03/2008 | 91 | 13605 | 172 | 193 | 4 | 2.11 |
| 93 | 01/04/2008 – 30/06/2008 | 91 | 13696 | 106 | 70 | 4 | 3.32 |
| 94 | 01/07/2008 – 30/09/2008 | 92 | 13788 | 199 | 117 | 18 | 8.82 |
| 95 | 01/10/2008 – 31/12/2008 | 92 | 13880 | 280 | 223 | 9 | 3.38 |
| 96 | 01/01/2009 – 31/03/2009 | 90 | 13970 | 81 | 154 | 2 | 2.10 |
| 97 | 01/04/2009 – 30/06/2009 | 91 | 14061 | 132 | 73 | 4 | 2.82 |
| 98 | 01/07/2009 – 30/09/2009 | 92 | 14153 | 252 | 133 | 21 | 8.18 |
| 99 | 01/10/2009 – 31/12/2009 | 92 | 14245 | 225 | 244 | 13 | 5.85 |
| 100 | 01/01/2010 – 31/03/2010 | 90 | 14335 | 251 | 195 | 6 | 2.35 |
| 101 | 01/04/2010 – 30/06/2010 | 91 | 14426 | 197 | 90 | 15 | 7.76 |
| 102 | 01/07/2010 – 30/09/2010 | 92 | 14518 | 205 | 106 | 30 | 14.53 |
| 103 | 01/10/2010 – 31/12/2010 | 92 | 14610 | 314 | 240 | 22 | 6.93 |
| 104 | 01/01/2011 – 31/03/2011 | 90 | 14700 | 293 | 198 | 18 | 6.18 |
| 105 | 01/04/2011 – 30/06/2011 | 91 | 14791 | 232 | 88 | 30 | 12.87 |
| 106 | 01/07/2011 – 30/09/2011 | 92 | 14883 | 278 | 117 | 38 | 13.71 |
| 107 | 01/10/2011 – 31/12/2011 | 92 | 14975 | 315 | 259 | 25 | 7.80 |
| 108 | 01/01/2012 – 31/03/2012 | 91 | 15066 | 335 | 216 | 22 | 6.41 |
| 109 | 01/04/2012 – 30/06/2012 | 91 | 15157 | 322 | 95 | 44 | 13.63 |
| 110 | 01/07/2012 – 30/09/2012 | 92 | 15249 | 202 | 125 | 32 | 15.76 |
| 111 | 01/10/2012 – 31/12/2012 | 92 | 15341 | 191 | 266 | 9 | 4.96 |

##### Model evapotranspiration

In MODFLOW the evapotranspiration (ET) package simulates ET from shallow water tables based on a root extinction depth (m) and a maximum groundwater extraction rate (mm/yr). The extinction depth identifies the water table depth below natural surface below which no ET will occur; see for example Shah et al. (2007). In the current application, the extinction depth was based on the vegetation root depth assigned in the various farming system models used to estimate recharge within the CAT modelling framework as reported in Beverly et al. (2011). The extinction depths vary from 800 mm for annual pasture to 7 m for forests. The maximum evapotranspiration rate was calculated as the difference between potential daily ET and estimated daily vegetative ET as predicted using the CAT modelling framework. This approach ensures that the sum of estimated daily vegetative ET from the unsaturated zone and the assigned maximum groundwater ET from the saturated zone cannot exceed daily potential ET calculated from meteorological data

##### Model rivers and drains

A total of 197 river gauges were identified within the model area representing all of the major rivers. Daily gauge level data was sourced from the Victorian Department of Environment, Land, Water and Planning Water Measurement Information System (WMIS, 2015). A list of the river gauges is provided in Table 12 to Table 18 for key river basins.

Only main stems of the major rivers were included in the model. These river reaches were identified using the DEPI hydro25 spatial data set (DEPI, 2014). The river classification was used to vary river incision depth (depth below the ground surface as defined by the digital elevation model) and width attributes. In the absence of recorded stage height information, river classification was used to estimate river stage heights. A total of 22,573 river cells are included in the model. Fifty-one gauges were selected to calibrate the catchment modelling framework in unregulated catchments based on Base Flow Indexes and observed stream flows.

Drainage channels and man-made drainage features in the Macalister Irrigation District (MID) were included in the model based on available drainage network mapping (Figure 121). This information was sourced from Southern Rural Water (SRW) and the DEPI Corporate Spatial Data library. Drainage cells are assigned to the uppermost cells within the model to capture groundwater discharge processes. Drain cells in MODFLOW can only act as groundwater discharge points and as such those cells outside drainage channels will be characterised as having a bed elevation equivalent to ground surface elevation. A total of 410,504 drainage cells are incorporated in the model. Apart from 3 river gauges sourced from the WMIS, SRW also has 15 gauges (Figure 121 and Table 19) monitored drainage from the MID. The measurements commenced between 1997 and 2005. Of the 15 gauges, six were selected to calibrate the catchment modelling framework based on observed discharge. The mean monthly discharge and annual discharge at these six gauges are presented in Figure 122 and Figure 123, respectively.

Table 12 	River gauges for East Gippsland

| Site ID | Site name | Easting | Northing | Gauge zero (Ahd) | Site start date | Site cease date | Drainage area (km2) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 221001 | GENOA RIVER @ ROCKTON | 705825 | 5887218 | 414.72 | 26/05/1993 |  | 125.6 |  | 0.28 |
| 221201 | CANN RIVER (WEST BRANCH) @ WEERAGUA | 694682 | 5861473 | 153.96 | 21/02/1957 |  | 311 |  | 0.51 |
| 221202 | GENOA RIVER @ WANGARABELL | 721597.1 | 5858290.5 | 0 | 13/10/1927 | 30/06/1929 | 780 |  | 0.42 |
| 221203 | BETKA RIVER @ MALLACOOTA | 737895.1 | 5836091.4 | 0 | 26/01/1966 | 24/07/1972 | 117 |  | 0.37 |
| 221204 | THURRA RIVER @ POINT HICKS | 699200 | 5822200 | 0 | 17/05/1966 | 28/07/1978 | 345 |  | 0.43 |
| 221205 | BEMM RIVER @ BEMM RIVER | 671893.8 | 5821410.8 | 0 | 28/06/1966 | 5/06/1975 | 935 |  | 0.55 |
| 221206 | CANN RIVER @ NOORINBEE | 694397 | 5854712.4 | 0 | 31/05/1967 | 3/02/1971 | 541 |  | 0.43 |
| 221207 | ERRINUNDRA RIVER @ ERRINUNDRA | 669418 | 5853798 | 0 | 28/07/1971 |  | 162 |  | 0.65 |
| 221208 | WINGAN RIVER @ WINGAN INLET NATIONAL PARK | 719710.3 | 5825779.5 | 0 | 14/12/1982 |  | 420 |  | 0.39 |
| 221209 | CANN RIVER (EAST BRANCH) @ WEERAGUA | 695470 | 5863398 | 0 | 23/10/1972 |  | 154 |  | 0.36 |
| 221210 | GENOA RIVER @ THE GORGE | 723046 | 5856007 | 0 | 22/08/1972 |  | 837 |  | 0.34 |
| 221211 | COMBIENBAR RIVER @ COMBIENBAR | 675439 | 5854433 | 0 | 12/08/1974 |  | 179 |  | 0.48 |
| 221212 | BEMM RIVER @ PRINCES HIGHWAY | 667855 | 5836059 | 0 | 30/04/1975 |  | 725 |  | 0.59 |
| 221213 | GOOLENGOOK RIVER @ D/S OF ARTE RIVER JUNCTION | 663700 | 5846900 | 0 | 1/05/1978 | 18/12/1986 | 171 |  | 0.69 |
| 221214 | CANN RIVER @ D/S OF CANN RIVER | 688655 | 5836283 | 0 | 26/07/1979 |  | 675 |  | 0.44 |
| 221216 | GENOA RIVER @ D/S OF BIG FLAT CREEK JUNCTION | 722308.7 | 5858209.9 | 0 | 23/05/1967 | 10/12/1970 | 829 |  | 0.43 |
| 221217 | GENOA RIVER @ GIPSY POINT (WOOD'S JETTY) | 736900 | 5848600 | -1.3 | 8/07/1992 | 3/11/1998 | 0 |  |  |
| 221218 | BETKA RIVER @ MINERS TRACK | 732688 | 5835801 | 0 | 16/01/1996 |  | 0 |  | 0.29 |
| 221222 | HENSLEIGH CREEK U/S COMBIENBAR | 679158 | 5863650 | 0 | 18/05/2006 | 17/11/2010 | 0 |  |  |
| 221223 | COMBIENBAR RIVER @ TIGER SNAKE CK | 680158 | 5864336 | 0 | 18/05/2006 | 17/11/2010 | 0 |  |  |
| 221224 | CANN RIVER U/S CANN RIVER OFFTAKE | 690544 | 5843171 | 0 | 11/03/2009 |  | 0 |  | 0.52 |
| 221225 | BEMM RIVER U/S OF PUMPHOUSE | 671929 | 5822377 | 0 |  |  | 0 |  | 0.44 |
| 221800 | RAINGAUGE (GENOA RIVER) @ THE GORGE | 722715 | 5855916.4 | 0 | 22/08/1972 |  | 0 |  |  |

Table 13 	River gauges for Latrobe Basin

| Site ID | Site name | Easting | Northing | Gauge zero (mAHD) | Site start date | Site cease date | Drainage area (km2) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 226005 | LATROBE RIVER @ THOMS BRIDGE | 448418 | 5775480 | 0 | 24/05/1960 |  | 2657 |  | 0.57 |
| 226006 | TYERS RIVER @ BOOLA | 448568.9 | 5781803 | 112.027 | 16/04/1958 |  | 293 |  | 0.44 |
| 226007 | TYERS RIVER @ BROWNS | 443476 | 5788600 | 169.359 | 17/08/1961 |  | 207 |  | 0.60 |
| 226008 | TYERS RIVER WEST BRANCH @ MORGANS MILL | 439256 | 5800120 | 0 | 27/04/1960 |  | 80 |  | 0.64 |
| 226012 | TANJIL RIVER EAST BRANCH @ TANJIL BREN | 429307.7 | 5812543.1 | 0 | 18/01/1961 | 11/08/1971 | 12 |  | 0.65 |
| 226016 | WATERHOLE CREEK @ MORWELL | 449045.2 | 5767997.4 | 60.56 | 22/08/1961 | 15/02/1982 | 41 |  | 0.42 |
| 226017 | JACOBS CREEK @ O'TOOLES | 446350 | 5788246 | 0 | 16/04/1962 |  | 36 |  | 0.43 |
| 226021 | NARRACAN CREEK @ MOE | 435969 | 5775832 | 0 | 26/06/1996 |  | 0 |  | 0.69 |
| 226023 | TRARALGON CREEK @ TRARALGON | 460064 | 5772541 | 32.673 | 18/09/1998 |  | 189 |  | 0.48 |
| 226027 | LATROBE RIVER AT SWING BRIDGE | 511200 | 5778084 | 0 | 7/05/2010 |  | 0 |  | 0.41 |
| 226028 | TYERS R @ PUMP HOUSE | 451503 | 5778560 | 0 | 5/04/2007 |  | 0 |  | 0.63 |
| 226033 | LATROBE RIVER @ SCARNES BRIDGE | 460763 | 5777041 | 21.185 | 20/12/1996 |  | 0 |  | 0.44 |
| 226039 | BILLY CREEK @ U/S OF OFFTAKE WEIR | 446116.2 | 5755155.9 | 0 | 2/03/1967 | 8/05/1968 | 22 |  | 0.57 |
| 226041 | LAKE WELLINGTON @ BULL BAY | 532876 | 5780415 | 0 | 25/02/1991 |  | 0 |  |  |
| 226202 | TYERS RIVER @ GOULD | 444451.2 | 5785659.5 | 0 | 19/05/1926 | 28/02/1941 | 220 |  | 0.59 |
| 226204 | LATROBE RIVER @ WILLOW GROVE | 426308 | 5784017 | 0 | 17/10/1966 |  | 580 |  | 0.74 |
| 226205 | LATROBE RIVER @ NOOJEE | 414035 | 5804037 | 0 | 1/05/1996 |  | 290 |  | 0.79 |
| 226206 | LATROBE RIVER @ APM MARYVALE | 452119.2 | 5774427.3 | 27.546 | 2/01/1946 | 3/05/1977 | 3002 |  | 0.62 |
| 226209 | MOE RIVER @ DARNUM | 412633 | 5770880 | 79.598 | 19/10/1960 |  | 214 |  | 0.48 |
| 226216 | TANJIL RIVER @ TANJIL SOUTH | 433560 | 5783395 | 69.245 | 5/04/1955 |  | 358 |  | 0.70 |
| 226217 | LATROBE RIVER @ HAWTHORN BRIDGE | 419490.7 | 5796547.9 | 0 | 2/06/1955 | 9/01/1989 | 440 |  | 0.73 |
| 226218 | NARRACAN CREEK @ THORPDALE | 428843 | 5763687 | 0 | 22/06/1955 |  | 66 |  | 0.75 |
| 226219 | TOORONGA RIVER @ NOOJEE | 415838.9 | 5810012 | 0 | 1/10/1924 | 1/07/1933 | 65 |  | 0.78 |
| 226220 | LOCH RIVER @ NOOJEE | 412644 | 5808497 | 0 | 25/04/1978 |  | 106 |  | 0.74 |
| 226222 | LATROBE RIVER @ NEAR NOOJEE (US ADA R JUNCT.) | 402650 | 5806658 | 272.304 | 10/05/1971 |  | 62 |  | 0.80 |
| 226223 | LATROBE RIVER @ NEERIM EAST | 414328.5 | 5800194.3 | 0 | 16/03/1972 | 8/01/1979 | 378 |  | 0.77 |
| 226224 | LATROBE RIVER @ ROSEDALE (ANABRANCH) | 481555 | 5777709 | 9.7 | 1/12/1936 |  | 4144 |  | 0.12 |
| 226226 | TANJIL RIVER @ TANJIL JUNCTION | 429155 | 5796100 | 0 | 25/05/1960 |  | 289 |  | 0.69 |
| 226227 | LATROBE RIVER @ KILMANY SOUTH | 492900 | 5778721 | 2.571 | 16/12/1976 |  | 4464 |  | 0.62 |
| 226228 | LATROBE RIVER @ ROSEDALE (MAIN STREAM) | 481748 | 5778351 | 9.7 | 1/12/1936 |  | 4144 |  | 0.62 |
| 226229 | TANJIL RIVER @ D/S OF BLUE ROCK DAM | 432764.2 | 5784646.8 | 75.315 | 11/05/1979 | 28/02/1982 | 352 |  | 0.71 |
| 226232 | TANJIL RIVER @ MOE-WALHALLA ROAD BRIDGE | 0 | 0 | 0 | 17/05/1990 | 5/06/1990 | 514 |  |  |
| 226233 | TANJIL RIVER @ U/S OF SERPENTINE CREEK | 435276.4 | 5783132.8 | 0 | 16/07/1992 | 12/12/2006 | 0 |  | 0.74 |
| 226235 | TYERS RIVER @ TYERS JUNCTION | 441506 | 5798197 | 0 | 20/08/2003 |  | 132.2 |  | 0.61 |
| 226244 | TYERS R EAST BRANCH @ CORRINGAL SCOUT CAMP | 441496 | 5798505 | 0 | 27/10/2004 | 18/09/2008 | 0 |  |  |
| 226245 | TYERS R WEST BRANCH U/S SOUTH FACE RD | 436327 | 5807354 | 0 | 1/11/2004 | 24/02/2009 | 0 |  |  |
| 226246 | TYERS R WEST BRANCH D/S SOUTH FACE RD | 436123 | 5807237 | 0 | 28/10/2004 | 24/02/2009 | 0 |  |  |
| 226247 | HOPE CK U/S SOUTHFACE ROAD BRIDGE | 434283 | 5810028 | 0 | 26/03/2007 | 24/02/2009 | 0 |  |  |
| 226248 | HOPE CK D/S SOUTHFACE ROAD BRIDGE | 434267 | 5809990 | 0 | 26/03/2007 | 24/02/2009 | 0 |  |  |
| 226400 | LATROBE RIVER @ YALLOURN | 437171.9 | 5776866.3 | 0 | 1/11/1923 |  | 1940 |  |  |
| 226402 | MOE DRAIN @ TRAFALGAR EAST | 431070 | 5774351 | 0 | 7/06/1957 |  | 622 |  |  |
| 226407 | MORWELL RIVER @ BOOLARRA | 439351 | 5748787 | 98.302 | 9/05/1972 |  | 114 |  |  |
| 226408 | MORWELL R @ YALLOURN | 444702 | 5770110.4 | 0 | 21/04/2004 |  | 622 |  |  |
| 226410 | TRARALGON CREEK @ KOORNALLA | 459050 | 5758231 | 0 | 9/07/1953 |  | 89 |  |  |
| 226415 | TRARALGON CREEK @ TRARALGON SOUTH (JONES RD) | 459805 | 5764127 | 60 | 27/05/1976 |  | 128 |  |  |
| 226602 | AREA 2 SITE1 @ DOWDS MORASS NTH | 515887 | 5777885 | -0.016 | 17/06/2003 | 24/09/2009 | 0 |  |  |
| 226603 | AREA 4 SITE 2 @ DOWDS MORASS STH | 516572 | 5776987 | -0.016 | 17/06/2003 |  | 0 |  |  |
| 226605 | LAKE VICTORIA AT MCLENNAN'S STRAIT | 539492.4 | 5787708 | 0 | 17/11/2010 |  | 0 |  |  |
| 226606 | LAKE VICTORIA AT LOCH SPORT | 551177 | 5789006 |  | 26/11/2010 |  |  |  |  |
| 226607 | MCMILLAN STRAIT AT PAYNESVILLE | 564019 | 5803744.2 |  | 21/10/2010 |  |  |  |  |
| 226608 | LAKE KING AT METUNG | 576509 | 5807127.8 |  | 17/11/2010 |  |  |  |  |
| 226609 | CUNNINGHAM ARM AT BULLOCK ISLAND | 585591 | 5806582.2 |  | 19/10/2010 |  |  |  |  |
| 226700 | LOY YANG OUTFALL TO TRARALGON CREEK | 0 | 0 | 0 | 17/12/1997 | 7/02/2000 | 0 |  |  |
| 226702 | MORWELL GROSS POLLUTION TRAP D/S CIRCULAR PIPE | 447900 | 5766700 | 0 | 1/06/1998 | 7/10/1998 | 0 |  |  |
| 226801 | RAINGAUGE (REPRESENTATIVE BASIN) @ LATROBE NO. 2 | 399410 | 5810382.1 | 0 | 19/01/1982 | 17/08/1988 | 0 |  |  |
| 226802 | RAINGAUGE (REPRESENTATIVE BASIN) @ LATROBE NO. 3 | 398477.4 | 5802787.5 | 0 | 19/01/1982 | 6/03/1986 | 0 |  |  |
| 226804 | RAINGAUGE (SITE 3) @ WALHALLA ST - NEWBOROUGH | 438000 | 5772500 | 0 | 6/12/1994 | 16/10/1995 | 0 |  |  |
| 226805 | RAINGAUGE (SITE 4) @ MOE INDOOR RECREATION CENTRE | 437018.3 | 5772948.1 | 0 | 7/12/1994 | 16/10/1995 | 0 |  |  |
| 226814 | RAIN GAUGE @ MT TASSIE | 461784 | 5750077 | 0 | 3/12/1998 |  | 0 |  |  |
| 226815 | RAIN GAUGE (TRARALGON CK) @ TRARALGON - EPA YARD | 458982 | 5772514 | 0 | 25/05/1999 |  | 0 |  |  |
| 226816 | RAIN GAUGE @ MT HOOGHLY | 453401 | 5750509 | 0 | 29/09/1999 | 15/05/2008 | 0 |  |  |
| 226817 | RAIN GAUGE @ LE ROY QUARRY | 457656 | 5750650 | 0 | 30/09/1999 | 31/10/2009 | 0 |  |  |
| 226818 | RAIN GAUGE @ BALOOK | 459952 | 5746341 | 0 | 10/06/1999 |  | 0 |  |  |
| 226819 | RAIN GAUGE @ CALIGNEE NORTH | 462063 | 5758780 | 0 | 25/05/1999 |  | 0 |  |  |
| 226825 | RAINGAUGE @ MOE SOUTH | 434231.7 | 5769648.9 |  | 6/10/2009 |  |  |  |  |
| 226826 | RAINGAUGE AT YARRAGON SOUTH | 420511 | 5763367.1 |  | 21/10/2009 |  |  |  |  |
| 226827 | RAINGAUGE @ THORPDALE PEAK | 431275 | 5760002.4 |  | 17/09/2009 |  |  |  |  |
| 226828 | RAINGAUGE @ JEERALANG SOUTH HALLAMS RD | 451008.4 | 5752916 |  | 6/10/2009 | 16/11/2011 |  |  |  |
| 226829 | RAINGAUGE AT JERRALANG, DOBBINS ROAD | 450883.6 | 5754699.5 |  | 29/01/2013 |  |  |  |  |

Table 14 	River gauges for Mitchell Basin

| Site ID | Site name | Easting | Northing | Gauge zero (Ahd) | Site start date | Site cease date | Drainage area (km2) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 224200 | MITCHELL RIVER @ BAIRNSDALE | 552241 | 5813267 | 0 |  |  | 4425 |  |  |
| 224201 | WONNANGATTA RIVER @ WATERFORD | 514714 | 5850745 | 175.337 | 15/07/1976 |  | 1979 |  |  |
| 224203 | MITCHELL RIVER @ GLENALADALE | 533010 | 5820334 | 28.951 | 22/02/1991 |  | 3903 |  |  |
| 224205 | DARGO RIVER @ DARGO (UPPER SITE) | 524129.9 | 5861627.7 | 0 | 1/06/1953 | 26/06/1974 | 539 |  |  |
| 224206 | WONNANGATTA RIVER @ CROOKED RIVER | 507891 | 5859903 | 240.829 | 13/04/1977 |  | 1096 |  |  |
| 224207 | WONGUNGARRA RIVER @ GUYS | 508800 | 5862200 | 247.234 | 7/05/1953 | 10/01/1989 | 736 |  |  |
| 224208 | MITCHELL RIVER @ HOWITT DAM SITE | 532433.8 | 5827894.3 | 51.586 | 14/02/1969 | 22/12/1975 | 3761 |  |  |
| 224209 | COBBANNAH CREEK @ NEAR BAIRNSDALE | 530900 | 5831600 | 0 | 15/05/1970 | 1/07/1987 | 106 |  |  |
| 224210 | WONNANGATTA RIVER @ KINGSWELL BRIDGE (HAWKHURST) | 510185.8 | 5859581.7 | 224.768 | 12/02/1970 | 25/03/1981 | 1883 |  |  |
| 224213 | DARGO RIVER @ LOWER DARGO ROAD | 523752 | 5850152 | 172.824 | 24/05/1973 |  | 676 |  |  |
| 224214 | WENTWORTH RIVER @ TABBERABBERA | 534808 | 5850099 | 0 | 4/07/1974 |  | 443 |  |  |
| 224215 | MITCHELL RIVER @ ANGUSVALE (TABBERABBERA) | 529928 | 5838289 | 92.644 | 28/05/1975 | 30/09/2008 | 3430 |  |  |
| 224216 | CLIFTON CREEK @ WY YUNG | 554294.5 | 5816639.7 | 0 | 28/01/1977 | 4/01/1979 | 129 |  |  |
| 224217 | MITCHELL RIVER @ ROSEHILL | 550366 | 5814542 | 0 | 2/04/2003 |  | 4413 |  |  |
| 224220 | RAIN GAUGE (BOGGY CREEK) @ BULLUMWAAL | 548565.5 | 5830013.3 | 178.885 | 26/02/1991 |  | 83 |  |  |
| 224222 | MITCHELL RIVER U/S GLENALADALE PUMPHOUSE | 532443 | 5823351 | 0 | 18/12/2009 |  | 0 |  |  |

Table 15 	River gauges for Snowy Basin

| Site Id | Site Name | Easting | Northing | Gauge Zero (Ahd) | Site Start Date | Site Cease Date | Drainage Area (Sq.km) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 222200 | SNOWY RIVER @ JARRAHMOND | 620071 | 5830901 | 0 | 30/06/1971 |  | 13421 |  |  |
| 222201 | SNOWY RIVER @ ORBOST | 627898 | 5825644.4 | -0.141 | 27/03/1997 |  | 13572 |  |  |
| 222202 | BRODRIBB RIVER @ SARDINE CREEK | 636752 | 5847240 | 78.591 | 16/06/1998 |  | 658 |  |  |
| 222203 | SNOWY RIVER @ MARLO JETTY | 634504.8 | 5815384.5 | -0.745 | 25/06/1934 | 18/01/1998 | 0 |  |  |
| 222204 | SNOWY RIVER @ BETE BELONG | 623487.8 | 5824589.9 | 0 | 21/10/1938 | 26/07/1949 | 13540 |  |  |
| 222205 | SNOWY RIVER @ NEWMERELLA-LOCHEND ROAD | 632611.9 | 5821910.2 | -0.479 | 1/03/1960 | 27/01/1983 | 13600 |  |  |
| 222206 | BUCHAN RIVER @ BUCHAN | 603711 | 5849459 | 74.85 | 27/03/1926 |  | 822 |  |  |
| 222207 | BUCHAN RIVER @ MURRINDAL | 609403 | 5848400.8 | 0 | 15/02/1951 | 21/11/1972 | 1204 |  |  |
| 222209 | SNOWY RIVER @ MCKILLOP BRIDGE | 625612 | 5894902 | 0 | 5/04/1967 |  | 10619 |  |  |
| 222210 | DEDDICK RIVER @ DEDDICK (CASEYS) | 626577.7 | 5894693.7 | 0 | 5/04/1973 |  | 857 |  |  |
| 222212 | SNOWY RIVER @ BASIN CREEK NEAR BUCHAN | 613458.7 | 5850738.1 | 0 | 4/05/1932 | 10/01/1934 | 11836 |  |  |
| 222213 | SUGGAN BUGGAN RIVER @ SUGGAN BUGGAN | 618048 | 5909278 | 0 | 25/06/1974 |  | 361 |  |  |
| 222214 | ROCKY RIVER @ NEAR ORBOST | 645109.5 | 5833372.7 | 0 | 8/12/1969 | 17/03/1978 | 20 |  |  |
| 222216 | MURRINDAL RIVER @ BASIN ROAD (BUCHAN) | 608500 | 5849900 | 0 | 4/02/1976 | 30/06/1987 | 302 |  |  |
| 222217 | RODGER RIVER @ JACKSONS CROSSING | 620346 | 5858744 | 0 | 3/06/1976 |  | 447 |  |  |
| 222218 | LITTLE RIVER @ WULGULMERANG | 616400 | 5897200 | 0 | 21/04/1977 | 18/07/1984 | 88 |  |  |
| 222219 | SNOWY RIVER @ D/S OF BASIN CREEK | 612580 | 5848984 | 0 | 12/12/1978 |  | 11964 |  |  |
| 222221 | BUCHAN RIVER @ EGW OFFTAKE | 603874 | 5852271 | 0 | 14/05/2009 |  | 0 |  |  |
| 222222 | ROCKY RIVER U/S OT THE WEIR | 645938 | 5834053 | 0 | 11/03/2009 |  | 0 |  |  |
| 222400 | MOYANGUL RIVER @ LOOKOUT NEAR TIN MINE | 613203.5 | 5941897.7 | 0 | 14/03/1955 | 14/11/1963 | 29 |  |  |
| 222401 | INGEEGOODBEE RIVER @ D/S OF TIN MINE HUTS | 613098.4 | 5933947.3 | 0 | 19/05/1955 | 14/11/1963 | 21 |  |  |
| 222403 | BUCHAN RIVER @ GLENMORE | 600499.4 | 5879843.5 | 0 | 27/01/1955 | 22/07/1969 | 513 |  |  |
| 222404 | MELLICK MUNJIE CREEK @ GILLINGALL | 598595.5 | 5877355.2 | 0 | 5/07/1955 | 22/07/1969 | 70 |  |  |

Table 16 	River gauges for South Gippsland region

| Site ID | Site name | Easting | Northing | Gauge zero (Ahd) | Site start date | Site cease date | Drainage area (km2) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 227001 | MERRIMAN CREEK @ SEASPRAY | 514294.7 | 5754491 | 0 | 29/09/1966 | 17/06/1971 | 525 |  |  |
| 227200 | TARRA RIVER @ YARRAM | 471649 | 5734286 | 0 | 18/02/1976 |  | 215 |  |  |
| 227201 | BRUTHEN CREEK @ WOODSIDE | 488800 | 5735750 | 0 | 1/03/1946 | 30/09/1960 | 174 |  |  |
| 227202 | TARWIN RIVER @ MEENIYAN | 412165 | 5729150 | 0 | 22/06/1955 |  | 1067 |  |  |
| 227203 | FRANKLIN RIVER @ HENWOODS BRIDGE | 436475.8 | 5725293.9 | 0 | 3/12/1946 | 8/01/1985 | 12 |  |  |
| 227205 | MERRIMAN CREEK @ CALIGNEE SOUTH | 469833 | 5755048 | 172.448 | 13/12/1965 |  | 36 |  |  |
| 227210 | BRUTHEN CREEK @ CARRAJUNG LOWER | 477651 | 5750309 | 0 | 7/08/1952 |  | 18 |  |  |
| 227211 | AGNES RIVER @ TOORA | 445387 | 5722973 | 0 | 10/01/1957 |  | 67 |  |  |
| 227213 | JACK RIVER @ JACK RIVER | 459739 | 5735728 | 0 | 1/12/1962 |  | 34 |  |  |
| 227216 | ALBERT RIVER @ HIAWATHA (BELOW FALLS) | 453863.5 | 5735533.5 | 0 | 25/06/1964 | 23/02/1989 | 41 |  |  |
| 227217 | LILLYPILLY CREEK @ STAIRCASE | 442292 | 5680222.7 | 0 | 2/09/1965 | 15/01/1974 | 5 |  |  |
| 227219 | BASS RIVER @ LOCH | 388609 | 5753722 | 0 | 1/04/1966 |  | 52 |  |  |
| 227220 | GREIG CREEK @ MUMFORDS | 473100 | 5743600 | 0 | 6/05/1968 | 2/12/1998 | 25 |  |  |
| 227221 | BODMAN CREEK @ BRIDGES | 476725.7 | 5746055.5 | 0 | 7/05/1968 | 20/10/1978 | 15 |  |  |
| 227222 | SPRING CREEK @ BOWDENS | 475282.4 | 5742352.5 | 65.909 | 20/05/1968 | 13/06/1975 | 10 |  |  |
| 227223 | MACKS CREEK @ RICHARDS | 467870.1 | 5741586.5 | 0 | 24/04/1968 | 15/06/1987 | 19 |  |  |
| 227224 | WOMERAH CREEK @ TARR VALLEY ROAD | 462201 | 5741376.9 | 0 | 29/04/1968 | 20/12/1982 | 1 |  |  |
| 227225 | TARRA RIVER @ FISCHERS | 461190 | 5741983 | 0 | 24/04/1968 |  | 16 |  |  |
| 227226 | TARWIN RIVER EAST BRANCH @ DUMBALK NORTH | 426838 | 5738399 | 0 | 8/01/1969 |  | 127 |  |  |
| 227227 | WILKUR CREEK @ LEONGATHA | 408948 | 5750056 | 0 | 31/07/1970 |  | 106 |  |  |
| 227228 | TARWIN RIVER EAST BRANCH @ MIRBOO | 433155.3 | 5737488.6 | 0 | 30/04/1971 | 18/06/1987 | 43 |  |  |
| 227231 | BASS RIVER @ GLEN FORBES SOUTH | 370463 | 5741320 | 0 | 30/03/1973 |  | 233 |  |  |
| 227232 | LANCE CREEK U/S LANCE CREEK RESERVOIR | 383783 | 5738738 | 0 | 31/03/2008 |  | 14 |  |  |
| 227234 | SPRING CREEK @ BEAUMONT | 475276.8 | 5744201.8 | 0 | 3/04/1975 | 15/12/1982 | 7 |  |  |
| 227235 | MIDDLE CREEK @ TALL TIMBERS | 460742.6 | 5742294.6 | 0 | 27/04/1978 | 18/01/1989 | 9 |  |  |
| 227236 | POWLETT RIVER @ D/S FOSTER CREEK JUNCTION | 387508 | 5731353 | 0 | 18/05/1979 |  | 228 |  |  |
| 227237 | FRANKLIN RIVER @ TOORA | 439771 | 5724088 | 0 | 7/04/1983 |  | 75 |  |  |
| 227238 | FOSTER CREEK @ DAM SITE | 387050.1 | 5739746.7 | 0 | 8/06/1979 | 17/01/1989 | 55 |  |  |
| 227239 | MERRIMAN CREEK @ STRADBROKE WEST | 492658 | 5764652 | 71.868 | 18/11/1983 |  | 256 |  |  |
| 227240 | MERRIMAN CREEK @ PROSPECT ROAD SEASPRAY | 514320 | 5754179 | 2.544 | 26/08/1983 |  | 529 |  |  |
| 227242 | MERRIMAN CREEK @ SEASPRAY TOWNSHIP | 516203 | 5752529 | 0 | 6/03/1990 |  | 8 |  |  |
| 227243 | BRUTHEN CREEK @ D/S REEDY CREEK | 484778 | 5747810 | 79.91 | 13/05/1992 |  | 124 |  |  |
| 227244 | DEEP CREEK @ FOSTER | 432312 | 5725707 | 0 | 29/04/1993 |  | 0 |  |  |
| 227245 | LITTLE BASS RIVER @ POOWONG U/S LITTLE BASS RES. | 39050 | 5753576 | 0 | 18/05/1999 |  | 0 |  |  |
| 227246 | COALITION CREEK | 396827 | 5748087.4 | 0 | 8/06/2004 |  | 0 |  |  |
| 227248 | BELLVIEW CREEK U/S BELLVIEW RESERVOIR | 396367 | 5749105 | 0 | 18/05/1999 |  | 0 |  |  |
| 227249 | RUBY CREEK @ ARAWATA | 402379 | 5748848 | 0 | 23/07/2008 |  | 0 |  |  |
| 227251 | TARRA RIVER @ TARRA WEIR OFFTAKE | 463301 | 5741305 | 0 | 27/10/2004 |  | 0 |  |  |
| 227264 | COALITION CREEK @ LEONGATHA (SPENCERS ROAD BRIDGE) | 409001 | 5743754 | 0 | 21/10/2008 |  | 0 |  |  |
| 227265 | GOLDEN CREEK @ BLACK SWAMP ROAD | 428266 | 5711026 | 0 | 15/02/2008 | 18/11/2008 | 0 |  |  |
| 227266 | TARWIN RIVER @ KOONWARRA | 408996 | 5735402 | 0 | 22/09/2008 |  | 0 |  |  |
| 227270 | FOSTER CREEK AT KORUMBURRA | 394617.6 | 5745540 |  | 13/10/2011 |  |  |  |  |

Table 17 	River gauges for Tambo Basin

| Site Id | Site Name | Easting | Northing | Gauge Zero (Ahd) | Site Start Date | Site Cease Date | Drainage Area (Sq.km) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 223202 | TAMBO RIVER @ SWIFTS CREEK | 564619 | 5875047 | 280.863 | 8/03/1977 |  | 943 |  |  |
| 223204 | NICHOLSON RIVER @ DEPTFORD | 561543 | 5839104 | 0 | 12/05/1961 |  | 287 |  |  |
| 223205 | TAMBO RIVER @ D/S OF RAMROD CREEK | 576710 | 5830147 | 15.452 | 9/06/1965 |  | 2681 |  |  |
| 223206 | TAMBO RIVER @ BINDI | 571424.2 | 5896694.3 | 0 | 8/08/1957 | 19/12/1974 | 401 |  |  |
| 223207 | TIMBARRA RIVER @ TIMBARRA | 592900 | 5869900 | 0 | 9/09/1957 | 4/06/1973 | 205 |  |  |
| 223208 | TAMBO RIVER @ BINDI (NEAR JUNCTION CREEK) | 568710 | 5887104 | 0 | 21/03/1974 | 14/07/2003 | 523 |  |  |
| 223209 | TAMBO RIVER @ BATTENS LANDING | 574796.1 | 5820757.9 | 0 | 26/01/1977 |  | 2781 |  |  |
| 223210 | NICHOLSON RIVER @ SARSFIELD | 562646 | 5823050 | 0 | 21/09/1977 |  | 471 |  |  |
| 223212 | TIMBARRA RIVER @ D/S OF WILKINSON CREEK | 594066 | 5855069 | 0 | 6/05/1982 |  | 438 |  |  |
| 223213 | TAMBO RIVER @ D/S OF DUGGAN CREEK | 578509 | 5904321 | 746.726 | 16/09/1987 |  | 96 |  |  |
| 223214 | TAMBO RIVER @ U/S OF SMITH CREEK | 582588 | 5909736 | 0 | 2/03/1989 |  | 32 |  |  |
| 223215 | HAUNTED STREAM @ HELLS GATE | 573015 | 5851389 | 153.892 | 8/02/1990 |  | 180 |  |  |
| 223216 | TAMBO RIVER U/S SWIFTS CK OFFTAKE | 563729 | 5877748 | 0 | 20/05/2009 |  | 0 |  |  |
| 223217 | NICHOLSON RIVER AT PUMP HOUSE | 564088 | 5821594 | 0 | 20/01/2011 |  | 0 |  |  |
| 223402 | TIMBARRA RIVER @ NUNNIONG PLAINS | 587028.3 | 5888208.6 | 0 | 22/06/1955 | 16/02/1960 | 16 |  |  |
| 223403 | TAMBO RIVER @ NUNNIONG PLAINS | 583943.9 | 5892462.9 | 0 | 21/06/1955 | 16/02/1960 | 39 |  |  |
| 223800 | RAINGAUGE (TAMBO RIVER) @ MOUNT ELIZABETH | 582300 | 5850700 | 0 | 15/01/1985 | 20/10/2004 | 0 |  |  |
| 223801 | RAIN GAUGE (TAMBO RIVER) @ MT ELIZABETH HELIPAD | 581909 | 5850809 | 0 | 20/10/2004 | 19/01/2011 | 0 |  |  |
| 223802 | RAINGAUGE AT MOUNT ELIZABETH SOMMERVILLE TRACK | 580733 | 5851999 |  | 19/01/2011 |  |  |  |  |

Table 18 	River gauges for Thomson Basin

| Site Id | Site Name | Easting | Northing | Gauge Zero (Ahd) | Site Start Date | Site Cease Date | Drainage Area (Sq.km) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 225019 | NORTH CASCADE CREEK @ THOMSON VALLEY ROAD | 441836.2 | 5815577.5 | 0 | 11/01/1962 | 6/06/1974 | 11 |  |  |
| 225105 | THOMSON RIVER @ NEWLAND ROAD | 427654.1 | 5819829.7 | 0 | 30/03/1954 | 1/05/1984 | 16 |  |  |
| 225114 | THOMSON RIVER @ D/S WHITELAWS CREEK | 436711 | 5825795 | 0 | 27/03/1987 |  | 155.3 |  |  |
| 225200 | THOMSON RIVER @ HEYFIELD | 480693 | 5795763 | 35.846 | 17/01/1991 |  | 1238 |  |  |
| 225201 | AVON RIVER @ STRATFORD | 506676 | 5797653 | 0 | 1/11/1976 |  | 1485 |  |  |
| 225204 | MACALISTER RIVER @ LAKE GLENMAGGIE (TAIL GAUGE) | 482885 | 5805021 | 47.015 | 28/09/1966 |  | 1891 |  |  |
| 225207 | THOMSON RIVER @ WALHALLA | 449064.3 | 5798590.9 | 0 | 9/03/1950 | 22/05/1952 | 875 |  |  |
| 225209 | MACALISTER RIVER @ LICOLA | 466762 | 5835198 | 2 | 1/08/1952 |  | 1233 |  |  |
| 225210 | THOMSON RIVER @ THE NARROWS | 447551 | 5805990 | 247.08 | 9/04/1957 |  | 518 |  |  |
| 225212 | THOMSON RIVER @ WANDOCKA | 489554 | 5792978 | 19.423 | 1/03/1977 |  | 1417 |  |  |
| 225213 | ABERFELDY RIVER @ BEARDMORE | 450135 | 5810238 | 305.552 | 27/06/1963 |  | 311 |  |  |
| 225216 | JORDAN RIVER @ ABERFELDY | 439747.8 | 5829099.3 | 0 | 4/10/1971 | 18/07/1972 | 124 |  |  |
| 225217 | BARKLY RIVER @ GLENCAIRN | 461700 | 5842800 | 263.1 | 12/05/1966 | 4/01/1989 | 248 |  |  |
| 225218 | FREESTONE CREEK @ BRIAGALONG | 508366.4 | 5815232.8 | 63.238 | 14/07/1975 |  | 309 |  |  |
| 225219 | MACALISTER RIVER @ GLENCAIRN | 461689 | 5847757 | 293.54 | 7/04/1967 |  | 570 |  |  |
| 225221 | MACALISTER RIVER @ STRINGYBARK CREEK | 470738 | 5819709 | 105.249 | 18/03/1968 |  | 1542 |  |  |
| 225222 | GLENMAGGIE CREEK @ SEATON (AUBREYS) | 471117.4 | 5803309.8 | 111.285 | 10/03/1970 | 18/12/1975 | 141 |  |  |
| 225223 | VALENCIA CREEK @ GILLIO ROAD | 499321 | 5822633 | 85.481 | 26/03/1991 |  | 195 |  |  |
| 225224 | AVON RIVER @ THE CHANNEL | 489868 | 5816150 | 72 | 12/07/1972 |  | 554 |  |  |
| 225225 | MACALISTER RIVER @ LAKE GLENMAGGIE (HEAD GAUGE) | 482418.6 | 5804322.1 |  | 26/01/1925 |  |  |  |  |
| 225228 | THOMSON RIVER @ COWWARR TIMBER WEIR H.G. | 469855.4 | 5794299.7 | -0.96 | 1/01/1958 |  | 1093 |  |  |
| 225230 | GLENMAGGIE CREEK @ THE GORGE | 469772 | 5803687 | 120.847 | 2/05/1975 |  | 139 |  |  |
| 225231 | THOMSON RIVER @ U/S OF COWWARR WEIR | 467065 | 5796878 | 69.4 | 1/04/1976 |  | 1080 |  |  |
| 225232 | THOMSON RIVER @ BUNDALAGUAH | 499308 | 5789148 | 5.094 | 3/11/1976 |  | 3538 |  |  |
| 225233 | PERRY RIVER @ PERRY BRIDGE | 523288.7 | 5793397.5 | -0.938 | 24/12/1976 | 19/11/1982 | 357 |  |  |
| 225234 | AVON RIVER @ CLYDEBANK (CHINN'S BRIDGE) | 515238 | 5791515 | 0 | 29/06/2004 |  | 1584 |  |  |
| 225236 | RAINBOW CREEK @ HEYFIELD | 480686 | 5793911 | 33.837 | 30/04/1992 |  | 0 |  |  |
| 225247 | MACALISTER RIVER @ RIVERSLEA | 498044.5 | 5791335.7 | 0 | 11/01/2001 |  | 0 |  |  |
| 225248 | BOGGY CREEK @ CORNWALLS ROAD | 492831 | 5793649.7 | 0 | 29/08/2008 |  | 0 |  |  |
| 225255 | AVON RIVER U/S VALENCIA CK JUNCTION | 498324 | 5813965 | 0 | 30/06/2004 |  | 0 |  |  |
| 225256 | MACALISTER R D/S MAFFRA (SMITHS BR.) | 498377 | 5793730 | 0 | 26/10/2005 |  | 0 |  |  |
| 225600 | LAKE WELLINGTON @ SALE | 519212 | 5781295.6 | 0 | 1/12/1973 | 1/07/1977 | 0 |  |  |
| 225703 | THOMSON RIVER DIVERSION CHANNEL @ COWWARR WEIR | 470010 | 5794250 | 0 | 5/02/2001 | 19/07/2007 | 0 |  |  |
| 225711 | LAKE WELLINGTON DRAIN @ 5 MILES 32 CHAIN MEASURING WEIR | 512208.2 | 5787288.3 | 0 | 12/11/1976 | 3/05/2001 | 0 |  |  |
| 225715 | CENTRAL GIPPSLAND DRAIN 3 @ NAMBROK (RD 1066M) | 488590 | 5786295 | 0 | 24/02/1980 | 15/04/2005 | 0 |  |  |
| 225716 | CENTRAL GIPPSLAND 1/2 DRAIN 3 @ DROP STRUCTURE (RD500) | 491500 | 5784700 | 0 | 24/02/1980 | 4/02/2004 | 0 |  |  |
| 225717 | CENTRAL GIPPSLAND 2/3 DRAIN @ US OF DRAIN 3 (RD 500M) | 491102 | 5783737 | 0 | 24/02/1980 | 15/04/2005 | 0 |  |  |
| 225721 | CENTRAL GIPPSLAND DRAIN NO 3/3 U/S OUTFALL | 487050 | 5788350 | 0 | 29/05/2000 | 13/02/2004 | 0 |  |  |
| 225722 | CENTRAL GIPPSLAND DRAIN NO 3 D/S NO3 O/FALL | 487100 | 5788300 | 0 | 29/05/2000 | 13/02/2004 | 0 |  |  |
| 225723 | CENTRAL GIPPSLAND DRAIN NO 1/3 @ D/S 11/4/1 JUNCTION | 488200 | 5784600 | 0 | 29/05/2000 | 13/02/2004 | 0 |  |  |
| 225724 | CHANNEL O/FALL 11/4/1 U/S JUNCTION CG DR 1/3 | 488150 | 5784500 | 0 | 29/05/2000 | 15/04/2005 | 0 |  |  |
| 225725 | CENTRAL GIPPSLAND DRAIN NO 3 D/S O/FALL 11/1 | 490400 | 5783500 | 0 | 26/05/2000 | 15/04/2005 | 0 |  |  |
| 225726 | CENTRAL GIPPSLAND DRAIN NO 3 D/S RAILWAY LINE | 491000 | 5781800 | 0 | 29/05/2000 | 15/04/2005 | 0 |  |  |
| 225727 | CENTRAL GIPPSLAND DRAIN NO 2/3 @ SOLDIERS RD | 490100 | 5785600 | 0 | 26/05/2000 | 13/02/2004 | 0 |  |  |
| 225737 | CENTRAL GIPPSLAND DRAIN NO 8/2 @ DENISON ROAD | 483978 | 5789779 | 0 | 28/05/2002 | 15/04/2005 | 0 |  |  |
| 225738 | CENTRAL GIPPSLAND DRAIN NO 2 @ SALE-TOONGABBIE ROAD | 487500 | 5782800 | 0 | 28/05/2002 | 15/04/2005 | 0 |  |  |
| 225739 | CENTRAL GIPPSLAND DRAIN NO 2/2 @ DESSENTS | 487311 | 5782836 | 0 | 28/05/2002 | 15/04/2005 | 0 |  |  |
| 225740 | CENTRAL GIPPSLAND DRAIN NO 2 U/S TINAMBA-ROSEDALE ROAD | 487800 | 5781900 | 0 | 16/05/2002 | 15/04/2005 | 0 |  |  |
| 225741 | CENTRAL GIPPSLAND DRAIN NO 2 U/S NAMBROK ROAD | 485335 | 5786592 | 0 | 16/05/2002 | 15/04/2005 | 0 |  |  |
| 225742 | CENTRAL GIPPSLAND DRAIN NO 2 U/S SALE-COWWARR ROAD | 484488 | 5788744 | 0 | 28/05/2002 | 15/04/2005 | 0 |  |  |
| 225743 | CENTRAL GIPPSLAND DRAIN NO 6/2 U/S DENISON ROAD | 483670 | 5788400 | 0 | 28/05/2002 | 15/04/2005 | 0 |  |  |
| 225747 | NOBLES O/FALL @ VALENCIA CK | 498563 | 5813297 | 0 | 24/02/2004 | 1/07/2005 | 0 |  |  |
| 225748 | TINAMBA MAIN O/FALL @ MENBURN PARK | 498214 | 5813767 | 0 | 25/02/2004 | 27/06/2005 | 0 |  |  |
| 225801 | RAIN GAUGE (MACALISTER RIVER) @ MURDERERS HILL | 460964 | 5810725 | 0 | 8/06/1970 |  | 0 |  |  |
| 225802 | RAIN GAUGE (MACALISTER RIVER) @ MOUNT TAMBORITHA | 472550 | 5853466 | 0 | 1/04/1986 |  | 0 |  |  |
| 225809 | RAIN GAUGE (AVON RIVER) @ MOUNT WELLINGTON | 487412 | 5850069 | 0 | 21/05/1997 |  | 0 |  |  |
| 225810 | RAIN GAUGE (AVON RIVER) @ REEVE KNOB | 500533.2 | 5846182 | 0 | 21/05/1997 |  | 0 |  |  |
| 225819 | RAINGAUGE @ MT USEFUL | 456353 | 5827872 | 0 | 27/05/2002 |  | 0 |  |  |
| 225823 | RAINGAUGE AT BLANKET HILL | 472533.6 | 5811892.1 |  | 26/07/2010 |  |  |  |  |
| 225824 | RAINGAUGE (MACALISTER RV) AT SNOWY RANGE | 0 | 0 |  | 22/02/2011 |  |  |  |  |
| 225825 | RAINGAUGE (MACALISTER RV) AT HIGH RIDGE | 0 | 0 |  |  |  |  |  |  |
| 225826 | RAINGAUGE (MACALISTER RV) AT MOUNT SUNDAY | 448995 | 5867264 |  | 6/04/2011 |  |  |  |  |

Figure 121 	Drainage channels and monitoring sites in the Macalister Irrigation District

Table 19 	Gauges monitoring drain from the Macalister Irrigation District (sourced from SWR)

| Site code | Site name | Northings_z55 | Eastings_z55 | Data Record |  | Mean Annual Discharge (ML/yr) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | From | To |  |
| 225248A | Boggy Creek | 5793110 | 493086 | 2001 | 2013 | 3710 |
| 225731A | Boisdale No 1 Drain | 5807657 | 499378 | 2001 | 2013 | 623 |
| 225732A | Bundalaguah | 5790315 | 500025 | 2001 | 2013 | 1696 |
| 225735A | CG1 | 5781567 | 478177 | 2001 | 2013 | 2106 |
| 225729A | CG2 | 5779780 | 489665 | 1999 | 2013 | 4960 |
| 225709A | CG3 | 5779400 | 492200 | 1997 | 2013 | 5431 |
| 225728A | CG4 | 5784308 | 499554 | 2000 | 2013 | 4161 |
| 225734A | CG6 | 5790162 | 493141 | 2001 | 2013 | 1955 |
| 225733A | Heyfield Ext | 5793889 | 491102 | 2001 | 2013 | 2685 |
| 225730A | Lake Wellington Main Drain | 5785744 | 515613 | 1997 | 2013 | 6133 |
| 225745A | Lake Wellington No 1 Drain | 5783956 | 513485 | 2004 | 2013 | 1366 |
| 225245A | Newry Creek | 5801277 | 495980 | 2000 | 2013 | 4251 |
| 225746A | Newry No 1 | 5800909 | 493696 | 2004 | 2013 | 388 |
| 225251A | Nuntin Creek | 5791751 | 509946 | 2001 | 2013 | 1799 |

Figure 122 	Mean monthly flow (ML/month) at key gauges monitored by SRW in the Macalister Irrigation District

Figure 123 	Annual Flow (ML/yr) at key gauges monitored by SRW in the Macalister Irrigation District

##### Surface water – groundwater interactions

Groundwater generally interacts with surface water through various processes and pathways. The development of groundwater often has impacts on major streams (and vice versa). As such it is necessary to manage groundwater and surface water resources in combination. This requires an understanding of the interconnectivity and processes underpinning surface water and groundwater interactions. In the context of water resource management it is important to understand and account for surface-groundwater water interaction when considering issues, including:

double accounting of water resources

impacts of groundwater pumping on stream flow, particularly flow depletion

surface water requirements for downstream users

water requirements for environmental purposes (e.g. floodplain, stream, wetland ecosystems)

health of groundwater-dependent ecosystems (GDEs)

conjunctive resource management strategy development and water allocation regime

salinity impacts on water quality, salt loads, and ecosystem health

management for climate variation/change and its impacts on groundwater–surface water systems.

To better inform the numerical groundwater model developed for the Gippsland region, a brief assessment of surface-groundwater water interaction across the study area was undertaken, based on available literature (e.g. DSE 2012; SKM 2012a, 2012b; Hofmann 2011) and analysis of groundwater and surface water information. DSE (2012) collated a state-wide dataset of groundwater and surface water interaction from numerous investigations across Victoria. The dataset described groundwater and surface water interaction in four broad classes: neutral/losing, gaining, variable and unclassified. SKM (2012a) undertook baseflow separation analysis for 180 stream gauges on unregulated rivers in Victoria. This included 51 gauges in the Gippsland region. The baseflow separation analysis was undertaken on historical river flow records up to 2012 and utilised a filter parameter of 0.98. The results of the analysis for the 51 stream gauges in the Gippsland region is summarised in Table 20.

The groundwater and surface water interaction in the main rivers in the Gippsland regions is summarised below.

In the East Gippsland catchment it is believed that the shallow aquifers are well connected to the rivers and all river reaches are generally gaining. The annual average base flow indices (BFI) are high, ranging from 0.72 to 0.79 (DSE, 2012; SKM, 2012b). As there are few groundwater monitoring bores in the catchment, these gaining conditions are not evidenced by a groundwater hydrograph.

In the Latrobe catchment, the river reaches in the upper part of the catchment (upstream of Traralgon) are generally gaining with high annual average BFI ranging from 0.71 to 0.79 (DSE, 2012; SKM, 2012b). The surface water – groundwater interaction in the lower part of the catchment is not well understood. Analysis of groundwater and stream flow data indicated that the flux exchange between the river and shallow aquifers is temporally and spatially variable. The river reaches near the Macalister Irrigation District (MID) might be dominated by gaining condition due to the elevated water table in the MID (Figure 125). Deep aquifers (e.g. Latrobe Group) are generally artesian and not in connection with rivers. Total licensed groundwater use in the catchment is estimated to be 14.4 GL/yr and extractions for mine dewatering are approximately twice this volume (SKM, 2012a). SKM (2012a) conclude that the impacts of current rates of groundwater extraction on the streamflow in the Latrobe River are low.

In the Mitchell catchment, the river reaches in the upper part of the catchment are generally gaining with annual average BFI ranging from 0.66 to 0.76 (DSE, 2012; SKM, 2012b). The surface-groundwater interaction in the lower part of the catchment was investigated by Monash University and SKM (2012a) using hydrogeochemistry and radon as a tracer (Hofmann, 2011) and using an analytical modelling approach respectively. There is a significant interaction between groundwater and surface water in the Mitchell River floodplain area. Hofmann (2011) found that these river reaches have gaining and losing sections and these invert depending on the flow conditions of the river. This is supported by groundwater hydrographs of the bores near the river (Figure 126). SKM (2012a) found that a high proportion of the Mitchell River catchment water yield is derived from the higher altitude areas in the upper catchment. Flows at Bairnsdale are much lower than those upstream and the Mitchell River becomes a losing stream as it emerges from the ranges (SKM, 2012a). SKM (2012a) estimated that the volume of discharge lost from the Mitchell River due to groundwater extraction is negligible when compared to the cumulative yearly discharge, but considerable during periods of low flow. During low flows periods, it is estimated that groundwater extraction can lead to a 13% reduction in streamflow. Groundwater levels are therefore considered to have a significant influence on the flow and aquatic habitat condition of the Mitchell River during low flow periods.

In the Snowy catchment, most river reaches are gaining with annual average BFI ranging from 0.69 to 0.79 (DSE, 2012; SKM, 2012a). The surface water – groundwater interaction in the lower part of the catchment is not well understood. Analysis of limited groundwater and stream flow data indicated that the flux exchange between the river and aquifers is variable (Figure 127).

In the South Gippsland catchment, based on stream flow analysis, most river reaches are gaining, with annual average BFI ranging from 0.64 to 0.78 (DSE, 2012; SKM, 2012a).

In the Tambo catchment, the river reaches in the upper part of the catchment are generally gaining, with annual average BFI ranging from 0.69 to 0.77 (DSE, 2012; SKM, 2012a). The surface water-groundwater interaction in the lower part of the catchment was investigated by Unland (2013) using hydrogeochemistry tracer techniques. Unland (2013) found that these river reaches have gaining and losing sections and these invert depending on the flow conditions of the river.

In the Thomson catchment, (including the Avon catchment) the river reaches in the upper part of the catchment are generally gaining, with annual average BFI ranging from 0.64 to 0.73 (DSE, 2012; SKM, 2012a). The river reaches in the MID are also dominated by gaining condition due to the elevated water table (Figure 128).

The surface water-groundwater interaction in the lower part of the catchment was investigated by Monash University using hydrogeochemistry and radon as a tracer (Hofmann, 2011). Hofmann (2011) found that these river reaches have gaining and losing sections and these invert depending on the flow conditions of the river.

There is significant groundwater/surface water interaction along the Avon River. SKM (2008) reported that the Avon River is strongly gaining along its main stem and along most tributaries. During periods of average rainfall, the groundwater baseflow component of stream flow is approximately 24–36% of average annual flow in the Avon River and 17–25% of average annual flow in Freestone Creek. Groundwater entitlement in the catchment is 13.8 GL/yr and current use is approximately 3.6 GL/yr. SKM (2008) suggests that current groundwater use reduces streamflow by approximately 3.4 GL/yr.

Based on these estimates it is suggested that if groundwater usage increases to full entitlement (assuming pumping infrastructure can support such usage) then groundwater extractions may have a significant impact on streamflow.

Figure 124 	Surface-groundwater connectivity in the Gippsland region (source:  data compiled by DSE in 2012)

Main townsStream gauge station

Table 20 	Estimated average baseflow index (BFI) (by season and annual) in 51 unregulated catchments in the Gippsland region (sourced from SKM 2012b)

| Gauge ID | Gauge name | River basin | BFI Summer | BFI Autumn | BFI Winter | BFI Spring | BFI Annual |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 221201 | Cann River (West Branch) @ Weeragua | East Gippsland | 0.81 | 0.83 | 0.71 | 0.74 | 0.77 |
| 221204 | Thurra River @ Point Hicks | East Gippsland | 0.79 | 0.78 | 0.61 | 0.7 | 0.72 |
| 221207 | Errinundra River @ Errinundra | East Gippsland | 0.83 | 0.86 | 0.72 | 0.75 | 0.79 |
| 221208 | Wingan River @ Wingan Inlet National Park | East Gippsland | 0.82 | 0.79 | 0.6 | 0.71 | 0.73 |
| 221209 | Cann River (East Branch) @ Weeragua | East Gippsland | 0.81 | 0.8 | 0.67 | 0.72 | 0.75 |
| 221210 | Genoa River @ The Gorge | East Gippsland | 0.8 | 0.78 | 0.67 | 0.72 | 0.74 |
| 221211 | Combienbar River @ Combienbar | East Gippsland | 0.82 | 0.84 | 0.72 | 0.74 | 0.78 |
| 221212 | Bemm River @ Princes Highway | East Gippsland | 0.83 | 0.86 | 0.71 | 0.75 | 0.79 |
| 226008 | Tyers River West Branch @ Morgans Mill | Latrobe River | 0.83 | 0.83 | 0.7 | 0.71 | 0.77 |
| 226012 | Tanjil River East Branch @ Tanjil Bren | Latrobe River | 0.79 | 0.77 | 0.69 | 0.61 | 0.72 |
| 226016 | Waterhole Creek @ Morwell | Latrobe River | 0.9 | 0.82 | 0.68 | 0.73 | 0.78 |
| 226017 | Jacobs Creek @ Otooles | Latrobe River | 0.79 | 0.81 | 0.61 | 0.64 | 0.71 |
| 226204 | Latrobe River @ Willow Grove | Latrobe River | 0.86 | 0.87 | 0.72 | 0.73 | 0.8 |
| 226217 | Latrobe River @ Hawthorn Bridge | Latrobe River | 0.85 | 0.87 | 0.69 | 0.72 | 0.78 |
| 226219 | Tooronga River @ Noojee | Latrobe River | 0.85 | 0.88 | 0.7 | 0.72 | 0.79 |
| 226220 | Loch River @ Noojee | Latrobe River | 0.87 | 0.89 | 0.68 | 0.73 | 0.79 |
| 226226 | Tanjil River @ Tanjil Junction | Latrobe River | 0.83 | 0.87 | 0.67 | 0.65 | 0.75 |
| 226407 | Morwell River @ Boolarra | Latrobe River | 0.85 | 0.82 | 0.58 | 0.66 | 0.72 |
| 226410 | Traralgon Creek @ Koornalla | Latrobe River | 0.84 | 0.81 | 0.55 | 0.64 | 0.71 |
| 224205 | Dargo River @ Dargo (Upper Site) | Mitchell River | 0.79 | 0.8 | 0.57 | 0.53 | 0.67 |
| 224209 | Cobbannah Creek @ Near Bairnsdale | Mitchell River | 0.67 | 0.69 | 0.66 | 0.68 | 0.67 |
| 224213 | Dargo River @ Lower Dargo Road | Mitchell River | 0.79 | 0.83 | 0.48 | 0.56 | 0.66 |
| 224214 | Wentworth River @ Tabberabbera | Mitchell River | 0.78 | 0.86 | 0.7 | 0.7 | 0.76 |
| 222202 | Brodribb River @ Sardine Creek | Snowy River | 0.83 | 0.86 | 0.71 | 0.75 | 0.79 |
| 222206 | Buchan River @ Buchan | Snowy River | 0.77 | 0.83 | 0.6 | 0.61 | 0.7 |
| 222210 | Deddick River @ Deddick (Caseys) | Snowy River | 0.79 | 0.83 | 0.76 | 0.77 | 0.74 |
| 222213 | Suggan Buggan River @ Suggan Buggan | Snowy River | 0.77 | 0.84 | 0.57 | 0.58 | 0.69 |
| 222216 | Murrindal River @ Basin Road (Buchan) | Snowy River | 0.76 | 0.75 | 0.62 | 0.65 | 0.7 |
| 222217 | Rodger River @ Jacksons Crossing | Snowy River | 0.79 | 0.85 | 0.67 | 0.71 | 0.75 |
| 227203 | Franklin River @ Henwoods Bridge | South Gippsland | 0.84 | 0.79 | 0.61 | 0.67 | 0.73 |
| 227210 | Bruthen Creek @ Carrajung Lower | South Gippsland | 0.82 | 0.83 | 0.63 | 0.68 | 0.74 |
| 227213 | Jack River @ Jack River | South Gippsland | 0.83 | 0.84 | 0.61 | 0.68 | 0.74 |
| 227220 | Greig Creek @ Mumfords | South Gippsland | 0.84 | 0.85 | 0.67 | 0.68 | 0.76 |
| 227223 | Macks Creek @ Richards | South Gippsland | 0.84 | 0.81 | 0.68 | 0.7 | 0.76 |
| 227225 | Tarra River @ Fischers | South Gippsland | 0.84 | 0.86 | 0.68 | 0.73 | 0.78 |
| 227226 | Tarwin River East Branch @ Dumbalk North | South Gippsland | 0.85 | 0.79 | 0.52 | 0.62 | 0.69 |
| 227227 | Wilkur Creek @ Leongatha | South Gippsland | 0.84 | 0.74 | 0.45 | 0.6 | 0.64 |
| 227228 | Tarwin River East Branch @ Mirboo | South Gippsland | 0.85 | 0.76 | 0.54 | 0.62 | 0.69 |
| 227236 | Powlett River @ D/S Foster Creek Junction | South Gippsland | 0.9 | 0.8 | 0.42 | 0.6 | 0.67 |
| 227237 | Franklin River @ Toora | South Gippsland | 0.85 | 0.8 | 0.55 | 0.65 | 0.71 |
| 223204 | Nicholson River @ Deptford | Tambo River | 0.78 | 0.8 | 0.71 | 0.73 | 0.75 |
| 223206 | Tambo River @ Bindi | Tambo River | 0.78 | 0.84 | 0.57 | 0.58 | 0.69 |
| 223207 | Timbarra River @ Timbarra | Tambo River | 0.81 | 0.85 | 0.71 | 0.69 | 0.76 |
| 223211 | Haunted Stream @ Stirling | Tambo River | 0.78 | 0.83 | 0.72 | 0.67 | 0.75 |
| 223212 | Timbarra River @ D/S Of Wilkinson Creek | Tambo River | 0.79 | 0.86 | 0.69 | 0.69 | 0.76 |
| 223215 | Haunted Stream @ Hells Gate | Tambo River | 0.77 | 0.86 | 0.71 | 0.73 | 0.77 |
| 225019 | North Cascade Creek @ Thomson Valley Road | Thomson River | 0.8 | 0.77 | 0.7 | 0.62 | 0.64 |
| 225217 | Barkly River @ Glencairn | Thomson River | 0.81 | 0.83 | 0.45 | 0.57 | 0.66 |
| 225218 | Freestone Creek @ Briagalong | Thomson River | 0.73 | 0.78 | 0.64 | 0.68 | 0.71 |
| 225219 | Macalister River @ Glencairn | Thomson River | 0.8 | 0.81 | 0.42 | 0.53 | 0.64 |
| 225230 | Glenmaggie Creek @ The Gorge | Thomson River | 0.75 | 0.8 | 0.67 | 0.71 | 0.73 |

Figure 125 	Groundwater hydrographs of three nested bores located in MID west of Sale plotted against daily stream water level of Latrobe River at Rosedale (gauge station no. 226228)

Figure 126 	Groundwater hydrographs of a transect of bores located near Mitchell River west of Bairnsdale plotted against daily stream water level of Mitchell River at Rose (gauge station no. 224203)

Figure 127 	Groundwater hydrographs of two nested bores located near Snowy River west of Orbost plotted 
against daily stream water level of Snowy River at Jarrahmond (gauge station no. 222200)

Figure 128 	Groundwater hydrographs of three nested bores located in the Macalister Irrigation District north of Sale plotted against daily stream water level of Avon River at Stratford (gauge station no. 225201)

#### Implementation and coal resource development pathway

Summary

The coal resource development pathway (CRDP) consists of expanded open-cut coal operations at Yallourn, Hazelwood and Loy Yang coal mines. The assessment of the implementation of both the CRDP and baseline pathway occurs under a modified future climate scenario.

The spatio-temporal pattern of recharge was computed using the Catchment Analysis Tool (CAT) which accounted for various farming systems. Future climate was scaled seasonally to account for climate change.

Potential evapotranspiration (ET) was based on the difference between daily potential ET calculated from meteorological data and daily vegetative ET estimated from farming system modelling. The extinction depth was based on land use data.

Water extractions accounted for registered bores, mine dewatering, stock and domestic use and offshore production. Mine dewatering was reduced following mine closure.

The rate of coal resource development was set such that mine development was completed by the expected mine closure date.

##### Baseline and CRDP climate change signal

The future 90-year climate was based on a trajectory of climate change constructed from the historical January 1983 to December 2012 climate time series. The 1983 to 2012 climate sequence was assumed short enough that a changing climate trend is not significant and assumed to be long enough to be representative of the climate variability (i.e. contains the millennium drought and the floods of 2011). The 30-year historical climate time series was repeated 3 times to create a 90-year time series. The only two forward modelling scenarios that are considered are the baseline and the development pathway. The future 90-year climate time series was used for both simulations to negate the impact of the future climate from the future development upon the assets and receptors.

The future climate series was based on the projections of a single global climate model (GCM) and a single emissions scenario. Climate projections from 15 global climate models (GCMs) were available. Associated with each GCM were local seasonal scaling factors which gave the change in rainfall expected per degree of global warming. The choice of the GCM was based on evaluating each GCM assuming a global warming of 1°C. The resultant average annual rainfall estimates from each GCM for the entire study area were spatially compiled from which the GCM corresponding to the median annual rainfall estimate was selected (Table 21). Based on results summarised in Table 21, the miroc3 GCM pattern of change data was used to seasonally scale 1982 to 2011 rainfall, while all other climate variables remained unchanged. The first block 2012 to 2041 used a global warming of 1°C, the second block 2042 to 2071 used 1.5°C, and the third block from 2072 to 2101 used 2°C.

Table 21 	Average annual rainfall estimates for the Gippsland Basin derived using various global climate models assuming a global warming of 1oC

| GCM | Average annual rainfall (mm/yr) | % of baseline (historical 1982-2011) |
| --- | --- | --- |
| Baseline (historical 1982-2011) | 904 | 100 |
| cccmacgcm31t47 | 867 | 95.8 |
| cccmacgcm31t63 | 901 | 99.6 |
| cnrmcm3 | 841 | 93.0 |
| csiromk3 | 865 | 95.7 |
| gfdlcm2 | 841 | 93.0 |
| Gissaom | 876 | 96.8 |
| iapfgoals1 | 890 | 98.5 |
| inmcm3 | 875 | 96.8 |
| ipslcm4 | 826 | 91.3 |
| miroc3 | 874* | 96.6 |
| Miubechog | 862 | 95.3 |
| mpiecham5 | 879 | 97.2 |
| mricgcm2 | 866 | 95.8 |
| ncarccsm3 | 899 | 99.4 |
| ncarpcm1 | 909 | 100.5 |

* Identified as the median rainfall estimate.

##### Baseline and CRDP recharge

For the baseline and coal resource development pathway (CRDP) simulations, recharge was estimated using the Catchment Analysis Tool (CAT) (Beverly et al., 2005; Beverly, 2009) under the future climate pattern scaled seasonally to account for climate change. The CAT landscape model comprises a suite of farming system models to simulate soil-water-plant interactions in the unsaturated zone. Daily water balance estimates were derived for the period 1970 to 2102 using a combination of historical and future climate conditions from which recharge was spatially assigned to the groundwater model. Although the CAT model accounts for elevated carbon dioxide impacts on plant performance, this was not considered in the current study.

##### Baseline and CRDP evapotranspiration

Potential groundwater evapotranspiration (ET) was based on the water balance estimates derived using the Catchment Analysis Tool (CAT) (Beverly et al., 2005; Beverly, 2009) and as used to derive recharge estimates under the future climate patterns (refer to above). The maximum evapotranspiration rate was calculated as the difference between potential daily ET and estimated daily vegetative ET as predicted using the CAT modelling framework. This approach ensures that the sum of estimated daily vegetative ET from the unsaturated zone and the assigned maximum groundwater ET from the saturated zone cannot exceed daily potential ET calculated from meteorological data.

The groundwater ET extinction depth that defines the water table depth (below natural surface below which no ET will occur) was based on the vegetation root depth assigned in the various farming system models used to estimate recharge within the CAT modelling framework as reported in Beverly et al. (2015). The extinction depths vary from 800 mm for annual pasture to 7 m for forests.

##### Baseline extractions

The groundwater model incorporated 1569 registered bores, 118 mine dewatering pumps, 53,245 stock and domestic bores and 22 offshore production bores.  The predictive modelling applied the baseline extraction volumes for operational developments as of December 2012.

##### CRDP extractions

The predictive modelling from January 2013 to December 2102 applied the 2010 to 2011 extraction volumes year-on-year with the exception of the mine dewatering pumps. The extraction volumes assigned to mine pumps applied the 2007 to 2008 annual volume distributed evenly across each month until the expected mine closure date, after which the volume was progressively reduced according to the following criteria:

Yallourn mine pumps: cease all pumping post-December 2032

Hazelwood mine pumps: maintain 2013 extractions until December 2036 after which extractions are reduced pro-rata over the remaining modelling period with a target date to cease pumping in 2123

Loy Yang mine pumps: maintain 2013 extractions until December 2048 after which extractions are reduced pro-rata over the remaining modelling period with a target date to cease pumping in 2123.

It must be stressed that the mine rehabilitation scenario described above is hypothetical and is not based on consultation with the appropriate water supply regulator. As such, it does not imply a preferred or intended rehabilitation strategy.

Figure 129	Groundwater extractions applied to the model for 2013 to 2102

##### CRDP mine development

As reported in Section 2.3.4.1 of the Conceptual Modelling report (product 2.3) for the Gippsland Basin bioregion, the development pathways of three coal mines are considered in the Gippsland Basin numerical model relating to the Yallourn Coal Field Development Project, the Hazelwood West Field Project and future mining blocks within the current Loy Yang Project. The planned development of these mines is described in Section 1.2.3 of the Coal and Coal Seam Gas Resource Assessment (product 1.2) for the Gippsland Basin bioregion.

The modelling assumed that the expansion of existing open-cut mine pits occurs in a sequenced and progressive lateral direction. The progressive development stages were based on mine work plans and broadly adopted the following pathways:

- Yallourn Coal Field Development Project: expand the existing void in a south-easterly direction

- Hazelwood West Field Project: expand the existing void in a westerly direction

- Loy Yang: expand in a north easterly direction and then travel in a southerly direction to align with the southern boundary of the extraction limit.

The rate of development was set such that mine development was completed by the expected mine closure date. Drain cells were used to simulate mine voids so as to enforce outflows from the groundwater model. The base of each drain cell was set to the base of the coal seam less 10 metres, where the coal seams of interest were the Yallourn Seam, Morwell interseams and the M2A sequence for the Yallourn, Hazelwood and Loy Yang mines respectively. The drain conductance’s were set to ensure inactive cells above the modelled layer representing the base of the void and set to the lateral conductivity of the coal seam in that layer representing the base of the void. Drain volumes within the void were compared for consistency to existing mine dewatering extractions.

#### Parameterisation

Summary

The regional groundwater model simulates 30 modelled layers representing key aquifers, aquitards and coal seams. There are up to four key hydraulic parameters associated with each modelled layer. The attribution of modelled groundwater layers were initially based on reported parameter ranges and then subsequently refined based on a spilt transient calibration/verification approach in which parameter optimisation was applied from 1990 to 2000 following which verification was undertaken over the subsequent 11 years from 2001 to 2012.

Initial parameter ranges presented in Table 22 are sourced from literature and previous numerical modelling and show that there is considerable variability in both the hydraulic conductivity and storage estimates. Initial model parameter attribution is also summarised in Table 22; this attribution is based upon the perceived likely value of the aquifer based upon the pre-existing studies cited. Initial model parameters are based on the average aquifer values, and by definition within the range cited, for the particular hydrogeological unit that the layer represents.

The depositional history of the basin is dominated by fluvial, deltaic, marginal marine and open marine environments (Schaeffer, 2008, after others). Typically, vertical hydraulic conductivity (Kz) is approximated as an order of magnitude less than horizontal hydraulic conductivity (Kxy), and as an initial calibration value, Kz values were set to 0.1 of the Kxy value, and the final, calibrated, value of this ratio is provided in Table 23.

The groundwater model calibration was based on a spilt transient calibration/verification approach in which parameter optimisation was applied from 1990 to 2000 following which verification was assessed over the subsequent 11 years from 2001 to 2012. Computational constraints impacted on the progress of the transient calibration. The calibration strategy was to produce modelled hydrographs that match the trends of the observation bores. It was acknowledged that any elevation offsets between observed and predicted heads would be addressed as part of future model refinement.

With the exception of zones 23–29 representing the Strzelecki Formation parameterisation, the post-calibration modelled aquifer parameterisation is summarised in Table 23. The aquifer parameterisation attributed to zones 23–29 were revised following a review of the Strzelecki Formation parameterisation. The model calibration results in the horizontal hydraulic conductivity for the uppermost Strzelecki units of the order 0.195 m/day and the vertical hydraulic conductivity of 0.0295 m/day which are inconsistent with the potential for this layer to act as a tight or shale gas reservoir. In order to better represent this potential, a lower value was used for scenario purposes. The spatial variability of key aquifer parameters for modelled layers 2 (the Quaternary deposits), 9 (lower M2 interseam), 18 (the Upper Latrobe Group), 19 (the T1 coal seam) and 24 (the Strzelecki 500–1000 m) are shown in Figure 130 to Figure 143 inclusive.

Table 22	Range and value of specific yield (Sy), specific storage (Ss) and lateral hydraulic conductivity (Kxy) initially assigned to each modelled layer in the Gippsland model

| Model layer | VAF no. | Coal name | Comment | VAF HGU | Layers (Schaeffer 2008) | Aquifer type1 | Ss (m–1) 2,3 | Sy2,3 | Kxy (m/d) 2,3 | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | Marine water thickness | N/A | N/A | N/A | (1.0 X 10-5) | (1.0) | (100) | Initial values in this study |
| 2 | 101 |  | Quaternary | Various Aeolian deposits, various fluvial, lacustrine, alluvial and colluvial sediments | 1 | UC | 2.5 X 10-5 | 0.15 | 0.01 to 10.26 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.02 to 0.05 | 0.04 to 0.08 | 0.1 to 1 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 1.0 X 10-5 to 1.0 X 10-4 | 0.04 to 0.25 | 2 to 50 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | N/A | 0.001 to 0.05 | 1.0 X 10-6 to 100 | Dahlhaus et al. (2004) |
|  |  |  |  |  |  |  | 2.4 X 10-6 | N/A | 59 | Mollica (1991) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.07) | (2.0) | Initial values in this study |
| 3 | 102 |  | Haunted Hill Formation | Haunted Hill Formation, Eagle Point Sand | 1 | UC | 1.0 X 10-5 | 0.1 to 0.15 | 0.01 to 10.26 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.02 to 0.05 | 0.04 to 0.08 | 1 to 10 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 2 to2.01 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | N/A | 0.001 to 0.05 | 1.0 X 10-6 to 100 | Dahlhaus et al. (2004) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.1) | (2.01) | Initial values in this study |
| 4 | 103 |  | Nuntin clay | Boisdale Fm (Nuntin Clay), Jemmys Point Fm Sale Grp | 1 | C/UC | 1.0 X 10-6 to 1.0 X 10-5 | 0.005 to 0.1 | 0 to 0.5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.001 | 0.02 | 0.2 to 0.5 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.1 to 0.23 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | (1.0 X 10-6) | (0.04) | (0.23) | Initial values in this study |
| 5 | 105 |  | Boisdale Formation | Boisdale Fm (Wurruk Sand), Jemmys Point Fm, Unnamed Tertiary Sands, Gravels and Clays | 1 | C/UC | 1.0 X 10-5 to 1.0 X 10-3 | 0.1 to 0.2 | 1 to 30 | Schaeffer (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 5.31 | Nahm (1977) |
|  |  |  |  |  |  |  | N/A | N/A | 6.5 | Nahm & Reid (1979a) |
|  |  |  |  |  |  |  | N/A | 0.1 | 0.1 to 1 | SKM (1999) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | N/A | 5 to 24 | Walker and Mollica (1990) |
|  |  |  |  |  |  |  | 0.001 | 0.02 | 15 to 25 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 2 to 12.38 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 1.0 X 10-4 to 5.0 X 10-4 | N/A | 4.7 to 13 | SKM (2006) |
|  |  |  |  |  |  |  | (1.0 X 10-4) | (0.1) | (12.38) | Initial values in this study |
| 6 | 106 |  | Jemmy’s Point Formation and upper Hazelwood Formation | Jemmy’s Point Formation and upper Hazelwood Formation | 1 | C/UC | 1.0 X 10-5 to 5.0 X 10-4 | 0.1 to 0.2 | 0.2 to 13 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.001 | 0.02 | 0.2 to 0.5 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 1.0 X 10-6 to 1.0 X 10-5 | 0.1 | 2 to 10 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | (2.0 X 10-5) | (0.1) | (0.23) | Initial values in this study |
| 7 | 106 | Yallourn Coal Seam | Y, Y1a, Y1b, Y2, Y1; y_all | Yallourn Formation | 2 | C/UC (Aquitard) | 2.5 X 10-5 to 2.1 X 10-4 | 0.001 to 0.05 | 2.0 X 10-6 to 0.1 | Schaeffer (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 0.005 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | 0.0001 | 0.005 | 0.04 | Aquaterra (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 0.015 to 0.1 | PDA (2006) |
|  |  |  |  |  |  |  | N/A | N/A | 6 X 10-5 to 1.8 X 10-1 | Harlow & LeCain (1993) |
|  |  |  |  |  |  |  | 3.0 X 10-6 to 5.0 X 10-4 | N/A | 0.0005 to 1.36 | USQ (2011) |
|  |  |  |  |  |  |  | (2.0 X 10-5) | (0.02) | (0.0002) | Initial values in this study |
| 8 | 106 | Yallourn Aquifer & interseam | Hazelwood Formation; all floor & M1a_all top | Hazelwood Formation, Yallourn Formation | 3 | C/UC | 1.0 X 10-6 to 2.5 X 10-5 | 0.05 to 0.1 | 0.2 to 8 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.001 | 0.02 | 0.2 to 0.5 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 2 to 2.44 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 0.2 to 0.5 | SKM (1999) |
|  |  |  |  |  |  |  | 6.8 X 10-5 | N/A | 8 | Blake (1972) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.1) | (2.44) | Initial values in this study |
| 9 | 107, 108 | Lower M2 interseam | Balook Formation Tambo River, Wuk Marl, Gippsland Limestone, | Balook Fm, LVG: Yarragon Fm, Alberton Fm, Tambo River Fm, Gippsland Limestone, Middle Lakes Entrance Fm | 5, 9 | C/UC | 1.0 X 10-6 to 1.0 X 10-4 | 0.04-0.06 | 2 to 57 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.0008 | 0.05 | 1.5 to 2 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 1 to 7.5 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | N/A | N/A | 10 to 58 | Reid (1985) |
|  |  |  |  |  |  |  | 1.0 X 10-5 | 0.06 | 1 | Golder Associates (1990) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 0.5 to 8 | SKM (1999) |
|  |  |  |  |  |  |  | N/A | N/A | 2 to 7 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | 1 X 10-5 to 09.0 X 10-4 | N/A | 2 to 30 | Walker & Mollica (1990) |
|  |  |  |  |  |  |  | (5.0 X 10-6) | (0.05) | (3.53) | Initial values in this study |
| 10 |  | M1A coal | Yarragon Formation, M10, M1a, M1b2, ML, M12; M1a_all | Yarragon Formation, Upper Gippsland Limestone | 4 | C/UC (Aquitard) | 1.0 X 10-5 to 1.0 X 10-4 | 0.001 to 0.05 | 2.0 X 10-6 to 0.1 | Schaeffer (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 0.015 to 0.1 | PDA (2006) |
|  |  |  |  |  |  |  | 0.0001 | 0.005 | 0.04 | Aquaterra (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 6 X 10-5 to 1.8 X 10-1 | Harlow & LeCain (1993) |
|  |  |  |  |  |  |  | 3.0 X 10-6 to 5.0 X 10-4 | N/A | 0.0005 to 1.36 | USQ (2011) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.02) | (0.0005) | Initial values in this study |
| 11 |  | Morwell 1A interseam/aquifer | M1a_all_floor & M1b_top | Morwell Formation, Middle Gippsland Limestone | 5 | C/UC | 1.0 X 10-6 to 1.0 X 10-4 | 0.05 to 0.1 | 7.7 X 10-5 to 8 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.0002 | 0.02 | 0.04 to 0.05 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 2 to 2.44 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 0.28 to 0.5 | SKM (1999) |
|  |  |  |  |  |  |  | 1.2 X 10-5 | N/A | 0.27 | Nahm (1972) |
|  |  |  |  |  |  |  | (5.0 X 10-6) | (0.1) | (2.44) | Initial values in this study |
| 12 |  | Morwell 1B coal | M1b, M1b1, M1b2, ML, M12 | Morwell Formation / Morwell seams, Lower Gippsland Limestone | 6 | C/UC (Aquitard) | 1.0 X 10-6 to 5.0 X 10-4 | 0.001-0.05 | 2.0 X 10-6 to 0.1 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.0002 | 0.02 | 0.04-0.05 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | N/A | N/A | 0.0001 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | N/A | N/A | 0.015 to 0.1 | PDA (2006) |
|  |  |  |  |  |  |  | 0.0001 | 0.005 | 0.04 | Aquaterra (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 6 X 10-5 to 1.8 X 10-1 | Harlow & LeCain (1993) |
|  |  |  |  |  |  |  | 3.0 X 10-6 to 5.0 X 10-4 | N/A | 0.0005 to 1.36 | USQ (2011) |
|  |  |  |  |  |  |  | (5.0 X 10-6) | (0.05) | (0.0015) | Initial values in this study |
| 13 |  | Morwell 1B interseam | Floor M1b_all & M2_all top | Morwell Formation / Morwell seams, Upper Lakes Entrance Formation | 7 | C/UC | 9.35 X 10-9 to 1 X 10-4 | 0.06-0.1 | 2.3 X 10-4 to 40.39 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.0002 | 0.02 | 0.04-0.05 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.97-1 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 1.0 X 10-6 to 9.4 X 10-9 | N/A | 0.08 to 4.07 | Nahm (1977) |
|  |  |  |  |  |  |  | 0.003 | N/A | 2.5 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | 0.001 | N/A | 10.83 | Fraser (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 4.48 to 47.68 | Barton (1971) |
|  |  |  |  |  |  |  | 1.0 X 10-6 to 1.0 X 10-4 | N/A | 2 to 7 | Thatcher (1976) |
|  |  |  |  |  |  |  | 2.5 X 10-5 | 0.06 | 0.2 to 5 | Golder Associates (1990) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 0.5 to 8 | SKM (1999) |
|  |  |  |  |  |  |  | N/A | N/A | 6.91 | Golder Brawner (unpubl. data) |
|  |  |  |  |  |  |  | (4.0 X 10-6) | (0.1) | (0.97) | Initial values in this study |
| 14 |  | Morwell 2 | M2, M2A, M2B coal; M2_all | Morwell Formation / Morwell seams / Middle Lakes Entrance Formation | 8, 10 | C/UC (Aquitard) | 8.8 X 10-7 to 9.2 X 10-4 | 0.001 to 0.05 | 1.0 X 10-5 to 0.1 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.0002 | 0.02 | 0.04 to 0.05 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.1 to 0.42 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | N/A | N/A | 0.015 to 0.1 | PDA (2006) |
|  |  |  |  |  |  |  | 0.0001 | 0.005 | 0.04 | Aquaterra (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 6 X 10-5 to 1.8 X 10-1 | Harlow and LeCain (1993) |
|  |  |  |  |  |  |  | 3.0 X 10-6 to 5.0 X 10-4 | N/A | 0.0005 to 1.36 | USQ (2011) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.02) | (0.42) | Initial values in this study |
| 15 | 108 |  | Lakes Entrance Formation | Lakes Entrance Fm /Morwell Formation | 8, 10, 12 | C/UC (Aquitard) | 3.4 X 10-7 to 1.0 X 10-4 | 0.001 to 0.05 | 1.0 X 10-5 to 0.1 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.0002 | 0.02 | 0.04 to 0.05 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.1 to 0.42 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | N/A | N/A | 0.03 | Thatcher (1976) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.02) | (0.05) | Initial values in this study |
| 16 | 109 |  | M2c aquifer/ Seaspray Sands | LVG: M2C aquifer, Seaspray Sand, Lower Lakes Entrance Fm, Seaspray Sands | 11, 13 | C/UC | 1 X 10-6 to 4.7 X 10-4 | 0.03 to 0.1 | 6.4 X 10-4 to 76.06 | Schaeffer (2008); |
|  |  |  |  |  |  |  | 0.0008 | 0.05 | 1.5 to 2 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.1 to 1.91 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 6.3 X 10-6 to 2.2 X 10-5 | N/A | 2 to 48 | Nahm (1973a, 1973b, 1977) |
|  |  |  |  |  |  |  | 4.0 X 10-4 | N/A | 5.7 to 32.35 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | 5.0 X 10-5 to 2.5 X 10-4 | N/A | 0.1 to 1.11 | Thatcher (1976) |
|  |  |  |  |  |  |  | N/A | N/A | 0.22 to 19.38 | Geo-Eng (1993, 1996, 2001) |
|  |  |  |  |  |  |  | N/A | N/A | 7.49 | Reid (1985) |
|  |  |  |  |  |  |  | N/A | N/A | 23.23 | Barton (1971) |
|  |  |  |  |  |  |  | 1.0 X 10-5 to 1.0 X 10-3 | 0.015 to 0.06 | 0.2 to 8 | Golder Associates (1990) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 0.5 to 8 | SKM (1999) |
|  |  |  |  |  |  |  | 3.0 X 10-4 | N/A | 13.6 | Fraser (1980) |
|  |  |  |  |  |  |  | (3.0 X 10-5) | (0.1) | (1.63) | Initial values in this study |
| 17 | 112 |  | Thorpdale volcanics | Thorpdale Volcanics | 7, 9, 11 | C/UC | 1.0 X 10-6 to 4.7 X 10-4 | 0.015 to 0.1 | 0.03 to 6.4 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.04 | 0.05 | 0.2 to 1 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.51 to 2 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 5.0 X 10-5 to 2.5 X 10-4 | N/A | 0.1 to 1.11 | Thatcher (1976) |
|  |  |  |  |  |  |  | 1.7 X 10-7 to 3.5 X 10-5 | N/A | 7.49 to 23.6 | Reid (1985a; 1985b) |
|  |  |  |  |  |  |  | 1.0 X 10-5 to 1.0 X 10-3 | 0.015 to 0.06 | 0.2 to 8 | Golder Associates (1990) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 8 | SKM (1999) |
|  |  |  |  |  |  |  | 6.3 X 10-6 to 2.2 X 10-5 | N/A | 2 to 48 | Nahm & Reid (1979a, 1979b) |
|  |  |  |  |  |  |  | N/A | <0.05 | 0.001 to 100 | Dahlhaus et al. (2004) |
|  |  |  |  |  |  |  | 5.0 X 10-5 | N/A | 6 to 35.6 | Pratt (1985) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.1) | (0.51) | Initial values in this study |
| 18 | 111 |  | Upper Latrobe Group | Childers Fm, M2 / M2C aquifer (when basal aquifer) | 13 | C/UC | 1.0 X 10-5 to 5.6 X 10-2 | 0.03 to 0.1 | 1.6 X 10-3 to 32.35 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.04 | 0.05 | 0.2 to 1 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.76 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 1.5 X 10-6 to 4.0 X 10-5 | N/A | 1 to 13.84 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | 1.0 X 10-5 to 1.1 X 10-4 | N/A | 0.25 to 7.3 | Thatcher (1976) |
|  |  |  |  |  |  |  | N/A | N/A | 2.34 to 76.06 | Geo-Eng (1993, 1996b, 2001) |
|  |  |  |  |  |  |  | 1.7 X 10-7 to 3.5 X 10-5 | N/A | 7.49 to 23.6 | Reid (1985a, 1985b) |
|  |  |  |  |  |  |  | 1.0 X 10-4 to 1.0 X 10-3 | 0.015 to 0.06 | 0.8 to 8 | Golder Associates (1990) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 8 | SKM (1999) |
|  |  |  |  |  |  |  | 3.0 X 10-4 | N/A | 13.6 | Fraser (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 2.0 | Nahm (1974) |
|  |  |  |  |  |  |  | 6.3 X 10-6 to 2.2 X 10-5 | N/A | 2 to 48 | Nahm & Reid (1979a, 1979b) |
|  |  |  |  |  |  |  | 5.0 X 10-5 | N/A | 6 to 35.6 | Pratt (1985) |
|  |  |  |  |  |  |  | (1.0 X 10-4) | (0.1) | (1.63) | Initial values in this study |
| 19 | 111 | T1 coal | TP, T1, TRU, TRM, TRL | Traralgon Fm/ Burong Fm, Carrajung Volcanics | 14 | C/UC (Aquitard) | 2.9 X 10-7 to 6.9 X 10-5 | 0.001 to 0.05 | 1 X 10-6 to 5.0 X 10-3 | Schaeffer (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 0.015 to 0.1 | PDA (2006) |
|  |  |  |  |  |  |  | 3.0 X 10-6 to 5.0 X 10-4 | N/A | 0.0005 to 1.36 | USQ (2011) |
|  |  |  |  |  |  |  | 0.0001 | 0.005 | 0.04 | Aquaterra (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 6 X 10-5 to 1.8 X 10-1 | Harlow & LeCain (1993) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.02) | (0.0025) | Initial values in this study |
| 20 | 111 | T1 interseam | Floor T1_all & Top T2_all | Traralgon Fm/ Burong Fm | 15 | C/UC | 2.5 X 10-6 to 9.8 X 10-4 | 0.015 to 0.1 | 1.6 X 10-3 to 48.26 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.04 | 0.05 | 1.1 to 2 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 1 to 2.02 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 1.0 X 10-6 | N/A | 4.07 | Nahm (1977) |
|  |  |  |  |  |  |  | 1.0 X 10-6 to 7.5 X 10-6 | N/A | 1.1 to 6.83 | Nahm & Reid (1979b, 1979c, 1979d, 1979e) |
|  |  |  |  |  |  |  | 2.5 X 10-5 to 4.0 X 10-4 | N/A | 2 to 88 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | N/A | N/A | 6.33 to 48.26 | Geo-Eng (1993, 2001) |
|  |  |  |  |  |  |  | 1.0 X 10-4 to 5.0 X 10-4 | 0.015 to 0.06 | 3 to 10 | Golder Associates (1990) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 8 | SKM (1999) |
|  |  |  |  |  |  |  | (1.0 X 10-4) | (0.1) | (2.02) | Initial values in this study |
| 21 | 111 | T2 coal |  | Traralgon Fm/ Burong Fm | 16 | C/UC (Aquitard) | 1.7 X 10-7 to 2.4 X 10-5 | 0.001 to 0.05 | 1 X 10-6 to 5 X 10-3 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 8.30 X 10-7 to 5 X 10-6 | N/A | 1.1 X 10-6 to 3.56 X10-6 | SKM (2001) |
|  |  |  |  |  |  |  | N/A | N/A | 0.015 to 0.1 | PDA (2006) |
|  |  |  |  |  |  |  | 3.0 X 10-6 to 5.0 X 10-4 | N/A | 0.0005 to 1.36 | USQ (2011) |
|  |  |  |  |  |  |  | 0.0001 | 0.005 | 0.04 | Aquaterra (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 6 X 10-5 to 1.8 X 10-1 | Harlow & LeCain (1993) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.02) | (0.0025) | Initial values in this study |
| 22 | 111 | T2 interseam | Lower Latrobe Group; T2_all floor | Lower Latrobe Group; T2_all floor & Traralgon Fm/Burong Fm, Honeysuckle Gravels, Yarram Fm | 17 | C/UC | 1.1 X 10-7 to 4.9 X 10-5 | 0.015 to 0.1 | 1.6 X 10-7 to 24.65 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.04 | 0.05 | 1.1 to 2 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 1 to 2.02 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 1.0 X 10-5 to 4.0 X 10-4 | N/A | 4.27 to 88 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | 1.0 X 10-6 | N/A | 4.02 | Thompson (1968) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 8 | SKM (1999) |
|  |  |  |  |  |  |  | (7.0 X 10-6) | (0.1) | (2.02) | Initial values in this study |
| 23 | 114 |  | Strzelecki 500 m; >0–500 m |  | 18 | C/UC | 4.5 X 10-6 to 1.0 X 10-5 | N/A | 1 X 10-10 to 1.0 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004-0.005 | 0.005-0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | N/A | 0.02 to 0.1 | 1.0 X 10-4 to 1 | Dahlhaus et al. (2004) |
|  |  |  |  |  |  |  | 2.2 X 10-5 | N/A | 0.02 to 1 | Shugg & Harris (1975) |
|  |  |  |  |  |  |  | 3.6 X 10-5 | N/A | 0.3 | Szabo (1979) |
|  |  |  |  |  |  |  | (1.0 X 10-6) | (0.02) | (0.002) | Initial values in this study |
| 24 | 114 |  | Strzelecki 500 m; 500–1000 m |  | 18 | C/UC | 4.5 X 10-6 to 1 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004-0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | (1.0 X 10-6) | (0.02) | (0.002) | Initial values in this study |
| 25 | 114 |  | Strzelecki 1 km; 1000–2000 m |  | 18 | C/UC | 4.5 X 10-6 to 1 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004 to 0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | (1 X 10-6) | (0.02) | (0.0004) | Initial values in this study |
| 26 | 114 |  | Strzelecki 1 km; 2000–3000 m |  | 18 | C/UC | 4.5 X 10-6 to 1.0 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004 to 0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | (1.0 X 10-6) | (0.02) | (0.0004) | Initial values in this study |
| 27 | 114 |  | Strzelecki 1km; 3000-4000m |  | 18 | C/UC | 4.5 X 10-6 to 1 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3.0 X 10-6 to 5.0 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004 to 0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | (1.0 X 10-6) | (0.02) | (0.0004) | Initial values in this study |
| 28 | 114 |  | Strzelecki 1km; 4000-6000m |  | 18 | C/UC | 4.5 X 10-6 to 1.0 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004 to 0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | (1.0 X 10-6) | (0.01) | (0.0004) | Initial values in this study |
| 29 |  |  | Strzelecki 4km; >6000m |  | 18 | C/UC | 4.5 X 10-6 to 1.0 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004 to 0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | (1 X 10-6) | (0.01) | (0.0004) | Initial values in this study |
| 30 |  |  | Palaeozoic basement 200m thick |  | 18 | 4.5 X 10-6 to 1 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) | 4.5 X 10-6 to 1 X 10-5 |
|  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) | <3 X 10-6 to 5 X 10-3 |
|  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) | N/A |
|  |  |  |  |  |  | 0.00004 to 0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) | 0.00004 to 0.005 |
|  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a; 2010b) | 0.01 |
|  |  |  |  |  |  | (1 X 10-6) | (0.02) | (0.0004) | Initial values in this study | (1 X 10-6) |

1. UC = unconfined, C/UC = confined/unconfined.

2. The ranges of aquifer parameter values were obtained from the publications listed in the table.

3. The aquifer parameter values in bracket are the initial values for the groundwater model in the study. They are primarily sourced from the previously calibrated groundwater models.

4. A Kx to Kz ratio of 10:1 was assumed prior to calibration

Table 23	Calibrated model aquifer parameterisation

| Zone | Aquifer | Kxy (m/d) | Kzz (m/d) | Specific yield | Specific storage (m-1) | Kzz/Kxy |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Marine water thickness | 1.063 X 102 | 1.594 X 101 | 0.1 | 1 X 10-5 | 0.150 |
| 2 | Quaternary | 6.507 | 9.761 X 10-1 | 0.1 | 1 X 10-5 | 0.150 |
| 3 | Haunted Hill Formation | 3.203 | 4.804 X 10-1 | 0.1 | 1 X 10-5 | 0.150 |
| 4 | Nuntin clay | 3.333 | 5 X 10-1 | 0.04 | 1 X 10-5 | 0.150 |
| 5 | Boisdale Formation | 2.986 X 101 | 4.479 | 0.1 | 1 X 10-5 | 0.150 |
| 6 | Jemmys Point & Upper Hazelwood Formation | 8.890 X 10-2 | 1.334 X 10-2 | 0.1 | 1 X 10-5 | 0.150 |
| 7 | Yallourn Coal Seam | 2 X 10-3 | 3 X 10-4 | 0.1 | 1 X 10-5 | 0.150 |
| 8 | Yallourn Aquifer and interseam | 7.667 X 10-1 | 1.15 X 10-1 | 0.1 | 1 X 10-5 | 0.150 |
| 9 | Lower M2 interseam | 1.605 X 101 | 1.202 | 0.1 | 1 X 10-5 | 0.075 |
| 10 | M1A coal | 2.230 X 10-1 | 3.345 X 10-2 | 0.02 | 1 X 10-5 | 0.150 |
| 11 | Morwell 1A interseam/aquifer | 7.450 | 7.45 X 10-1 | 0.1 | 1 X 10-5 | 0.100 |
| 12 | Morwell 1B coal | 4.525 X 10-2 | 6.79 X 10-3 | 0.1 | 1 X 10-5 | 0.150 |
| 13 | Morwell 1B interseam | 9.691 | 1.602 X 10-1 | 0.1 | 1 X 10-5 | 0.017 |
| 14 | Morwell 2 | 4.108 X 10-1 | 6.162 X 10-2 | 0.1 | 1 X 10-5 | 0.150 |
| 15 | Lakes Entrance Formation | 9.572 X 10-2 | 8.860 X 10-3 | 0.05 | 1 X 10-5 | 0.093 |
| 16 | M2c aquifer/Seaspray sands | 6.119 | 3.902 X 10-1 | 0.1 | 1 X 10-5 | 0.064 |
| 17 | Thorpdale volcanics | 6.522 X 10-1 | 1.820 X 10-2 | 0.1 | 1 X 10-5 | 0.028 |
| 18 | Upper Latrobe Group | 2.350 | 3.524 X 10-1 | 0.1 | 1 X 10-5 | 0.150 |
| 19 | T1 coal | 1.92 X 10-3 | 2.9 X 10-4 | 0.02 | 1 X 10-5 | 0.151 |
| 20 | T1 interseam | 1.262 | 1.894 X 10-1 | 0.1 | 1 X 10-5 | 0.150 |
| 21 | T2 coal | 1.420 X 10-3 | 8 X 10-5 | 0.02 | 1 X 10-5 | 0.056 |
| 22 | T2 interseam | 8.862 X 10-1 | 9.566 X 10-2 | 0.1 | 1 X 10-5 | 0.108 |
| 23 | Strzelecki top 500m | 1 X 10-2 | 1 X 10-3 | 0.02 | 1 X 10-5 | 0.100 |
| 24 | Strzelecki 500-1000m | 2 X 10-4 | 1 X 10-5 | 0.02 | 1 X 10-5 | 0.050 |
| 25 | Strzelecki 1km-2km | 2 X 10-4 | 1 X 10-5 | 0.02 | 1 X 10-5 | 0.050 |
| 26 | Strzelecki 2km-3km | 2 X 10-4 | 1 X 10-5 | 0.02 | 1 X 10-5 | 0.050 |
| 27 | Strzelecki 3km-4km | 2 X 10-4 | 1 X 10-5 | 0.02 | 1 X 10-5 | 0.050 |
| 28 | Strzelecki 4km-6km | 2 X 10-4 | 1 X 10-5 | 0.02 | 1 X 10-5 | 0.050 |
| 29 | Strzelecki >6km | 2 X 10-4 | 1 X 10-5 | 0.02 | 1 X 10-5 | 0.050 |
| 30 | Palaeozoic basement 200m thick | 1.012 X 10-1 | 1.518 X 10-2 | 0.02 | 1 X 10-5 | 0.150 |
| 31 | Lower M2 interseam | 8.747 | 1.312 | 0.1 | 1 X 10-5 | 0.150 |
| 32 | Lower M2 interseam | 6.077 | 6.273 X 10-1 | 0.1 | 1 X 10-5 | 0.103 |
| 33 | Upper Latrobe Group | 2.665 | 3.997 X 10-1 | 0.1 | 1 X 10-5 | 0.150 |
| 34 | Upper Latrobe Group | 2.930 | 3.163 X 10-1 | 0.12 | 1 X 10-5 | 0.108 |
| 35 | Palaeozoic basement 200m thick | 1.950 | 2.337 X 10-2 | 0.04 | 1 X 10-5 | 0.012 |
| 36 | Quaternary | 1.501 X 10-1 | 2.009 X 10-2 | 0.07 | 1 X 10-5 | 0.134 |

Figure 130	Lateral hydraulic conductivity (m/day) for modelled layer 2

Figure 131	Lateral hydraulic conductivity (m/day) for modelled layer 9

Figure 132 	Lateral hydraulic conductivity (m/day) for modelled layer 18

Figure 133 	Lateral hydraulic conductivity for modelled layer 19

Figure 134	Lateral hydraulic conductivity (m/day) for modelled layer 24

Figure 135	Vertical hydraulic conductivity (m/day) for modelled layer 2

Figure 136	Vertical hydraulic conductivity (m/day) for modelled layer 9

Figure 137	Vertical hydraulic conductivity (m/day) for modelled layer 18

Figure 138	Vertical hydraulic conductivity (m/day) for modelled layer 19

Figure 139	Vertical hydraulic conductivity (m/day) for modelled layer 24

Figure 140	Specific yield for modelled layer 2

Figure 141	Specific yield for modelled layer 18

Figure 142	Specific yield for modelled layer 19

Figure 143	Specific yield for modelled layer 24

#### Observations and predictions

Summary

The calibration statistics for the transient groundwater model achieved the target scaled a normalised (scaled) root mean squared error (RMS) of less than 5%. Of note was that the 2000 to 2010 verification period displayed improved calibration statistics with a scaled RMS of 1.42%, a mean sum of residuals of 5 m and a coefficient of determination of 0.9. Analysis of the spatial residual error suggest that the model over predicts the heads in the outcropped regions and under predicts the heads in the lower parts of the landscape.

Results from the sensitivity analysis indicate high sensitivity of simulated shallow water table extent to variations in recharge and maximum groundwater evapotranspiration. This is not surprising and suggests that the groundwater model is well calibrated to key inputs. The sensitivity analysis shows model parameters fit within type I or II classification of the Murray-Darling Basin Groundwater Flow Modelling Guideline.

The difference between CRDP and baseline is known as the additional coal resource development (ACRD).  The ACRD is the change that is primarily reported in the BA (see Section 2.3.1.1 of companion product 2.3 for the Gippsland Basin bioregion).  An evaluation of the impact of the ACRD estimates that the greatest spatial impact of 167,700 ha is likely to occur around 2072. This is consistent with the timing of the peak reduction in annual baseflow of 78 ML/yr to the Latrobe River. Simulation results suggest that:

- the maximum impact of the CRDP is localised around the mine pits

- drawdowns of greater than 2 m, 5 m and 10 m impact an area of 167,696 ha, 87,200 ha and 50, 976 ha respectively with maximum drawdowns of greater than 100 m impacting 1,328 ha in the vicinity of the mine pits

- the greatest impact on baseflow to the Latrobe River occurs post mine closure.

##### Observation bore data

All available groundwater observation bore data was compiled and included in the model as calibration targets. A total of 822 target bores are utilised for the transient calibration model whereas only 71 bores have data available to calibrate the predevelopment steady-state condition. Figure 144 shows the location of the observation bores used for model calibration. Bore hydrographs identified in SKM (2014) and GHD (2014a) are considered “high reliability” and were assigned a calibration weight of 5; all other observation bore data were assigned a calibration weight of 1.

Summarised in Table 24 are all the bores used to calibrate the groundwater model. Included in the table are the layer allocations based on the VAF data (VAF Lay) and the corresponding layer allocation based on bore depth and surface elevation data (New Lay). The top and bottom elevations of the VAF assigned layer (columns TopLay and BotLay respectively) are included for comparison with the associated bore depth. The tabled information suggests that in most cases the VAF assigned bore layer varies from the depth based layer as reported by the last column of Table 24.

Table 24	Calibration bore attribution

| Bore_ID | VAF Lay | Bore_x | Bore_y | Depth (m) | Surface (mAHD) | Depth (mAHD) | TopLay (mAHD) | BotLay (mAHD) | New Lay | Diff |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 114161 | 3 | 2568202 | 2315100 | 6 | 9. 56 | 3. 56 | 1.66 | -0.34 | 2 | yes |
| 114156 | 3 | 2568974 | 2314144 | 38 | 0 | -38 | -5.73 | -7.73 | 19 | yes |
| 114157 | 3 | 2568974 | 2314142 | 17 | 0 | -17 | -5.73 | -7.73 | 8 | yes |
| 114158 | 3 | 2568974 | 2314141 | 11 | 0 | -11 | -5.73 | -7.73 | 5 | yes |
| 94807 | 2 | 2568974 | 2314146 | 4 | 0 | -4 | -3.73 | -5.73 | 2 | no |
| 94814 | 3 | 2569007 | 2313783 | 9 | 12 | 3 | 3.16 | 1.01 | 3 | no |
| 94813 | 3 | 2570892 | 2310487 | 21 | 12.15 | -8.85 | 9.7 | 6.99 | 9 | yes |
| 94804 | 2 | 2571195 | 2311454 | 4 | 7.48 | 3.48 | 7.11 | 3.41 | 2 | no |
| 94816 | 3 | 2571454 | 2310076 | 12 | 5.97 | -6.03 | 8.43 | 6.27 | 9 | yes |
| 94815 | 2 | 2571634 | 2310081 | 1 | 5.97 | 4.97 | 16.82 | 8.43 | 4 | yes |
| 94803 | 3 | 2572449 | 2311040 | 6 | 3 | -3 | -0.63 | -2.63 | 4 | yes |
| 94802 | 2 | 2572605 | 2310763 | 4 | 6.99 | 2.99 | 4.9 | 0.85 | 2 | no |
| 113124 | 3 | 2572605 | 2310763 | 22 | 6.99 | -15.01 | 0.85 | -1.15 | 10 | yes |
| 113125 | 3 | 2572606 | 2310763 | 12 | 6.99 | -5.01 | 0.85 | -1.15 | 5 | yes |
| 75405 | 17 | 2572685 | 2329154 | 16 | 74.84 | 58.84 | 48 | 40.09 | 11 | yes |
| 75404 | 18 | 2574268 | 2332762 | 70 | 68.94 | -1.06 | 29.8 | 20.57 | 21 | yes |
| 61429 | 18 | 2574390 | 2323889 | 28 | 66 | 38 | 40.62 | 33.35 | 18 | no |
| 75566 | 18 | 2574974 | 2330073 | 29 | 59 | 30 | 23.75 | 20.93 | 14 | yes |
| 61430 | 18 | 2576510 | 2323093 | 25 | 86 | 61 | 41.25 | 38.08 | 8 | yes |
| 75399 | 17 | 2576679 | 2325405 | 16 | 55 | 39 | 30.79 | 12.22 | 12 | yes |
| 75400 | 17 | 2576892 | 2327769 | 20 | 38 | 18 | 5.79 | -5.95 | 10 | yes |
| 75401 | 18 | 2578498 | 2330054 | 26 | 32.87 | 6.87 | -0.9 | -4.75 | 14 | yes |
| 61184 | 30 | 2578770 | 2375435 | 38 | 174 | 136 | 58.57 | -141.43 | 23 | yes |
| 75565 | 18 | 2580601 | 2336533 | 68 | 94 | 26 | 94.26 | 78.82 | 22 | yes |
| 75403 | 18 | 2581386 | 2330524 | 84 | 38 | -46 | -6.19 | -14.54 | 22 | yes |
| 75563 | 18 | 2582856 | 2337821 | 29 | 73 | 44 | 59.78 | 48.99 | 19 | yes |
| 71148 | 18 | 2585325 | 2337641 | 79 | 46 | -33 | -14.24 | -19.08 | 21 | yes |
| 71149 | 18 | 2585360 | 2337644 | 65 | 46 | -19 | -14.24 | -19.08 | 18 | no |
| 100979 | 2 | 2596905 | 2296875 | 3 | 8.17 | 5.17 | 3.95 | -0.02 | 0 | yes |
| 100978 | 2 | 2597197 | 2297400 | 1 | 4.94 | 3.94 | -2.9 | -4.9 | 0 | yes |
| 100980 | 2 | 2597499 | 2296386 | 4 | 11.24 | 7.24 | 2.14 | -0.03 | 0 | yes |
| 100977 | 2 | 2598224 | 2296343 | 3 | 8.67 | 5.67 | 16.94 | -0.06 | 2 | no |
| 100975 | 2 | 2598284 | 2295929 | 3 | 11.09 | 8.09 | 12.05 | -0.04 | 2 | no |
| 100976 | 2 | 2598608 | 2296044 | 3 | 6.31 | 3.31 | 9.39 | -0.09 | 2 | no |
| 79775 | 18 | 2600542 | 2364722 | 141 | 75 | -66 | -32.4 | -45.44 | 20 | yes |
| 79774 | 30 | 2600569 | 2364712 | 0 | 75 | n/a | -189.08 | -389.08 | 30 | no |
| 110729 | 30 | 2601696 | 2376617 | 68 | 105 | 37 | 29.05 | -170.95 | 26 | yes |
| YG55 | 17 | 2602034 | 2367878 | 187 | 59.99 | -127.01 | -110.96 | -156.59 | 17 | no |
| 79784 | 18 | 2603345 | 2356315 | 21 | 252.83 | 231.83 | 249.6 | 229.92 | 18 | no |
| YG49 | 17 | 2604822 | 2368537 | 221 | 56 | -165 | -148.32 | -164.89 | 18 | yes |
| YG43 | 17 | 2605523 | 2370851 | 168 | 63 | -105 | -90.02 | -131.46 | 17 | no |
| YG34 | 10 | 2608727 | 2371814 | 58 | 68.1 | 10.1 | 32.18 | 27.29 | 14 | yes |
| 107971 | 17 | 2609634 | 2371458 | 111 | 78.97 | -32.03 | 20.67 | -58.85 | 17 | no |
| 107970 | 17 | 2609634 | 2371458 | 66 | 78.97 | 12.97 | 20.67 | -58.85 | 17 | no |
| 107972 | 18 | 2609981 | 2369233 | 226 | 65 | -161 | -139.37 | -141.37 | 28 | yes |
| 107973 | 17 | 2609981 | 2369233 | 136 | 65 | -71 | -52.9 | -139.37 | 17 | no |
| N3788 | 17 | 2611466 | 2370191 | 98 | 58 | -40 | -10.2 | -59.61 | 17 | no |
| 84155 | 18 | 2612123 | 2364811 | 156 | 133.83 | -22.17 | -5.59 | -7.59 | 26 | yes |
| N3789 | 23 | 2612432 | 2369327 | 216 | 75.39 | -140.61 | -73.07 | -75.07 | 30 | yes |
| N3787 | 12 | 2613479 | 2368347 | 222 | 86 | -136 | -40.22 | -54.95 | 30 | yes |
| 110731 | 18 | 2613789 | 2357471 | 69 | 90.11 | 21.11 | 39.36 | 10.53 | 18 | no |
| N3726 | 23 | 2614021 | 2367934 | 197 | 103.98 | -93.02 | -52.08 | -54.08 | 30 | yes |
| 84156 | 18 | 2614468 | 2365921 | 209 | 163.05 | -45.95 | -50.33 | -52.33 | 17 | yes |
| N3607 | 17 | 2614508 | 2361004 | 97 | 131.69 | 34.69 | 56.98 | 54.98 | 19 | yes |
| N3615 | 14 | 2614602 | 2359409 | 69 | 89.21 | 20.21 | 57.85 | 55.85 | 18 | yes |
| N3799 | 23 | 2614973 | 2366950 | 236 | 140.35 | -95.65 | -39.56 | -41.56 | 30 | yes |
| N3567 | 17 | 2615132 | 2361777 | 135 | 139.88 | 4.88 | 40.84 | 38.84 | 21 | yes |
| Y135 | 10 | 2615627 | 2350900 | 330 | 79.89 | -250.11 | -106.88 | -108.88 | 19 | yes |
| N3270 | 13 | 2615782 | 2358819 | 49 | 66 | 17 | 39.35 | 25.36 | 17 | yes |
| N3288 | 10 | 2615836 | 2357563 | 49 | 75 | 26 | 48.16 | 46.16 | 14 | yes |
| N3694 | 14 | 2615904 | 2357479 | 75 | 75 | 0 | 32.37 | 24.17 | 18 | yes |
| N5952 | 14 | 2616110 | 2360737 | 144 | 87 | -57 | -14.87 | -19.87 | 18 | yes |
| N4558 | 14 | 2616386 | 2362578 | 178 | 88.73 | -89.27 | -64.61 | -81.05 | 15 | yes |
| N3263 | 14 | 2616411 | 2359248 | 102 | 67.94 | -34.06 | 11.07 | 4.84 | 18 | yes |
| N4651 | 9 | 2616643 | 2362376 | 194 | 79 | -115 | 30.23 | 28.23 | 15 | yes |
| N3780 | 17 | 2616695 | 2361984 | 200 | 81 | -119 | -114.94 | -130.54 | 17 | no |
| N3369 | 10 | 2616696 | 2361995 | 145 | 81 | -64 | 33.37 | 28.84 | 14 | yes |
| N4652 | 9 | 2616749 | 2363272 | 219 | 80.36 | -138.64 | 31.76 | 29.76 | 16 | yes |
| N3570 | 23 | 2616929 | 2358071 | 283 | 62 | -221 | -211.1 | -482.18 | 23 | no |
| N3271 | 14 | 2617255 | 2359681 | 120 | 59 | -61 | -45.73 | -65.57 | 14 | no |
| N2491 | 14 | 2617641 | 2358877 | 117 | 52 | -65 | -22.33 | -41.69 | 16 | yes |
| Y122 | 17 | 2617726 | 2351085 | 410 | 74 | -336 | -278.86 | -283.34 | 20 | yes |
| N4541 | 9 | 2618455 | 2361561 | 228 | 49 | -179 | -11.47 | -13.47 | 16 | yes |
| H1095 | 14 | 2618694 | 2357662 | 106 | 72 | -34 | 21.44 | 12.17 | 17 | yes |
| H1502 | 10 | 2618826 | 2354188 | 347 | 73 | -274 | -102.74 | -148.07 | 18 | yes |
| M3787 | 3 | 2619925 | 2364195 | 10 | 44.93 | 34.93 | 35.04 | 31.35 | 3 | no |
| Y152 | 23 | 2620417 | 2350817 | 645 | 96.97 | -548.03 | -533.59 | -705.45 | 23 | no |
| M2758 | 14 | 2620950 | 2363802 | 276 | 69 | -207 | -184.49 | -211.95 | 14 | no |
| H1320 | 23 | 2621603 | 2355955 | 571 | 82 | -489 | -474.01 | -974.01 | 23 | no |
| H1691 | 18 | 2622148 | 2352910 | 429 | 105 | -324 | -202.01 | -259.54 | 20 | yes |
| M3282 | 9 | 2623325 | 2364252 | 250 | 72 | -178 | -30.31 | -32.31 | 14 | yes |
| TE1694 | 17 | 2623554 | 2369626 | 355 | 33.99 | -321.01 | -249.75 | -259.32 | 30 | yes |
| H1726 | 22 | 2623761 | 2351447 | 365 | 154.81 | -210.19 | -181.26 | -219.72 | 22 | no |
| H1719 | 20 | 2624946 | 2354097 | 352 | 112 | -240 | -183.63 | -239.99 | 21 | yes |
| H1348 | 12 | 2625037 | 2356496 | 643 | 109 | -534 | -130.55 | -216.32 | 23 | yes |
| H1632 | 17 | 2625505 | 2358509 | 703 | 88.1 | -614.9 | -519.68 | -521.68 | 22 | yes |
| H1631 | 12 | 2625507 | 2358524 | 298 | 88.1 | -209.9 | -197.54 | -269.22 | 12 | no |
| H1333 | 16 | 2625515 | 2358500 | 600 | 88.1 | -511.9 | -509.26 | -519.68 | 16 | no |
| M3101 | 17 | 2625832 | 2362004 | 694 | 62 | -632 | -489.94 | -496.25 | 23 | yes |
| M3054 | 14 | 2626343 | 2368322 | 357 | 57.25 | -299.75 | -266.75 | -307.76 | 14 | no |
| M942 | 7 | 2626659 | 2367886 | 529 | 43 | -486 | -27.79 | -124.35 | 20 | yes |
| T487 | 9 | 2627182 | 2355413 | 303 | 116.99 | -186.01 | 1.97 | -1.73 | 15 | yes |
| M3190 | 14 | 2627377 | 2365373 | 463 | 47.41 | -415.59 | -341.02 | -456.66 | 14 | no |
| 110724 | 9 | 2627413 | 2313451 | 119 | 9 | -110 | -43.98 | -55.23 | 14 | yes |
| T426 | 12 | 2628599 | 2357891 | 202 | 106.98 | -95.02 | -85.74 | -126.72 | 12 | no |
| BB195 | 23 | 2629104 | 2370602 | 480 | 43 | -437 | -359.07 | -460.48 | 23 | no |
| T493 | 10 | 2629164 | 2363251 | 267 | 60.3 | -206.7 | -210.32 | -246.92 | 8 | yes |
| T494 | 23 | 2629164 | 2363251 | 712 | 60.3 | -651.7 | -618.72 | -1118.7 | 23 | no |
| T454 | 23 | 2630325 | 2359681 | 644 | 100.97 | -543.03 | -544.64 | -1044.6 | 22 | yes |
| T440 | 14 | 2630346 | 2359679 | 294 | 100.97 | -193.03 | -174.39 | -293.37 | 14 | no |
| BB196 | 9 | 2630715 | 2369875 | 360 | 35.98 | -324.02 | -71.88 | -90.01 | 15 | yes |
| T256 | 23 | 2631780 | 2357668 | 536 | 159.39 | -376.61 | -358.35 | -858.35 | 23 | no |
| T485 | 23 | 2632831 | 2367857 | 704 | 44 | -660 | -598.85 | -1098.8 | 23 | no |
| TS37 | 23 | 2632959 | 2370884 | 607 | 34 | -573 | -557.39 | -856.45 | 23 | no |
| T489 | 23 | 2633154 | 2360587 | 627 | 120.34 | -506.66 | -439.08 | -939.08 | 23 | no |
| T445 | 18 | 2633252 | 2363852 | 632 | 87 | -545 | -451.29 | -473.66 | 22 | yes |
| T466 | 23 | 2633307 | 2364240 | 722 | 77.06 | -644.94 | -574.96 | -1075 | 23 | no |
| T442 | 23 | 2633472 | 2354615 | 149 | 143.63 | -5.37 | -32.83 | -532.83 | 21 | yes |
| T495 | 23 | 2633534 | 2355971 | 242 | 122.24 | -119.76 | -146.93 | -646.93 | 22 | yes |
| T496 | 19 | 2633534 | 2355973 | 182 | 122.24 | -59.76 | 47.83 | 25.14 | 21 | yes |
| LY3298 | 23 | 2634759 | 2355782 | 243 | 84 | -159 | -108.99 | -608.99 | 23 | no |
| LY3299 | 20 | 2634760 | 2355786 | 115 | 84 | -31 | 34.89 | -13.07 | 21 | yes |
| LY3055 | 20 | 2634887 | 2362019 | 420 | 55 | -365 | -323.31 | -366.79 | 20 | no |
| T433 | 12 | 2634984 | 2366881 | 353 | 39.76 | -313.24 | -287.33 | -308.72 | 13 | yes |
| T491 | 20 | 2635423 | 2362597 | 529 | 62.98 | -466.02 | -385.32 | -419.78 | 22 | yes |
| LY2477 | 23 | 2635544 | 2357010 | 226 | 82.31 | -143.69 | -150.18 | -650.18 | 22 | yes |
| TS40 | 11 | 2636291 | 2371312 | 330 | 31.6 | -298.39 | -168.16 | -305.8 | 11 | no |
| 110721 | 9 | 2637094 | 2324642 | 25 | 26 | 1 | 11.19 | -13.5 | 9 | no |
| LY3118 | 23 | 2637659 | 2364370 | 607 | 90 | -517 | -510.39 | -1010.4 | 23 | no |
| LY3119 | 10 | 2637670 | 2364367 | 150 | 90 | -60 | -79.2 | -129.89 | 8 | yes |
| TS38 | 20 | 2637732 | 2372083 | 575 | 30 | -545 | -507.3 | -527.89 | 21 | yes |
| TS32 | 17 | 2639294 | 2375777 | 430 | 54 | -376 | -302.87 | -338.98 | 23 | yes |
| LY2268 | 14 | 2639531 | 2367013 | 451 | 51 | -400 | -352.69 | -442.46 | 14 | no |
| LY2676 | 20 | 2639563 | 2367251 | 705 | 51 | -654 | -571.26 | -606.51 | 22 | yes |
| TS42 | 23 | 2640827 | 2372258 | 621 | 31 | -590 | -578.57 | -1078.6 | 23 | no |
| LY2472 | 23 | 2642202 | 2363504 | 570 | 60 | -510 | -500.11 | -1000.1 | 23 | no |
| 76074 | 30 | 2643006 | 2367953 | 0 | 31 | n/a | -2530.2 | -2730.2 | 30 | no |
| 110722 | 9 | 2643027 | 2367954 | 0 | 31 | n/a | -177.81 | -185.53 | 9 | no |
| TB212 | 23 | 2643128 | 2354433 | 193 | 186.11 | -6.89 | 35.64 | -464.36 | 23 | no |
| TN25 | 30 | 2643402 | 2381160 | 433 | 70.83 | -362.17 | -477.9 | -677.9 | 23 | yes |
| TS34 | 30 | 2643796 | 2378073 | 514 | 52 | -462 | -1051.7 | -1251.7 | 23 | yes |
| TB214 | 12 | 2644238 | 2359737 | 117 | 91.87 | -25.13 | 63.6 | -22.36 | 13 | yes |
| 76079 | 18 | 2644285 | 2368211 | 703 | 33.71 | -669.29 | -569.95 | -571.95 | 21 | yes |
| TB213 | 12 | 2644367 | 2360412 | 147 | 88 | -59 | 56.17 | -38.24 | 14 | yes |
| TB220 | 12 | 2644636 | 2358912 | 256 | 110.4 | -145.6 | 7.73 | -89.14 | 19 | yes |
| LY2678 | 23 | 2644987 | 2366319 | 691 | 63 | -628 | -639.33 | -1139.3 | 22 | yes |
| LY2310 | 14 | 2644989 | 2366328 | 347 | 63 | -284 | -264.49 | -339.06 | 14 | no |
| TB205 | 19 | 2645186 | 2357946 | 189 | 153.35 | -35.65 | 56.21 | -37.66 | 19 | no |
| TB198 | 20 | 2645269 | 2358285 | 199 | 167.84 | -31.16 | -22.09 | -76.9 | 20 | no |
| TB176 | 20 | 2645685 | 2359864 | 528 | 119.92 | -408.08 | -308.82 | -348.52 | 22 | yes |
| TB165 | 9 | 2645751 | 2359904 | 271 | 119.92 | -151.08 | 78.42 | 76.42 | 15 | yes |
| TB167 | 22 | 2645828 | 2354613 | 196 | 217.38 | 21.38 | 55.43 | -88.28 | 22 | no |
| LY1967 | 23 | 2646204 | 2364366 | 656 | 77 | -579 | -619.58 | -1119.6 | 22 | yes |
| LY1979 | 14 | 2646328 | 2364358 | 400 | 77 | -323 | -310.14 | -373.82 | 14 | no |
| LY2883 | 3 | 2646335 | 2364355 | 71 | 77 | 6 | 50.74 | 31.28 | 5 | yes |
| LY2809 | 9 | 2646402 | 2364342 | 456 | 77 | -379 | -122.91 | -124.91 | 15 | yes |
| LY2810 | 20 | 2646418 | 2364339 | 658 | 77 | -581 | -472.35 | -521.43 | 22 | yes |
| 105220 | 18 | 2646432 | 2327644 | 329 | 27.7 | -301.3 | -223.96 | -225.96 | 21 | yes |
| TS36 | 23 | 2647000 | 2372700 | 743 | 29 | -714 | -717.22 | -1217.2 | 22 | yes |
| LY2269 | 14 | 2647013 | 2368728 | 403 | 44.36 | -358.64 | -331.64 | -408.88 | 14 | no |
| TN24 | 30 | 2647139 | 2386406 | 358 | 62 | -296 | -214.75 | -414.75 | 30 | no |
| R330 | 23 | 2647174 | 2361701 | 665 | 85.25 | -579.75 | -563.82 | -1063.8 | 23 | no |
| TS43 | 20 | 2648186 | 2375355 | 644 | 27.98 | -616.02 | -550.06 | -570.42 | 23 | yes |
| 105222 | 18 | 2648660 | 2325903 | 318 | 24 | -294 | -173.26 | -230.01 | 20 | yes |
| 105221 | 18 | 2649379 | 2327361 | 185 | 40.77 | -144.23 | -43.06 | -45.06 | 21 | yes |
| 45759 | 3 | 2650353 | 2378495 | 20 | 41 | 21 | 29.06 | 21.69 | 4 | yes |
| 45760 | 3 | 2650353 | 2378495 | 12 | 41 | 29 | 29.06 | 21.69 | 3 | no |
| 103811 | 8 | 2650359 | 2378494 | 78 | 41 | -37 | -103.25 | -134.25 | 6 | yes |
| 147173 | 18 | 2650869 | 2325257 | 238 | 16 | -222 | -217.72 | -219.72 | 19 | yes |
| R324 | 9 | 2651453 | 2369213 | 400 | 35 | -365 | -119.14 | -139.12 | 14 | yes |
| R340 | 9 | 2651482 | 2369201 | 558 | 35 | -523 | -119.14 | -139.12 | 15 | yes |
| R344 | 3 | 2651644 | 2362903 | 451 | 71.78 | -379.22 | 84.51 | 69.33 | 23 | yes |
| 147174 | 9 | 2652295 | 2317015 | 156 | 4.72 | -151.28 | -216.4 | -218.4 | 7 | yes |
| 110726 | 18 | 2652400 | 2324979 | 520 | 19 | -501 | -259.87 | -261.87 | 21 | yes |
| 45757 | 3 | 2653706 | 2377948 | 13 | 29 | 16 | 21.81 | 13.81 | 3 | no |
| 45756 | 3 | 2653706 | 2377948 | 28 | 29 | 1 | 21.81 | 13.81 | 5 | yes |
| WN47 | 9 | 2654438 | 2372380 | 452 | 60 | -392 | -174.54 | -196.14 | 13 | yes |
| WL196 | 23 | 2654621 | 2358697 | 402 | 135.54 | -266.46 | -216.36 | -716.36 | 23 | no |
| WL197 | 21 | 2654640 | 2358703 | 358 | 135.54 | -222.46 | -133.56 | -147.24 | 23 | yes |
| 45758 | 2 | 2655668 | 2374732 | 6 | 23 | 17 | 34.07 | 11.35 | 2 | no |
| R343 | 22 | 2656046 | 2364512 | 358 | 87.86 | -270.14 | -200.25 | -305.38 | 22 | no |
| 103822 | 4 | 2657145 | 2377374 | 91 | 23 | -68 | -9.06 | -129.65 | 4 | no |
| 103820 | 8 | 2658261 | 2384698 | 64 | 37 | -27 | -27.73 | -43.22 | 7 | yes |
| 45764 | 3 | 2658276 | 2375990 | 20 | 21 | 1 | 12.46 | -45.73 | 3 | no |
| 45765 | 3 | 2658276 | 2375990 | 12 | 21 | 9 | 12.46 | -45.73 | 3 | no |
| WN52 | 20 | 2658441 | 2379797 | 703 | 28 | -675 | -577.22 | -608.74 | 23 | yes |
| DN58 | 9 | 2659538 | 2372849 | 572 | 31 | -541 | -158.8 | -219.8 | 15 | yes |
| 45761 | 3 | 2660163 | 2375868 | 24 | 23 | -1 | 11.04 | -15.53 | 3 | no |
| 45762 | 3 | 2660163 | 2375868 | 11 | 23 | 12 | 11.04 | -15.53 | 2 | yes |
| 45763 | 2 | 2660163 | 2375868 | 4 | 23 | 19 | 34.82 | 11.04 | 2 | no |
| R323 | 11 | 2660757 | 2366250 | 219 | 109.16 | -109.84 | 54.83 | 4.51 | 14 | yes |
| 89809 | 18 | 2661060 | 2370220 | 804 | 15 | -789 | -708.65 | -710.65 | 22 | yes |
| R325 | 12 | 2661080 | 2370219 | 373 | 15 | -358 | -324.71 | -357.05 | 13 | yes |
| 145094 | 5 | 2661499 | 2332102 | 0 | 57.12 | n/a | 6.56 | -17.14 | 5 | no |
| DN56 | 9 | 2662121 | 2373826 | 434 | 42 | -392 | -133.49 | -135.49 | 14 | yes |
| 58937 | 18 | 2662747 | 2379792 | 710 | 23.89 | -686.11 | -674.39 | -676.39 | 19 | yes |
| HP207 | 21 | 2663024 | 2361507 | 359 | 165.04 | -193.96 | -107.14 | -121.52 | 23 | yes |
| S51 | 23 | 2663451 | 2353016 | 362 | 196.51 | -165.49 | -165.84 | -665.84 | 22 | yes |
| HP221 | 19 | 2664291 | 2362982 | 327 | 161.24 | -165.76 | 17.32 | -7.26 | 23 | yes |
| 59308 | 4 | 2664623 | 2381390 | 65 | 23 | -42 | 2.46 | -89.14 | 4 | no |
| HP196 | 23 | 2665014 | 2362962 | 339 | 154.74 | -184.26 | -143.19 | -643.19 | 23 | no |
| HP204 | 10 | 2665364 | 2369310 | 313 | 17.16 | -295.84 | -157.11 | -167.82 | 14 | yes |
| 92118 | 18 | 2665789 | 2349930 | 214 | 154.47 | -59.53 | -28.7 | -30.7 | 19 | yes |
| DN341 | 22 | 2665917 | 2381168 | 692 | 25.5 | -666.5 | -675.35 | -697.84 | 21 | yes |
| 58934 | 5 | 2666053 | 2374972 | 122 | 17 | -105 | -82.96 | -109.81 | 5 | no |
| 145090 | 5 | 2666243 | 2324016 | 0 | 12 | n/a | -52.3 | -79.06 | 5 | no |
| HP220 | 19 | 2667289 | 2368318 | 302 | 41.45 | -260.55 | -219.6 | -245.22 | 20 | yes |
| HP177 | 21 | 2667300 | 2365125 | 183 | 130.59 | -52.41 | -24.93 | -47.56 | 22 | yes |
| HP188 | 21 | 2667311 | 2365094 | 229 | 130.59 | -98.41 | -24.93 | -47.56 | 23 | yes |
| HP189 | 21 | 2667314 | 2365075 | 216 | 130.59 | -85.41 | -24.93 | -47.56 | 23 | yes |
| 95482 | 5 | 2667529 | 2388040 | 67 | 26 | -41 | -30.89 | -42.92 | 5 | no |
| 67442 | 30 | 2669134 | 2369326 | 120 | 19.06 | -100.94 | -3067.4 | -3267.4 | 8 | yes |
| 105134 | 9 | 2669366 | 2376001 | 0 | 31.72 | n/a | -143.23 | -206.54 | 9 | no |
| 110725 | 18 | 2670480 | 2376025 | 668 | 32 | -636 | -556.96 | -558.96 | 22 | yes |
| 67441 | 18 | 2671405 | 2368736 | 889 | 39.1 | -849.9 | -628.01 | -639.27 | 22 | yes |
| C97 | 21 | 2671536 | 2359557 | 254 | 118.72 | -135.28 | 2.25 | -41.48 | 22 | yes |
| 105132 | 3 | 2671954 | 2376198 | 28 | 31 | 3 | 21.05 | -24.92 | 3 | no |
| 105196 | 5 | 2672137 | 2378998 | 123 | 18 | -105 | -91.06 | -103.21 | 6 | yes |
| C93 | 21 | 2672162 | 2363449 | 126 | 142.33 | 16.33 | 29.92 | -4.53 | 21 | no |
| 105547 | 5 | 2673156 | 2374585 | 154 | 13 | -141 | -112.64 | -128.83 | 6 | yes |
| C96 | 21 | 2673608 | 2361845 | 222 | 152.61 | -69.39 | 17.12 | -17.52 | 22 | yes |
| 98028 | 9 | 2673618 | 2404058 | 55 | 52 | -3 | 33.72 | 27.5 | 14 | yes |
| C94 | 23 | 2673622 | 2361830 | 233 | 152.61 | -80.39 | -76.9 | -576.9 | 23 | no |
| 104536 | 18 | 2673691 | 2328292 | 993 | 2 | -991 | -834.63 | -836.63 | 21 | yes |
| BDL11 | 2 | 2674741 | 2382600 | 0 | 12 | n/a | 9.97 | 7.97 | 2 | no |
| 52752 | 9 | 2675691 | 2390117 | 115 | 32 | -83 | -47.92 | -102.93 | 9 | no |
| 51532 | 30 | 2676358 | 2405156 | 0 | 49 | n/a | -39.38 | -239.38 | 30 | no |
| 105548 | 5 | 2677006 | 2374317 | 145 | 24 | -121 | -104.08 | -126.04 | 5 | no |
| 52754 | 18 | 2677284 | 2377910 | 879 | 8 | -871 | -760.31 | -762.31 | 21 | yes |
| 52753 | 5 | 2677730 | 2379523 | 169 | 17 | -152 | -104.62 | -115.14 | 9 | yes |
| 110172 | 2 | 2679710 | 2404836 | 3 | 53 | 50 | 39.31 | 37.31 | 0 | yes |
| 64835 | 18 | 2679945 | 2359719 | 233 | 78.46 | -154.54 | -66.05 | -68.05 | 20 | yes |
| 110171 | 3 | 2679975 | 2404783 | 6 | 53 | 47 | 37.31 | 33.59 | 0 | yes |
| 92176 | 9 | 2680120 | 2396546 | 73 | 35.43 | -37.57 | -31.77 | -57.61 | 9 | no |
| 92175 | 18 | 2681697 | 2401558 | 87 | 67.8 | -19.2 | -12.4 | -35.13 | 18 | no |
| 145092 | 5 | 2682235 | 2344458 | 0 | 27 | n/a | -56.05 | -73.52 | 5 | no |
| 110166 | 3 | 2682626 | 2406700 | 6 | 65 | 59 | 60.56 | 58.56 | 3 | no |
| 110167 | 3 | 2682829 | 2406593 | 5 | 65 | 60 | 60.56 | 58.56 | 3 | no |
| 110168 | 3 | 2683040 | 2406454 | 6 | 64 | 58 | 54.51 | 52.51 | 0 | yes |
| 90149 | 5 | 2683141 | 2370724 | 143 | 3.88 | -139.12 | -108.83 | -122.79 | 7 | yes |
| G46 | 19 | 2683676 | 2364817 | 200 | 59.58 | -140.42 | -13.29 | -56.53 | 22 | yes |
| WWK7 | 23 | 2683864 | 2357055 | 799 | 51.5 | -747.5 | -721.54 | -1221.5 | 23 | no |
| 86669 | 5 | 2684013 | 2386136 | 85 | 9 | -76 | -73.44 | -81.29 | 5 | no |
| 92177 | 5 | 2684210 | 2394150 | 38 | 49 | 11 | 11.1 | -3.26 | 5 | no |
| 86464 | 5 | 2685541 | 2378892 | 105 | 8 | -97 | -99.36 | -111.68 | 4 | yes |
| 86670 | 4 | 2685542 | 2378884 | 59 | 8 | -51 | -19.53 | -99.36 | 4 | no |
| 86726 | 3 | 2685548 | 2378915 | 12 | 8 | -4 | 0.57 | -19.53 | 3 | no |
| G54 | 19 | 2685958 | 2367270 | 234 | 54 | -180 | -113.42 | -162.36 | 20 | yes |
| 90148 | 5 | 2688532 | 2374035 | 106 | 3 | -103 | -81.79 | -92.85 | 6 | yes |
| 105483 | 18 | 2688625 | 2351378 | 1029 | 28 | -1001 | -736.02 | -738.02 | 21 | yes |
| 105484 | 5 | 2688656 | 2351346 | 52 | 28 | -24 | -20.12 | -47 | 5 | no |
| 145091 | 5 | 2692353 | 2345871 | 0 | 1.99 | n/a | -47.37 | -63.28 | 5 | no |
| 109044 | 5 | 2693425 | 2391702 | 0 | 44.84 | n/a | -11.44 | -40.29 | 5 | no |
| G66 | 22 | 2693472 | 2368179 | 801 | 6 | -795 | -688.26 | -895.73 | 22 | no |
| 90400 | 15 | 2694503 | 2373976 | 787 | 0.73 | -786.27 | -682.64 | -840.38 | 15 | no |
| 90366 | 16 | 2694529 | 2374029 | 830 | 0.73 | -829.27 | -840.38 | -842.87 | 15 | yes |
| 86465 | 5 | 2694549 | 2380502 | 112 | 4.99 | -107.01 | -91.65 | -114.1 | 5 | no |
| 86466 | 5 | 2694551 | 2380500 | 112 | 4.99 | -107.01 | -91.65 | -114.1 | 5 | no |
| 77947 | 8 | 2700775 | 2386061 | 107 | 30.17 | -76.83 | -91.3 | -100.86 | 6 | yes |
| 77945 | 18 | 2700814 | 2386079 | 775 | 30.17 | -744.83 | -721.9 | -723.9 | 19 | yes |
| 105478 | 9 | 2708356 | 2411556 | 48 | 39 | -9 | -15.12 | -17.12 | 5 | yes |
| 105479 | 2 | 2708360 | 2411571 | 13 | 39 | 26 | 29 | -3.12 | 2 | no |
| 105392 | 2 | 2709725 | 2411335 | 22 | 37 | 15 | 30.5 | -16.12 | 2 | no |
| 50876 | 5 | 2709899 | 2370039 | 57 | 5 | -52 | -50.62 | -110.61 | 5 | no |
| 110177 | 9 | 2709953 | 2411714 | 39 | 37 | -2 | -28.12 | -30.12 | 2 | yes |
| 145093 | 5 | 2710101 | 2362746 | 0 | 10 | n/a | -34.85 | -74.67 | 5 | no |
| 105477 | 2 | 2710822 | 2410369 | 3 | 28.23 | 25.23 | 23.91 | -10.07 | 0 | yes |
| 56545 | 15 | 2711368 | 2394903 | 43 | 46.67 | 3.67 | -371.08 | -424.47 | 4 | yes |
| 105476 | 2 | 2711423 | 2411015 | 6 | 32 | 26 | 30.24 | -1.97 | 2 | no |
| 56541 | 2 | 2711424 | 2409624 | 0 | 32 | n/a | 37.01 | 27.78 | 2 | no |
| 56546 | 2 | 2711451 | 2409610 | 5 | 32 | 27 | 37.01 | 27.78 | 3 | yes |
| 56548 | 9 | 2712147 | 2409756 | 6 | 30 | 24 | 0.79 | -4.03 | 2 | yes |
| 56540 | 9 | 2712152 | 2409786 | 3 | 30 | 27 | 0.79 | -4.03 | 2 | yes |
| 111800 | 9 | 2712155 | 2409797 | 58 | 30 | -28 | 0.79 | -4.03 | 14 | yes |
| 56539 | 9 | 2712249 | 2410310 | 5 | 31 | 26 | -3.78 | -7.43 | 0 | yes |
| 56536 | 9 | 2713306 | 2409284 | 4 | 26 | 22 | 9.28 | 2.13 | 2 | yes |
| 56537 | 9 | 2713402 | 2409814 | 5 | 26 | 21 | 0.4 | -4.14 | 2 | yes |
| 56550 | 9 | 2713411 | 2409842 | 6 | 26 | 20 | 0.4 | -4.14 | 2 | yes |
| 56538 | 9 | 2713520 | 2410442 | 5 | 27 | 22 | -2.57 | -6.09 | 2 | yes |
| 56533 | 2 | 2714924 | 2409073 | 3 | 23 | 20 | 22.69 | -5.37 | 2 | no |
| 56534 | 2 | 2715048 | 2409763 | 5 | 24 | 19 | 22.14 | -17.1 | 2 | no |
| 56535 | 2 | 2715143 | 2410293 | 3 | 23 | 20 | 20.95 | -7.22 | 2 | no |
| 90615 | 5 | 2716462 | 2376783 | 65 | 8 | -57 | -60.44 | -89.95 | 4 | yes |
| 56531 | 2 | 2716561 | 2408898 | 3 | 21 | 18 | 20.33 | -27.94 | 2 | no |
| 56532 | 2 | 2716677 | 2409530 | 4 | 23 | 19 | 19.94 | -33.21 | 2 | no |
| 140692 | 5 | 2717298 | 2389460 | 69 | 26 | -43 | -20.03 | -90.37 | 5 | no |
| 56744 | 2 | 2718291 | 2409242 | 4 | 19.95 | 15.95 | 19.03 | -3.13 | 2 | no |
| 110165 | 2 | 2718609 | 2408008 | 4 | 18 | 14 | 19.2 | -7.09 | 2 | no |
| 56530 | 2 | 2719538 | 2406888 | 4 | 17 | 13 | 19.89 | 3.52 | 2 | no |
| 56528 | 2 | 2719629 | 2407381 | 2 | 18 | 16 | 19 | -4.99 | 2 | no |
| 56529 | 2 | 2719718 | 2407957 | 7 | 18 | 11 | 18.9 | -7.89 | 2 | no |
| 105480 | 2 | 2720462 | 2407322 | 3 | 17 | 14 | 18 | -3.82 | 2 | no |
| 80760 | 2 | 2721827 | 2406046 | 2 | 12 | 10 | 17 | 0.79 | 2 | no |
| 80761 | 2 | 2722005 | 2406657 | 4 | 15 | 11 | 17.96 | -19.54 | 2 | no |
| 80762 | 2 | 2722108 | 2407287 | 2 | 15 | 13 | 17.84 | -5.37 | 2 | no |
| 140279 | 2 | 2723314 | 2405478 | 6 | 11 | 5 | 18.25 | -4.03 | 2 | no |
| 65762 | 5 | 2723813 | 2388361 | 71 | 25 | -46 | -23.71 | -92.66 | 5 | no |
| 140281 | 15 | 2727673 | 2405062 | 0 | 8 | n/a | -110.67 | -127.08 | 15 | no |
| 140280 | 3 | 2728606 | 2406724 | 11 | 4.22 | -6.78 | -1.79 | -22 | 3 | no |
| 105733 | 18 | 2730124 | 2408060 | 149 | 53 | -96 | -97.33 | -117.05 | 17 | yes |
| 46968 | 5 | 2730196 | 2394362 | 36 | 37 | 1 | -7.97 | -50.85 | 4 | yes |
| 105725 | 30 | 2730404 | 2413138 | 0 | 28 | n/a | -532.58 | -732.58 | 30 | no |
| 47063 | 18 | 2730849 | 2396570 | 553 | 36 | -517 | -502.22 | -507.36 | 20 | yes |
| 105728 | 18 | 2731009 | 2408220 | 132 | 30 | -102 | -69.75 | -81.19 | 20 | yes |
| 65764 | 5 | 2731255 | 2389067 | 45 | 0 | -45 | -41.48 | -80.4 | 5 | no |
| 105730 | 18 | 2732144 | 2408798 | 83 | 36 | -47 | -19.54 | -30.56 | 20 | yes |
| 144468 | 3 | 2797767 | 2417344 | 15 | 6 | -9 | 9.69 | 4.37 | 7 | yes |
| 144469 | 30 | 2798169 | 2419147 | 31 | 3.14 | -27.86 | -47.19 | -247.19 | 20 | yes |
| 115733 | 30 | 2798901 | 2418392 | 29 | 12 | -17 | -46.03 | -246.03 | 15 | yes |
| 144467 | 15 | 2799610 | 2415561 | 32 | 14 | -18 | -20.92 | -22.92 | 13 | yes |
| 115734 | 15 | 2801774 | 2416059 | 27 | 9 | -18 | -18.89 | -20.89 | 14 | yes |
| 115732 | 15 | 2804218 | 2414545 | 29 | 7 | -22 | -24.46 | -26.46 | 13 | yes |
| 41973 | 3 | 2660288 | 2376718 | 13.71 | 22 | 8.29 | 12.93 | 2.45 | 3 | no |
| 42001 | 2 | 2656732 | 2377395 | 10.05 | 23 | 12.95 | 27.18 | 16.01 | 3 | yes |
| 42002 | 3 | 2663477 | 2376112 | 14.32 | 19 | 4.68 | 11.7 | -23.04 | 3 | no |
| 42003 | 3 | 2666460 | 2375529 | 10.66 | 23.95 | 13.29 | 15.47 | -0.6 | 3 | no |
| 42004 | 3 | 2655966 | 2377510 | 14.63 | 26 | 11.37 | 17.64 | -22.75 | 3 | no |
| 42005 | 2 | 2659365 | 2376894 | 12.49 | 23 | 10.51 | 34.08 | 13.2 | 3 | yes |
| 42006 | 3 | 2658709 | 2377052 | 16.15 | 25 | 8.85 | 14.34 | 2.49 | 3 | no |
| 42007 | 3 | 2665085 | 2375827 | 17.37 | 21 | 3.63 | 12.13 | -22.33 | 3 | no |
| 42020 | 3 | 2660710 | 2376652 | 18.59 | 22 | 3.41 | 12.81 | 1.35 | 3 | no |
| 42026 | 3 | 2657303 | 2377286 | 19.2 | 24 | 4.8 | 14.15 | -3.73 | 3 | no |
| 42027 | 3 | 2656397 | 2377442 | 18.89 | 22 | 3.11 | 16.75 | -33.32 | 3 | no |
| 42028 | 3 | 2657317 | 2378300 | 18.89 | 21 | 2.11 | 18.33 | -2.54 | 3 | no |
| 42038 | 3 | 2665681 | 2379023 | 13.71 | 21 | 7.29 | 18.75 | 6.25 | 3 | no |
| 42039 | 3 | 2658760 | 2378541 | 18.28 | 23 | 4.72 | 19.12 | -15.77 | 3 | no |
| 42041 | 3 | 2665377 | 2377422 | 15.84 | 21 | 5.16 | 17 | 1.51 | 3 | no |
| 42045 | 2 | 2655864 | 2377547 | 10.97 | 26 | 15.03 | 27.12 | 17.64 | 3 | yes |
| 42051 | 4 | 2664100 | 2379330 | 44 | 22 | -22 | -1.06 | -102.19 | 4 | no |
| 42058 | 3 | 2663738 | 2377525 | 20.42 | 20 | -0.42 | 17.25 | -0.61 | 3 | no |
| 42059 | 3 | 2654903 | 2385150 | 25.9 | 44.6 | 18.7 | 35.33 | 25.33 | 4 | yes |
| 42063 | 3 | 2657676 | 2379947 | 17.37 | 28 | 10.63 | 21.78 | 13.42 | 4 | yes |
| 42066 | 3 | 2667692 | 2380777 | 23.16 | 20.9 | -2.26 | 18.3 | 2.21 | 4 | yes |
| 42067 | 3 | 2664453 | 2381417 | 32.61 | 23 | -9.61 | 19.2 | 2.46 | 4 | yes |
| 42068 | 3 | 2662883 | 2381711 | 20.42 | 26 | 5.58 | 20.04 | -29.95 | 3 | no |
| 42069 | 3 | 2661361 | 2381995 | 19.2 | 31.1 | 11.9 | 20.08 | 1.41 | 3 | no |
| 42070 | 3 | 2659404 | 2382358 | 14.93 | 30 | 15.07 | 22.79 | 10.44 | 3 | no |
| 42071 | 3 | 2657925 | 2382929 | 12.19 | 36 | 23.81 | 27.12 | 17.42 | 3 | no |
| 42072 | 3 | 2656373 | 2383538 | 15.24 | 37 | 21.76 | 29.85 | 20.25 | 3 | no |
| 42073 | 9 | 2655126 | 2384033 | 85 | 40 | -45 | -43.81 | -74.33 | 9 | no |
| 42074 | 3 | 2656599 | 2384879 | 14.02 | 39 | 24.98 | 30.11 | 21.66 | 3 | no |
| 42075 | 3 | 2658253 | 2384589 | 17.67 | 37 | 19.33 | 28.51 | 20.1 | 4 | yes |
| 42076 | 3 | 2659861 | 2384312 | 13.1 | 32 | 18.9 | 25.19 | 15.77 | 3 | no |
| 42077 | 3 | 2661654 | 2383970 | 19.2 | 35 | 15.8 | 21.73 | 12.47 | 3 | no |
| 42078 | 3 | 2663670 | 2383687 | 18.89 | 28.99 | 10.1 | 19 | -9.47 | 3 | no |
| 42079 | 3 | 2665061 | 2384478 | 18.59 | 28 | 9.41 | 18 | -0.62 | 3 | no |
| 42080 | 2 | 2666693 | 2384226 | 10.05 | 28 | 17.95 | 19.95 | 17.95 | 3 | yes |
| 42081 | 3 | 2668281 | 2383852 | 14.93 | 26.24 | 11.31 | 12.78 | 0.14 | 3 | no |
| 42083 | 3 | 2669022 | 2379228 | 15.24 | 20 | 4.76 | 17 | 0.55 | 3 | no |
| 42084 | 3 | 2667005 | 2377117 | 15.24 | 21 | 5.76 | 16.53 | 5.54 | 3 | no |
| 42085 | 2 | 2656914 | 2386440 | 10.66 | 42 | 31.34 | 34.9 | 30.43 | 2 | no |
| 42086 | 3 | 2658488 | 2386159 | 18.89 | 37.7 | 18.81 | 26.62 | 21.66 | 4 | yes |
| 42087 | 3 | 2660108 | 2385873 | 18.89 | 37 | 18.11 | 23.27 | 18.41 | 4 | yes |
| 42088 | 3 | 2661859 | 2385103 | 18.89 | 34 | 15.11 | 20.95 | 16.13 | 4 | yes |
| 42095 | 3 | 2662487 | 2379615 | 22.25 | 26 | 3.75 | 22.95 | -63.88 | 3 | no |
| 42108 | 2 | 2660954 | 2379913 | 10.97 | 27 | 16.03 | 30.01 | 20.39 | 3 | yes |
| 42117 | 3 | 2661090 | 2378288 | 18.89 | 22.22 | 3.33 | 19.61 | -11.72 | 3 | no |
| 42118 | 3 | 2662146 | 2377826 | 14.1 | 20 | 5.9 | 19.19 | -3.53 | 3 | no |
| 42121 | 3 | 2670362 | 2377738 | 11.27 | 20.98 | 9.71 | 16.48 | -7.34 | 3 | no |
| 42122 | 3 | 2671190 | 2381876 | 14.32 | 23 | 8.68 | 14.8 | -7.23 | 3 | no |
| 42123 | 2 | 2672604 | 2380876 | 9.14 | 18 | 8.86 | 12.99 | 10.99 | 3 | yes |
| 42128 | 3 | 2672289 | 2378975 | 13.41 | 18 | 4.59 | 13.11 | -21.44 | 3 | no |
| 42134 | 2 | 2665666 | 2374594 | 11.35 | 18 | 6.65 | 16.79 | 11.33 | 3 | yes |
| 42135 | 2 | 2665652 | 2374445 | 6.75 | 18 | 11.25 | 16.51 | 10.74 | 2 | no |
| 42136 | 2 | 2666359 | 2374612 | 9.8 | 17 | 7.2 | 16.03 | 11.14 | 3 | yes |
| 42137 | 2 | 2667385 | 2374192 | 9.8 | 18 | 8.2 | 18.82 | 12.87 | 3 | yes |
| 42181 | 5 | 2664919 | 2374924 | 110 | 17 | -93 | -65.8 | -89.72 | 6 | yes |
| 45962 | 18 | 2643354 | 2317957 | 229.3 | 2 | -227.3 | -178.77 | -180.77 | 21 | yes |
| 50867 | 3 | 2708672 | 2372152 | 6.55 | 5 | -1.55 | -2.05 | -21.08 | 2 | yes |
| 50868 | 2 | 2708672 | 2372152 | 2.8 | 5 | 2.2 | 2.19 | -2.05 | 0 | yes |
| 50870 | 3 | 2709643 | 2369543 | 23 | 10 | -13 | 1.25 | -13.49 | 3 | no |
| 50871 | 3 | 2708309 | 2372760 | 21.3 | 0.1 | -21.2 | -2.69 | -20.93 | 4 | yes |
| 50872 | 3 | 2709643 | 2369543 | 23 | 10 | -13 | 1.25 | -13.49 | 3 | no |
| 50877 | 2 | 2708309 | 2372760 | 4.75 | 0.1 | -4.65 | 1.05 | -2.69 | 3 | yes |
| 50878 | 2 | 2708309 | 2372760 | 1.9 | 0.1 | -1.8 | 1.05 | -2.69 | 2 | no |
| 50891 | 2 | 2708794 | 2371999 | 3 | 3 | 0 | 2.83 | -2.06 | 2 | no |
| 51535 | 3 | 2683008 | 2406070 | 20 | 69 | 49 | 53.86 | 51.03 | 4 | yes |
| 51539 | 2 | 2682716 | 2405765 | 10.4 | 63 | 52.6 | 53.76 | 51.76 | 2 | no |
| 51575 | 2 | 2682229 | 2405778 | 10.66 | 61 | 50.34 | 51.09 | 49.09 | 2 | no |
| 51590 | 3 | 2682263 | 2405901 | 13.41 | 61 | 47.59 | 49.09 | 47.09 | 3 | no |
| 51592 | 2 | 2682001 | 2405356 | 28 | 60 | 32 | 48.89 | 46.89 | 10 | yes |
| 52872 | 3 | 2680268 | 2383579 | 25 | 15 | -10 | 9.99 | -26.28 | 3 | no |
| 52873 | 3 | 2677763 | 2383982 | 20 | 20 | 0 | 9.91 | -23.45 | 3 | no |
| 52874 | 3 | 2679068 | 2380040 | 20 | 16 | -4 | 9.24 | -22.87 | 3 | no |
| 52875 | 3 | 2679451 | 2381595 | 17.5 | 15 | -2.5 | 9.77 | -24.31 | 3 | no |
| 52877 | 3 | 2675601 | 2387655 | 14.87 | 15.71 | 0.84 | 15.61 | -0.36 | 3 | no |
| 52878 | 2 | 2675601 | 2387655 | 5.95 | 15.71 | 9.76 | 18.15 | 15.61 | 3 | yes |
| 52879 | 3 | 2680560 | 2386750 | 13.97 | 21 | 7.03 | 12.84 | -29.28 | 3 | no |
| 52880 | 2 | 2680560 | 2386750 | 10.02 | 21 | 10.98 | 18.59 | 12.84 | 3 | yes |
| 52882 | 3 | 2677763 | 2383982 | 12.69 | 20 | 7.31 | 9.91 | -23.45 | 3 | no |
| 52883 | 2 | 2677763 | 2383982 | 5.42 | 20 | 14.58 | 15.01 | 9.91 | 2 | no |
| 52885 | 2 | 2680268 | 2383579 | 7.08 | 15 | 7.92 | 14.8 | 9.99 | 3 | yes |
| 52886 | 2 | 2680268 | 2383579 | 4.99 | 15 | 10.01 | 14.8 | 9.99 | 2 | no |
| 52888 | 3 | 2679068 | 2380040 | 13.34 | 16 | 2.66 | 9.24 | -22.87 | 3 | no |
| 52889 | 3 | 2679068 | 2380040 | 10.78 | 16 | 5.22 | 9.24 | -22.87 | 3 | no |
| 52890 | 2 | 2679068 | 2380040 | 5.09 | 16 | 10.91 | 11.24 | 9.24 | 2 | no |
| 52891 | 3 | 2676431 | 2380996 | 13.86 | 11 | -2.86 | 6.98 | -23.61 | 3 | no |
| 52892 | 2 | 2676431 | 2380996 | 4.89 | 11 | 6.11 | 9.06 | 6.98 | 3 | yes |
| 52893 | 2 | 2679451 | 2381595 | 10.18 | 15 | 4.82 | 11.77 | 9.77 | 3 | yes |
| 52894 | 2 | 2679451 | 2381595 | 5.42 | 15 | 9.58 | 11.77 | 9.77 | 3 | yes |
| 56497 | 18 | 2680887 | 2368715 | 774.14 | 27 | -747.14 | -501.13 | -554.29 | 23 | yes |
| 56547 | 9 | 2711758 | 2409462 | 13 | 30 | 17 | 8.55 | 2.22 | 4 | yes |
| 56549 | 9 | 2712564 | 2409113 | 12 | 31 | 19 | 3.72 | -3.28 | 2 | yes |
| 56551 | 9 | 2713298 | 2409256 | 14 | 26 | 12 | 9.28 | 2.13 | 7 | yes |
| 56552 | 2 | 2718110 | 2408193 | 10 | 19 | 9 | 19.27 | -11.97 | 2 | no |
| 56553 | 2 | 2718243 | 2408935 | 11 | 19.95 | 8.95 | 19.03 | -3.13 | 2 | no |
| 58935 | 9 | 2666399 | 2375524 | 862 | 17 | -845 | -183.86 | -185.86 | 23 | yes |
| 59328 | 2 | 2664419 | 2381449 | 5.36 | 22.82 | 17.46 | 21.32 | 19.32 | 3 | yes |
| 59329 | 3 | 2664567 | 2382021 | 10.05 | 24 | 13.95 | 19.05 | 0.32 | 3 | no |
| 59330 | 2 | 2664567 | 2382021 | 5.41 | 24 | 18.59 | 21.05 | 19.05 | 3 | yes |
| 59331 | 2 | 2665106 | 2384501 | 4.99 | 28 | 23.01 | 20 | 18 | 0 | yes |
| 59336 | 2 | 2657303 | 2377286 | 6.17 | 24 | 17.83 | 28.98 | 14.15 | 2 | no |
| 59337 | 2 | 2657339 | 2377269 | 3.02 | 24 | 20.98 | 28.98 | 14.15 | 2 | no |
| 60082 | 9 | 2643050 | 2326627 | 30.5 | 21 | -9.5 | -37.39 | -39.39 | 6 | yes |
| 66881 | 9 | 2621756 | 2359451 | 100 | 14 | -86 | -1.56 | -3.56 | 15 | yes |
| 76860 | 3 | 2669776 | 2396182 | 20.13 | 33 | 12.87 | 29.16 | 26.24 | 7 | yes |
| 76888 | 3 | 2666044 | 2395310 | 25 | 38 | 13 | 33.58 | 27.9 | 7 | yes |
| 76891 | 3 | 2669776 | 2396182 | 10.08 | 33 | 22.92 | 29.16 | 26.24 | 5 | yes |
| 76892 | 2 | 2669776 | 2396182 | 5.75 | 33 | 27.25 | 33.08 | 29.16 | 3 | yes |
| 76894 | 2 | 2665829 | 2397604 | 5 | 40 | 35 | 39.87 | 31.37 | 2 | no |
| 76895 | 2 | 2666044 | 2395310 | 10.3 | 38 | 27.7 | 35.58 | 33.58 | 4 | yes |
| 76896 | 2 | 2666044 | 2395310 | 5.45 | 38 | 32.55 | 35.58 | 33.58 | 3 | yes |
| 77914 | 2 | 2700547 | 2381991 | 10.7 | 1.1 | -9.6 | 4.4 | -2.21 | 3 | yes |
| 77915 | 3 | 2705710 | 2383644 | 21 | 10.81 | -10.19 | 4.71 | -24.78 | 3 | no |
| 77918 | 3 | 2705591 | 2382981 | 19 | 10 | -9 | 1.12 | -23.36 | 3 | no |
| 77919 | 3 | 2705591 | 2382981 | 10.01 | 10 | -0.01 | 1.12 | -23.36 | 3 | no |
| 77920 | 2 | 2705591 | 2382981 | 6.65 | 10 | 3.35 | 7.69 | 1.12 | 2 | no |
| 77921 | 3 | 2705632 | 2383248 | 18.75 | 10 | -8.75 | 1.12 | -23.36 | 3 | no |
| 77922 | 2 | 2705632 | 2383248 | 12.02 | 10 | -2.02 | 7.69 | 1.12 | 3 | yes |
| 77924 | 3 | 2705470 | 2382325 | 19 | 3.58 | -15.42 | -6.09 | -20.77 | 3 | no |
| 77925 | 2 | 2705470 | 2382325 | 10.31 | 3.58 | -6.73 | 4.74 | -6.09 | 3 | yes |
| 77926 | 2 | 2705470 | 2382325 | 5.63 | 3.58 | -2.05 | 4.74 | -6.09 | 2 | no |
| 77927 | 3 | 2705284 | 2381449 | 20.97 | 1.6 | -19.37 | -8.65 | -19.72 | 3 | no |
| 77928 | 2 | 2705284 | 2381449 | 10.23 | 1.6 | -8.63 | 1.8 | -8.65 | 2 | no |
| 77929 | 2 | 2705284 | 2381449 | 5.49 | 1.6 | -3.89 | 1.8 | -8.65 | 2 | no |
| 77930 | 3 | 2702941 | 2383982 | 21.28 | 22.68 | 1.4 | 13.11 | -23.18 | 3 | no |
| 77933 | 3 | 2702676 | 2383292 | 21.28 | 17.66 | -3.62 | 3.09 | -19.4 | 3 | no |
| 77936 | 3 | 2702580 | 2382756 | 21.7 | 4 | -17.7 | -1.02 | -18.41 | 3 | no |
| 77937 | 3 | 2702580 | 2382756 | 9.7 | 4 | -5.7 | -1.02 | -18.41 | 3 | no |
| 77938 | 2 | 2702580 | 2382756 | 4.91 | 4 | -0.91 | 9.6 | -1.02 | 2 | no |
| 77939 | 3 | 2702476 | 2382096 | 21.14 | 1 | -20.14 | -6.67 | -16.77 | 4 | yes |
| 77940 | 2 | 2702476 | 2382096 | 10.27 | 1 | -9.27 | 5.59 | -6.67 | 3 | yes |
| 77941 | 2 | 2702476 | 2382096 | 4.92 | 1 | -3.92 | 5.59 | -6.67 | 2 | no |
| 77942 | 3 | 2702281 | 2381024 | 21.35 | 1 | -20.35 | -7.94 | -14.64 | 4 | yes |
| 77943 | 2 | 2702281 | 2381024 | 10.27 | 1 | -9.27 | 2.14 | -7.94 | 3 | yes |
| 77944 | 2 | 2702281 | 2381024 | 4.92 | 1 | -3.92 | 2.14 | -7.94 | 2 | no |
| 80866 | 2 | 2721083 | 2407535 | 10 | 18 | 8 | 18 | -7.35 | 2 | no |
| 86653 | 3 | 2697822 | 2382551 | 19.37 | 1.69 | -17.68 | -2.8 | -15.49 | 4 | yes |
| 86654 | 3 | 2695174 | 2379864 | 20 | 2.42 | -17.58 | -2.05 | -19.29 | 3 | no |
| 86655 | 3 | 2695174 | 2379864 | 9.23 | 2.42 | -6.81 | -2.05 | -19.29 | 3 | no |
| 86656 | 3 | 2691898 | 2378707 | 21 | 4 | -17 | -1.17 | -20.33 | 3 | no |
| 86657 | 2 | 2691898 | 2378707 | 4.77 | 4 | -0.77 | 2.68 | -1.17 | 2 | no |
| 86658 | 3 | 2690815 | 2379555 | 20 | 3 | -17 | -0.17 | -21.44 | 3 | no |
| 86659 | 2 | 2690815 | 2379555 | 5.4 | 3 | -2.4 | 5.31 | -0.17 | 3 | yes |
| 86660 | 3 | 2688409 | 2379916 | 17.26 | 5 | -12.26 | 0.01 | -21.91 | 3 | no |
| 86661 | 2 | 2688409 | 2379916 | 4.77 | 5 | 0.23 | 6.8 | 0.01 | 2 | no |
| 86662 | 3 | 2687427 | 2380215 | 20.5 | 8 | -12.5 | -0.05 | -22.74 | 3 | no |
| 86663 | 2 | 2687427 | 2380215 | 5.26 | 8 | 2.74 | 7.75 | -0.05 | 2 | no |
| 86664 | 3 | 2692667 | 2380142 | 10.25 | 6 | -4.25 | -1.91 | -21.28 | 3 | no |
| 86665 | 2 | 2692667 | 2380142 | 5.6 | 6 | 0.4 | 5.13 | -1.91 | 2 | no |
| 86666 | 3 | 2695267 | 2384265 | 19.74 | 1.33 | -18.41 | -3.31 | -18.98 | 3 | no |
| 86667 | 2 | 2695267 | 2384265 | 5.65 | 1.33 | -4.32 | 3.5 | -3.31 | 3 | yes |
| 86725 | 4 | 2685548 | 2378915 | 50 | 8 | -42 | -19.53 | -99.36 | 4 | no |
| 86727 | 2 | 2685548 | 2378915 | 8 | 8 | 0 | 5 | 0.57 | 3 | yes |
| 86736 | 3 | 2697822 | 2382551 | 13.45 | 1.69 | -11.76 | -2.8 | -15.49 | 3 | no |
| 86737 | 2 | 2697822 | 2382551 | 4.53 | 1.69 | -2.84 | 2.6 | -2.8 | 3 | yes |
| 86739 | 3 | 2682856 | 2379347 | 12.45 | 10 | -2.45 | 5.99 | -20.27 | 3 | no |
| 86740 | 2 | 2682856 | 2379347 | 5.66 | 10 | 4.34 | 7.99 | 5.99 | 3 | yes |
| 86741 | 3 | 2694215 | 2383036 | 16.49 | 1.65 | -14.84 | -7.77 | -21.12 | 3 | no |
| 86742 | 2 | 2694215 | 2383036 | 9.33 | 1.65 | -7.68 | 3.86 | -7.77 | 2 | no |
| 86743 | 2 | 2694215 | 2383036 | 5.5 | 1.65 | -3.85 | 3.86 | -7.77 | 2 | no |
| 86744 | 3 | 2689996 | 2382237 | 25.25 | 7 | -18.25 | -3.17 | -26.23 | 3 | no |
| 86745 | 2 | 2689996 | 2382237 | 9.73 | 7 | -2.73 | 6.73 | -3.17 | 2 | no |
| 86746 | 2 | 2689996 | 2382237 | 5.23 | 7 | 1.77 | 6.73 | -3.17 | 2 | no |
| 86747 | 3 | 2685658 | 2384116 | 17.4 | 11 | -6.4 | 0.01 | -31.08 | 3 | no |
| 86748 | 2 | 2685658 | 2384116 | 6.51 | 11 | 4.49 | 9 | 0.01 | 2 | no |
| 86749 | 2 | 2685658 | 2384116 | 6.45 | 11 | 4.55 | 9 | 0.01 | 2 | no |
| 89810 | 30 | 2651492 | 2369190 | 702.02 | 35 | -667.02 | -3440.4 | -3640.4 | 23 | yes |
| 89818 | 3 | 2658117 | 2369716 | 22.55 | 18 | -4.55 | 10.15 | 6.4 | 6 | yes |
| 89841 | 2 | 2657221 | 2369554 | 9.3 | 17 | 7.7 | 17.45 | 11.15 | 4 | yes |
| 89842 | 2 | 2657221 | 2369554 | 5.32 | 17 | 11.68 | 17.45 | 11.15 | 2 | no |
| 89845 | 3 | 2658117 | 2369716 | 30 | 18 | -12 | 10.15 | 6.4 | 6 | yes |
| 89850 | 2 | 2660328 | 2370169 | 4.75 | 13.94 | 9.19 | 18.32 | 9.69 | 3 | yes |
| 90138 | 18 | 2694513 | 2374002 | 1049.8 | 0.73 | -1049.03 | -845.36 | -847.36 | 22 | yes |
| 90357 | 3 | 2691134 | 2372094 | 21.6 | 0.02 | -21.58 | -4.62 | -28.95 | 3 | no |
| 90358 | 2 | 2691134 | 2372094 | 10.34 | 0.02 | -10.32 | 2 | -4.62 | 3 | yes |
| 90359 | 2 | 2691134 | 2372094 | 6.57 | 0.02 | -6.55 | 2 | -4.62 | 3 | yes |
| 90360 | 3 | 2691200 | 2375245 | 20 | 3 | -17 | -1.18 | -44.15 | 3 | no |
| 90361 | 2 | 2691200 | 2375245 | 5.84 | 3 | -2.84 | 2.24 | -1.18 | 3 | yes |
| 90423 | 3 | 2683852 | 2373197 | 21.17 | 13 | -8.17 | 0.11 | -39.77 | 3 | no |
| 90424 | 3 | 2684537 | 2374124 | 21.5 | 11 | -10.5 | 0.55 | -42.78 | 3 | no |
| 90428 | 3 | 2686816 | 2373735 | 14.07 | 7 | -7.07 | -0.03 | -55.51 | 3 | no |
| 90429 | 2 | 2686816 | 2373735 | 10.34 | 7 | -3.34 | 5 | -0.03 | 3 | yes |
| 90430 | 2 | 2686816 | 2373735 | 5.12 | 7 | 1.88 | 5 | -0.03 | 2 | no |
| 90431 | 3 | 2688593 | 2373773 | 21.55 | 3 | -18.55 | -2.66 | -63.71 | 3 | no |
| 90432 | 2 | 2688493 | 2373775 | 10.71 | 3 | -7.71 | 3.89 | -2.66 | 3 | yes |
| 90433 | 2 | 2688493 | 2373775 | 5.3 | 3 | -2.3 | 3.89 | -2.66 | 2 | no |
| 90434 | 3 | 2688607 | 2374448 | 22.04 | 4 | -18.04 | -2.02 | -59.4 | 3 | no |
| 90435 | 2 | 2688607 | 2374448 | 8.15 | 4 | -4.15 | 3.98 | -2.02 | 3 | yes |
| 90436 | 2 | 2688607 | 2374448 | 4.44 | 4 | -0.44 | 3.98 | -2.02 | 2 | no |
| 90437 | 3 | 2694679 | 2374109 | 15.7 | 0.73 | -14.97 | -2.32 | -29.1 | 3 | no |
| 90438 | 3 | 2694679 | 2374109 | 11.05 | 0.73 | -10.32 | -2.32 | -29.1 | 3 | no |
| 90439 | 3 | 2694679 | 2374109 | 6.32 | 0.73 | -5.59 | -2.32 | -29.1 | 3 | no |
| 90614 | 18 | 2716462 | 2376783 | 1246.4 | 8 | -1238.39 | -1014.5 | -1016.5 | 22 | yes |
| 92296 | 4 | 2680164 | 2396489 | 33 | 38 | 5 | 10.63 | -21.78 | 4 | no |
| 92297 | 3 | 2680164 | 2396489 | 15 | 38 | 23 | 25.57 | 10.63 | 3 | no |
| 94805 | 3 | 2569818 | 2312072 | 11.5 | 6.9 | -4.6 | 7.44 | 4.91 | 7 | yes |
| 94806 | 3 | 2569395 | 2313647 | 10 | 5 | -5 | -5.49 | -7.5 | 2 | yes |
| 94808 | 3 | 2568601 | 2314320 | 10 | 8.95 | -1.05 | 4.41 | 2.18 | 5 | yes |
| 94809 | 3 | 2568284 | 2313546 | 11.5 | 13.08 | 1.58 | 7.71 | 5.37 | 5 | yes |
| 94810 | 3 | 2568745 | 2313304 | 22 | 18.73 | -3.27 | 1.9 | -0.2 | 5 | yes |
| 94811 | 3 | 2568572 | 2313057 | 26.5 | 18.73 | -7.77 | 1.9 | -0.2 | 7 | yes |
| 95196 | 18 | 2660434 | 2394286 | 96 | 75.09 | -20.91 | 34.87 | 32.87 | 23 | yes |
| 95401 | 2 | 2672432 | 2385695 | 5.22 | 23 | 17.78 | 13.05 | 11.05 | 0 | yes |
| 95485 | 4 | 2667519 | 2388050 | 34 | 26 | -8 | 11.47 | -30.89 | 4 | no |
| 95486 | 3 | 2667529 | 2388049 | 15 | 26 | 11 | 15.43 | 11.47 | 4 | yes |
| 95487 | 3 | 2661349 | 2388017 | 41 | 42 | 1 | 30.82 | 27.22 | 7 | yes |
| 95488 | 9 | 2662140 | 2392315 | 28 | 36.74 | 8.74 | 17.58 | 11.8 | 10 | yes |
| 95489 | 8 | 2665364 | 2393361 | 33 | 38.06 | 5.06 | 10.66 | 5.47 | 9 | yes |
| 95491 | 2 | 2665364 | 2393361 | 8.81 | 38.06 | 29.25 | 34.96 | 29.64 | 3 | yes |
| 95492 | 2 | 2665364 | 2393361 | 6.51 | 38.06 | 31.55 | 34.96 | 29.64 | 2 | no |
| 95493 | 2 | 2669865 | 2390902 | 8.39 | 29 | 20.61 | 23.25 | 20.52 | 2 | no |
| 95494 | 2 | 2669865 | 2390902 | 5.49 | 29 | 23.51 | 23.25 | 20.52 | 0 | yes |
| 95495 | 2 | 2671771 | 2395185 | 8.35 | 27 | 18.65 | 27.4 | 23.75 | 5 | yes |
| 95496 | 2 | 2671771 | 2395185 | 5.48 | 27 | 21.52 | 27.4 | 23.75 | 4 | yes |
| 95497 | 3 | 2669041 | 2386192 | 17.4 | 26 | 8.6 | 13.78 | 4.02 | 3 | no |
| 95498 | 2 | 2669041 | 2386192 | 8.35 | 26 | 17.65 | 15.78 | 13.78 | 0 | yes |
| 95499 | 2 | 2669041 | 2386192 | 5.48 | 26 | 20.52 | 15.78 | 13.78 | 0 | yes |
| 95500 | 3 | 2665108 | 2386529 | 19.53 | 27.45 | 7.92 | 16.89 | -27.41 | 3 | no |
| 95501 | 2 | 2665108 | 2386529 | 7.69 | 27.45 | 19.76 | 18.89 | 16.89 | 0 | yes |
| 95502 | 2 | 2665108 | 2386529 | 4.89 | 27.45 | 22.56 | 18.89 | 16.89 | 0 | yes |
| 95503 | 3 | 2661349 | 2388017 | 18.69 | 42 | 23.31 | 30.82 | 27.22 | 5 | yes |
| 95504 | 2 | 2662140 | 2392315 | 8.87 | 36.74 | 27.87 | 40.74 | 29.58 | 3 | yes |
| 95505 | 2 | 2662140 | 2392315 | 5.45 | 36.74 | 31.29 | 40.74 | 29.58 | 2 | no |
| 96560 | 18 | 2635010 | 2366948 | 583 | 34.78 | -548.22 | -518.07 | -546.7 | 19 | yes |
| 98114 | 8 | 2674804 | 2400464 | 26 | 35 | 9 | 12.82 | 6.85 | 8 | no |
| 98115 | 3 | 2672955 | 2397251 | 42 | 73 | 31 | 50.49 | 40.73 | 4 | yes |
| 98119 | 3 | 2679319 | 2389598 | 19.2 | 21 | 1.8 | 11.61 | -19.48 | 3 | no |
| 98120 | 2 | 2679319 | 2389598 | 10.8 | 21 | 10.2 | 18.99 | 11.61 | 3 | yes |
| 98121 | 2 | 2679319 | 2389598 | 4.68 | 21 | 16.32 | 18.99 | 11.61 | 2 | no |
| 98122 | 2 | 2679053 | 2391234 | 9.63 | 19 | 9.37 | 18.17 | 13.35 | 3 | yes |
| 98123 | 2 | 2679053 | 2391234 | 4.24 | 19 | 14.76 | 18.17 | 13.35 | 2 | no |
| 98124 | 2 | 2675572 | 2397024 | 5.31 | 30 | 24.69 | 31.02 | 27.85 | 4 | yes |
| 98125 | 3 | 2674794 | 2400454 | 14.01 | 35 | 20.99 | 30.77 | 28.77 | 6 | yes |
| 98126 | 3 | 2674794 | 2400454 | 10.88 | 35 | 24.12 | 30.77 | 28.77 | 6 | yes |
| 98127 | 2 | 2674794 | 2400454 | 3.81 | 35 | 31.19 | 38.88 | 30.77 | 2 | no |
| 103582 | 9 | 2653126 | 2384365 | 460.33 | 61 | -399.33 | -47.98 | -77.14 | 20 | yes |
| 103583 | 18 | 2651685 | 2375860 | 727.67 | 21 | -706.67 | -617.23 | -619.23 | 23 | yes |
| 103734 | 3 | 2653116 | 2375225 | 20 | 18 | -2 | 8.08 | -1.97 | 4 | yes |
| 103825 | 2 | 2653051 | 2375271 | 10.44 | 18 | 7.56 | 18.1 | 8.08 | 3 | yes |
| 103826 | 2 | 2653116 | 2375225 | 4.86 | 18 | 13.14 | 18.1 | 8.08 | 2 | no |
| 103828 | 2 | 2655864 | 2377547 | 7.47 | 26 | 18.53 | 27.12 | 17.64 | 2 | no |
| 104537 | 4 | 2673518 | 2327830 | 39 | 4 | -35 | -14.28 | -31.29 | 5 | yes |
| 105199 | 3 | 2674346 | 2378582 | 20.46 | 14 | -6.46 | 9.44 | -25.41 | 3 | no |
| 105200 | 2 | 2674334 | 2378590 | 9.4 | 14 | 4.6 | 11.91 | 9.44 | 3 | yes |
| 105201 | 2 | 2674346 | 2378582 | 5.43 | 14 | 8.57 | 11.91 | 9.44 | 3 | yes |
| 109039 | 9 | 2698908 | 2386140 | 121.92 | 4 | -117.92 | -98.96 | -168.22 | 9 | no |
| 110199 | 2 | 2689066 | 2378191 | 9 | 2 | -7 | 3.81 | 0.04 | 3 | yes |
| 110201 | 2 | 2688604 | 2380983 | 8.3 | 7 | -1.3 | 8.53 | -0.04 | 3 | yes |
| 110206 | 3 | 2688274 | 2379098 | 13.4 | 4 | -9.4 | -0.02 | -20.8 | 3 | no |
| 110208 | 3 | 2688163 | 2378590 | 10.95 | 4 | -6.95 | -0.04 | -20.53 | 3 | no |
| 110720 | 18 | 2659138 | 2338232 | 333 | 92.79 | -240.21 | -116.22 | -118.22 | 21 | yes |
| 110727 | 9 | 2600232 | 2365476 | 193.3 | 69.46 | -123.84 | -19.58 | -32.9 | 16 | yes |
| 110728 | 3 | 2600200 | 2365480 | 80 | 69.46 | -10.54 | 60.55 | 47.19 | 8 | yes |
| 110732 | 9 | 2651454 | 2369213 | 400.2 | 35 | -365.2 | -119.14 | -139.12 | 14 | yes |
| 110733 | 9 | 2661081 | 2370219 | 373.2 | 15 | -358.2 | -189.05 | -272.22 | 13 | yes |
| 110749 | 3 | 2695267 | 2384265 | 10.47 | 1.33 | -9.14 | -3.31 | -18.98 | 3 | no |
| 110976 | 9 | 2710822 | 2410345 | 116 | 28.23 | -87.77 | -22.07 | -24.29 | 23 | yes |
| 110978 | 9 | 2708309 | 2411580 | 74.5 | 39 | -35.5 | -15.12 | -17.12 | 19 | yes |
| 112982 | 2 | 2688703 | 2369493 | 9 | 11 | 2 | 6.75 | -2.19 | 2 | no |
| 112983 | 2 | 2688703 | 2369493 | 5.1 | 11 | 5.9 | 6.75 | -2.19 | 2 | no |
| 112984 | 3 | 2688480 | 2368397 | 12.95 | 19 | 6.05 | 12.04 | 1.79 | 3 | no |
| 112987 | 3 | 2694523 | 2370272 | 13.3 | 1 | -12.3 | -2.2 | -34.72 | 3 | no |
| 112988 | 3 | 2694523 | 2370272 | 10.25 | 1 | -9.25 | -2.2 | -34.72 | 3 | no |
| 112989 | 2 | 2694523 | 2370272 | 5.05 | 1 | -4.05 | 1.24 | -2.2 | 3 | yes |
| 112991 | 2 | 2677658 | 2377230 | 11.17 | 5 | -6.17 | 7.59 | -0.93 | 3 | yes |
| 112992 | 2 | 2677658 | 2377230 | 4.78 | 5 | 0.22 | 7.59 | -0.93 | 2 | no |
| 112993 | 3 | 2679253 | 2372193 | 13.46 | 5 | -8.46 | -1.29 | -30.79 | 3 | no |
| 112994 | 2 | 2679253 | 2372193 | 4.77 | 5 | 0.23 | 8.09 | -1.29 | 2 | no |
| 112995 | 3 | 2672975 | 2373426 | 20.45 | 9 | -11.45 | 6 | -48.26 | 3 | no |
| 112996 | 3 | 2650006 | 2374078 | 19.32 | 25 | 5.68 | 9.94 | 0.82 | 3 | no |
| 112997 | 3 | 2652942 | 2367743 | 9.37 | 37 | 27.63 | 21.54 | 17.7 | 0 | yes |
| 112998 | 3 | 2648550 | 2363533 | 22.05 | 61 | 38.95 | 40.67 | 26.92 | 3 | no |
| 113000 | 3 | 2665385 | 2369383 | 21.69 | 11.98 | -9.71 | 9.78 | -37.35 | 3 | no |
| 113668 | 30 | 2571699 | 2310853 | 50 | 11.32 | -38.68 | -742.42 | -942.42 | 23 | yes |
| 113669 | 3 | 2571699 | 2310854 | 19 | 11.32 | -7.68 | 7.61 | 5.61 | 10 | yes |
| 114155 | 30 | 2571699 | 2310855 | 32 | 11.32 | -20.68 | -742.42 | -942.42 | 17 | yes |
| 114159 | 3 | 2568204 | 2315100 | 50 | 9.56 | -40.44 | 1.66 | -0.34 | 23 | yes |
| 114160 | 3 | 2568203 | 2315099 | 23 | 9.56 | -13.44 | 1.66 | -0.34 | 10 | yes |
| 115218 | 2 | 2684708 | 2384637 | 14.3 | 16 | 1.7 | 10.49 | 2.74 | 3 | yes |
| 115220 | 3 | 2687560 | 2378473 | 15.7 | 7 | -8.7 | -0.07 | -19.73 | 3 | no |
| 115221 | 3 | 2684421 | 2379069 | 14.1 | 9 | -5.1 | 3.3 | -20.31 | 3 | no |
| 115223 | 3 | 2686471 | 2376614 | 14.6 | 8 | -6.6 | 0.16 | -22.78 | 3 | no |
| 115225 | 3 | 2686471 | 2376614 | 20.6 | 8 | -12.6 | 0.16 | -22.78 | 3 | no |
| 115226 | 3 | 2687049 | 2376502 | 16.7 | 7 | -9.7 | 0.12 | -27.43 | 3 | no |
| 115245 | 2 | 2687049 | 2376502 | 8.9 | 7 | -1.9 | 5.01 | 0.12 | 3 | yes |
| 115630 | 3 | 2681968 | 2379501 | 13.33 | 11 | -2.33 | 7.31 | -21.2 | 3 | no |
| 115631 | 3 | 2681856 | 2378923 | 12.91 | 12.98 | 0.07 | 7.95 | -20.58 | 3 | no |
| 115937 | 3 | 2664427 | 2381049 | 20.5 | 26 | 5.5 | 19.64 | -6.78 | 3 | no |
| 115939 | 3 | 2661734 | 2384545 | 20.5 | 33 | 12.5 | 21.05 | 14.9 | 4 | yes |
| 115940 | 3 | 2667702 | 2380907 | 20.5 | 20.9 | 0.4 | 18.3 | 2.21 | 4 | yes |
| 115941 | 3 | 2667877 | 2381778 | 20.5 | 23 | 2.5 | 18.78 | 1.67 | 3 | no |
| 119103 | 3 | 2654568 | 2320729 | 0 | 3 | n/a | -0.1 | -29.35 | 3 | no |
| 119104 | 2 | 2654573 | 2320724 | 0 | 3 | n/a | 7.18 | -0.1 | 2 | no |
| 119105 | 2 | 2654573 | 2320719 | 0 | 3 | n/a | 7.18 | -0.1 | 2 | no |
| 119106 | 2 | 2654559 | 2320689 | 0 | 3 | n/a | 7.18 | -0.1 | 2 | no |
| 119107 | 2 | 2654568 | 2320675 | 0 | 3 | n/a | 7.18 | -0.1 | 2 | no |
| 119108 | 2 | 2656313 | 2319385 | 0 | 5 | n/a | 2.75 | -2.45 | 2 | no |
| 119109 | 3 | 2656307 | 2319382 | 0 | 5 | n/a | -2.45 | -47.77 | 3 | no |
| 119110 | 3 | 2656423 | 2323075 | 0 | 12.13 | n/a | 6.2 | -18.69 | 3 | no |
| 119113 | 3 | 2665743 | 2321355 | 0 | 8 | n/a | 0.03 | -30.19 | 3 | no |
| 119114 | 2 | 2665745 | 2321356 | 0 | 8 | n/a | 3.76 | 0.03 | 2 | no |
| 119115 | 3 | 2667011 | 2326948 | 0 | 16 | n/a | 9.09 | -13.19 | 3 | no |
| 119116 | 2 | 2667424 | 2323704 | 0 | 8 | n/a | 5.58 | -0.24 | 2 | no |
| 119117 | 3 | 2667422 | 2323704 | 0 | 8 | n/a | -0.24 | -27.87 | 3 | no |
| 119118 | 2 | 2662140 | 2321449 | 0 | 7 | n/a | 8.56 | 0.01 | 2 | no |
| 119119 | 2 | 2662139 | 2321453 | 0 | 7 | n/a | 8.56 | 0.01 | 2 | no |
| 119120 | 2 | 2661150 | 2321142 | 0 | 5 | n/a | 6.64 | 0.01 | 2 | no |
| 119121 | 2 | 2661149 | 2321138 | 0 | 5 | n/a | 6.64 | 0.01 | 2 | no |
| 119122 | 2 | 2672405 | 2328230 | 0 | 2 | n/a | 5.68 | -0.19 | 2 | no |
| 119123 | 3 | 2672405 | 2328228 | 0 | 2 | n/a | -0.19 | -11.56 | 3 | no |
| 119132 | 2 | 2665655 | 2322353 | 0 | 7 | n/a | 4.76 | 0.02 | 2 | no |
| 119136 | 3 | 2645306 | 2320962 | 0 | 13 | n/a | 9.5 | 1.25 | 3 | no |
| 119137 | 3 | 2649317 | 2332391 | 0 | 58.81 | n/a | 46.22 | 44.22 | 3 | no |
| 119138 | 3 | 2649029 | 2332819 | 0 | 50 | n/a | 47.1 | 45.1 | 3 | no |
| 119892 | 3 | 2689529 | 2379798 | 12.2 | 7.3 | -4.9 | 0.02 | -21.79 | 3 | no |
| 120793 | 3 | 2692906 | 2373778 | 25 | 4 | -21 | -1.5 | -48.88 | 3 | no |
| 120794 | 3 | 2694222 | 2373580 | 20.5 | 3.99 | -16.51 | -1.57 | -42.03 | 3 | no |
| 120795 | 3 | 2694065 | 2373223 | 20.5 | 2.53 | -17.97 | -2.02 | -48.77 | 3 | no |
| 120796 | 3 | 2693678 | 2379595 | 14.5 | 2.45 | -12.05 | -1.69 | -19.13 | 3 | no |
| 120799 | 3 | 2693873 | 2378891 | 20.5 | 0.98 | -19.52 | -0.49 | -16.9 | 4 | yes |
| 120800 | 3 | 2693714 | 2378464 | 20.5 | 3 | -17.5 | -1.57 | -15.9 | 4 | yes |
| 120801 | 3 | 2692396 | 2378572 | 13 | 3.99 | -9.01 | -1.83 | -19.97 | 3 | no |
| 120802 | 3 | 2692561 | 2379289 | 12 | 0 | -12 | -1.02 | -19.91 | 3 | no |
| 120997 | 3 | 2693233 | 2384648 | 18.1 | 2.23 | -15.87 | -3.95 | -22.48 | 3 | no |
| 121802 | 3 | 2709866 | 2386171 | 23.5 | 20.96 | -2.54 | 11.46 | -21.26 | 3 | no |
| 121803 | 3 | 2712227 | 2383520 | 25 | 3 | -22 | -5.46 | -23.46 | 3 | no |
| 121804 | 3 | 2706711 | 2384376 | 20.5 | 18 | -2.5 | 9.37 | -26.33 | 3 | no |
| 121805 | 3 | 2713407 | 2388988 | 25 | 14 | -11 | 14.52 | -9.51 | 4 | yes |
| 121806 | 3 | 2718562 | 2384778 | 16 | 10 | -6 | -7.28 | -24.99 | 2 | yes |
| 121807 | 3 | 2715586 | 2387942 | 19.5 | 15 | -4.5 | 8.06 | -14.61 | 3 | no |
| 121808 | 3 | 2725593 | 2389414 | 23.5 | 14 | -9.5 | 9.83 | -4.8 | 4 | yes |
| 121809 | 3 | 2719211 | 2388067 | 20.5 | 29 | 8.5 | 13.79 | -9.24 | 3 | no |
| 121810 | 3 | 2723768 | 2388272 | 20.5 | 25 | 4.5 | 17.31 | -8.01 | 3 | no |
| 122449 | 2 | 2710505 | 2385647 | 3 | 5 | 2 | 6.66 | -0.69 | 2 | no |
| 122450 | 3 | 2710505 | 2385647 | 14.5 | 5 | -9.5 | -0.69 | -20.3 | 3 | no |
| 122451 | 2 | 2713007 | 2384714 | 6.1 | 3 | -3.1 | 3.99 | -4.77 | 2 | no |
| 122452 | 2 | 2712403 | 2380704 | 7 | 4 | -3 | 1.02 | -1.9 | 3 | yes |
| 122453 | 2 | 2716187 | 2382226 | 2 | 4 | 2 | 1 | -2.86 | 0 | yes |
| 122454 | 2 | 2722295 | 2387202 | 5 | 5 | 0 | 16.81 | -3.02 | 2 | no |
| 122455 | 2 | 2722295 | 2387202 | 9 | 5 | -4 | 16.81 | -3.02 | 3 | yes |
| 122456 | 3 | 2722295 | 2387202 | 20 | 5 | -15 | -3.02 | -10.5 | 4 | yes |
| 122457 | 2 | 2718869 | 2386073 | 5 | 6 | 1 | 10.82 | -7.6 | 2 | no |
| 122458 | 2 | 2718869 | 2386073 | 10 | 6 | -4 | 10.82 | -7.6 | 2 | no |
| 122459 | 3 | 2718869 | 2386073 | 16 | 6 | -10 | -7.6 | -18.77 | 3 | no |
| 122461 | 3 | 2711719 | 2388674 | 16.79 | 6 | -10.79 | 9.87 | -13.53 | 3 | no |
| 122462 | 3 | 2711719 | 2388674 | 9.74 | 6 | -3.74 | 9.87 | -13.53 | 3 | no |
| 122463 | 3 | 2711719 | 2388674 | 20.99 | 6 | -14.99 | 9.87 | -13.53 | 4 | yes |
| 122464 | 3 | 2711724 | 2388924 | 20 | 6 | -14 | 9.87 | -13.53 | 4 | yes |
| 122465 | 3 | 2711070 | 2388737 | 20 | 5.49 | -14.51 | 6.86 | -16.4 | 3 | no |
| 122466 | 3 | 2710709 | 2388244 | 20.43 | 10 | -10.43 | 2.69 | -18.51 | 3 | no |
| 122467 | 3 | 2713574 | 2388885 | 10.8 | 6 | -4.8 | 1.59 | -10.61 | 3 | no |
| 122468 | 2 | 2713572 | 2388785 | 5.48 | 6 | 0.52 | 15.89 | 1.59 | 3 | yes |
| 122469 | 2 | 2713570 | 2388685 | 5.13 | 6 | 0.87 | 15.89 | 1.59 | 3 | yes |
| 122470 | 3 | 2723300 | 2387431 | 16.03 | 5 | -11.03 | -1.9 | -11.58 | 3 | no |
| 122471 | 2 | 2723300 | 2387431 | 9.49 | 5 | -4.49 | 17.05 | -1.9 | 3 | yes |
| 122472 | 2 | 2723300 | 2387431 | 5.51 | 5 | -0.51 | 17.05 | -1.9 | 2 | no |
| 122473 | 3 | 2723820 | 2388371 | 20.83 | 25 | 4.17 | 17.31 | -8.01 | 3 | no |
| 123721 | 3 | 2666401 | 2388173 | 13 | 28 | 15 | 20 | 5.89 | 3 | no |
| 126436 | 3 | 2680435 | 2380824 | 35 | 14 | -21 | 9 | -23.44 | 3 | no |
| 126437 | 3 | 2683019 | 2380474 | 16.5 | 11 | -5.5 | 6.57 | -21.25 | 3 | no |
| 127134 | 3 | 2691340 | 2376422 | 18.6 | 3 | -15.6 | -0.98 | -26.57 | 3 | no |
| 127135 | 3 | 2691388 | 2376792 | 17.4 | 5 | -12.4 | -1 | -23.31 | 3 | no |
| 127137 | 3 | 2691496 | 2377180 | 17 | 3 | -14 | -0.99 | -21.34 | 3 | no |
| 127144 | 3 | 2691546 | 2377639 | 14 | 3 | -11 | -0.92 | -20.57 | 3 | no |
| 127302 | 3 | 2672127 | 2378746 | 14.5 | 16 | 1.5 | 13.25 | -22.33 | 3 | no |
| 127303 | 3 | 2672568 | 2378922 | 11 | 16 | 5 | 12.42 | -23.8 | 3 | no |
| 127611 | 3 | 2711712 | 2388364 | 15 | 6.49 | -8.51 | -1.48 | -14.91 | 3 | no |
| 127612 | 2 | 2711712 | 2388364 | 5 | 6.49 | 1.49 | 9.79 | -1.48 | 2 | no |
| 127613 | 2 | 2692110 | 2383070 | 5 | 1 | -4 | 4.99 | -6.32 | 2 | no |
| 127614 | 2 | 2693807 | 2383355 | 5 | 0 | -5 | 3.86 | -7.5 | 2 | no |
| 127615 | 3 | 2693807 | 2383355 | 15 | 0 | -15 | -7.5 | -21.61 | 3 | no |
| 127616 | 3 | 2693467 | 2378149 | 15 | 1.99 | -13.01 | -0.26 | -16.74 | 3 | no |
| 127617 | 3 | 2693467 | 2378149 | 5 | 1.99 | -3.01 | -0.26 | -16.74 | 3 | no |
| 127618 | 3 | 2692431 | 2375009 | 15 | 3 | -12 | -1.43 | -35.6 | 3 | no |
| 127619 | 3 | 2692431 | 2375009 | 5 | 3 | -2 | -1.43 | -35.6 | 3 | no |
| 127620 | 3 | 2692110 | 2383070 | 14 | 1 | -13 | -6.32 | -24.35 | 3 | no |
| 127621 | 3 | 2694724 | 2372669 | 15 | 1.65 | -13.35 | -2.67 | -49.33 | 3 | no |
| 127622 | 3 | 2694724 | 2372669 | 5 | 1.65 | -3.35 | -2.67 | -49.33 | 3 | no |
| 127623 | 3 | 2687929 | 2370740 | 15 | 0 | -15 | -4.04 | -25.34 | 3 | no |
| 127624 | 2 | 2687929 | 2370740 | 5 | 0 | -5 | 2.35 | -4.04 | 3 | yes |
| 127625 | 3 | 2690899 | 2369227 | 15 | 6.94 | -8.06 | -1.46 | -11.45 | 3 | no |
| 127626 | 2 | 2690899 | 2369227 | 5 | 6.94 | 1.94 | 1.22 | -1.46 | 0 | yes |
| 127896 | 3 | 2681312 | 2380655 | 16 | 14 | -2 | 8 | -23.1 | 3 | no |
| 127897 | 3 | 2680684 | 2380769 | 16 | 13 | -3 | 8.85 | -23.38 | 3 | no |
| 127898 | 3 | 2679957 | 2380904 | 17.5 | 15 | -2.5 | 9 | -23.5 | 3 | no |
| 127899 | 3 | 2680219 | 2379107 | 19 | 14 | -5 | 8.9 | -21.64 | 3 | no |
| 127996 | 3 | 2691992 | 2379361 | 13 | 3 | -10 | -1.19 | -20.96 | 3 | no |
| 127997 | 3 | 2691564 | 2380430 | 11.5 | 4 | -7.5 | -1.5 | -21.97 | 3 | no |
| 128028 | 3 | 2648483 | 2373659 | 13 | 25 | 12 | 11.38 | 2.9 | 2 | yes |
| 128029 | 3 | 2646285 | 2372498 | 20.5 | 27 | 6.5 | 19.16 | 8.09 | 4 | yes |
| 128030 | 3 | 2644525 | 2372823 | 15 | 29 | 14 | 20.08 | 12.95 | 3 | no |
| 128031 | 3 | 2641280 | 2373330 | 19 | 34 | 15 | 28.93 | 19.56 | 4 | yes |
| 128033 | 3 | 2641613 | 2374433 | 14.5 | 38 | 23.5 | 30.94 | 23.18 | 3 | no |
| 129031 | 3 | 2664660 | 2382406 | 12.5 | 25 | 12.5 | 19 | -1.58 | 3 | no |
| 129032 | 3 | 2664851 | 2383490 | 12 | 28 | 16 | 18 | -1.21 | 3 | no |
| 130367 | 3 | 2673705 | 2387669 | 11 | 20 | 9 | 14.93 | 5.93 | 3 | no |
| 130368 | 3 | 2674095 | 2384349 | 10 | 16 | 6 | 9.97 | -9.13 | 3 | no |
| 130369 | 2 | 2668751 | 2391936 | 11.6 | 31 | 19.4 | 27.74 | 22.9 | 3 | yes |
| 130370 | 2 | 2671345 | 2393012 | 10 | 26 | 16 | 25.2 | 19.94 | 3 | yes |
| 130371 | 2 | 2669928 | 2394623 | 10 | 31 | 21 | 29.66 | 23 | 3 | yes |
| 130372 | 2 | 2671415 | 2388238 | 11.75 | 26 | 14.25 | 19.04 | 17.04 | 3 | yes |
| 130373 | 2 | 2662613 | 2395537 | 7 | 45 | 38 | 44.15 | 31.61 | 2 | no |
| 130374 | 2 | 2667404 | 2389723 | 11 | 29.99 | 18.99 | 20.92 | 18.92 | 2 | no |
| 131246 | 3 | 2675496 | 2383379 | 10.5 | 13 | 2.5 | 9.88 | -18.28 | 3 | no |
| 131249 | 3 | 2679167 | 2393348 | 13 | 23 | 10 | 18.95 | -4.87 | 3 | no |
| 131250 | 2 | 2676821 | 2395499 | 10 | 28 | 18 | 26.36 | 21.36 | 4 | yes |
| 131253 | 3 | 2666038 | 2389942 | 10 | 28 | 18 | 22.73 | 20.73 | 5 | yes |
| 131255 | 2 | 2674784 | 2403816 | 6.5 | 44 | 37.5 | 37.83 | 35.83 | 2 | no |
| 131257 | 2 | 2668746 | 2395008 | 11 | 32 | 21 | 30.7 | 26.82 | 4 | yes |
| 131259 | 3 | 2684843 | 2382953 | 14 | 13 | -1 | 2.08 | -29.14 | 3 | no |
| 135289 | 2 | 2682217 | 2383238 | 10 | 15 | 5 | 12.15 | 9.04 | 3 | yes |
| 135416 | 3 | 2684086 | 2377895 | 17.5 | 11 | -6.5 | 2.5 | -20.9 | 3 | no |
| 136519 | 3 | 2678617 | 2383364 | 18 | 18 | 0 | 9.9 | -25.01 | 3 | no |
| 136529 | 3 | 2678970 | 2380154 | 20.5 | 16 | -4.5 | 9 | -23.29 | 3 | no |
| 136531 | 3 | 2678981 | 2380174 | 21 | 16 | -5 | 9 | -23.29 | 3 | no |
| 136533 | 3 | 2678125 | 2381843 | 19 | 17 | -2 | 9.1 | -24.3 | 3 | no |
| 136537 | 3 | 2683171 | 2378175 | 10.97 | 13 | 2.03 | 5.01 | -19.93 | 3 | no |
| 136539 | 3 | 2681699 | 2383359 | 24 | 13 | -11 | 9.66 | -26.34 | 3 | no |
| 136540 | 3 | 2685888 | 2380789 | 14.5 | 8 | -6.5 | 0.52 | -23.4 | 3 | no |
| 136718 | 3 | 2693543 | 2376046 | 20.5 | 2.93 | -17.57 | -1.88 | -17.49 | 4 | yes |
| 136719 | 2 | 2693553 | 2376046 | 5.9 | 2.93 | -2.97 | 1 | -1.88 | 3 | yes |
| 136999 | 3 | 2665923 | 2381163 | 20 | 25.5 | 5.5 | 20.09 | 7.94 | 4 | yes |
| 137476 | 2 | 2678696 | 2383792 | 7.5 | 18 | 10.5 | 15.19 | 10.02 | 2 | no |
| 137478 | 3 | 2681548 | 2383312 | 14 | 15 | 1 | 9.79 | -26.04 | 3 | no |
| 137479 | 2 | 2681553 | 2383312 | 7 | 15 | 8 | 12.99 | 9.79 | 3 | yes |
| 137481 | 3 | 2678679 | 2380090 | 20 | 15 | -5 | 8.67 | -22.95 | 3 | no |
| 137539 | 3 | 2678696 | 2383782 | 18 | 18 | 0 | 10.02 | -25.05 | 3 | no |
| 139365 | 2 | 2654096 | 2374994 | 18 | 19 | 1 | 22.15 | 9.95 | 3 | yes |
| 139366 | 3 | 2651180 | 2374394 | 20 | 18 | -2 | 9.62 | 0.81 | 4 | yes |
| 139367 | 3 | 2653588 | 2375115 | 14 | 17 | 3 | 8.9 | -2.59 | 3 | no |
| 139370 | 3 | 2650900 | 2375697 | 20 | 25 | 5 | 15.63 | 8.15 | 4 | yes |
| 139374 | 3 | 2655426 | 2376387 | 30 | 22 | -8 | 11.35 | -31.02 | 3 | no |
| 139379 | 3 | 2665260 | 2386009 | 19 | 27 | 8 | 16.63 | -12.6 | 3 | no |
| 139380 | 3 | 2660191 | 2386345 | 17 | 35 | 18 | 23.3 | 19.29 | 4 | yes |
| 139381 | 3 | 2656390 | 2387026 | 15 | 39 | 24 | 30.19 | 25.47 | 4 | yes |
| 140201 | 3 | 2667171 | 2387227 | 13.7 | 24 | 10.3 | 19.78 | 0.75 | 3 | no |
| 140202 | 3 | 2677244 | 2373096 | 15 | 4 | -11 | -3.91 | -22.9 | 3 | no |
| 140204 | 3 | 2676701 | 2372493 | 13 | 3 | -10 | -4.19 | -31.01 | 3 | no |
| 140213 | 3 | 2666872 | 2387253 | 16 | 24 | 8 | 19.78 | 0.75 | 3 | no |
| 140277 | 3 | 2678206 | 2371734 | 17 | 8 | -9 | -5.65 | -32.03 | 3 | no |
| 140691 | 5 | 2709255 | 2389826 | 80 | 17 | -63 | -30.43 | -56.65 | 7 | yes |
| 143099 | 3 | 2654794 | 2389638 | 22 | 46 | 24 | 42.25 | 36.28 | 6 | yes |
| 143100 | 2 | 2654794 | 2389638 | 6 | 46 | 40 | 46 | 42.25 | 3 | yes |
| 143101 | 2 | 2655890 | 2389618 | 8.3 | 42 | 33.7 | 47.21 | 35.72 | 3 | yes |
| 143102 | 3 | 2655895 | 2389615 | 21 | 42 | 21 | 35.72 | 31.18 | 6 | yes |
| 143103 | 3 | 2653131 | 2389533 | 8 | 56 | 48 | 45.68 | 41.51 | 0 | yes |
| 143104 | 3 | 2652817 | 2389800 | 8.3 | 54 | 45.7 | 48.77 | 44.86 | 3 | no |
| 143747 | 3 | 2686847 | 2383581 | 15 | 3.49 | -11.51 | -0.57 | -30.24 | 3 | no |
| 143748 | 2 | 2685426 | 2384541 | 10 | 5 | -5 | 9.01 | 0.11 | 3 | yes |
| 143749 | 3 | 2684009 | 2386122 | 9 | 7.69 | -1.31 | 5.56 | -31.49 | 3 | no |
| 144943 | 3 | 2668684 | 2396519 | 11 | 33 | 22 | 28.47 | 25.37 | 5 | yes |
| 145207 | 3 | 2681633 | 2375102 | 27 | 14 | -13 | 0.64 | -22.26 | 3 | no |
| 145214 | 2 | 2681633 | 2375102 | 27 | 14 | -13 | 6.98 | 0.64 | 3 | yes |
| 145215 | 3 | 2680897 | 2377188 | 19.5 | 13.93 | -5.57 | 1.8 | -20.49 | 3 | no |
| 145216 | 2 | 2680897 | 2377188 | 19.5 | 13.93 | -5.57 | 9.3 | 1.8 | 3 | yes |
| 145217 | 3 | 2683658 | 2376258 | 20 | 13 | -7 | 1.71 | -24.76 | 3 | no |
| 145218 | 2 | 2683658 | 2376258 | 20 | 13 | -7 | 8.13 | 1.71 | 3 | yes |
| 145219 | 3 | 2682544 | 2377315 | 22 | 11 | -11 | 3.11 | -21.03 | 3 | no |
| 145220 | 2 | 2682544 | 2377315 | 22 | 11 | -11 | 9 | 3.11 | 3 | yes |
| WRK059121 | 18 | 2642696 | 2334157 | 23 | 71.73 | 48.73 | 19.65 | 12.54 | 3 | yes |
| Sole1 | 18 | 2853712 | 2368609 | n/a | 0 | n/a | -795.3 | -1376.6 | 18 | no |
| Scallop1 | 18 | 2814360 | 2359237 | n/a | 0 | n/a | -1697.4 | -2131.5 | 18 | no |
| Grunter1 | 18 | 2807716 | 2353253 | n/a | 0 | n/a | -1987.6 | -2463.6 | 18 | no |
| Patricia1 | 18 | 2802636 | 2380240 | n/a | 0 | n/a | -701.47 | -727.65 | 18 | no |
| Flounder1 | 18 | 2799462 | 2348982 | n/a | 0 | n/a | -2000.3 | -2640.5 | 18 | no |
| Tuna1 | 18 | 2799392 | 2364629 | n/a | 0 | n/a | -1324.7 | -1662.4 | 18 | no |
| SpermWhale1 | 18 | 2795214 | 2377488 | n/a | 0 | n/a | -822.87 | -847 | 18 | no |
| Kahawai1 | 18 | 2795163 | 2364658 | n/a | 0 | n/a | -1403.5 | -1766.4 | 18 | no |
| Mackerel1 | 18 | 2792860 | 2330670 | n/a | 0 | n/a | -2551 | -3272 | 18 | no |
| Halibut2 | 18 | 2790885 | 2339980 | n/a | 0 | n/a | -2342 | -3122.3 | 18 | no |
| Anemone1A | 18 | 2789397 | 2299109 | n/a | 0 | n/a | -2597.8 | -2933.1 | 18 | no |
| Marlin2 | 18 | 2788238 | 2358496 | n/a | 0 | n/a | -2070.2 | -2589.1 | 18 | no |
| Cobia1 | 18 | 2786606 | 2333302 | n/a | 0 | n/a | -2443.4 | -3141.7 | 18 | no |
| Turrum7 | 18 | 2785594 | 2354766 | n/a | 0 | n/a | -1676.6 | -2286.1 | 18 | no |
| Sunfish2 | 18 | 2784391 | 2368653 | n/a | 0 | n/a | -1683.8 | -1944.4 | 18 | no |
| Turrum5 | 18 | 2780180 | 2356691 | n/a | 0 | n/a | -1411.7 | -1846.7 | 18 | no |
| Kingfish9 | 18 | 2774207 | 2314644 | n/a | 0 | n/a | -2314 | -2987.9 | 18 | no |
| Kingfish8 | 18 | 2766768 | 2319041 | n/a | 0 | n/a | -2289.3 | -3010.8 | 18 | no |
| Snapper1 | 18 | 2763929 | 2362938 | n/a | 0 | n/a | -1172.8 | -1709 | 18 | no |
| Snapper | 18 | 2763929 | 2362938 | n/a | 0 | n/a | -1172.8 | -1709 | 18 | no |
| WestMoonfish1 | 18 | 2760997 | 2368277 | n/a | 0 | n/a | -1523.4 | -1946.5 | 18 | no |
| ZaneGrey1 | 18 | 2760502 | 2321223 | n/a | 0 | n/a | -2297.7 | -2989.3 | 18 | no |
| Bream1 | 18 | 2743954 | 2327976 | n/a | 0 | n/a | -1878.8 | -2595.4 | 18 | no |
| Omeo | 18 | 2736696 | 2317734 | n/a | 0 | n/a | -2280.9 | -2512.5 | 18 | no |
| Seahorse1 | 18 | 2734125 | 2364019 | n/a | 0 | n/a | -1406 | -1453.3 | 18 | no |
| Barracouta3 | 18 | 2728797 | 2350406 | n/a | 0 | n/a | -1001.5 | -1463.7 | 18 | no |
| Bullseye | 18 | 2723620 | 2320362 | n/a | 0 | n/a | -2100.8 | -2267 | 18 | no |
| WestWhiptail1 | 18 | 2719026 | 2350092 | n/a | 0 | n/a | -1172.4 | -1229.2 | 18 | no |
| Blenny1 | 18 | 2710713 | 2333805 | n/a | 0 | n/a | -1243.7 | -1592.6 | 18 | no |
| Dolphin1 | 18 | 2707487 | 2332052 | n/a | 0 | n/a | -1242.1 | -1650.7 | 18 | no |
| Pearch2 | 18 | 2703421 | 2323090 | n/a | 0 | n/a | -1161.1 | -1273.7 | 18 | no |
| Kyarra1A | 18 | 2690428 | 2311385 | n/a | 0 | n/a | -964.2 | -1010.9 | 18 | no |
| TommyRuff1 | 18 | 2686766 | 2319025 | n/a | 0 | n/a | -952.69 | -1114.6 | 18 | no |
| Drummer1 | 18 | 2783672 | 2331495 | n/a | 0 | n/a | -2460.2 | -3137.7 | 18 | no |
| Fortescue3 | 18 | 2785434 | 2340855 | n/a | 0 | n/a | -2407.3 | -3210 | 18 | no |
| Barracouta5 | 18 | 2732789 | 2352535 | n/a | 0 | n/a | -1052.7 | -1485.1 | 18 | no |
| Barracouta4 | 18 | 2737315 | 2355029 | n/a | 0 | n/a | -1097.6 | -1508.5 | 18 | no |
| Whiptail1A | 18 | 2720415 | 2350188 | n/a | 0 | n/a | -1149.7 | -1206.1 | 18 | no |
| Tarwhine1 | 18 | 2720950 | 2341321 | n/a | 0 | n/a | -1418.5 | -1826.4 | 18 | no |
| Seahorse2 | 18 | 2728764 | 2363182 | n/a | 0 | n/a | -1409.6 | -1450.5 | 18 | no |
| West Seahorse | 18 | 2729819 | 2363297 | n/a | 0 | n/a | -1414.9 | -1457.3 | 18 | no |
| Luderick1 | 18 | 2737252 | 2337069 | n/a | 0 | n/a | -1826.8 | -2499.2 | 18 | no |
| Wirrah1 | 18 | 2746783 | 2364504 | n/a | 0 | n/a | -1505.6 | -2058.9 | 18 | no |
| Wirrah2 | 18 | 2747519 | 2365142 | n/a | 0 | n/a | -1491.9 | -2021.7 | 18 | no |
| Whiting1 | 18 | 2752555 | 2359104 | n/a | 0 | n/a | -1265.5 | -1880.2 | 18 | no |
| Cod1 | 18 | 2760055 | 2345032 | n/a | 0 | n/a | -2096.8 | -2698.3 | 18 | no |
| Snapper3 | 18 | 2761498 | 2361584 | n/a | 0 | n/a | -1281.6 | -1813.1 | 18 | no |
| Snapper5 | 18 | 2761883 | 2360486 | n/a | 0 | n/a | -1279.3 | -1830.8 | 18 | no |
| Veilfin1 | 18 | 2762314 | 2338728 | n/a | 0 | n/a | -1939.3 | -2519.7 | 18 | no |
| Moonfish1 | 18 | 2763787 | 2368350 | n/a | 0 | n/a | -1379.7 | -1722.7 | 18 | no |
| Moonfish2 | 18 | 2764969 | 2368408 | n/a | 0 | n/a | -1332.3 | -1683.9 | 18 | no |
| West Kingfish | 18 | 2770563 | 2318779 | n/a | 0 | n/a | -2247.7 | -2999.5 | 18 | no |
| Turrum6 | 18 | 2777935 | 2358316 | n/a | 0 | n/a | -1492.2 | -1967.5 | 18 | no |
| Remora1 | 18 | 2779805 | 2367419 | n/a | 0 | n/a | -2187.9 | -2447.5 | 18 | no |
| Kingfish1 | 18 | 2779609 | 2318261 | n/a | 0 | n/a | -2297.3 | -3094.4 | 18 | no |
| Sunfish1 | 18 | 2782979 | 2368800 | n/a | 0 | n/a | -1752.3 | -1997.1 | 18 | no |
| Turrum1 | 18 | 2784149 | 2361923 | n/a | 0 | n/a | -2069.4 | -2462 | 18 | no |
| Turrum2 | 18 | 2784501 | 2357221 | n/a | 0 | n/a | -1538 | -2039.1 | 18 | no |
| Turrum3 | 18 | 2784501 | 2355320 | n/a | 0 | n/a | -1579.7 | -2174 | 18 | no |
| Marlin4 | 18 | 2786130 | 2357629 | n/a | 0 | n/a | -1838.3 | -2359.6 | 18 | no |
| Trumpeter1 | 18 | 2792616 | 2338145 | n/a | 0 | n/a | -2477.7 | -3188.3 | 18 | no |

Figure 144 	Location of observation bores used for model calibration

##### Baseflow estimates

Baseflow separation and analysis were conducted on all stream flow data within the study area to estimate the contribution of groundwater to streamflow. The estimated groundwater discharge volumes to streams were then used as a calibration flux target within ungauged catchments.

Baseflow separation was estimated using the approach developed by the USGS and subsequently implemented in the BFI software. Daily streamflow input data was sourced from the Victorian Water data warehouse (http://www.vicwaterdata.net/vicwaterdata/home) and extracted as computed daily flows. Periods with poor quality data and/or missing records were infilled to ensure a complete data record at each location. The program implements a digital filter procedure and combines a local minimums approach with a recession slope test. The program estimates the annual base-flow volume of unregulated rivers and streams and computes an annual base-flow index (BFI, the ratio of base flow to total flow volume for a given year) for multiple years of data at one or more gage sites. Although the method may not yield the true base flow as might be determined by a more sophisticated analysis, the index has been found to be consistent and indicative of base flow, and thus may be useful for analysis of long term base-flow trends (Arnold et al., 2010).

As a comparison, the derived BFI estimates were compared to independent values based on the Lyne and Hollick digital filter method (SKM, 2012) and recent GHD’s study (GHD, 2013). The comparative values adopted a single consistent baseflow filter parameter of 0.98 applied to all sites thereby allowing a comparison from site to site on an equivalent basis.

##### Model calibration procedure

The calibration procedure included both a manual approach and application of parameter estimation procedures. Initial model calibration was made by using a manual trial-and-error approach. Due to the large number of solution points and the large number of model layers, it was found to be inefficient and difficult to make notable enhancements in the calibration process. As a result, automated calibration methods were adopted.

The automated calibration approach utilised three independent parameter estimation software packages, namely:

- BeoPest implemented in Groundwater Vistas

- DOS version of PEST (Doherty, 2010)

- A simplex optimisation approach.

The same calibration targets were used by each package to enable a comparison of results.

The automated calibration procedure adopted a zonal approach. The zone calibration approach was based on a series of parameter zones in which the horizontal and vertical hydraulic conductivities were modified in order to minimise the variation between observed data and modelled prediction. A total of 36 zones based on geological extents and attributes were mapped as shown in Figure 145 to Figure 174.

Figure 145	Spatial extent of zones used to calibrate modelled layer 1

Figure 146	Spatial extent of zones used to calibrate modelled layer 2

Figure 147 	Spatial extent of zones used to calibrate modelled layer 3

Figure 148 	Spatial extent of zones used to calibrate modelled layer 4

Figure 149 	Spatial extent of zones used to calibrate modelled layer 5

Figure 150 	Spatial extent of zones used to calibrate modelled layer 6

Figure 151 	Spatial extent of zones used to calibrate modelled layer 7

Figure 152 	Spatial extent of zones used to calibrate modelled layer 8

Figure 153 	Spatial extent of zones used to calibrate modelled layer 9

Figure 154 	Spatial extent of zones used to calibrate modelled layer 10

Figure 155 	Spatial extent of zones used to calibrate modelled layer 11

Figure 156 	Spatial extent of zones used to calibrate modelled layer 12

Figure 157 	Spatial extent of zones used to calibrate modelled layer 13

Figure 158 	Spatial extent of zones used to calibrate modelled layer 14

Figure 159 	Spatial extent of zones used to calibrate modelled layer 15

Figure 160 	Spatial extent of zones used to calibrate modelled layer 16

Figure 161 	Spatial extent of zones used to calibrate modelled layer 17

Figure 162 	Spatial extent of zones used to calibrate modelled layer 18

Figure 163 	Spatial extent of zones used to calibrate modelled layer 19

Figure 164 	Spatial extent of zones used to calibrate modelled layer 20

Figure 165 	Spatial extent of zones used to calibrate modelled layer 21

Figure 166 	Spatial extent of zones used to calibrate modelled layer 22

Figure 167 	Spatial extent of zones used to calibrate modelled layer 23

Figure 168 	Spatial extent of zones used to calibrate modelled layer 24

Figure 169 	Spatial extent of zones used to calibrate modelled layer 25

Figure 170 	Spatial extent of zones used to calibrate modelled layer 26

Figure 171 	Spatial extent of zones used to calibrate modelled layer 27

Figure 172 	Spatial extent of zones used to calibrate modelled layer 28

Figure 173 	Spatial extent of zones used to calibrate modelled layer 29

Figure 174 	Spatial extent of zones used to calibrate modelled layer 30

##### Calibration criteria

The calibration procedure adopted a split calibration/verification approach in which the calibration period was 1990 to 1999 and the verification period was 2000 to 2012. Model calibration was based on matching groundwater hydrograph response and groundwater discharge to streams. In addition to these data sets, model predictions were also compared to mapped discharge extents in regions where this information was available.

Numerous statistical performance measures were applied to assess the goodness of fit model predicted and observed groundwater heads as summarised in Table 25. Analysis includes the coefficient of determination (CD) and the Nash–Sutcliffe index (NSI) (Nash and Sutcliffe, 1970). Each coefficient is defined by the number of observations n, measured data at time i, (Yi), and the corresponding modelled prediction at time i (Xi). The CD value is a measure of the relationship between observed and simulated values whereas the NSI value indicates how well the plot of the observed versus simulated values fit the 1:1 line. If the CD and NSI values approach zero (or are negative), the model performance is considered to be unacceptable or poor. If the values are equal to one, the model prediction is considered perfect (Middlemis et al., 2000). An NSI value of 0.6 is viewed as “satisfactory” whereas an NSI of 0.8 or higher is considered “good” (Chiew et al., 1993).

Both manual and automated calibration procedures were adopted in this study. In the case of automated calibration three independent software packages were utilised, namely:

- Groundwater Vistas

- DOS version of PEST

- A simplex optimisation approach.

A common underpinning data set was used by each package to enable a comparison of results. A zonal parameter distribution approach was employed.

Table 25	Statistical measure of model performance

| Statistical Measure | Definition | Units | Description |
| --- | --- | --- | --- |
| Mean sum of squares |  | m | MSSQ is the average of the square of the differences between predicted values and observed values. |
| Root mean square (RMS) error |  | m | Root Mean Square (RMS) error is the sample standard deviation of the differences between predicted values and observed values. |
| Scaled root mean square error |  | % | Scales the RMS error to the range of the observed values and is expressed as a percentage. The SRMS is more meaningful than root mean square error as it is independent of scale. |
| Root mean fraction square error |  | % | RMFS represents the sample standard deviation of the differences between predicted values and observed values as a fraction of the observed value expressed as a percentage. |
| Scaled root mean fraction square error |  | % | Scales the RMFS error by the ratio of the mean observed value to the range of the observed values expressed as a percentage. |
| Mean sum of residual |  | m | MSR or residual mean is not an unbiased estimator of the error variance. |
| Scaled mean sum of residuals |  | % | As per Root Mean Square error but further scaled by the range of observation data and expressed as a percentage. |
| Coefficient of determination |  |  | Describes the causation of change in Y by changes in X. |
| Nash–Sutcliffe index |  |  | The NSI value is a measure of the relationship between observed and simulated values. |

##### Steady-state predictions

The steady-state model assumed 1970 pre-development conditions in which there were no groundwater extractions. Simulated steady-state water balance results for the active model domain are summarised in Table 26. It is noteworthy that all domain cells remained saturated. The percentage mass balance closure error reported by MODFLOW-2005 was 0.7%, which satisfies the pre-defined target.

Table 26	Steady-state mass balance predictions

|  | Flow in ML/day | Flow out ML/day | Net flow ML/day | % Recharge |
| --- | --- | --- | --- | --- |
| Recharge | 9737.7 |  | 9737.7 |  |
| Evapotranspiration |  | 4002 | –4002 | 41 |
| Major rivers | 15 | 353 | –338 | 4 |
| Drains/streams |  | 3468 | –3468 | 36 |
| Constant head | 777 | 2632 | –1855 | 19 |

Water balance results suggest that:

- 40% of groundwater recharge discharges to river and drains

- 41% of groundwater recharge is “lost” through evapotranspiration

- 19% of groundwater recharge is discharged as a net outflow across the model boundaries predominantly through flux offshore.

The calibration statistics for the steady-state groundwater model based on 71 observation points are summarised in Table 27. These target statistics meet the calibration criteria as specified by the project. The observed versus computed target values are shown in Figure 175 whereas the observed versus residual error is shown in Figure 176.

Table 27	Steady-state model calibration statistics

| Statistic | Code | Unit | Steady-state |
| --- | --- | --- | --- |
| Mean sum of squares | MSSQ | m | 3.50 |
| Root mean square | RMS | m | 1.87 |
| Scale root mean square | SRMS | % | 5.34 |
| Root mean fraction square | RMFS | % | 10.24 |
| Scaled root mean fraction square | SRMFS | % | 0.60 |
| Sum of residuals | SR | m | 398.07 |
| Mean sum of residuals | MSR | m | 0.52 |
| Scaled mean sum of residuals | SMSR | % | 1.49 |
| Coefficient of determination | CD |  | 0.66 |
| Nash–Sutcliffe index | NSI |  | 0.69 |

Figure 175 	Observed versus computed steady-state heads (mAHD)

Figure 176 	Residual error (observed head – modelled head) (m) for each steady-state calibration bore

The simulated 1970 unconfined potentiometric surface (water table) and depth-to-water table map across the entire model domain are shown in Figure 177 and Figure 178 respectively.

Visual comparison of the Victorian SAFE depth to water table map (Figure 179) with the steady-state simulated depth to water table map shows the water table surface is reasonable within the alluvial systems, however in some upland locations the water table appears to be in greater connection with surface features than presented in the Victorian Aquifer Framework (VAF) data. This is not considered a significant issue as these areas are well beyond the zone of interest.

It must be noted that the VAF depth to water table map was derived using a combination of terrain analysis and interpolated bore data, and in part based on proximity to streams within the exposed basement areas. Additionally, the VAF reflects the 1990 conditions whereas the simulated steady-state depth to water table represents pre-development conditions. As such, it is expected that that the simulated steady-state depth to water table map would have a greater area of shallow water table than that reported in the VAF spatial layer.

Figure 177 	Simulated 1970 unconfined aquifer potentiometric surface (mAHD)

Figure 178 	Simulated 1970 steady-state depth to water table (m)

Figure 179 	1990 water table depth (m) within the West Gippsland CMA region

##### Transient predictions

Simulated transient volumetric water balance components for the active model domain are shown in Figure 180. Presented results clearly show the transition phase from the initial steady-state conditions to the calibration period commencing in 1990. Also of note is the lack of seasonal responses during the 1970 to 1990 simulation period during which annual stress periods have been assigned.

Figure 180 	Transient model mass balance components (m3/day per time step)

###### Calibration statistics

The calibration statistics for the transient groundwater model based on 88,799 observation points are summarised in Table 28 for the transition calibration and verification periods. The transient calibration statistics suggest there is a poor match between the modelled and observed data during the initial 20 years of simulation, which is also when the stress periods are large and the transition from the pseudo steady-state conditions would have the greatest impact on model predictions. This suggests that the simulated pseudo steady-state solution describes the 1970 groundwater condition poorly. Importantly the calibration statistics improve during the 1990-1999 calibration period as evidenced by an improved scaled RMS of 5.62% compared to the pre-calibration period scaled RMS of 8.82%. The 2000–2010 verification period has improved calibration statistics with a scaled RMS of 1.42%, a mean sum of residuals of 5 m and a coefficient of determination of 0.9. Key calibration criteria traces are presented in Figure 181 to Figure 184.

Table 28	Transient model calibration statistics

| Name of statistics | Code | Unit | 1970–1989 | 1990–1999 | 2000–2010 |
| --- | --- | --- | --- | --- | --- |
| Number of observations |  |  | 16050 | 30158 | 42591 |
| Mean Sum of Squares | MSSQ | m | 876.30 | 354.53 | 231.78 |
| Root Mean Square | RMS | m | 29.60 | 18.83 | 15.22 |
| Scale Root Mean Square | SRMS | % | 8.82 | 5.62 | 1.42 |
| Root Mean Fraction Square | RMFS | % | 42186.37 | 5403.267 | 10633.07 |
| Scaled Root Mean Fraction Square | SRMFS | % | 3318.30 | 271.58 | 139.77 |
| Sum of Residuals | SR | m | 59826.58 | 221472.12 | 223509.96 |
| Mean Sum of Residuals | MSR | m | 3.73 | 7.34 | 5.25 |
| Scaled Mean Sum of Residuals | SMSR | % | 1.11 | 2.19 | 0.49 |
| Coefficient of Determination | CD |  | 0.45 | 0.68 | 0.90 |

Figure 181 	Number of observation points throughout the simulation period

Figure 182 	Temporal trace of RMS error (%) throughout simulation period

Figure 183 	Temporal trace of mean residual error (m) throughout simulation period

Figure 184 	Temporal trace of mean absolute error (m) throughout the simulation period

###### Spatial residual error

The spatial distributions of residual errors are presented in Figure 185, Figure 186, Figure 187 and Figure 188 for 1980, 1990, 2000 and 2010 respectively. The mean residual water level is defined such that negative numbers refer to points where predicted hydraulic head values are higher than observed and positive values indicate that predicted hydraulic head values are lower than observed.

Results shown in Figures 42 to 45 suggest that the model over predicts the hydraulic heads in the outcropped regions and under predicts the heads in the lower parts of the landscape. The greatest errors are shown to be in the outcropping regions of the Strzelecki Formation suggesting that the model attribution associated with model layer 23 requires further analysis. This is consistent with observations reported in the Observational Analysis companion product (2.1-2.2) for the Gippsland Basin bioregion that considered the hydraulic conductivities attributed to zones 23–29 were on the lower bounds of previously reported values

Figure 185 	Spatial mean residual water level error (m) for 1980 based on observed and predicted heads

Figure 186 	Spatial mean residual water level error (m) for 1990 based on observed and predicted heads

Figure 187 	Spatial mean residual water level error (m) for 2000 based on observed and predicted heads

Figure 188 	Spatial mean residual water level error (m) for 2010 based on observed and predicted heads

##### Sensitivity analysis

A comprehensive analysis was undertaken to assess the sensitivity of key modelled outputs to variations in input data. Modelled outputs considered included baseflow rates, saturated area and groundwater discharges (evapotranspiration, boundary fluxes and aquifer interflows). The sensitivity analysis procedure involved altering by up to three orders of magnitude the calibrated input data sets (including horizontal hydraulic conductivities, specific storage, specific yield, vertical hydraulic conductivities, recharge, maximum evapotranspiration rates, river and boundary conductance terms) and recording the groundwater response relative to the calibrated condition. Each input data set was systematically modified while maintaining all other input data. In total, approximately 4,200 simulations were undertaken to test the robustness of the groundwater model and the uniqueness of the combination of input data required to meet the calibration criteria.

The sensitivity analysis was undertaken to assess the performance of the groundwater model simulation to changes in various input parameter values. The model performance was assessed based on scaled RMS, coefficient of determination and NSI criteria (see Section 4) using observed and predicated calibration bore hydrograph responses. A total of 4,200 simulations were examined in detail, in which changes in recharge, groundwater evapotranspiration rates, river conductance, horizontal and vertical hydraulic conductivities, specific storage and specific yields were modified by three orders of magnitude from the calibrated values. Although 4,200 simulations were examined in detail, model performance results based on variations in recharge, groundwater evapotranspiration rates, river conductance, drain conductance and horizontal hydraulic conductivities are presented for the steady-state model in Figure 189 to Figure 192 respectively.

The sensitivity of the model to recharge estimates is shown in Figure 189. This figure shows the variation in MAE, coefficient of determination and NSI criteria due to altered recharge. The trajectory of each response curve reflects the solution uniqueness and stability of the model. The flatter the curve, the less sensitive the output to the associated model attribute.

Results from the sensitivity analysis show a comparatively high sensitivity of simulated shallow water table extent to variations in recharge and maximum groundwater evapotranspiration. This is to be expected as these parameter changes are applied across large extents of the model, whereas hydraulic conductivity changes are applied on a zone by zone basis which may be of minor/limited areal extents. A similar outcome applies to river conductance changes which impact localised processes and is limited by the density of the river network incorporated in the model.

In terms of the sensitivity of predicted baseflow to parameter change, the most significant parameters are (in order of significance) recharge, potential groundwater evapotranspiration rates and river conductance. The least significant parameters are those associated with the deeper aquifers.

With respect to the modelled area of shallow water table, the most significant parameters are (in order of significance) recharge, potential groundwater evapotranspiration rates and conductivities of the unconfined aquifers.

Based on this analysis, model uncertainty is most strongly impacted by uncertainty in the estimates of recharge and potential groundwater evapotranspiration rates. The hydraulic conductivities of the unconfined and outcropping aquifers also have significant influence on predicted baseflow and water table extent.

In general, the sensitivity analysis show model parameters generally fit within type I or II classification as described by Middlemis et al. (2000).

Figure 189 	Sensitivity of the steady-state model to variations in recharge

Figure 190 	Sensitivity of the steady-state model to variations in potential groundwater evapotranspiration rate

Figure 191 	Sensitivity of the steady-state model to variations in river conductance

Figure 192 	Sensitivity of the steady-state model to variations in drain conductance

##### CRDP predictions

Section 2.6.2.1.5 outlines the methodology in which the distributed multi-layered groundwater model was used at the basin scale to account for the CRDP under a climate change signal. The future climate change projections translated into an average reduction in rainfall of 6.8% from 2073 to 2102 relative to the 1983 to 2012 conditions as reported in Section 2.5.2 of companion product 2.5 for the Gippsland Basin bioregion. The resultant reduction in surface water inflows and groundwater baseflows was predicted to be 12% and 75% respectively. It is in this context that the CRDP simulation results are presented.

The modelling assumed that the expansion of existing open-cut mine pits occurs in a sequenced and progressive development phasing based on mine work plans. The rate of development was set such that mine development was completed by the expected mine closure date. Model simulation targets were based on matching estimated mine dewatering volumes derived from previous groundwater modelling (GHD, 2014b) and the stabilisation of groundwater heads within the basement of the mine.

The current parameterisation results in the bulk of the water extracted from expanded open-cut coal mine dewatering to be sourced from the aquifer beneath the associated target coal seam where the hydraulic conductivity and storages are considered to be higher. There is therefore a higher probability of this parameterisation to over predict groundwater extraction volumes associated with the CRDP. The predicted extraction volumes associated with the CRDP for the Hazelwood, Loy Yang and Yallourn mines are shown in Figure 193, Figure 194 and Figure 195 respectively.

Figure 193	Predicted time varying mine dewatering extraction volume for the Hazelwood mine required to meet the CRDP targets. The predicted volume is the sum of the extraction from the base of the M2A seam (Haz_mine) and the aquifer below (Haz_below)

Figure 194	Predicted time varying mine dewatering extraction volume for the Loy Yang mine required to meet the CRDP targets. The predicted volume is the sum of the extraction from the base of the M2A seam (Loy_mine) and the aquifer below (Loy_below)

Figure 195	Predicted time varying mine dewatering extraction volume for the Yallourn mine required to meet the CRDP targets. The predicted volume is the sum of the extraction from the base of the Yallourn coal seam (Yal_mine) and the aquifer below (Yal_below)

The change in water table elevation within each open-cut mine pit during the coal mine development phasing is shown in Figure 196. At the end of the simulation the average hydraulic heads within the mine pits were -52 mAHD, -92 mAHD and 12 mAHD for the Hazelwood, Loy Yang and Yallourn mines respectively with 37% and 47% of the Hazelwood and Loy Yang mine cells becoming dry.

The impact of climate change and projected baseline groundwater extractions on the predicted depth to water table relative to the 2012 condition (Figure 197) at 2042, 2072 and 2102 are shown in Figure 198, Figure 199 and Figure 200 respectively. In general, the depth to water table from ground surface increases on the basin margins and outcropped areas due to the reduced recharge whilst in predominantly lowland alluvial systems the depth to water table is shallower due to a reduction in groundwater extractions.

The impact of the additional coal resource development (depth to water table under the CRDP relative to the corresponding baseline condition at 2042, 2072 and 2102) is shown in Figure 201 to Figure 203 respectively. As outlined in Section 2.5.2 of companion product 2.5 for the Gippsland Basin bioregion, the CRDP has most impact on those regions in close proximity to the proposed open-cut coal mine pits. Simulation results estimate that the greatest spatial impact is likely to occur at 2072. This is consistent with the timing of the peak reduction in annual baseflow (ML/yr) to the Latrobe River as shown in Figure 204.

Figure 196	Average water table elevation within the open cut mine pits in response to CRDP mine work plans

Figure 197	2012 baseline depth to water table (m) from ground surface

Figure 198	Change in depth to water table (m) at 2042 under baseline conditions relative to 2012 condition

Figure 199	Change in depth to water table (m) at 2072 under baseline conditions relative to 2012 condition

Figure 200	Change in depth to water table (m) at 2102 under baseline conditions relative to 2012 condition

Figure 201	Change in depth to water table (m) at 2042 under CRDP conditions relative to 2042 baseline condition

Figure 202	Change in depth to water table (m) at 2072 under CRDP conditions relative to 2072 baseline condition

Figure 203	Change in depth to water table (m) at 2102 under CRDP conditions relative to 2102 baseline condition

Figure 204	Change in predicted annual baseflow (ML/yr) to the Latrobe River relative to the baseline condition

#### Limitations

Summary

The distributed, multi-layered MODFLOW groundwater model described in this report was an enhancement of an existing model developed to investigate and understand the potential impacts of future onshore gas developments and to understand the possible impacts of a potential onshore natural gas industry on groundwater and surface waters within the Gippsland region.

In the immediate vicinity of the open-cut mines, the resolution of the model and the reliability of predictions is inferior to a local-scale model that accounts for local geological and hydrogeological detail.

The interpretation of predictions should account for model assumptions and limitations.

The key assumptions underpinning the development of the Gippsland region groundwater model are summarised in Table 29. These assumptions provide a foundation for the development of the model; while there is always some level of uncertainty the best available data has been used to limit predictive errors. It is likely that in some areas detailed hydrogeological processes have not been fully captured in the conceptualisation. Sub-catchment or site-specific investigations would be required to better define local variations and responses

Table 29	Key model assumptions

| Assumption | Basis |
| --- | --- |
| There is no dual porosity. | Dual porosity models are only applicable to solute transport models and are not meaningful for flow models. |
| Groundwater concentration gradients are acknowledged to have some impact on groundwater heads but have not been explicitly included in the simulation. | Data limitations make inclusion into a dispersion-convective transient model problematic and would require significant additional computational resources and time. |
| Groundwater temperature gradients are acknowledged to have some impact on groundwater heads but have not been explicitly included in the simulation. | As for density driven. |
| Bores used in the model have been correctly assigned to aquifers. | Significant data cleansing work used to preparing data sets, however minor inaccuracies may exist. |
| The digital elevation model (DEM) has adequate vertical resolution to allow reasonable calibration of the model and represents the land surface topology over the entire domain. | Self-evident. |
| Land use change data is limited in availability and extent and has not been incorporated in this study. | Development scenarios based on existing land use. |
| The model cell size is adequate to meet project objectives. | Self-evident. |

The developed groundwater model has been shown to broadly quantify groundwater flow and match historical hydrograph trends. The groundwater model is capable of predicting relative changes in hydraulic heads/water table and baseflow. The model is deemed appropriate to assess the potential regional cumulative impacts of gas developments and other groundwater users (including coal mines and offshore oil and gas extraction) on existing groundwater users and the environment against a baseline condition of the current development.

Key observations arising from this study are as follows:

The need to recognise the assumptions and limitations associated with the model so as to ensure the appropriate use of the model and interpretation of model predictions. This groundwater model may be used to consider relative changes in the catchment water balance. However, the spatial accuracy of simulation results may not correlate well with gauged data due to the model construct and spatial resolution and therefore should be used with regards to these considerations.

Caution should be exercised when using the solver option available in Groundwater Vistas by enabling the solution to continue if the convergence criteria are met for only the outer iterations without satisfying all convergence tolerances.

The following recommendations could improve the robustness and confidence of the groundwater model predictions as follows:

- Improve knowledge of groundwater abstraction across the region. This should extend to the compilation of basic information including the (1) location of pumping wells, (2) the groundwater volumes abstracted and (3) the nature of the aquifers from which abstractions are sourced.

- Translate the groundwater model into the recently developed UnStructured Grid (USG) version of MODFLOW, which would likely increase accuracy in the representation of processes in regions of steep hydraulic gradients (such as near mines, near connected surface features, adjacent to groundwater extraction points and in regions of stream/aquifer interaction) thereby improving groundwater predictions and water budget estimates.

- Further assess groundwater recharge estimates and associated hydrograph response would likely increase mass balance prediction accuracy.

- Incorporate temperature and density correction functions.

- Review model layers assigned to groundwater extraction bores.

- Review the attribution of the model layer representing the uppermost Strzelecki Formation. This suggestion is based on the analysis of spatial residual error presented in this report.

References

Section 2.6.2.1

Beverly C (2009) Models of the Catchment Analysis Tool (CAT1D Version 32). August 2009, Technical Reference Manual (ed. C. Beverly), Department of Primary Industries, Victoria.

Beverly C, Bari M, Christy B, Hocking M  and Smettem, K (2005) Predicted salinity impacts from land use change: a comparison between rapid assessment approaches and a detailed modelling framework. Australian Journal of Experimental Agriculture 45, pp. 1453–1469.

Beverly C, Hocking M, Cheng X, O’Neil C, Schroers R  . and Baker S (2015) The Gippsland groundwater model. Technical report prepared for the Department of Environment, Land and Water, Victoria, pp. 410.

Barnett B, Townley LR, Post, V Evans RE, Hunt RJ, Peeters L, Richardson S, Werner AD, Knapton A and Boronkay A (2012) Australian groundwater modelling guidelines. Waterlines report, National Water Commission, Canberra.

Section 2.6.2.2

Beverly C (2009) Models of the Catchment Analysis Tool (CAT1D Version 32). August 2009, Technical Reference Manual (ed. C. Beverly), Department of Primary Industries, Victoria.

Beverly C, Hocking M and Day G, (2012) Understanding the potential significance of energy and earth resource development on water resources. Department of Primary Industries Technical Report, pp. 67.

Beverly C, Hocking M, Cheng X, O’Neil C, Schroers R and Baker S (2015) The Gippsland groundwater model. Technical report prepared for the Department of Environment, Land and Water, Victoria, pp. 410.

Bolger PF (1987) Regional groundwater investigations of the Latrobe Valley — Stage III mathematical modelling studies. Geological Report 41, Coal Resources Division, Fuel Department, State Electricity Commission of Victoria.

Evans RS (1983) Regional groundwater investigation of the Latrobe Valley, Stage 2 mathematical modelling studies. Fuel Department Report DD187. State Electricity Commission of Victoria (unpubl.).

Fraser CJ (1980) Regional groundwater investigation of the Latrobe Valley — Initial mathematical modelling studies Volume 1. Fuel Department Report DD 149. State Electricity Commission of Victoria (unpubl.).

Freij-Ayoub R, Strand J, Ahmed S (2011a) Coal seam gas water production management. Project Report EP114078, CSIRO Earth Science and Resource Engineering.

Freij-Ayoub R, Strand J, Ahmed S (2011b) A Numerical Model for the Prediction of Produced Methane and Water from Coal Seams and the Associated Ground Surface Subsidence. 4th International Conference GeoProc2011: Cross Boundaries Through THMC Integration, Perth, Western Australia, 6–9 July 2011.

Geo-Eng (1994) Estimates of long term pumping requirements for Loy Yang Mine. Report No. 1205/3, Geo-Eng Pty Ltd (unpubl.).

Geo-Eng (1996a) Predictive modelling of the aquifer depressurisation requirements for Latrobe Valley Mines. Report 1205/10, Geo-Eng Pty Ltd (unpubl.).

Geo-Eng (1996b) The Latrobe Valley regional groundwater model and the Loy Yang Mine groundwater model - A review of work completed to October 1996. Draft Report 1205/12, Geo-Eng Pty Ltd (unpubl.)

GHD (2008) East Gippsland CMA Groundwater Model, Interim steady-state model development report. Report prepared by GHD for Department of Sustainability and Environment, Victoria.

GHD (2010a) West Gippsland CMA Groundwater Model. Interim steady-state model development report. Report prepared by GHD for the Department of Sustainability and Environment, Victoria.

GHD (2010b) West Gippsland CMA Groundwater Model. Transient model development report. Report prepared by GHD for Department of Sustainability and Environment, Victoria.

GHD (2010c) East Gippsland CMA Groundwater Model. Transient model development report, report for the Dept. of Sustainability and Environment, Victoria.

GHD (2012) Victorian Aquifer Framework updates for seamless mapping of aquifer surfaces. Report prepared by GHD for the Secure Allocation Future Entitlements (SAFE) Project, Department of Sustainability and Environment, Victoria.

Golder Associates (1990) Loy Yang regional groundwater model. Report No 89612362, prepared by Golder Associates Pty Ltd for State Electricity Commission of Victoria (unpubl.).

Golder Associates (1991) Groundwater model for Loy Yang and Morwell and open cuts, (Interim report). Report No. 91612011, Golder Associates Pty Ltd.

Golder Associates (1992a) Update and recalibration of groundwater model — progress report. Report 92612275/02. Report prepared by Golder Associates Pty Ltd for State Electricity Commission of Victoria.

Golder Associates Pty Ltd (1992b) Groundwater model for the Loy Yang and Morwell open cuts, Latrobe Valley, Victoria. Second Interim Report, Report No. 91612011.

Golder Associates Pty Ltd (1992c) Groundwater Model for the Loy Yang and Morwell Open Cuts, Latrobe Valley, Victoria (Third Interim Report). 92612275/09, Geo Eng Pty Ltd.

Harbaugh AW (2005) MODFLOW-2005, The U.S. Geological Survey modular ground-water model—the ground-water flow process. Techniques and Methods Report 6-A16. U.S. Geological Survey.

Henzell ST, Irrgang HR, Jenssen EJ, Mitchell RAM, Morrell GO, Palmer ID, Seage NW, Hicks GJ, Hordern MJ, and Kable CW (1985) Fortescue reservoir development and reservoir studies. The APEA Journal 25(1): 95-106.

Middlemis, H (2000) Murray Darling Basin Commission Groundwater Flow Modelling Guidelines. Report prepared for the Murray Darling Basin Commission by Aquaterra Pty Ltd.

Nahm GY (2002) The hydrogeology of the Gippsland Basin and its role in the genesis and accumulation of petroleum. PhD Thesis, University of Melbourne.

PB (2010a) Review of West Gippsland CMA groundwater model. Transient Model Development Report. Prepared by Parsons Brinckerhoff for the Department of Sustainability and Environment, Victoria.

PB (2010b) Review of East Gippsland CMA groundwater model. Transient model development report. Prepared by Parsons Brinckerhoff for the Department of Sustainability and Environment, Victoria.

Schaeffer J (2008) Scaling Point Based Aquifer Data for Developing Regional Groundwater Models: Application to the Gippsland Groundwater System. PhD thesis. The University of Melbourne.

SKM (1995) Groundwater modelling of the Generation Victoria and Esso/BHP drawdown cones. Report prepared by Sinclair Knight Merz for the Department of Conservation and Natural Resources, Victoria (unpubl.).

SKM (1996) Numerical modelling of declining groundwater levels in Gippsland. Report prepared by Sinclair Knight Merz for the Department of Conservation and Natural Resources, Victoria (unpubl.).

SKM (1999) Yarram subregional model. Report prepared by Sinclair Knight Merz for the Department of Natural Resources and Environment, Victoria (unpubl).

SKM (2011a) Gippsland Basin Model Scoping Study. Review of groundwater management tools in the Gippsland Basin. Report prepared by Sinclair Knight Merz.

SKM (2011b) A review of the Victorian Aquifer Framework. Report prepared by Sinclair Knight Merz for the Secure Allocation, Future Entitlements (SAFE) Project, Department of Sustainability and Environment, Victoria

Section 2.6.2.3.

Beverly C, Bari M, Christy B., Hocking M., Smettem K (2005) Predicted salinity impacts from land use change: a comparison between rapid assessment approaches and a detailed modelling framework. Australian Journal of Experimental Agriculture 45, pp. 1453–1469.

Beverly C (2009) Models of the Catchment Analysis Tool (CAT1D Version 32). August 2009, Technical Reference Manual (ed. C. Beverly), Department of Primary Industries, Victoria.

Blevin 2013.

Constantine A (2001) Sedimentology, Stratigraphy and Palaeoenvironment of the Upper Jurassic – Lower Cretaceous Non-Marine Strzelecki Group, Gippsland Basin, Southern Australia. PhD Thesis, Monash University.

GHD (2010a) West Gippsland CMA Groundwater Model. Interim steady-state model development report, report prepared by GHD for the Department of Sustainability and Environment, Victoria.

GHD (2010b) West Gippsland CMA Groundwater Model. Transient model development report. Report prepared by GHD for Department of Sustainability and Environment, Victoria.

GHD (2010c) East Gippsland CMA Groundwater Model. Transient model development report. Report for the Dept. of Sustainability and Environment, Victoria.

GHD (2010d) West Gippsland CMA Groundwater Model. Transient model development report. Report for the Dept. of Sustainability and Environment, Victoria.

GHD (2011a) Latrobe Valley Coal Model completion report. Report prepared by GHD for Clean Coal Victoria, Department of Primary Industries, Victoria.

GHD (2011b) Latrobe Valley regional groundwater and land level monitoring report. Five Year Review. Prepared by GHD.

GHD (2012a) Victorian Aquifer Framework updates for seamless mapping of aquifer surfaces. Report prepared by GHD for the Secure Allocation Future Entitlements (SAFE) Project, Department of Sustainability and Environment, Victoria.

GHD (2012b) Development of State Wide 3D Aquifer surfaces. Report prepared by GHD for the Secure Allocation, Future Entitlements (SAFE) Project, Department of Sustainability and Environment, Victoria.

Hatton T, Otto C and Underschultz J (2004) Falling water levels in the Latrobe Aquifer, Gippsland Basin: Determination of cause and recommendations for future work. Prepared by CSIRO Petroleum Resources Division.

Holdgate GR, Gallagher SJ (2003) Tertiary. In: Birch, W.D. (ed.), 2003. Geology of Victoria, pp. 289–335. Geological Society of Australia Special Publication 23. Geological Society of Australia (Victoria Division).

Jansen B and Maher S (2003) Digital geological model of the Latrobe Valley Coal Resource. Report 2003/2. Geological Survey of Victoria (unpubl.).

McLean and Blackburn (2013).

Middlemis H (2000) Murray Darling Basin Commission Groundwater Flow Modelling Guidelines. Report prepared for the Murray Darling Basin Commission by Aquaterra Pty Ltd.

Osbourne C, (2014) Updates to the Latrobe Valley Coal Model. Prepared by Geological Survey of Victoria, Department of State Development and Business Innovation.

SKM, GHD (2009) Hydrogeological Mapping of Southern Victoria. July 2009. Report prepared by Sinclair Knight Merz and GHD for Southern Rural Water.

SKM (2011) A review of the Victorian Aquifer Framework. Report prepared by Sinclair Knight Merz for the Secure Allocation, Future Entitlements (SAFE) Project, Department of Sustainability and Environment, Victoria

Stuart-Smith P, Gou B, Cunningham MM, Prendergast L, Monoury S, (2010) Gippsland Basin Study Modules: Onshore & Offshore, Department of Primary Industries, Victoria. The Cooperative Research Centre for Greenhouse Gas Technologies, Canberra.

Varma S, Karsten M, Bekele E and Ciftci B (2012) Hydrodrnamic modelling of the Gippsland Basin. Prepared by CSIRO Petrolium and Geothermal Research Division, Perth in collaboration with the Department of Primary Industries, Victoria. Unpublished report

Whiteway (2009).

Section 2.6.2.4

Beverly C, Bari M, Christy B, Hocking M, Smettem K (2005) Predicted salinity impacts from land use change: a comparison between rapid assessment approaches and a detailed modelling framework. Australian Journal of Experimental Agriculture 45, pp. 1453–1469.

Beverly C (2009) Models of the Catchment Analysis Tool (CAT1D Version 32), August 2009, Technical Reference Manual (ed. C. Beverly), Department of Primary Industries, Victoria.

DEPI (2014) Watercourse Network 1:25,000 – Vicmap Hydro. Spatial Information Infrastructure (DSE), Department of Environment and Primary Industries, Victoria.

DSE (2012) Victorian groundwater-surface water interaction spatial data. Compiled for the Secure Allocation, Future Entitlements (SAFE) Project by the Department of Sustainability and Environment, Victoria.

Hofmann H (2011) Understanding connectivity within groundwater systems and between groundwater and rivers, PhD Thesis, Monash University, Clayton, Victoria.

Shah N, Nachabe M and Ross M (2007) Extinction Depth and Evapotranspiration from Ground Water under Selected Land Covers. Groundwater, 45: 329–338. doi: 10.1111/j.1745-6584.2007.00302.x

SKM (2008) Investigation of groundwater/surface water interaction in the Avon River catchment. A report prepared by Sinclair Knight Merz for the Western Gippsland Catchment Management Authority. Final Report. December 2008.

SKM (2012a) Impacts of groundwater extraction on streamflow in selected catchments throughout Australia. Waterlines Report Series No 84. Report prepared by Sinclair Knight Merz for the National Water Commission, Canberra.

SKM (2012b) Baseflow separation and analysis. Report prepared by Sinclair Knight Merz for the Secure Allocation, Future Entitlements (SAFE) Project, Department of Sustainability and Environment, Victoria.

Unland NP, Cartwright I, Andersen MS, Rau GC, Reed J, Gilfedder BS, Atkinson A P , Hofmann H (2013) Investigating the spatio-temporal variability in groundwater and surface water interactions: a multi-technique approach. Hydrology and Earth Systems Science 17: pp. 3437–3453.

WMIS (2015) DELWP Water Management Information System, Water monitoring data, Available from <http://data.water.vic.gov.au/monitoring.htm>. [15 May 2015]

Section 2.6.2.5

Beverly C (2009) Models of the Catchment Analysis Tool (CAT1D Version 32), August 2009, Technical Reference Manual (ed. C. Beverly), Department of Primary Industries, Victoria

Beverly C, Bari  M, Christy B, Hocking M and Smettem, K (2005) Predicted salinity impacts from land use change: a comparison between rapid assessment approaches and a detailed modelling framework. Australian Journal of Experimental Agriculture 45, 1453–1469.

Beverly C, Hocking M, Cheng X, O’Neil C, Schroers R and Baker S (2015) The Gippsland groundwater model, Technical report prepared for the Department of Environment, Land and Water, Victoria, pp410.

Section 2.6.2.6

Aquaterra (2008) Ashton Underground Mine Pikes Gully Seam groundwater modelling report. Report submitted to ACOL in support of the Longwall/Miniwall 5-9 SMP Application.

Barton CM (1971) The Morwell "interseam" sands. Journal of the Geological Society of Australia 17(2): 191–204.

Batu V (1998) Aquifer Hydraulics: A comprehensive guide to hydrogeologic data analysis, John Wiley & Sons, New York.

Blake WJR (1972) An investigation of a groundwater supply for the township of Trafalgar, Parish of Moe. Report No. 1972/3, Geological Survey of Victoria.

Brumley JC, Barton, CM, Holdgate GR and Reid MA (1981) Regional groundwater investigation of the Latrobe Valley: 1976-1981. State Electricity Commission of Victoria and Victorian Department of Minerals and Energy, Melbourne.

Dahlhaus PG, Heislers DS, Brewin D, Leonard JL, Dyson PR, Cherry DP (2004) Port Phillip and Westernport groundwater flow systems. A report prepared for the Port Phillip and Westernport Catchment Management Authority, Frankston, Victoria.

Domenico PA and FW Schwartz (1990) Physical and chemical hydrogeology. John Wiley & Sons, New York.

Fraser CJ (1980) Regional groundwater investigation of the Latrobe Valley — Initial mathematical modelling studies Volume 1. Fuel Department Report DD 149. State Electricity Commission of Victoria (unpubl.).

Geo-Eng (1993) Aquifer depressurisation report. Report No. 1204/1. Geo-Eng Pty Ltd. (unpubl.).

Geo-Eng (1996) Predictive modelling of the aquifer depressurisation requirements for Latrobe Valley Mines. Report 1205/10. Geo-Eng Pty Ltd (unpubl.).

Geo-Eng (2001) Aquifer depressurisation strategic assessment. A unpubl. report prepared by Geo-Eng Pty Ltd. Report No 1100/1204/21. Geo-Eng Pty Ltd.

GHD (2008a) East Gippsland CMA Groundwater Model, Interim steady-state model development report. Report prepared by GHD for Department of Sustainability and Environment, Victoria.

GHD (2008b) Hydrogeological assessment to support licence application BCL 9030173. Report prepared by GHD for East Gippsland Water.

GHD (2010a) West Gippsland CMA Groundwater Model. Interim steady-state model development report. Report prepared by GHD for the Department of Sustainability and Environment, Victoria.

GHD (2010b) West Gippsland CMA Groundwater Model. Transient model development report. Report prepared by GHD for Department of Sustainability and Environment, Victoria.

Golder Associates (1990) Loy Yang regional groundwater model. Report No 89612362, prepared by Golder Associates Pty Ltd for State Electricity Commission of Victoria (unpubl.).

Golder Brawner.

Harlow GE, LeCain GD (1993) Hydraulic characteristics of, and ground-water flow in, coal-bearing rocks of southwestern Virginia. Water-supply Paper 2388. U.S. Geological Survey.

Leonard JG (2006) Hydrogeology of the Melbourne area. Australian Geomechanics 41(3), pp. 63–74.

Mollica FJ (1991) Results of pumping test carried out on Bore 3468-15060, Sale. Technical Note TN/5. Rural Water Commission of Victoria.

Nahm GY (1972) Report on the pumping test of Bore No. 12, Parish of Harrow. Report No. 1972/6 Geological Survey of Victoria (unpubl.).

Nahm GY (1973a) Report on Bore No. 5, Parish of Wy Yung. Report No. 1973/71. Geological Survey of Victoria (unpubl.).

Nahm GY (1973b) Report on Bore Nos. 7, 8 and 9, Parish of Wy Yung. Report No. 1973/72. Geological Survey of Victoria (unpubl.).

Nahm GY (1974) Geology and groundwater resources of the Thorpdale area including aquifer test of Bore No. 46, Parish of Moe. Report No. 1974–36. Geological Survey of Victoria (unpubl.).

Nahm GY (1977) Groundwater resources in Gippsland. Report No. 1977/1, Geological Survey of Victoria.

Nahm GY, Reid MA (1979a) Report on the aquifer tests of the D.M.E. and S.E.C. joint project (1), Bore No. 197, Parish of Willung. Report No. 1979/29. Geological Survey of Victoria (unpubl.).

Nahm GY, Reid MA (1979b) Report on the aquifer tests of the D.M.E. and S.E.C. joint project (3), Bore No. 96, Parish of Coolungoolun. Report No. 1979/58. Geological Survey of Victoria (unpubl.).

Nahm GY, Reid MA (1979c) Report on the aquifer tests of the D.M.E. and S.E.C. joint project (5), Bore No. 307, Parish of Rosedale. Report No. 1979/61. Geological Survey of Victoria (unpubl.).

Nahm GY, Reid MA (1979d) The second aquifer tests on Bore Nos. 17 and 19, Parish of Sale. Report No. 1979/70.1979/64. Geological Survey of Victoria (unpubl.).

Nahm GY, Reid MA (1979e) Report on the aquifer tests of the D.M.E. and S.E.C. joint project (6), Bore No. 286, Parish of Traralgon, Report No. 1979/70. Geological Survey of Victoria (unpubl.).

PDA (2006) Ashton Coal Mine Longwall Panels 1–4 Subsidence Management Plan; groundwater assessment. Report prepared by Peter Dundon and Associates Pty Ltd for Ashton Coal Operations Ltd.

Pratt M (1985) Groundwater investigation — Shires of Bass and Phillip Island. Report No. 1985/48. Geological Survey Victoria (unpubl.).

Reid MA (1985a) Completion Report on Sale 21, Sale town supply bore, Report No. 1985/57. Geological Survey Victoria (unpubl.).

Reid MA (1985b) Narracan 3737 Latrobe Valley Groundwater Investigation, Report No. 1985/66. Geological Survey Victoria (unpubl.).

Schaeffer J(2008) Scaling Point Based Aquifer Data for Developing Regional Groundwater Models: Application to the Gippsland Groundwater System. PhD thesis. The University of Melbourne.

Shugg A, Harris IF (1975) Evaluation of pumping test data of an unconfined fractured rock aquifer, Monbulk 3, Parish of Monbulk. Report No. 1975/47. Geological Survey Victoria (unpubl.).

SKM (1999) Yarram subregional model. Report prepared by Sinclair Knight Merz for the Department of Natural Resources and Environment, Victoria (unpubl).

SKM (2001) Gippsland subsidence modelling — Yarram. Report prepared by Sinclair Knight Merz for the Department of Natural Resources and Environment, Victoria (unpubl).

SKM (2006) Sale Water Supply Protection Area Groundwater Resource Appraisal. Report WC03438.1, prepared by Sinclair Knight Merz for Southern Rural Water.

Szabo LG (1979) Pumping test on Bore 2, Mount Franklin Mineral Water Spring. Scanex Minerals Pty Ltd, Melbourne.

Thatcher RC (1976) Loy Yang Project aquifer characteristic from pump tests conducted during 1975. DD111, Fuel Department, Development and Planning Division, State Electricity Commission of Victoria (unpubl.).

Thompson BR (1968) Longford Gippsland, Glenco South 80001 Esso BHP Water Bore No 2.

Todd DK (1980) Groundwater hydrology (2nd edn). John Wiley & Sons, New York.

USQ (2011) Preliminary assessment of cumulative drawdown impacts in the Surat Basin associated with the coal seam gas industry. Investigation of parameters and features for a regional model of Surat Basin coal seam gas developments. University of Southern Queensland.

Walker G, Molica F (1990) Review of the Groundwater resources in the South East Region. A report for the Department of Water Resources Victoria. South East Region Water Management Strategy. Water Resources Management Report Series. No. 54. Department of Water Resources, Victoria.

Section 2.6.2.7

Arnold JG, Allen PM, Muttiah R, Bernhardt G (2010) Automated base flow separation and recession analysis techniques.

Chiew FHS, Stewardson, MJ, McMahon TA (1993) Comparison of six rainfall-runoff modelling approaches. Journal of Hydrology 147, 1–36.

Doherty J (2010) PEST Model-independent parameter estimation, User Manual (5th edn). Watermark Numerical Computing.

GHD (2013) Groundwater Assessment – baseflow dependent rivers characterising groundwater contribution to baseflow dependent waterways. Report prepared for the Department of Environment and Primary Industries, Victoria.

GHD (2014a) Statewide groundwater level reporting. Report prepared by GHD for Department of Environment and Primary Industries, Victoria.

GHD (2014b) Groundwater Impacts and Management for Lignite Mining in the Latrobe Valley, a report prepared for the Department of State Development, Business and Innovation, Vic, pp 239.

Middlemis H (2000) Murray Darling Basin Commission Groundwater Flow Modelling Guidelines. Report prepared for the Murray Darling Basin Commission by Aquaterra Pty Ltd.

SKM (2012) Baseflow separation and analysis. Report prepared by Sinclair Knight Merz for the Secure Allocation, Future Entitlements (SAFE) Project, Department of Sustainability and Environment, Victoria.

SKM (2014) SOBN hydrograph review: State-wide groundwater hydrograph catalogue. Report prepared by by Sinclair Knight Merz for the Department of Environment and Primary Industries, Victoria.